3GPP TSG-SA5 Meeting #157	S5-245363

Hyderabad, India, 14 - 18 October 2024

# Source:	Huawei

# Title:	WT alignment for control plane congestion analytics

# Document for:	Approval

Agenda Item:	6.19.2

# 1	Decision/action requested

For approval

# 2	References

[1] 	3GPP TR 28.866: “Study on Management Data Analytics (MDA) – Phase 3”.

# 3	Rationale

Control plane congestion analytics should be studied as part of WT-9. However, currently, this area is studied as part of WT-7.

This contribution proposes to move the text related to control plane congestion analytics to the correct clauses.

The technical content is not changed, except to align clause numbering.

# 4	Detailed proposal

It is proposed to make the following changes to TR 28.866 [1].

1st change

2nd change

## 5.9	Control plane congestion analytics

### 5.9.1	Use case 1: Enhancing control plane congestion analysis

#### 5.9.1.1	Description

The use case for control plane congestion analysis is described in TS 28.104 [2]. It proposes to use MDA to assist control plane congestion analysis in order to detect, prevent or resolve congestion issue happened at the control plane. NWDAF as an MDA MnS consumer may need to take further analysis on control plane congestion issue based on the analytics output from MDAF. Existing MDA analytics output for 5GC control plane congestion analysis only includes 5GC NFs affected by congestion issues, analysis report identifier and the recommended actions, which may need enhancement to support further analytics of NWDAF. Missing attributes include the root cause of the congestion issue and the predicted duration of congestion issue.

The examples of root cause could be the huge amount of UE access network in short period of time. Congestion issues with different root causes need different mitigation solutions. For example, if congestion issue is caused by network failure, it is possible for OAM to resolve the issue with analytics performed by MDAF and may not need NWDAF to take further actions. If congestion issue is caused by signalling overload, the MDA report may include the identified object which is the bottleneck of network and the recommended action to eliminate the bottleneck issue. NWDAF may perform further analytics to recommend actions to eliminate the congestion issue, such as adjusting the backoff timer or controlling the signalling access rate.

Besides, information about predicted duration of identified congestion issue may be also needed by NWDAF. For example, if the congestion is predicted to be self-recovered in a short time, it may not necessary for NWDAF to take actions. If the predicted duration of congestion issue is long, timely actions may need to be taken by NWDAF to resolve the issue.

#### 5.9.1.2	Potential requirements

REQ-CP_ANA-ENHANCE-01: MDA capability for control plane congestion analysis should be able to provide the analytics output including root cause of the congestion issue and the predicted duration of congestion issue to the consumer.

#### 5.9.1.3	Potential solutions

The analytics output for MDA assisted control plane congestion analysis may be enhanced with the following attributes.

- rootCause. This attribute may indicate the root cause of control plane congestion issue, such as network failure, signalling overload, etc.- predictedDuration. This attribute may indicate the predicted time duration of control plane congestion issue.

#### 5.9.1.4	Evaluation of solutions

Only potential solution #1 is proposed, the requirements are satisfied and this solution is feasible for normative work.

3rd change

## 6.7	Fault management related analytics and alarm prediction

The use case, requirements and solution for Use case: Providing threshold statistics information for fault management is described in clause 5.7.1. It is recommended to add new attribute for threshold information in the MDARequest in TS 28.104 [2] to support providing statistics information in the analytics output. The detailed solution is described in clause 5.7.1.3.

The use case, requirements and solution for the capability of enhancing failure prediction is described in clause 5.7.2.3. It is recommended to extend the output of failure prediction as described in clause 5.7.2.3 in TS 28.104 [2].

The MDA MnS producer needs to provide a new MDA capability for service failure recovery:

- analysis of the probable cause of the service failure

- recommended actions to resolve the service failure

The use case, requirements and solution for Use case: NF failure evaluation is described in clause 5.7.5. It is recommended to add new attribute in the MDA analytics output for failure prediction analysis in TS 28.104 [2] to support providing predicted network impact information in the analytics output. The detailed solution is described in clause 5.7.5.3.

4th change

## 6.9	Control plane congestion analytics

The use case, requirements and solution for Use case: Enhancing control plane congestion analysis is described in clause 5.9.1. It is recommended to add new attributes in the MDA analytics output for control plane congestion analysis in TS 28.104 [2] to support providing root cause and predicted time duration of control plane congestion issue in the analytics output. The detailed solution is described in clause 5.9.1.3.

5th change

End of changes