3GPP TSG-SA5 Meeting #160	S5-251444

Goteborg, Sweden, 7 - 11 April 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 28.105 | CR | draftCR | rev | - | Current version: | 19.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Input to DraftCR Rel19 TS28.105 New use case on Coordination between the ML capabilities |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia, Nokia Shanghai Bell, China mobile, TELUS |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA5 |  |  |  |  |  |  |  |  |  |
| Work item code: | AIML_MGT_Ph2 |  |  |  |  |  | Date: |  |  | 2025-03-27 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | This CR to introduce the agreed Coordination between the ML capabilities use case and requirements in TR28.858 clause 5.4.1. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add the use case and requirements. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Coordination between the ML capabilities will be missing. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.5.3.2.x (new), 6.5.3.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

Start of First change

##### 6.5.3.2.X	Coordination between the ML capabilities

For ML in 5GC or RAN, the ML capabilities in 5GC or RAN may be needed to coordinate with 3GPP management analytics and possibly other aspects to improve the overall performance. Typical 5GS data, such as performance measurements and KPI, can be modeled as time series data to identify patterns like seasonality and trends. Combining analytics results from RAN nodes, 5G core, and the 3GPP management system can enhance prediction accuracy by capturing both fine-grained and high-level seasonality patterns. Additionally, integrating data from multiple sources (e.g., RAN node, management node) mitigates bias inherent in single-node analytics, further improving overall prediction accuracy.

To enable the coordination between the ML capabilities, the configuration (e.g. a triggering condition, i.e. when a result is needed from the RAN analytics to MDA) may be needed. On the other hand, the result of the coordination may be communicated towards the consumer(s) and hence there is a need to enhance the reporting to capture the deviation of predictions (and/or the related context) so the consumer can gain a better understanding regarding the coordination of ML capabilities.

When coordinating with management ML capability, more than one ML capability may be involved. From a single ML capability perspective, one ML model may support one (or a set of) specific type of inference(s) capability. One ML inference function may employ two or more ML models to perform more sophisticated inference(s) instead of developing a new specific ML model. The involved ML models may work together in a coordinated manner, for which a coordination of ML capabilities would be needed.

#### 6.5.3.3	Requirements for AI/ML inference capabilities management

Table 6.5.3.3-1

| Requirement label | Description | Related use case(s) |
| --- | --- | --- |
| REQ-ML_CAP-01 | The AI/ML inference MnS Producer shall have a capability allowing an authorized MnS consumer to request the capabilities of existing ML models available within the AI/ML inference producer. | Identifying capabilities of ML models (clause 6.5.3.2.1) |
| REQ- ML_CAP-02 | The AI/ML inference MnS Producer shall have a capability to report to an authorized MnS consumer the capabilities of an ML model as a decision described as a triplet <object(s), parameters, metrics> with the entries respectively indicating: the object or object types for which the ML model can undertake optimization or control; the configuration parameters on the stated object or object types, which the ML model optimizes or controls to achieve the desired outcomes; and the network metrics which the ML model optimizes through its actions. | Identifying capabilities of ML models (clause 6.5.3.2.1) |
| REQ-ML_CAP-03 | The AI/ML inference MnS Producer shall have a capability to report to an authorized MnS consumer the capabilities of an ML model as an analysis described as a tuple <object(s), characteristics> with the entries respectively indicating: the object or object types for which the ML model can undertake analysis; and the network characteristics (related to the stated object or object types) for which the ML model produces analysis | Identifying capabilities of ML models (clause 6.5.3.2.1) |
| REQ-ML_CAP-04 | The AI/ML inference MnS Producer shall have a capability allowing an authorized MnS consumer to request a mapping of the consumer's inference targets to the capabilities of one or more ML models. | Mapping of the capabilities of ML models (clause 6.5.3.2.2) |
| REQ-ML_CAP-0x | 3GPP management system should have the capability to allow an authorized consumer to request a coordination of different ML capabilities, e.g. between MDAS and RAN function, or MDAF and NWDAF, or among MDAF(s) or among ML Model(s) | Coordination between the ML capabilities (clause 6.5.3.2.x) |
| REQ-ML_CAP-0y | 3GPP management system should have the capability to report the result of coordination of different ML capabilities | Coordination between the ML capabilities (clause 6.5.3.2.x) |

End of changes