3GPP TSG-SA5 Meeting #160 S5-251457 # Stor-Göteborg, Sweden, 7th Apr 2025 - 11th Apr 2025 CR-Form-v12.3. CHANGE REQUEST. 28.552; CR; 0686; rev; -; Current version:; 19.3.0. For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests..

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Rel-19 CR TS28.552 Enhance Number of Active UEs related measurements for RedCap UEs |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ZTE Corporation |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S5 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_RedCap_OAM |  |  |  |  |  | Date: |  |  | 2025-02-06 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | This submission implements the solution for Use case #6: Performance evaluation on resource load of RedCap network service in TR 28.876 |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add UE type as subcounter for: Mean/Max number of Active UEs in the DL/UL per cell Mean/Max number of Active UEs per cell |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  |  |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.1.1.23.1,5.1.1.23.2,5.1.1.23.3,5.1.1.23.4,5.1.1.23.5,5.1.1.23.6 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

Start of change

#### 5.1.1.23	Number of Active UEs

##### 5.1.1.23.1	Mean number of Active UEs in the DL per cell

a)	This measurement provides the mean number of active UEs in the DL in an NRCellDU. The measurement is filterable per PLMN ID and per QoS level (mapped 5QI or/and QCI in EN-DC) and per supported S-NSSAI and filterable based on whether UE is (e)RedCap UE(See procedure in TS 38.473 clause 8.4.1 Initial UL RRC Message Transfer or TS 38.331 clause 5.6.1 UE capability transfer).

b)	DER (n=1).

c)	This measurement is obtained by aggregating the measurement "Mean number of Active UEs in the DL per DRB per cell" (see clause 4.2.1.3.2 in TS 38.314 [29]). The measurement is performed per PLMN ID and per QoS level (mapped 5QI or/and QCI in EN-DC) and per supported S-NSSAI.

d)	Each measurement is a single integer value. The number of measurements is equal to the number of PLMNs multiplied by the number of QoS levels multiplied by the number of supported S-NSSAIs.

[Total No. of measurement instances] x [No. of filter values for all measurements] (DL and UL) ≤ 100.

e)	The measurement name has the form DRB.MeanActiveUeDl_Filter, 
where filter is a combination of PLMN ID and QoS level and S-NSSAI and RedCapUE,

where PLMN ID represents the PLMN ID, QoS represents the mapped 5QI or/and QCI level, and S-NSSAI represents S-NSSAI, and RedCapUE is the filter with value 1 indicating (e)RedCapUE (see in 3GPP TS 38.306 clause  4.2.21.1 Definition of RedCap UE and 4.2.22.1 Definition of eRedCap UE), it represents the mean number of active UEs in the DL in an NRCellDU of (e)RedCap UEs. Value 0 indicates non-(e)RedCapUE.

f)	NRCellDU.

g)	Valid for packet switched traffic.

h)	5GS.

i)	One usage of this measurement is for performance assurance within integrity area (user plane connection quality).

##### 5.1.1.23.2	Max number of Active UEs in the DL per cell

a)	This measurement provides the max number of active UEs in the DL in an NRCellDU. The measurement is filterable per PLMN ID and per QoS level (mapped 5QI or/and QCI in EN-DC) and per supported S-NSSAI and filterable based on whether UE is (e)RedCap UE(See procedure in TS 38.473 clause 8.4.1 Initial UL RRC Message Transfer or TS 38.331 clause 5.6.1 UE capability transfer).

b)	DER (n=1).

c)	This measurement is defined according to measurement "Max number of Active UEs in the DL per DRB per cell" (see clause 4.2.1.3.3 in TS 38.314 [29]). The measurement is performed per PLMN ID and per QoS level (mapped 5QI or/and QCI in EN-DC) and per supported S-NSSAI.

d)	Each measurement is a single integer value. The number of measurements is equal to the number of PLMNs multiplied by the number of QoS levels multiplied by the number of supported S-NSSAIs.

[Total No. of measurement instances] x [No. of filter values for all measurements] (DL and UL) ≤ 100.

e)	The measurement name has the form DRB.MaxActiveUeDl_Filter, 
where filter is a combination of PLMN ID and QoS level and S-NSSAI and RedCapUE,

where PLMN ID represents the PLMN ID, QoS represents the mapped 5QI or/and QCI level, and S-NSSAI represents S-NSSAI, and RedCapUE is the filter with value 1 indicating (e)RedCapUE (see in 3GPP TS 38.306 clause  4.2.21.1 Definition of RedCap UE and 4.2.22.1 Definition of eRedCap UE), it represents the max number of active UEs in the DL in an NRCellDU of (e)RedCap UEs. Value 0 indicates non-(e)RedCapUE.

f)	NRCellDU.

g)	Valid for packet switched traffic.

h)	5GS.

i)	One usage of this measurement is for performance assurance within integrity area (user plane connection quality).

##### 5.1.1.23.3	Mean number of Active UEs in the UL per cell

a)	This measurement provides the mean number of active UEs in the UL in an NRCellDU. The measurement is filterable per PLMN ID and per QoS level (mapped 5QI or/and QCI in EN-DC) and per supported S-NSSAI and filterable based on whether UE is (e)RedCap UE(See procedure in TS 38.473 clause 8.4.1 Initial UL RRC Message Transfer or TS 38.331 clause 5.6.1 UE capability transfer).

b)	DER (n=1)

c)	This measurement is obtained by aggregating the measurement "Mean number of Active UEs in the UL per DRB per cell" (see clause 4.2.1.3.4 in TS 38.314 [29]). The measurement is performed per PLMN ID and per QoS level (mapped 5QI or/and QCI in EN-DC) and per supported S-NSSAI.

d)	Each measurement is a single integer value. The number of measurements is equal to the number of PLMNs multiplied by the number of QoS levels multiplied by the number of supported S-NSSAIs.

[Total No. of measurement instances] x [No. of filter values for all measurements] (DL and UL) ≤ 100.

e)	The measurement name has the form DRB.MeanActiveUeUl_Filter, 
where filter is a combination of PLMN ID and QoS level and S-NSSAI and RedCapUE,

where PLMN ID represents the PLMN ID, QoS represents the mapped 5QI or/and QCI level, and S-NSSAI represents S-NSSAI, and RedCapUE is the filter with value 1 indicating (e)RedCapUE (see in 3GPP TS 38.306 clause  4.2.21.1 Definition of RedCap UE and 4.2.22.1 Definition of eRedCap UE), it represents the mean number of active UEs in the UL in an NRCellDU of (e)RedCap UEs. Value 0 indicates non-(e)RedCapUE.

f)	NRCellDU.

g)	Valid for packet switched traffic.

h)	5GS.

i)	One usage of this measurement is for performance assurance within integrity area (user plane connection quality).

##### 5.1.1.23.4	Max number of Active UEs in the UL per cell

a)	This measurement provides the max number of active UEs in the UL in an NRCellDU. The measurement is filterable per PLMN ID and per QoS level (mapped 5QI or/and QCI in EN-DC), subcounters per S-NSSAI and per supported PLMN ID and filterable based on whether UE is (e)RedCap UE(See procedure in TS 38.473 clause 8.4.1 Initial UL RRC Message Transfer or TS 38.331 clause 5.6.1 UE capability transfer).

b)	DER (n=1)

c)	This measurement is defined by the measurement "Max number of Active UEs in the UL per DRB per cell" (see clause 4.2.1.3.5 in TS 38.314 [29]). The measurement is performed per PLMN ID and per QoS level (mapped 5QI or/and QCI in EN-DC) and per supported S-NSSAI.

d)	Each measurement is a single integer value. The number of measurements is equal to the number of PLMNs multiplied by the number of QoS levels multiplied by the number of supported S-NSSAIs.
[Total No. of measurement instances] x [No. of filter values for all measurements] (DL and UL) ≤ 100.

e)	The measurement name has the form DRB.MaxActiveUeUl_Filter, 
where filter is a combination of PLMN ID and QoS level and S-NSSAI and RedCapUE,
where PLMN ID represents the PLMN ID, QoS represents the mapped 5QI or/and QCI level, and SNSSAI represents S-NSSAI, and RedCapUE is the filter with value 1 indicating (e)RedCapUE (see in 3GPP TS 38.306 clause  4.2.21.1 Definition of RedCap UE and 4.2.22.1 Definition of eRedCap UE), it represents the max number of active UEs in the UL in an NRCellDU of (e)RedCap UEs. Value 0 indicates non-(e)RedCapUE.

f)	NRCellDU.

g)	Valid for packet switched traffic.

h)	5GS.

i)	One usage of this measurement is for performance assurance within integrity area (user plane connection quality).

##### 5.1.1.23.5	Mean number of Active UEs per cell

a)	This measurement provides the mean number of active UEs in an NRCellDU. This measurement refers to UEs for which there is data available for transmission for the UL for DRBs, or there is data available for transmission for the DL for DRBs, or both. This measurement can’t be calculated from the Mean number of active UEs in the DL per cell and Mean number of active UEs in the UL per cell according to 2 out of 3 approach. The measurement is filterable per PLMN ID and per supported S-NSSAI and filterable based on whether UE is (e)RedCap UE(See procedure in TS 38.473 clause 8.4.1 Initial UL RRC Message Transfer or TS 38.331 clause 5.6.1 UE capability transfer).

b)	DER (n=1).

c)	This measurement is obtained by aggregating the measurement "Mean number of Active UEs per cell" (see clause 4.2.1.3.6 in TS 38.314 [29]). The measurement is performed per PLMN ID and per supported S-NSSAI.

d)	Each measurement is a single integer value. The number of measurements is equal to the number of PLMNs multiplied by the number of supported S-NSSAIs.

[Total No. of measurement instances] x [No. of filter values for all measurements] ≤ 100.

e)	The measurement name has the form DRB.MeanActiveUe_Filter,
where filter is a combination of PLMN ID and S-NSSAI and RedCapUE, 
where PLMN ID represents the PLMN ID, and S-NSSAI represents S-NSSAI, and RedCapUE is the filter with value 1 indicating (e)RedCapUE (see in 3GPP TS 38.306 clause  4.2.21.1 Definition of RedCap UE and 4.2.22.1 Definition of eRedCap UE), it represents the mean number of active UEs in an NRCellDU of (e)RedCap UEs. Value 0 indicates non-(e)RedCapUE

f)	NRCellDU.

g)	Valid for packet switched traffic.

h)	5GS.

i)	One usage of this measurement is for performance assurance within integrity area (user plane connection quality).

##### 5.1.1.23.6	Max number of Active UEs per cell

a)	This measurement provides the max number of active UEs in an NRCellDU. This measurement refers to UEs for which there is data available for transmission for the UL for DRBs, or there is data available for transmission for the DL for DRBs, or both. This measurement can’t be calculated from the Max number of active UEs in the DL per cell and Max number of active UEs in the UL per cell according to 2 out of 3 approach. The measurement is filterable per PLMN ID and per supported S-NSSAI and filterable based on whether UE is (e)RedCap UE(See procedure in TS 38.473 clause 8.4.1 Initial UL RRC Message Transfer or TS 38.331 clause 5.6.1 UE capability transfer).

b)	DER (n=1).

c)	This measurement is defined according to measurement "Max number of Active UEs per cell " (see clause 4.2.1.3.7 in TS 38.314 [29]). The measurement is performed per PLMN ID and per supported S-NSSAI.

d)	Each measurement is a single integer value. The number of measurements is equal to the number of PLMNs multiplied by the number of supported S-NSSAIs.

[Total No. of measurement instances] x [No. of filter values for all measurements] ≤ 100.

e)	The measurement name has the form DRB.MaxActiveUe_Filter,
where filter is a combination of PLMN ID and S-NSSAI and RedCapUE, 
where PLMN ID represents the PLMN ID, and S-NSSAI represents S-NSSAI, and RedCapUE is the filter with value 1 indicating (e)RedCapUE (see in 3GPP TS 38.306 clause  4.2.21.1 Definition of RedCap UE and 4.2.22.1 Definition of eRedCap UE), it represents the Max number of active UEs in an NRCellDU of (e)RedCap UEs. Value 0 indicates non-(e)RedCapUE.

f)	NRCellDU.

g)	Valid for packet switched traffic.

h)	5GS.

i)	One usage of this measurement is for performance assurance within integrity area (user plane connection quality).

End of change