3GPP TSG-SA5 Meeting #160	S5-251700

Goteborg, Sweden, 07 - 11 April 2025

# Source:	Nokia

# Title:	Rel-19 pCR TS 28.579 Add use case solution to enable the publishing management services to the CCF

# Document for:	Approval

Agenda Item:	6.19.21.1

# 1	Decision/action requested

The group is asked to discuss and approve the proposals.

# 2	References

[1] 3GPP TS 28.579, "Management services exposure to external consumers through CAPIF".

# 3	Rationale

This pCR proposes to add a solution for publishing management services to the CCF.

# 4	Detailed proposal

It is proposed to add the following use case solution to TS 28.579[1].

First Change

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]                       3GPP TS 23.222: "Common API Framework for 3GPP Northbound APIs".

[X]	3GPP TS 28.622: “Telecommunication management; Generic Network Resource Model (NRM) Integration Reference Point (IRP); Information Service (IS)”.

[Z]	3GPP TS 29.222: "Common API Framework for 3GPP Northbound APIs; stage 3".

[Y]	TS 32.158: "Management and orchestration; Design rules for REpresentational State Transfer (REST) Solution Sets (SS)".

…

[x]	<doctype> <#>[ ([up to and including]{yyyy[-mm]|V<a[.b[.c]]>}[onwards])]: "<Title>".

Second Change

# 6 Solutions for exposing management services using CAPIF

## 6.X Publishing of management services into the CCF

### 6.X.1 Introduction

To publish management services to the CCF, the following steps occur :

-    The management service shall be mapped into a service API, since CAPIF publishes service APIs and not management services. The management service information (described by the MnSInfo IOC (see clause 4.3.42 of TS 28.622[X])) is mapped into the service API information using the "ServiceAPIDescription" data type (see clause 8.2.4.2.2 in 3GPP TS 29.222 [Y]). The "ServiceAPIDescription" data type describes a service API, and the network operator can decide to publish a single management service as one or more service APIs at the CCF.

-	The management services now mapped into service APIs are published to the CCF using the Publish_Service_API (see clause 5.3 of TS 29.222 [Y]) over the CAPIF-4 interface. The APF of the MSED performs the publishing functionality.

### 6.X.2 Mapping of management service information into service API information

Table 6.X.2-1 presents the attributes of the ServiceAPIDescription data type (see clause 8.2.4.2.2-1 of TS 29.222 [Y]) and clarifies which attributes (of the ServiceAPIDescription data type) can be mapped from management service information. Refer to Table 8.2.4.2.2-1 of TS 29.222 [Y] for detailed information on the attributes of ServiceAPIDescription data type (e.g., the attribute data type, presence indicator, cardinality, description and applicability information).

Table 6.X.2-1: Mapping of management service information into ServiceAPIDescription data type attributes

| Attribute name | Attribute additional information | Equivalent MnSInfo IOC attribute/comments |
| --- | --- | --- |
| apiName | The data type of this attribute is defined as "string" and presence qualifier is defined as "M". | Corresponds to the following MnSInfo IOC attribute: mnsType |
| apiId | The data type of this attribute is defined as "string" and presence qualifier is defined as "O". |  |
| aefProfiles | The data type of this attribute is defined as " array(AefProfile)" and presence qualifier is defined as "C". | See Table 6.X.2-2 |
| description | The data type of this attribute is defined as "string" and presence qualifier is defined as "O". |  |
| supportedFeatures | The data type of this attribute is defined as "SupportedFeatures" and presence qualifier is defined as "O". | N/A |
| shareableInfo | The data type of this attribute is defined as "ShareableInformation" and presence qualifier is defined as "O". | N/A |
| serviceAPICategory | The data type of this attribute is defined as "string" and presence qualifier is defined as "C". |  |
| ccfId | The data type of this attribute is defined as "string" and presence qualifier is defined as "C". | N/A |
| apiSuppFeats | The data type of this attribute is defined as "SupportedFeatures" and presence qualifier is defined as "O". | N/A |
| pubApiPath | The data type of this attribute is defined as "PublishedApiPath" and presence qualifier is defined as "C". | N/A |

Table 6.X.2-2 presents the attributes contained in the AefProfile data type (see clause 8.2.4.2.4 of TS 29.222 [Z]) and clarifies which attributes (of the AefProfile data type) can be mapped from the management service information. See Table 8.2.4.2.4-1 of TS 29.222 [Z] for detailed information on the attributes of the AefProfile data type (e.g., the attribute data type, presence indicator, cardinality, description and applicability information).

Table 6.X.2-2: Mapping of management service information into AefProfile data type attributes

| Attribute name | Attribute additional information | Equivalent MnSInfo IOC attribute/comments |
| --- | --- | --- |
| aefId | The data type of this attribute is defined as "string" and presence qualifier is defined as "M". | Corresponds to the AEF identifier provided by the CCF upon MSED registration (see clause 6.Z). |
| versions | The data type of this attribute is defined as "array(Version)" and presence qualifier is defined as "M". | See Table 6.X.2-3. |
| protocol | The data type of this attribute is defined as "Protocol" and presence qualifier is defined as "O". | Only "HTTP_1_1" and "HTTP_2" values are applicable. |
| dataFormat | The data type of this attribute is defined as "DataFormat" and presence qualifier is defined as "O". | Only applicable value is “JSON”. |
| securityMethods | The data type of this attribute is defined as "array(SecurityMethod)" and presence qualifier is defined as "O". | Only applicable value is "OAUTH" (i.e. TLS with OAuth token). |
| domainName | The data type of this attribute is defined as "string" and presence qualifier is defined as "O". |  |
| interfaceDescriptions | The data type of this attribute is defined as "array (InterfaceDescription)" and presence qualifier is defined as "O". | See Table 6.X.2-5. |
| aefLocation | The data type of this attribute is defined as "AefLocation" and presence qualifier is defined as "O". |  |

Table 6.X.2-3 presents the attributes of the Version data type (see clause 8.2.4.2.5 of TS 29.222 [Z]) and clarifies which attributes (of the Version data type) can be mapped from management service information. See Table 8.2.4.2.5-1 of TS 29.222 [Z] for detailed information on the attributes of the Version data type (e.g., the attribute data type, presence indicator, cardinality, description and applicability information).

Table 6.X.2-3: Mapping of management service information into Version datatype attributes

| Attribute name | Attribute additional information | Equivalent MnSInfo IOC attribute/comments |
| --- | --- | --- |
| apiVersion | The data type of this attribute is defined as "string" and presence qualifier is defined as "M". | Corresponds to the following MnSInfo IOC attribute: mnsVersion |
| expiry | The data type of this attribute is defined as "DateTime" and presence qualifier is defined as "O". | N/A. |
| resources | The data type of this attribute is defined as "array(Resource)" and presence qualifier is defined as "O". | See Table 6.X.2-4. Each Resource corresponds to an MOI accessed through this MnS. |
| custOperations | The data type of this attribute is defined as "array(CustomOperation)" and presence qualifier is defined as "O". |  |

Table 6.X.2-4 presents the attributes of the Resource data type (see clause 8.2.4.2.6 of TS 29.222 [Z]) and clarifies which attributes (of the Resource data type) can be mapped from management service information. See Table 8.2.4.2.6-1 of TS 29.222 [Z] for detailed information on the attributes of the Resource data type (e.g., the attribute data type, presence indicator, cardinality, description and applicability information).

Table 6.X.2-4: Mapping of management service information into Resource data type attributes

| Attribute name | Attribute additional information | Equivalent MnSInfo IOC attribute/comments |
| --- | --- | --- |
| resourceName | The data type of this attribute is defined as "string" and presence qualifier is defined as "M". | IOC name of the MOI. In the URI structure of the MnS API (see clause 6.X.4.2), this attribute corresponds to the variable parameter {className}. |
| commType | The data type of this attribute is defined as "CommunicationType" and presence qualifier is defined as "M". | Only "REQUEST_RESPONSE" value is applicable for SA5 MnS of type Provisioning. |
| uri | The data type of this attribute is defined as "string" and presence qualifier is defined as "M". | Corresponds to the URI LDN of the IOC/MOI to be published (see clause 4.4.2 of TS 32.158[Y]). In the URI structure of the MnS API (see clause 6.X.4.2), this attribute corresponds to: {URI-LDN-first-part}/{className} = {id}. |
| custOpName | The data type of this attribute is defined as "string" and presence qualifier is defined as "O". |  |
| custOperations | The data type of this attribute is defined as "array(CustomOperation)" and presence qualifier is defined as "O". |  |
| operations | The data type of this attribute is defined as "array(Operation)" and presence qualifier is defined as "C". | This attribute represents an array of HTTP methods applicable for the published IOC/MOI. The valid values are POST, GET, PUT, PATCH, DELETE. It is up to the network operator’s discretion to determine the allowable HTTP operations on an IOC/MOI to be published. |
| description | The data type of this attribute is defined as "string" and presence qualifier is defined as "O" (see table 8.2.4.2.6-1 of TS 29.222 [Z] ). |  |

Table 5.1.2.3.1.2-5 presents the attributes of the InterfaceDescription data type (see clause 8.2.4.2.3 of TS 29.222 [Z]) and clarifies which attributes (of the InterfaceDescription data type) can be mapped from the management service information. See Table 8.2.4.2.3-1 of TS 29.222 [Z] for the data type, presence indicator, cardinality, description and applicability information for attributes of InterfaceDescription.

Table 5.1.2.3.1.2-5: Mapping of management service information into InterfaceDescription datatype attributes

| Attribute name | Attribute additional information | Equivalent MnSInfo IOC attribute/comments |
| --- | --- | --- |
| ipv4Addr | The data type of this attribute is defined as "Ipv4Addr" and presence qualifier is defined as "C". | N/A |
| ipv6Addr | The data type of this attribute is defined as "Ipv6Addr" and presence qualifier is defined as "C". | N/A |
| fqdn | The data type of this attribute is defined as "Fqdn" and presence qualifier is defined as "C". | Corresponds to the URI-DN-prefix of the IOC/MOI to be published. The FQDN of the IOC/MOI can be constructed from the DN prefix as detailed in clause 4.2.3 of TS 32.158 [Y]. In the URI structure of the MnS API (see clause 6.X.4.2), this attribute corresponds to the following: {URI-DN-prefix}. |
| port | The data type of this attribute is defined as "Port" and presence qualifier is defined as "O". |  |
| apiPrefix | The data type of this attribute is defined as "string" and presence qualifier is defined as "O". | Corresponds to the optional “root” path component of the IOC/MOI to be published (see clause 4.4.2 of TS 32.158). In the URI structure of the MnS API (see clause 6.X.4.2), this attribute corresponds to the following "/{root}”. |
| securityMethods | The data type of this attribute is defined as "array(SecurityMethod)" and presence qualifier is defined as "O". | Only applicable value is "OAUTH" (i.e. TLS with OAuth token). |

With this mapping, management service(s) can be translated into CAPIF service API(s) described by the corresponding ServiceAPIDescription datatype(s).

### 6.X.3 Service operations supported by the APF of MSED to enable the publishing functionality

To publish the service APIs (from clause 6.X.2) to the CCF, the APF functionality of MSED invokes the Publish_Service_API service operation as described in clause 5.3.2.2 of TS 29.222 [Z] via the CAPIF-4 interface. The body of the HTTP POST request shall include the Service API description details as described in clause 6.X.2.

To retrieve the published service APIs from the CCF, the APF functionality of MSED invokes the Get_Service_API service operation as described in clause 5.3.2.4 of TS 29.222 [Z] via the CAPIF-4 interface.

To update the published service APIs from the CCF, the APF functionality of MSED invokes the Update_Service_API service operation as described in clause 5.3.2.5 of TS 29.222 [Z] via the CAPIF-4 interface. The body of the HTTP PUT request shall include the updated Service API description details as described in clause 6.X.2.

To unpublish the published service APIs from the CCF, the APF functionality of MSED invokes the Unpublish_Service_API service operation as described in clause 5.3.2.3 of TS 29.222 [Z] via the CAPIF-4 interface.

### 6.X.4	Mapping of the MnS API URI to the Service API URI

#### 6.X.4.1	Introduction

After publishing management services (now published service APIs) to the CCF as described clause 6.X.3, the published service APIs are available for discovery by the external MnS consumers via the CAPIF-1 interface. To discover the published service APIs, the external MnS consumer invokes the GET request operation of the CAPIF_Discover_Service_API service (see clause 8.2 of TS 29.222[Z]).  The response body of the GET request returns the DiscoveredAPIs datatype (see Table 8.1.4.2.2-1 of TS 29.222[Z]). The DiscoveredAPI data type consists of an array of the published service APIs each represented by the corresponding serviceAPIDescription data type (see clause 6.X.2).

On discovering the service APIs, the external MnS consumer can use the discovered service API information to construct the URI of the service API that will be invoked at the AEF of the MSED (via the CAPIF-2e interface). Clause 6.X.4.2 describes how the external MnS consumer can construct the URI of the service API to be invoked.

#### 6.X.4.2	Description

The solution shows how the external MnS consumer can construct the service API URIs from the discovered service APIs. To construct the service API URI, it is needed first to compare and map the URI structure of the service API to the MnS API URI structure as shown in Table 6.X.4.2-1.

Table 6.X.4.2-1: URI structure for Service API and MnS API

| Service API | MnS API (see clause 4.4 of TS 32.158[Y]) |
| --- | --- |
| URI: = <apiRoot>/<apiName>/<apiVersion>/<APISpecificSuffixes> See NOTE 1 | URI: = {MnSRoot}/{MnSName}/{MnSVersion}/{URI-LDN-first-part}/{className} = {id} See NOTE 2 |
| NOTE 1: <apiRoot>:= https://<authority>/<API-prefix>, with <API-prefix> being optional. NOTE 2: {MnSRoot} = https://{URI-to-DN-prefix}/{root}, with {root} being optional. |  |

From Table 6.X.4.2-1, the following mapping applies:

-	{MnSRoot} corresponds to the <apiRoot>. The {apiRoot} variable of the URI structure for the service API can be constructed by the external MnS consumer based on the value of the "interfaceDescriptions" attribute of the AefProfile data type (see Table 6.X.2-2).

-	{URI-to-DN-prefix} corresponds to the <authority> (host and optional TCP port). The host name is constructed from the DN prefix as detailed in clause 4.2.3 of TS 32.158 [Y].

-	{root} corresponds to <API-prefix>.  The value of the <API-prefix> shall be the value of the “apiPrefix” attribute of the InterfaceDescription datatype (see Table 5.1.2.3.1.2-5).

-	{MnSName} corresponds to <apiName>. The value of the <apiName> shall be the value of the “apiName” attribute of the ServiceAPIDescription data type (see Table 6.X.2-1).

-	{MnSVersion} corresponds to <apiVersion>. The value of the <apiVersion> shall be the value of the “apiVersion” attribute of the Version datatype (see Table 6.X.2-3).

-	{URI-LDN-first-part}/{className} = {id} corresponds to <apiSpecificSuffixes>. The value of the <apiSpecificSuffixes> will be the value of the “uri” attribute of the Resource data type (see Table 6.X.2-4).

End of Changes