3GPP TSG-SA5 Meeting #160	S5-251708

Goteborg, Sweden, 7- 11 April 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 28.315 | CR | 0001 | rev | - | Current version: | 18.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Procedure IAB-node connects to management system |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA5 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_MOBILE_IAB_OAM |  |  |  |  |  | Date: |  |  | 2025-03-28 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | This is nomative work for Rel-19 study on management of IAB nodes |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add procedure for IAB-node to connect to management system |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  |  |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.1, 5.1.x(new), A.1, A.1.x(new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

1st Change

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP TS 28.314: "Management and orchestration; Plug and Connect; Concepts and requirements".

[3]	3GPP TS 28.316: "Management and orchestration; Plug and Connect; Data formats".

[y]	3GPP TS 28.532: "Technical Specification; Management and orchestration; Generic management services".

2nd Change

## 5.1.x IAB-node connects to management system

IAB-node connects to management system at power up and during mobiltiy by following PnC steps as described in clause 6.1.x of TS 28.314 [2]. The management system that IAB-node connects to may change due to IAB-node mibility behaviours. The procedures for IAB-node connectiong to 3GPP management system based on its locations are illustrated in Figure 5.1.x.1, this includes step 2 of obtaining IP configuration via notification mechanism instead of initial IP autoconfiguration procedure.

In step 1, IAB-node has established a connection with 3GPP management system via PnC

In step 2, It reuses the existing notifications and operations as defined in TS 28.532 [y] to obtain IP configuration for OAM connectivity:

- The provisioning MnS producer of IAB-node should provide IAB-node location information via any of the following notifications:

notifyMOICreation

notifyMOIAttributeValueChanges

notififyMOIChanges

- The provisioning MnS consumer (management system) should provide IP configuration for OAM connectivity via any of the following operations upon the reception of notifications:

createMOI

modifyMOIAttributes

In step 3, IAB-node connects to new 3GPP management system by following subsequent PnC steps including connection to CA/RA, SeGW and SCS.

Figure 5.1.x.1 Procedure flows for IAB-node connects to management system

3rd Change

## A.1.x	IAB-node connects to management system

The following PlantUML source code is used to describe the procedure flows for IAB-node connects to management system, as depicted by Figure 5.1.1:

@startuml

actor “IAB-node” as IAB

participant "OAM 1" as OAM_1

participant "OAM 2" as OAM_2

Ref over IAB, OAM_1: 1. Precondition: IAB-node has established connection with 3GPP management system (OAM 1) via PnC

Ref over IAB, OAM_1: 2. Obtain IP configuration for new management system via notifications based on IAB-node location

Ref over IAB, OAM_2: 3. Connect to new 3GPP management system (OAM 2) by following subsequent PnC procedures including connection to CA/RA, SeGW and SCS

hide footbox

skinparam DefaultFontSize 11

skinparam sequenceActorBackgroundColor #FFFFFF

skinparam sequenceParticipantBackgroundColor #FFFFFF

skinparam noteBackgroundColor #FFFFFF

autonumber "#'.'"

skinparam monochrome true

skinparam shadowing false

@enduml

End of change