3GPP TSG-SA5 Meeting #160	S5-251720

# Stor-Göteborg, Sweden, 7th Apr 2025 - 11th Apr 2025

# Source:	Ericsson GmbH, Eurolab

# Title:	DP on Solution-agnostic request for service provisioning

# Document for:	Endorsement

Agenda Item:	6.19.13

# 1	Decision/action requested

The group is asked to discuss and endorse the proposal

# 2	References

[1]	3GPP TS 28.530, “Management and orchestration; Concepts, use cases and requirements”

[2]	3GPP TS 28.541, “Management and orchestration; 5G Network Resource Model (NRM); Stage 2 and 3”

[3]	TM Forum Open Digital Architecture (ODA). Link: https://www.tmforum.org/oda/

[4] 	TM Forum Open APIs. Link: https://www.tmforum.org/oda/implementation/open-apis/

# 3	Rationale

## 3.1 	Introduction

The 3GPP SA5 group specifies model-based solutions that helps Mobile Network Operators (MNOs) to automate the provisioning of communication services, such that service requirements from hosted use cases/customers can be met as needed.

An MNO can play two roles:

- Communication Service Provider (CSP), which is responsible for providing communication services. The CSP is focused on the management of requested services (ensuring their requirements can be fulfilled and assured) and customer relationship (onboarding, charging, ticketing, etc.).

- Network Operator (NOP), which is responsible for managing 5G network resources, and combining them to deliver mobile network solutions (e.g., mobile VPN, network slicing, DNN provisioning). These mobile network solutions aim to provide individual CSP services with necessary capacity and functionality, while making an efficient resource usage.

For the CSP role, clause 4.8 of TS 28.530 [2] notes that “CSP provided communication service can be built with or without network slices”. Though this assertion may lead to think that communication service and network slicing are concepts that can be decoupled, the reality in 3GPP specifications is rather different, with multiple references that show the opposite, i.e. communication service concept is rooted/anchored to network slicing.

Regarding stage 1, proofs of this issue can be found in different clauses from TS 28.530 [1], including:

- clause 4.1.3 (“communication services using network slices”): it illustrates how multiple communication services can be realized with different network slices, emphasizing the N:M relationship between them. However, the specification does not have another clause that reports how communication services can be realized with network solutions other than slicing.

- clause 4.1.4  (“communication service requirements”): it explicitly notes that different service types may include different network slice related requirements

- clause 4.1.5 (“NetworkSlice instance Lifecycle and relationship to service instances”): it highlights the impact that service instances have on the lifecycle of network slice instances hosting them. However, the specification does not have another clause reporting the impact of service instances on the lifecycle of another network solutions.

- clauses 4.1.6 (“Network Slice as a Service”) and 4.1.7 (“Network slices as NOP internals”). These clauses define two alternative service delivery models for a CSP to offer communication services to customers. Despite their differences, both models assume the usage of network slicing.

- clause 4.1.6 (“Network slice as a Service”): in this CSP model, the concepts of communication service and network slices are even intertwined, taking into account that what the CSP offers is precisely a network slice; the relationship between NOP and CSP with regards to network slice management capability consumption becomes vague as well.

Regarding stages 2 and 3, proofs of this issue can be found in 3GPP TS 28.541 [2], which details the fragments forming the 5G NRM, including NR (clause 4), 5GC (clause 5) and network slicing (clause 6). In this specification, the only construction that captures service-related requirements, and therefore could be closest to representing the concept of communication service, is ServiceProfile. However, multiple aspects prevent this aspiration:

- ServiceProfile belongs to Network Slice NRM fragment (clause 6 of TS 28.541 [3]). This makes it a construction in the realm of NOP.

- ServiceProfile contains slice-specific attributes that are mandatory to support.

- ServiceProfile can only be contained by a NetworkSlice IOC, making ServiceProfile a construction rooted/anchored to network slice.

## 3.2 	Problems with existing solution

## 3.2.1	Issues

## 3.2.1.1	CSP offering is not limited to wireless-only services.

In the context of 3GPP, the concept of communication service is introduced in TS 28.530 [2], with multiple references in different clauses: 4.1.2, 4.1.3, 4.1.4 and 4.8. All these clauses assume that communication services are provisioned with 3GPP networks. However, the CSP offering is not limited to these wireless-only services (aka “3GPP communication services”). As seen in Figure 1, other communication services can also be part of the CSP offering. Furthermore, the CSP can use these communication services and combine them with IT/OT applications to define new communication services. The TM Forum defines an architectural blueprint called Open Digital Architecture (ODA) [3], which aims to help CSP structure and catalogue their service offerings into Customer Facing Services (CFS) and Resource Facing Services (RFS), defining their dependencies and their composition patterns.

## 3.2.1.2	CSP systems needs to communicate with (sub)systems other than 3GPP management system

To manage the requested communication services and customer relationships, the CSP system includes service orchestration and BSS tools implementing TM Forum Open APIs [4], e.g. TMF641 (Service Ordering API), TMF638 (Service Inventory API), TMF633 (Service Catalogue management API), TMF645 (Service Qualification API). To issue service requirements to NOP, the CSP system need to communicate with domain-specific management systems, each targeting different network domains (e.g., mobile, transport, data network) and typically managed by a different forum (e.g., 3GPP, O-RAN, IETF, BBF, etc).

## 3.2.2	Problem statement

According to TMF ODA [3], it is required that lower layers hide complexity and solution details to upper layers, to facilitate scalability and maintain separation of concerns. This means that domain-specific management system should provide abstraction to the CSP system, such that the latter focus on service requirements, without worrying on the solution used for their fulfilment and assurance.

The NOP hosting 3GPP management system offers MnSs to operate over IOCs and data types defined in the different NRMs. This means that when becoming MnS consumer, the CSP system (e.g., Service Orchestration, BSS) hall then understand the IOCs and data types accessible from this MnS. To comply with TMF ODA on abstraction, it is required that these IOCs and data types include technology-agnostic details.

In 5G systems, communication services are delivered using network slicing. This means that CSP system shall become network slice provisioning MnS consumer. The CSP system issues an NSI allocation request with ServiceProfile [2] expressing input requirements. As seen in Table 1, this data type includes slicing-specific attributes, which are marked in green. These attributes are arranged into two groups.

The first group is formed of those attributes with support qualifier set to “O (Optional)”. These attributes are six (6): “networkSliceSharingIndicator”, “dlThputPerSlice”, “ulThputPerSlice”, “sliceSimultaneousUse”, “nssaaSupport”, “sliceAvailability”. The implementation of these attributes in ServiceProfile is at MnS producer’s discretion. If not implementing them, the MnS producer provides no means for the MnS consumer to specify or read them.

The second group includes those attributes with support qualifier set to “M (Mandatory)”, i.e. attributes that the MnS producer shall always support. These attributes are two (2):

- “sST”, which expresses the slice/service type (SST) associated to the service requirements (see standardized SST values in clause 5.15.2 of TS 23.501 [11]). “ssT” attribute is writable with cardinality = 1, which means that MnS consumer always need to provide it as input requirement when issuing a NSI allocation request.

- “pLMNInfoList”, which is an array of tuples {PLMN ID, S-NSSAI}, each univocally identifying a network slice.  This attribute is set by the MnS producer, by first generating the S-NSSAI and then appending it to the PLMN ID. This attribute is readable with cardinality = 1..*, which means that it is always made available in the ServiceProfile for the MnS consumer to read.

Problem formulation: For a CSP system that wants to request the fulfilment of service requirements in a 5G network, this system always need to understand details on network slicing (i.e. the network solution used by NOP). These details are captured in “sST” and “pLMNInfoList” attributes, which are the non-nullable attributes that MnS producer needs to always support in ServiceProfile. This represents a problem for CSP systems that do not understand slice semantics, since it prevents them from integrating as 3GPP management system consumers.  In addition, this slicing-awareness on CSP system side violates the ODA principle on abstraction, while breaking the decoupling between CSP (focus on issuing service requirements and get reported about their fulfilment) and NOP (focus on delivering network solutions to meet and assure these requirements).

Table 1: ServiceProfile attributes (see TS 28.541 [2], clause 6.3.3.2).

| Attribute name | S | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| serviceProfileId | M | T | F | T | T |
| pLMNInfoList | M | T | F | F | T |
| maxNumberofUEs | O | T | T | F | T |
| coverageArea | O | T | T | F | T |
| dLLatency | O | T | T | F | T |
| uLLatency | O | T | T | F | T |
| uEMobilityLevel | O | T | T | F | T |
| networkSliceSharingIndicator | O | T | T | F | T |
| sST | M | T | T | F | T |
| availability | O | T | T | F | T |
| delayTolerance | O | T | T | F | T |
| dLDeterministicComm | O | T | T | F | T |
| uLDeterministicComm | O | T | T | F | T |
| dLThptPerSlice | O | T | T | F | T |
| dLThptPerUE | O | T | T | F | T |
| uLThptPerSlice | O | T | T | F | T |
| uLThptPerUE | O | T | T | F | T |
| dLMaxPktSize | O | T | T | F | T |
| uLMaxPktSize | O | T | T | F | T |
| maxNumberofPDUSessions | O | T | T | F | T |
| kPIMonitoring | O | T | T | F | T |
| userMgmtOpen | O | T | T | F | T |
| v2XCommModels | O | T | T | F | T |
| termDensity | O | T | T | F | T |
| activityFactor | O | T | T | F | T |
| uESpeed | O | T | T | F | T |
| survivalTime | O | T | T | F | T |
| radioSpectrum | O | T | T | F | T |
| dLReliability | O | T | T | F | T |
| uLReliability | O | T | T | F | T |
| maxDLDataVolume | O | T | T | F | T |
| maxULDataVolume | O | T | T | F | T |
| nBIoT | O | T | T | F | T |
| synchronicity | O | T | T | F | T |
| positioning | O | T | T | F | T |
| sliceSimultaneousUse | O | T | T | F | T |
| energyEfficiency | O | T | T | F | T |
| nssaaSupport | O | T | T | F | T |
| n6Protection | O | T | T | F | T |
| nonIPSupport | O | T | T | F | T |
| supportedDataNetworks | O | T | T | F | T |
| dataNetworkAccess | O | T | T | F | T |
| sliceAvailability | O | T | T | F | T |
| dLPktDelayVariation | O | T | T | F | T |
| uLPktDelayVariation | O | T | T | F | T |

## 3.3 	Solution Description

It is proposed to enhance the existing Network Slice NRM fragment (3GPP TS 28.541 [2], clause 6), such that 3GPP management system supports receiving requests to fulfil 5G service requirements in a technology-agnostic way, i.e. without the need for CSP system to include slice-specific attributes in the request.

The focus of the solution is to make ServiceProfile a data type that can be used to capture service-related requirements in a technology-agnostic way. This gives the possibility for CSP system to become consumer of 3GPP management system, even when the CSP system does not understand slicing semantics.

This solution would require applying four main changes that are detailed below.

## 3.3.1 	Change #1: Improve ServiceProfile definition

The definition of this data type can be found in TS 28.541 [1], clause 6.3.3.1. The excerpt is as follows:

“This data type represents the properties of the network slice related requirements that should be supported by a NetworkSlice instance in a 5G network. The network slice related requirements apply to a one-to-one relationship between a Network Slice Customer (NSC) and a Network Slice Provider (NSP). A network slice can be tailored based on the specific requirements adhered to an SLA agreed between NSC and NSP, see clause 2 of [50]. An NSP may add additional requirements not directly derived from SLA’s, associated to the NSP internal [business] goals. The GST defined by GSMA (see [50]) and the service performance requirements defined in 3GPP TS 22.261 [28] and TS 22.104 [51] are all considered as input for the network slice related requirements”

As seen, the description is quite tied to network slicing. The aim is to simplify the data type definition, focusing now on what is represents (communication service related requirements) instead of the actual underlying network solution (network slice). Nonetheless, the new definition still clarifies that the service requirements (CSP requirements) expressed with ServiceProfile will be fulfilled with network slicing (NOP network solution).

The proposed new text would be as follows:

“This data type represents the properties of the communication service related requirements in a 5G network. These requirements will be fulfilled with network slicing. The GST defined by GSMA (see [50]) and the service performance requirements defined in 3GPP TS 22.261 [28] and TS 22.104 [51] are all considered as input for the communication service related requirements”

## 3.3.2 	Change #2: Improve ServiceProfile specification

As noted in section 3.2.2, there are two attributes in ServiceProfile that force CSP system to be slicing-aware: “sST” and “pLMNInfoList”, as both are non-nullable attributes and are mandatory to support by the MnS producer.

For “SST” attribute, it is proposed to make it a nullable attribute, by changing the cardinality to “0..1”. This will give the possibility for slicing-agnostic CSP systems to not specify this attribute, while allowing slicing-aware CSP systems to keep providing it. The solution is backwards compatible.

For the “pLMNInfoList”, it is proposed to keep it unchanged. If slicing-agnostic CSP system consumes ServiceProfile, it can simply ignore the S-NSSAI component, focusing on PLMN ID.  The solution is backwards compatible.

## 3.3.3 	Change #3: Improve SliceProfile definition

The definition of this data type can be found in TS 28.541 [3], clause 6.3.4.1. The excerpt is as follows:

“This data type represents the properties of network slice subnet related requirement that should be supported by the NetworkSliceSubnet instance in a 5G network.”

The aim is to make SliceProfile a datatype able to capture service requirements with necessary slicing information, regardless whether this information represents requirements applicable to the whole slice or parts of it (network slice subnet); this is consistent with the recursion property, whereby a network slice (subnet) can consist of other network slice subnets.

The proposed new text would be as follows:

“This data type represents the properties of network slicing related requirements in a 5G network”

With the new definition, SliceProfile represents slicing-specific information (valuable for NOP), while ServiceProfile represents slicing-agnostic information (valuable for CSP).

## 3.3.4	Change #4: Add a new capability on network slice provisioning MnS producer

In SliceProfile the “pLMNInfoList” attribute is mandatory to support (by the network slice subnet provisioning MnS producer) and writable (by the network slice subnet provisioning MnS consumer) – see table below. This means that the NOP’s entity playing the role the role of network slice provisioning MnS producer shall, based on input ServiceProfile, be able to generate “plmnInfoList” attribute. The attribute value will be included in the SliceProfile (see below) that this entity, when playing the role of network slice subnet provisioning MnS consumer, will send to the network slice subnet provisioning MnS producer.

| Attribute name | S | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| sliceProfileId | M | T | F | T | T |
| pLMNInfoList | M | T | T | F | T |
| CNSliceSubnetProfile | CM | T | T | F | T |
| RANSliceSubnetProfile | CM | T | T | F | T |
| TopSliceSubnetProfile | CM | T | T | F | T |

The procedure that network slice provisioning MnS producer follows to generate “pLMNInfoList” attribute is as follows:

- The MnS producer takes the “SST” value from input ServiceProfile, and optionally indexes a “SD” value to create a S-NSSAI.

- The MnS producer appends the created S-NSSAI to one (or more) PLMN ID, resulting into one (or more) “PLMNInfo”.

The procedure above assumes that the input ServiceProfile includes “sST” value. However, with the change #2 (section 3.3.2), it is possible that the ServiceProfile  does not include “sST” value. In such a situation, the network slice provisioning MnS producer needs to have the ability to ascertain the SST value based on the rest of service requirements provided in ServiceProfile . This represents a new capability on MnS producer side, and therefore on 3GPP management system.

It is proposed to add a new requirement in TS 28.531 [4] to reflect on this new capability for the network slice MnS producer.

## 3.4 	Advantages of the proposed solution

- For a CSP system (e.g., Service Orchestrator, BSS) that wants to request the fulfilment of service requirements in a 5G network, there is no need for this system to have knowledge on network slicing.  The CSP system only needs to focus on issuing service requirements, without the need to include information (e.g., SST attribute in Figure 2) on how they need to get fulfilled; how these requirements are fulfilled with which network slice attributes are left to 3GPP management system.

- 3GPP management system can attract more consumers. The current portfolio (slicing-aware consumers) can be extended with new consumers that do not understand slicing details.

# 4	Detailed proposal

The group is asked to:

- Agree on the issues listed in clause 3.2.1, and problem statement reported in clause 3.2.2.

- Endorse the changes #1-#3 specified in clause 3.3, and agree on the Rel19 CR 28.541 addressing them.

- Endorse the change #4 in clause 3.3, and agree on the Rel19 CR 28.531 addressing them.