3GPP TSG-SA5 Meeting #160	S5-251753

# Stor-Göteborg, Sweden, 7th Apr 2025 - 11th Apr 2025

# Source:	Ericsson

# Title:	Rel-19 pCR TS 28.572 Plan Activation Results

# Document for:	Approval

Agenda Item:	6.19.9.1 - Management of planned configurations

# 1	Decision/action requested

The group is requested to discuss and approve the pCR below.

# 2	References

[1]		3GPP TS 28.572: "Management of planned configurations"

# 3	Rationale

None.

# 4	Detailed proposal

The following changes are proposed for TS 28.572[1].

Begin of modifications

### 6.6.3	Activation process

Activation is the process of applying the updates specified by the operations contained in a planned configuration to the managed system. After the managed system is updated, the updates are reflected in the current configuration. Note that the system may not apply the operations to the managed system exactly as specified. For example, to change the configuration of a function, the function may have to be locked. Locking operations may not be included in the planned configuration and therefore the configuration update operation is augmented with locking and unlocking operations.

In atomic mode, the system may have to roll back to the current configuration that was effective when the activation process started. Therefore, the system needs to take and store a (conceptual) snapshot of the impacted part of the current configuration upon starting the activation process.

If not in atomic mode, the activation process may stop or continue on occurrence of error(s).

The activation process may run for quite some time, for example when large configurations are downloaded to the Network Functions as part of the activation process.

The planned configuration to be activated should be validated first.

Problems related to the activation of a planned configuration shall be reported.

Next modification

## 6.6	Activation of planned configurations

### 6.6.1	Definition

Activation is the process of applying a planned configuration to the managed system.

### 6.6.2	Potential problems

A planned configuration is validated before activation. Therefore a planned configuration that is activated should not have any of the problems that are discovered by validation. Only valid or partially valid planned configurations or planned configuration groups should be activated. Therefore, other types of problems are encountered during activation. These include

- A Network Function cannot be updated, because it is not reachable.

- A Network Function cannot be updated, because it is locked.

- A Network Function cannot be updated, because it does not exist.

- A Network Function cannot be updated, because it lacks necessary resources.

- A Network Function cannot be updated, because necessary access rights are not in place.

### 6.6.3	Activation process

Activation is the process of applying the updates specified by the operations contained in a planned configuration to the managed system. After the managed system is updated, the updates are reflected in the current configuration. Note that the system may not apply the operations to the managed system exactly as specified. For example, to change the configuration of a function, the function may have to be locked. Locking operations may not be included in the planned configuration and therefore the configuration update operation is augmented with locking and unlocking operations.

In atomic mode, the system may have to roll back to the current configuration that was effective when the activation process started. Therefore, the system needs to take and store a (conceptual) snapshot of the impacted part of the current configuration upon starting the activation process.

If not in atomic mode, the activation process may stop or continue on occurrence of the first error.

The activation process may run for quite some time, for example when large configurations are downloaded to the Network Functions as part of the activation process.

The planned configuration to be activated should be validated first.

Problems related to the activation of a planned configuration shall be reported.

### 6.6.4	Activation of planned configuration groups

The planned configuration group members are activated in the order specified. If no order is specified, they can be activated in any order. Each member is activated as specified in the previous clause. The activation of atomic members concludes with either activated or failed. The activation of best effort members concludes with either activated, partially activated or failed.

An atomic group is considered as activated, if atomic members can be activated, and best effort members can be activated or partially activated. If the activation of one (atomic or best effort) member fails, the activation of the complete group shall fail (and all updates must be undone).

A best effort group is considered as activated, if atomic members can be activated, and best effort members can be activated or partially activated. If the activation of at least one (atomic or best effort) member fails, the group is considered as partly activated. If the activation of all members fails, the activation of the complete group shall fail (and all updates must be undone).

### 6.6.5	Activation control parameters

Editor's note: Explain "enable-fallback", "service-impact and "immediate-activation". Also "cancel-request".

The MnS Consumer may configure different activation control parameter options to suit their wanted/preferred activation execution behavior.  These options shall include the following :

| Activation Option | Description |
| --- | --- |
| Enable Fallback | This option offers the MnS consumer the ability to undo/revert the current configuration of the managed systems to their pre-activation configuration state for the resources applied during the plan configuration activation. |
| Service Impact | This option offers the MnS producer to choose if they want to apply the changes such that it has the least possibility of impacting service or whether they want to execute in a way that favours applying the changes as quickly as possible to the network. |
| Immediate Activation | Normally when an MnS Consumer requests a plan configuration activation the associated plan configuration changes are applied immediately to the related managed systems. However, this option offers the MnS consumer the ability to suspend the activation process and defer applying the plan configuration changes to the current configuration of managed systems until some external event triggers the activation process to resume and apply the plan configuration changes to the managed systems. The same activation plan configuration may be triggered to resume it activation process multiple times. |
| Cancel Request | A plan configuration that is in the process of being activated may be cancelled at any point prior to finishing the activation. A cancel shall trigger the following behaviors : - The activation state shall reflect the cancel request and the process of applying the changes to the managed systems is halted for the remaining unapplied configuration plan changes - Plan configurations with atomic apply option selected : The current configuration of the managed systems shall be reverted to their state prior to activation for the network resources referenced in the plan configuration - Plan configurations with best effort apply option selected : The current configuration of the managed systems shall reflect only the plan configuration changes up to the point of the cancellation |

### 6.6.6 Activation Reporting

During the activation process the configuration changes are written to the current configuration of the managed systems.  These changes will normally be applied successfully.  In some cases, the activation plan configuration may contain a very large number of changes that may take some time to apply to the managed systems. An MnS Consumer should have the ability to track the progress of a long running activation with reporting on how many plan configuration changes are applied, how many failed to apply and how many are remaining to be applied.  The ongoing recording of these configuration change result details will help the MnS Consumer plan their network operations in a more efficient manner.

Figure 1 Plan Configuration Results

In some cases, the plan configurations changes will fail to be applied successfully for various reasons.  It is important for the MnS Consumer to know what changes were applied successfully and which were not.  Failure reasons with supporting error information shall be made available to the MnS consumer to allow them to troubleshoot the issues and take corrective action.  A result report detailing the status of each of the plan configuration changes shall

Next modification

## 7.6	ActivationJob

### 7.6.1	Definition

This definition represents an activation job.

### 7.6.2	Information Elements

The following table specifies the information elements of an activation job.

| Information element name | S | Documentation and Allowed Values | Properties |
| --- | --- | --- | --- |
| id | M | The identifier of the activation job. | type: String multiplicity: 1 isInvariant: True isWritable: False |
| name | M | The name of the activation job. | type: String multiplicity: 0..1 isInvariant: False isWritable: True |
| description. | M | The textual human-readable description of the activation job. | type: String multiplicity: 0..1 isInvariant: False isWritable: True |
| planConfigDescrId | M | The identifier of the planned configuration descriptor, whose operation set is requested to be activated. | type: String multiplicity: 0..1 isInvariant: True isWritable: True |
| planConfigGroupDescrId | M | or, alternatively, the identifier of the planned configuration group descriptor, whose operation sets are requested to be activated. | type: String multiplicity: 0..1 isInvariant: True isWritable: True |
| fallbackConfigDescrId | M | or, alternatively, the identifier of the fallback configuration descriptor, whose operation sets are requested to be activated. | type: String multiplicity: 0..1 isInvariant: True isWritable: True |
| planConfigDescr | M | or, alternatively, the planned configuration descriptor to be activated, if no planned configuration descriptor was created earlier. | type: String multiplicity: 0..1 isInvariant: True isWritable: True |
| planConfigGroupDescr | M | or, alternatively, the planned configuration group descriptor to be activated, if no planned configuration group descriptor was created earlier. | type: String multiplicity: 0..1 isInvariant: True isWritable: True |
| isEnableFallback | M | This boolean attribute allows to specify if the MnS producer shall create a plan with the current configuration to allow for a later fallback. Default value is "False", e.g. no fallback is enabled. The plan-id of the fallback plan shall be returned to the MnS consumer. | type: Boolean multiplicity: 1 isInvariant: True isWritable: True |
| serviceImpact | O | The service impact mode. allowedValues: - LEAST_SERVICE_IMPACT - SHORTEST_TIME | type: ENUM multiplicity: 1 isInvariant: True isWritable: True |
| isImmediateActivation | O | This Boolean attribute specifies if the activation job shall start immediately or, alternatively, by conditional activation. | type: Boolean multiplicity: 1 isInvariant: True isWritable: True |
| cancelRequest | M | This boolean attribute allows to request to cancel the activation process. | type: Boolean multiplicity: 1 isInvariant: False isWritable: True |
| state | M | The activation job state. allowedValues: - NOT_STARTED - RUNNING - CANCELLED - CANCELLING - COMPLETED - FAILED Editor's note: It is ffs if more detailed state and progress info needs to be added | type: ENUM multiplicity: 1 isInvariant: False isWritable: False |
| startedAt | M | The date and time at which the activation process started, i.e. the time when the job state transition from "NOT_STARTED" to "RUNNING" occurred. In the "NOT_STARTED" state the information element is absent or carries no information. | type: DateTime multiplicity: 0..1 isInvariant: False isWritable: FalseType: DateTime multiplicity: 0..1 isOrdered: NA isUnique: NA |
| stoppedAt | M | The date and time at which the activation process stopped, i.e. the job state transition from "RUNNING" to "COMPLETED" or "FAILED", or from "CANCELLING" to "CANCELLED". In the "NOT_STARTED", "RUNNING" or "CANCELLING" state the information element is absent or carries no information. | type: DateTime multiplicity: 0..1 isInvariant: False isWritable: False |
| result | M | The activation result, either " FINISHED_SUCCESSFUL " or " FINISHED_FAILED ". When the activated operation set is non-atomic or the activated planned configuration group is non-atomic, also " FINISHED_PARTIALLY_SUCCESSFUL ". allowedValues: FINISHED_SUCCESSFUL FINISHED_FAILED FINISHED_FAILED_ROLLBACK FINISHED_FAILED_ROLLBACK_FAILED FINISHED_PARTIALLY_SUCCESSFUL NOT_AVAILABLE | type: ENUM multiplicity: 0..1 isInvariant: False isWritable: False |
| errors | M | In case activation job failed Editor's note: This parameter is ffs | type: multiplicity: 0..1 isInvariant: False isWritable: False |
| resultDetails | M | The detailed result information for the individual configuration changes | type: ResultDetails multiplicity: 1 isInvariant: False isWritable: False |

### 7.6.3 Data Types

##### 7.6.3.3 Data type: PlanConfiguration

The PlanConfiguration contains the attributes defined in table 7.6.3.3-1.

Table 7.6.3.3-1: Definition of type PlanConfiguration

| Attribute Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| patch-id | string | M | 1 | Id of the configuration changes patch |
| comment | string | O | 1 | A comment to describe this set of configuration changes |
| edit | array(Edit) | M | 1..N | A set of configuration change edits |

##### 7.6.3.4 Data type: Edit

The Edit contains the attributes defined in table 7.6.3.4-1.

Table 7.6.3.4-1: Definition of type Edit

| Attribute Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| edit-id | string | M | 1 | A unique identifier for the change edit in the plan configuration patch/descriptor |
| operation | Operation | M | 1 | The configuration change operation |
| comment | String | O | 1 | Comment/description of the operation |
| target | string | M | 1 | As per section 2.4 of [1] but with the relaxed requirement to allow for the omission of the module prefix. The server shall accept this relaxed value if it determines the overall target is unambiguous. |
| value | object | O | 0..1 | Encoded according to [2] but with the relaxed requirement to allow for the omission of the module prefix. The server shall accept this relaxed value if it determines the overall value is unambiguous. |
| customProperties | Map(KeyValueObject) | O | 1 | Optional custom properties provided by the MnS consumer. |

##### 7.6.3.5 Data type: PlanActivationJob

The PlanActivationJob contains the attributes defined in table 7.6.3.5-1.

Table 7.6.3.5-1: Definition of type PlanActivationJob

| Attribute Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| id | String | M | 1 | id |
| name | String | O | 0..1 | The name |
| description | String | O | 0..1 | The description |
| planDescriptor | Link* | M | 1 | Link to the Plan Configuration Descriptor passed in the request |
| status | Link* | M | 1..N | Link to the Plan Activation Job status. A link to the status information |
| resultDetails | Link* | M | 1 | Link to the Plan Activation Job result detail information |
| isFallbackEnabled | boolean | M | 1 | Fallback enabled option |
| _links | Links | M | 1 | Links to properties associated with the plan activation job |

##### 7.6.3.6 Data type: PlanActivationStatus

Table 7.6.3.6-1: Definition of type PlanActivationStatus

| Attribute Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| state | JobState | M | 1 | The current lifecycle state of the plan activation job. |
| result | ResultState | M | 1 | The current overall result of the activation of the Plan Configuration Descriptor |
| startedAt | DateTime | M | 1 | The time the activation execution started its execution |
| stoppedAt | DateTime | CM | 1 | The time the activation execution completed its execution |
| cancelRequest | boolean | M | 0..1 | If set to true the job will be cancelled. Default value is 'false'. The consumer can cancel a job by setting this attribute from false to true after the job is created. |

##### 7.6.3.7 Data type: Links

The Links contains the attributes defined in table 7.6.3.7-1.

Table 7.6.3.7-1: Definition of type Links

| Attribute Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| self | Link* | M | 1 | Link to the Job resource itself |
| fallback | Link* | CM | 1 | Link to the fallback plan descriptor. Only available if isFallBackEnabled=true in the request |

* Link defined in 3GPP specifications

##### 7.6.3.8 Data type: ResultDetails

The ResultDetails contains the attributes defined in table 7.6.3.8-1.  This type is based on the 'yang-patch-status' type defined in RFC 8072 but with the following extensions/deviations :

Extension :

- 'summary'  : New property to show a results summary/overview of the edit/operation execution status

Deviations :

- error-path : modified to follow URI path formatting as per clause 3.5.3 of RFC 8040

Table 7.6.3.8-1: Definition of type ResultDetails

| Attribute Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| patch-id | String | M | 1 | Id of the patch configuration |
| edit-status | EditStatus | M | 1 | The status of the config edit operations |
| errors | Errors | CM | 1 | Global error associated with the overall configuration activation request. Included if there are global failures associated with the activation of the plan configuration. Defined as per definition in section 7 of RFC 8040 |
| summary | ResultSummary | M | 1 | Summary result information |

##### 7.6.3.9 Data type: ResultSummary

The ResultSummary contains the attributes defined in table 7.6.3.9-1.

Table 7.6.3.9-1: Definition of type ResultSummary

| Attribute Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| total | integer | M | 1 | Total number of operations (edits) in the plan activation job |
| completed | integer | M | 1 | Total number of executed operations (edits) in the plan activation job |
| succeeded | integer | M | 1 | Total number of successfully executed operations (edits) in the plan activation job |
| failed | integer | M | 1 | Total number of failed operations (edits) in the plan activation job |

##### 7.6.3.10 Data type: EditStatus

The EditStatus contains the attributes defined in table 7.6.3.10-1.

Table 7.6.3.10-1: Definition of type EditStatus

| Attribute Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| edit | EditResult | M | 1..N | Array of edit operation results. Only the failed edits are returned by default, if 'expand=all parameter is passed then ALL edit statuses are returned |

##### 7.6.3.11 Data type: EditResult

The EditResult contains the attributes defined in table 7.6.3.11-1.

Table 7.6.3.11-1: Definition of type EditResult

| Attribute Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| edit-id | String | M | 1 | Unique edit id matching the edit-id in the request |
| errors | Errors | O | 0..1 | As per definition in section 7 of RFC 8040 (see [5]) |
| state | EditState | M | 1 | Activation state of the individual edit. Default : NOT_STARTED |

##### 7.6.3.12 Data type: CancelJobRequest

The CancelJobRequest contains the attributes defined in table 7.6.3.12-1.

Table 7.6.3.12-1: Definition of type CancelJobRequest

| Attribute Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| isCancelled | boolean | M | 1 | Can only be modified to 'true' from an initial value of 'false'. Otherwise a HTTP 409 conflict error is thrown |

#### 7.6.3.13 Enumeration: ApplyMode

Table 7.6.3.13-1: Enumeration type of ApplyMode

| Enumerations Value | Description |
| --- | --- |
| BEST_EFFORT | Attempt to apply all the changes to the network regardless of config change apply succeeds or fails |
| ATOMIC | Apply all the changes to the network. In case of any failure revert to original configuration state |
| EXIT_ON_ERROR | Stop activation on encountering first error |

#### 7.6.3.14 Enumeration: Operation

Table 7.6.3.14-1: Enumeration type of Operation

| Enumerations Value | Description |
| --- | --- |
| create | As defined in section 2.5 of [1] |
| remove | As defined in section 2.5 of [1] |
| merge | As defined in section 2.5 of [1] |
| action | Supports action execution as per RFC 7950 |

#### 7.6.3.15 Enumeration: JobState

This enum reflects the overall execution state of the job.

Table 7.6.3.15-1: Enumeration type of JobState

| Enumerations Value | Description |
| --- | --- |
| NOT_STARTED | Plan Activation Job Created |
| SCHEDULED | Plan Activation Job Scheduled |
| RUNNING | Plan Activation Job RUNNING |
| COMPLETED | Plan Activation Job COMPLETED |
| FAILED | Plan Activation Job FAILED |
| CANCELLING | Plan Activation Job CANCELLING |
| CANCELLED | Plan Activation Job CANCELLED |

#### 7.6.3.16 Enumeration: ResultState

This enum reflects the overall status of the activation of the plan configuration(s) toward the network

Table 7.6.3.16-1: Enumeration type of ResultState

| Enumerations Value | Description |
| --- | --- |
| FINISHED_SUCCESSFUL | All requested config changes were successfully applied to the network |
| FINISHED_FAILED | The activation failed and no config changes were successfully applied |
| FINISHED_FAILED_ROLLBACK | The activation failed and an automatic rollback to original state was applied (reported with ATOMIC applyMode) |
| FINISHED_PARTIALLY_SUCCESSFUL | Some of the operations in the requested configuration failed to be successfully applied to the network |
| NOT_AVAILABLE | Results are not yet available |

#### 7.6.3.17 Enumeration: EditState

This enum reflects the activation state of an individual edit within a plan configuration

Table 7.6.3.17-1: Enumeration type of EditState

| Enumerations Value | Description |
| --- | --- |
| SUCCEEDED | All requested network changes were successfully activated and applied to the network |
| FAILED | The activation failed and no (automatic) rollback was performed |
| NOT_STARTED | The activation failed and an automatic rollback to original state was applied (reported with with ATOMIC applyMode) |

#### 7.6.3.18 Media Types

Media types are described in this section

##### 7.6.3.18.1 Media Type application/3gpp-yang-patch+json

Table 7.6.3.18.1-1: application/3gpp-yang-patch+json Media Type

| Media Type | Description |
| --- | --- |
| application/3gpp-yang-patch+json | Extension to the media type application/yang-patch+json (see section 4.4.2 of [1]) with the following additions/deviations - 'target' and 'value' definitions are relaxed to allow for the omission of the module prefix. The producer (server) shall accept the target and value parameters without the prefix if it can determine that the resources are uniquely identifiable and are unambiguous. Otherwise, a Conflict error shall be reported - 'edit' shall support 'comment’ and 'customProperties' fields to support more verbose operation tracking and logging. - For the 'operation' field as defined in [1], only 'create', 'remove' and 'merge' operations are supported and follow the definition as laid out in [1]. |

End of modifications