3GPP TSG-SA5 Meeting #160	S5-251796

# Goteborg, Sweden, 7th Apr 2025 - 11th Apr 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 28.104 | CR | 0198 | rev | 1 | Current version: | 19.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Rel19 CR TS28.104 add use case and requirement on correlation analytics for NF Scaling and dimensioning |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia, Nokia Shanghai Bell, Orange |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA5 |  |  |  |  |  |  |  |  |  |
| Work item code: | eMDAS_Ph3 |  |  |  |  |  | Date: |  |  | 2025-03-28 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | The use case of “Measurement data correlation analytics for NF Scaling and dimensioning” has been discussed and agreed upon in the study (clause 5.3.6 in TR 28.866 and clause 7.2.9.2 in TS28.104). This CR is to normatively propose the solution. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Adding solution for the use case description and requirements |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | No solution for the use case. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.4.x.2, 8.5.1.2, 8.6.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  | X |  | O&M Specifications |  |  |  | TS28.104 CR0197 |  |  |
| Other comments: |  | The new clause 8.4.x.2 is to be after 8.4.x.1 (new clause 8.4.x is from CR0197) |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | Revision of S5-251441 |  |  |  |  |  |  |  |  |

Start of First change

#### 8.4.x.2	Analytics for NF Scaling and dimensioning

##### 8.4.x.2.1	MDA type

The MDA type for NF Scaling and dimensioning analysis is: CorrelationAnalytics.NFScalingDimensioningDataAnalysis.

##### 8.4.x.2.2	Enabling data

The enabling data for CorrelationAnalytics.NFScalingDimensioningDataAnalysis is provided in table 8.4.x.2.2-1.

For general information about enabling data, see clause 8.2.1.

Table 8.4.x.2.2-1: Enabling data for correlation analysis for NF Scaling and dimensioning

| Data category | Description | References |
| --- | --- | --- |
| Performance measurements | The performance metric or KPI data for management data correlation analysis for NF Scaling and dimensioning. | The performance metric data defined in TS 28.552 [4] And KPI data defined in TS 28.554 [5] |
| Geographical data | The geographical information (longitude, latitude, altitude) of the deployed 5GC Core NF. | The geographical information (longitude, latitude, altitude) information (it may be provided in the peeParametersList attribute of the ManagedFunction IOC in TS 28.622 [19]). |
| Configuration data | The NRMs of the 5GC NF, Network Slice, SliceSub Network | The 5GC Functions, slice related NRM defined in TS 28.541 [15]. |

##### 8.4.x.2.3	Analytics output

The specific information elements of the analytics output for NF Scaling and dimensioning analysis, in addition to the common information elements of the analytics outputs (see clause 8.3), are provided in table 8.4.x.2.3-1.

Table 8.4.x.2.3-1: Analytics output for NF Scaling and dimensioning analysis

| Information element | Definition | Support qualifier | Properties |
| --- | --- | --- | --- |
| recommendedActions | The recommended actions to change the resource allocation for NFs. The recommended action may be (but not limited to): - scale in a list of NFs; - scale out a list of NFs. | M | type: RecommendedAction multiplicity: * isOrdered: False isUnique: True defaultValue: None isNullable: False |

Next change

#### 8.5.1.2	Information elements

Table 8.5.1.2-1

| Name | Definition | Support qualifier | Properties |
| --- | --- | --- | --- |
| recommended3GPPActions | It contains the recommendations actions concerning 3GPP defined operations on MOIs. The order of the list elements indicates the recommended order that the actions should be performed. | O | type: Recommended3GPPAction multiplicity: * isOrdered: True isUnique: True defaultValue: None isNullable: False |
| recommendedNon3GPPActions | It contains the recommended actions related to non-3GPP operations for 3GPP management system to interact with non-3GPP management system. The order of the list elements indicates the recommended order that the actions should be performed. | O | type: String multiplicity: * isOrdered: True isUnique: True defaultValue: None isNullable: False |
| recommendedHumanReadableActions | It contains the recommendations on human readable actions. The order of the list elements indicates the recommended order that the human readable actions should be performed. NOTE: Further details of recommended human readable actions are not specified. | O | type: String multiplicity: * isOrdered: True isUnique: True defaultValue: None isNullable: False |
| actionInterval | It indicates the interval of the order of operations, for example, it may indicate the interval of the order of NF scaling operations. The unit is second. allowedValue: none zero Integer | O | type: Integer multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| timeWindow | It indicate the time window for the recommended operation. The order of the list elements indicates the recommended order that the actions should be performed. allowedValue: N/A | O | type: TimeWindow multiplicity: * isOrdered: True isUnique: False defaultValue: None isNullable: False |

Next change

### 8.6.1	MDAType <<enumeration>>

Table 8.6.1-1: <<enumeration>> MDAType

| Enumeration value | Description |
| --- | --- |
| "COVERAGEANALYTICS_COVERAGEPROBLEMANALYSIS" | Indicates that the MDA type for the Coverage problem analysis defined in clause 8.4.1.1 |
| "COVERAGEANALYTICS_PAGINGOPTIMIZATION" | Indicates that the MDA type for the Paging Optimization defined in clause 8.4.1.2 |
| "SLSANALYSIS_SERVICEEXPERIENCEANALYSIS" | Indicates that the MDA type for the Service experience analysis defined in clause 8.4.2.1 |
| "SLSANALYSIS_NETWORKSLICETHROUGHPUTANALYSIS" | Indicates that the MDA type for the Network slice throughput analysis defined in clause 8.4.2.2 |
| "SLSANALYSIS_NETWORKSLICETRAFFICANALYSIS" | Indicates that the MDA type for the Network slice traffic prediction defined in clause 8.4.2.3 |
| "SLSANALYSIS_E2ELATENCYANALYSIS" | Indicates that the MDA type for the E2E latency analysis defined in clause 8.4.2.4 |
| "SLSANALYSIS_NETWORKSLICELOADANALYSIS" | Indicates that the MDA type for the Network slice load analysis defined in clause 8.4.2.5 |
| "MDAASSISTEDFAULTMANAGEMENT_FAILUREPREDICTION" | Indicates that the MDA type for the MDA assisted failure prediction defined in clause 8.4.3.1 |
| "MDAASSISTEDENERGYSAVING_ENERGYSAVINGANALYSIS" | Indicates that the MDA type for the Energy saving analysis defined in clause 8.4.4.1 |
| "MOBILITYMANAGEMENTANALYTICS_MOBILITYPERFORMANCEANALYSIS" | Indicates that the MDA type for the Mobility performance analysis defined in clause 8.4.5.1 |
| "MOBILITYMANAGEMENTANALYTICS_HANDOVEROPTIMIZATION" | Indicates that the MDA type for the Handover Optimization analysis defined in clause 8.4.5.2 |
| "MAINTENANCE_MAINTENANCEANALYTICS" | Indicates that the MDA type for the Maintenance.MaintenanceAnalytics defined in clause 8.4.6.1 |
| "RESOURCEANALYTICS_VIRTUALIZEDRESOURCEUTILIZATIONANALYSISNF" | Indicates that the MDA type for the Virtualized resource utilization analysis defined in clause 8.4.7.1.1 |
| "RESOURCEANALYTICS_PHYISCALRESOURCEUTILIZATIONANALYSISNF" | Indicates that the MDA type for the Physical resource utilization analysis defined in clause 8.4.7.1.2 |
| "RESOURCEANALYTICS_5GCCONTROLPLANECONGESTIONANALYSIS" | Indicates that the MDA type for the 5GC Control plane congestion analysis defined in clause 8.4.7.1.3 |
| "PREDICTIONS_PMDATA" | Indicates that the MDA type for the MDA assisted PM predictions defined in clause 8.4.8.1 |
| "UETHROUGHPUTANALYTICS_UETHROUGHPUTPROBLEMANALYSIS" | Indicates that the MDA type for the UE throughput analysis defined in clause 8.4.2.6 |
| “CORRELATIONANALYTICS_NFSCALINGDIMENSIONINGDATAANALYSIS” | Indicates that the MDA type for the NF Scaling and dimensioning analysis defined in clause 8.4.x.2 |

End of First change