3GPP TSG-SA5 Meeting #160	S5-251947

Goteborg, Sweden, 07 - 11 April 2025

Source:	Huawei, Nokia, Samsung, AT&T, Ericsson

Title:	Pseudo-CR Add concepts content

Document for:	Approval

Agenda item:	6.19.21.1

Spec:	3GPP TS 28.579

Version:	0.1.0

Work Item:	MExpo

Comments

This contribution proposes to add content for concepts of draft TS 28.579.

Proposed Changes

* * * First Change * * * *

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]                       3GPP TS 23.222: "Common API Framework for 3GPP Northbound APIs".

[X]	3GPP TS 28.533: "Management and orchestration; Architecture Framework".

…

[x]	<doctype> <#>[ ([up to and including]{yyyy[-mm]|V<a[.b[.c]]>}[onwards])]: "<Title>".

* * * Next Change * * * *

# 4 Concepts and overview

## 4.1	Exposure of management services

### 4.1.1	Overview

Common API Framework (CAPIF) is a framework comprising common API aspects (e.g., publishing, discovery, access control) that are required to support service APIs (see TS 23.222 [2]). Service APIs are produced by API provider domain and consumed by API invokers. An external MnS consumer consuming management services using CAPIF is equivalent to an API invoker consuming the CAPIF-1e and CAPIF-2e interfaces (i.e. API invoker outside PLMN operator trust domain (see NOTE1)).

NOTE 1: the concept of the API invoker is defined in TS 23.222 [2].

A management service (MnS) is identified by different component types, i.e. MnS component type A (management operations and/or notifications), MnS component type B (managed objects), and MnS component type C (performance and fault information) (as defined in clause 4.2 of 3GPP TS 28.533 [X]). Accordingly, in order for the external MnS consumer to consume management services through CAPIF, the appropriate access rights have to be configured for the external MnS consumer within the management system. These access rights determine which MnS resources (i.e. component type B and component type C) can be accessed. In addition, the access rights determine what operations (i.e., MnS component A) can or cannot be performed on the MnS resources by the external MnS consumer.

To provide a MnS to be consumed by the external MnS consumer through CAPIF, first the network operator determines what MnS APIs will be published into CAPIF. Next, these MnS APIs are published into CAPIF as service APIs. The network operator needs to configure the required access control information within the CAPIF core function (CCF) to ensure that the external MnS consumers can only discover the MnS APIs that they can be able to successfully invoke within the management system. Upon completion of the above procedures, external MnS consumers can discover and invoke the MnS APIs that are exposed using CAPIF.

* * * End of Changes * * * *