3GPP TSG-SA5 Meeting #161 	S5-252263

Fukuoka City, Fukuoka, Japan, 19th – 23rd May 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 28.552 | CR | 0698 | rev | - | Current version: | 19.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | x | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Rel-19 CR TS 28.552 correction on paging record measurement |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S5 |  |  |  |  |  |  |  |  |  |
| Work item code: | PM_KPI_5G_Ph4 |  |  |  |  |  | Date: |  |  | 2025-05-01 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | When developing the functionality for inter gNB RAN paging, the descriptions are inconsistent between subclauses 5.1.1.27.7 and 5.1.1.27.8. Subclause 5.1.1.27.7 seems to be described from the target node side (where UE answers the page), while subclause 5.1.1.27.8 is described from the last serving gNB side (where DL data arrives). At inter-gNB RAN paging, it would not be possible to relate those counters to each other since they refer to different gNBs and cells. And since they show attempts and successes, they are intended to be used to calculate a success rate. To make that possible, subclause 5.1.1.27.7 should be redefined to also refer to the last serving gNB (i.e., where DL data arrives). It shall also be clarified further that each page attempt is only counted in one gNB and only once (even if a re-attempt is made). Since the cell where the paging is successful (if successful) could be an external cell to this gNB, it is better to peg the counter on node level. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Rewording on the paging measurements |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Inconsistent specification, which may lead to incorrect implementations. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.1.1.27.7, 5.1.1.27.8 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

*** START OF NEXT CHANGE ***

##### 5.1.1.27.7	Number of NG-RAN Initiated paging records sent by the gNB-CU

a)	This measurement provides number of NR RAN initiated paging records sent by the last serving gNB-CU.

b)	CC.

c)	Transmission of a RAN PAGING message by the gNB-CU on the F1 interface which is used to request the gNB-DU to page UEs, or transmission of a RAN PAGING message to another gNB via the Xn interface (3GPP TS 38.423[13]). The IE in "CHOICE Paging Identity" of this message is "RAN UE Paging identity". (3GPP TS 38.473[6]).
Multiple paging records for the same UE are counted as one.

d)	A single integer value.

e)	PAG.SentNbrRanIntiated.

f)	GNBCUCPFunction.

g)	Valid for packet switched traffic.

h)	5GS.

*** START OF NEXT CHANGE ***

##### 5.1.1.27.8	Number of successful NG-RAN Initiated paging sent by the gNB-CU

a)	This measurement provides number of successful NR RAN Initiated paging response received by the last serving gNB-CU.

b)	CC.

c)	On reception of RRC RESUME Request (RRCResumeRequest or RRCResumeRequest1) message from the UE, or reception of RETRIEVE UE CONTEXT REQUEST message (which carries the I-RNTI of the UE and the MAC-I verification passed) from another gNB through the Xn interface (3GPP TS 38.423[13]) which indicating a successful NR RAN Initiated paging when this gNB-CU was the last serving gNB, the counter is stepped by 1.

d)	A single integer value.

e)	PAG.SentNbrRanIntiatedSucc.

f)	GNBCUCPFunction.

g)	Valid for packet switched traffic.

h)	5GS.

*** END OF CHANGE ***