3GPP TSG-SA5 Meeting #161	S5-252319

# Fukuoka, JP, 19 - 23 May 2025

Title:	Reply to LS on clarification on UE performance measurement at PDCP level

Response to:	LS on clarification on UE performance measurement at PDCP level (R3-252436)

Release:	Rel-19

Work Item:	NR_AIML_NGRAN_enh-Core

Source:	SA5

To:	RAN3

Cc:	-

Contact person:	Zu Qiang
Zu.Qiang@ericsson.com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments: -

# 1	Overall description

SA5 thanks RAN3 for the incoming LS on clarification on UE performance measurement at PDCP level.

RAN3 asking: According to TS 28.552, RAN3 thinks that the DL UE Buffered Throughput per DRB, as defined in clause 5.1.1.3.7, can be used to measure DL UE throughput at the PDCP level by deriving it from the transmitted PDCP SDU volume over a specific DDDS period. Similarly, RAN3 thinks that a similar metric, such as the received PDCP PDU volume over a certain period, can be used to measure UL UE throughput at the PDCP level.

RAN3 would like to seek confirmation from SA5 on following:

- Whether it is feasible to reuse DL UE Buffered Throughput at the PDCP level for both split and non-split gNB scenario. If not, RAN3 would like to ask SA5 to define DL UE Throughput measurement at PDCP level for both split and non-split architecture.

SA5 responses: SA5 does not think it is feasible to reuse DL UE buffered throughput at PDCP level for both split and non-split gNB scenario. The current measurement is based on Desired buffer size communicated within DDDS from DU to CU UP and is intended for services with burst duration spanning over the time interval of a couple of consequent DDDSs. However, this measurement does not represent the total PDCP level throughput over the air for the UE in a precise way as the DDDS is not defined clearly enough and the frequency of sending the DDDS message is not enough to provide reliable throughput measurement.

- Whether it is feasible to measure the corresponding UL UE throughput measurement at PDCP level for both split and non-split architecture. If feasible, RAN3 would like to ask SA5 to specify this measurement for both split and non-split architecture.

SA5 responses: For the corresponding UL UE throughput measurement, there is no UL PDCP throughput measurement designed today. This is because that only the UE knows the amount of data at RLC level which needs to be sent. The DU could measure the RLC level measurement, but there is no way to translate it into PDCP level measurement. However, the existing RLC throughput measurements (e.g., UE throughput measurements defined in subclause 5.1.1.3 of TS 28.552) could be used to measure throughput for both UL and DL. Those RLC throughput measurements could provide more precise measurements than the ones at PDCP level.

- The feasibility of measuring Packet Loss Rate in UL and DL at PDCP level for both split and non-split architecture. And if it is feasible, RAN3 would like to ask SA5 to specify these measurements for both split and non-split architecture.

SA5 responses: the current measurements can be used for this purpose. Those measurements are defined in subclause 5.1.3.1.1 of TS 28.552 for UL PDCP SDU Loss Rate, 5.1.3.2.1 of TS 28.552 for DL PDCP SDU Drop rate in gNB-CU-UP.

# 2	Actions

To RAN3

ACTION: 	SA5 respectfully asks RAN3 to take into account the information above.

# 3	Dates of next TSG SA WG 5 meetings

SA5#162		25 Aug – 29 Aug 2025		Goteborg, SE

SA5#163		13 Oct – 17 Oct 2025		China (TBC), CN