3GPP TSG-SA5 Meeting #161	S5-252402

# Fukuoka, Japan, 19 - 23 May 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 28.312 | CR | 0309 | rev | 2 | Current version: | 19.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Rel-19 CR TS 28.312 Add procedure for intent exploration |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ZTE Corporation |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA5 |  |  |  |  |  |  |  |  |  |
| Work item code: | IDMS_MN_Ph3 |  |  |  |  |  | Date: |  |  | 2025-05-08 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | - SA5#160: The use case, requirements and solution for intent exploration are documented in TS 28.312. However, the procedure for intent exploration is missing. It proposes to add procedure for intent exploration in clause 6.3 based on the procedure in TR 28.914. - SA5#161: - In SA5#161, S5-251917 is agreed to add the exploration procedure. But in S5-251918, the allowed value of “intentMgmtPurpose” is “EXPLORATION” instead of “EXPLORE”, the value should be aligned in the specification. - The description of step 1 is confusing, as the “intentMgmtPurpose” is also an attribute of Intent IOC with the “isWritable” property as True, i.e., “intentMgmtPurpose” is a part of the so-called “intent information”. But current wording implies they are different. - Since the multiplicity of targetExplorationResults is “1..*”, MnS consumer may further negotiate with the MnS Producer to determine the best value of the target(s) to be fulfilled when multiple best values of target(s) are provided in the exploration report. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - SA5#160: Add procedure for intent exploration in clause 6.3 - SA5#161: - Align the value of “intentMgmtPurpose” and rewording for clarification. - Clarification on the possible action of the MnS consumer when multiple best possible values are received. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Incosistent definition may lead to confusion. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.3.X(new), Annex.1.X(new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | - S5-251917 is the revision of S5-251275 - S5-252402 is the revision of S5-251917 |  |  |  |  |  |  |  |  |

1st Change

6.3.X	Intent exploration

Figure 6.3.X-1 illustrates the procedure for intent exploration for intent negotiation during intent pre-evaluation phase.

Figure 6.3.X-1: Procedure for intent exploration for intent negotiation during intent pre-evaluation phase

1.	MnS Consumer sends a request to create an intent instance for intent exploration (see createMOI operation defined in TS 28.532 [3]) to MnS Producer, providing intent information for the new intent to be created. The detailed intent information sees attributes (attribute which "isWritable" property is "True") of the concrete intent IOC defined in clause 6.2. In the provided intent information, the intentMgmtPurpose is specified as "EXPLORATION" and the value of specific intent targets or contexts that need to be explored is initially set to 'NULL', indicating that the MnS consumer requests the MnS Producer to send the intent exploration report.

2.	Based on the received request, the MnS Producer creates the concrete intent instance (i.e., instance of intent IOC) and configure the new created intent MOI with the received intent information.

3.	MnS Producer sends a response (see createMOI operation defined in TS 28.532[3]) to the MnS Consumer with attribute "objectInstance" of the created intent instance.

4.	Based on the created intent instance with intentMgmtPurpose as " EXPLORATION ", MnS Producer performs the exploration process to evaluate and determine the best possible values for the specified targets and/or contexts, considering current resource situation and capabilities of the system. The intent instance will not impact the network as the MnS producer will not take any action on the network based on the intent exploration request.

5.	MnS Producer should notify (see notifyMOIAttributeValueChanges notification) MnS Consumer about attribute "objectInstance" of intent report instance and corresponding intent exploration report information that contains the best possible values  (targetExplorationResults and/or contextExplorationResults) which are required by the MnS Consumer. Then, MnS consumer can further negotiate with the MnS Producer based on the intent exploration report to determine the best value of the target(s) to be fulfilled.

Next Change

A.1.X	Intent exploration

@startuml

title "[Intent Exploration]"

actor "MnS Consumer" as MnS_Consumer

participant "MnS Producer" as MnS_Producer

Collections "NE(s)" as ManagedEntity

MnS_Consumer -> MnS_Producer: 1. Request to create an intent instance (intentMgmtPurpose=EXPLORE, list of attributes of intent IOC, \n    value of specific intent target(s) and/or context(s) to be explored = 'NULL')

MnS_Producer -> MnS_Producer: 2. Create and configure intent MOI

MnS_Producer -> MnS_Consumer: 3. Response for create an intent instance

MnS_Producer -> MnS_Producer: 4. Perform the exploration for the best value of specified target(s) and/or context(s)

MnS_Producer -> MnS_Consumer: 5. Notify of intent exploration report (DN of IntentReport MOI, ExpectationExplorationResult )

hide footbox

@enduml