3GPP TSG-SA5 Meeting #161	S5-252666

Fukuoka, Japan, 19 - 23 May 2025

Source:	Nokia, Ericsson

Title:	Pseudo-CR on Rel-19 pCR TR 28.869 Add evaluation and conclusion for data streaming for cloud-native NF deployments

Document for:	Approval

Agenda item:	6.19.6

Spec:	3GPP TR 28.869

Version:	1.4.0

Work Item:	FS_Cloud_OAM

Comments

The pCR proposes to add the evaluation and conclusion for the data streaming of cloud-native NF deployment use case in clause 5.2.2 of TR 28.869

Proposed Changes

* * * First Change * * * *

#### 5.2.2.4	Evaluation of solutions

The proposed solution #1 in clause 5.2.2.3.1 implies major impacts on the 3GPP management system to support the streaming of management data based on message bus. Further, the proposed solution is not limited to NFs running in the cloud but would apply for all types of NFs. Accordingly, a detailed study is required in 5G-A Rel-20 SBMA to determine how the proposed mechanism could be supported.

The proposed solution #2 in clause 5.2.2.3.2 shows how management data can be streamed in cloud native deployments with minimal impact to the existing MnS producers. The proposed solution relies on existing SA5 defined mechanisms for streaming management data between the MnS producer and the MnS consumer and supports having a message bus beyond the MnS consumer. This approach brings in the benefits of using the message bus architecture (e.g., scalability, low latency, fault tolerance and high throughput) while mitigating major architectural impacts on the 3GPP management system. Accordingly, the proposed solution satisfies the use case requirements in clause 5.2.2.2 and no further normative work is required to support the proposed solution.

* * * Next Change * * * *

# 6 			Conclusions and recommendations

Editor's Note: This clause captures the conclusions and the recommendations of the study.

## 6.1	General conclusion

The present document mainly studied the following several categories:

-	In clause 4, the terminology and concepts has been studied.

-	In clause 5.1, the use cases are about the configuration, maintenance (i.e., upgrade and traffic management) and policy management for cloud-native VNFs. The related different solutions are based on existing functionalities provided by the 3GPP management system and/or the usage of VNF generic OAM functions/PaaS Services.

-	In clause 5.2.1, 5.2.3, 5.2.4, 5.2.5 and 5.2.6, the use cases are about the lifecycle management of NF Deployment. The related different solutions can include ETSI NFV-MANO, but are not limited to it.

-	In clause 5.2.2, the use cases are about the management data streaming for Network Functions in supporting cloud native.

-	In clause 5.2.7, the use case is about observability for Network Functions in supporting cloud native.

-	In clause 5.3, the use case is about the placement of Network Functions to support different cloud deployment scenarios while there are no solutions available for this use case in this document.

## 6.X	Data streaming for cloud-native NF deployments

It is recommended to further study solution #1 (i.e., data streaming solution based on message bus) presented in clause 5.2.2.3.1 with broader scope in R20 5G-A SBMA topic. The solution#2 presented in clause 5.2.2.3.2 is already supported and no further normative work is required.

* * * End of Changes * * * *