3GPP TSG-SA5 Meeting #161	S5-252853

Fukuoka, Japan, 19 - 23 May 2025

Source:	Samsung

Title:	Rel-19 pCR TS 28.567 Metrics for CCL performance monitoring

Document for:	Approval

Agenda item:	6.19.4.1

Spec:	3GPP TS 28.567

Version:	0.1.0

Work Item:	Closed Control Loop Management

Comments

This pCR is to add solution of metrics for CCL performance monitoring as was agreed in the CCL study in TR 28.867 and to support procedure defined in clause 7.1 of TS 28.567

Proposed Changes

* * * Start of Change * * * *

# 8	CCL Performance Metrics

The performance metrics to evaluate performance of a CCL for its optimal execution are defined in order to enable operators to track the effectiveness of closed loop automation, identify areas for improvement, and make informed adjustments to CCL functionalities.

## 8.1	Total number of occurrences of a goal breach:

a)	This measurement provides the total number of occurrences when a goal, as defined in CCL is breached during an observation time period i.e.granularityPeriod.

b)	CC.

c)	This is measured by counting each incidence when a goal is breached and incrementing the corresponding counter by one for each such occurrence within an observation time period i.e. granularityPeriod.

d)	An integer value.

e)	The measurement name has the form TotalGoalBreach.

f)	ClosedControlLoop

g)	Valid for packet switched traffic.

h)	5GS.

## 8.2	Time taken by CCL to meet a breached goal:

a)	This measurement provides the time taken by a CCL to meet a breached goal after activating it again.

b)	DER.

c)	This is measured by considering the time stamp when a goal is breached and subtracting it from the time stamp when that goal is met after activating the CCL with required changes.

d)	Each measurement is an integer representing the mean delay in milliseconds.

e)	The measurement name has the form TimeBreachedGoalRecovery.

f)	ClosedControlLoop

g)	Valid for packet switched traffic.

h)	5GS.

## 8.3	Total number of conflicts occurred by a CCL:

a)	This measurement provides the total number of conflicts that occur between a CCL under consideration and any other CCL during an observation time period i.e. granularityPeriod.

b)	CC.

c)	This is measured by counting each incidence when conflict occurs between a CCL under consideration and the other CCL and incrementing the corresponding counter by one for each such occurrence within an observation time period i.e. granularityPeriod

d)	An integer value.

e)	The measurement name has the form TotalCclConflicts_Filter, where filter is either Implicit or Explicit. Implicit represents the action conflict i.e. conflict between two existing CCL and target represents the explicit conflict i.e. conflict between an existing CCL and a requested CCL.

f)	ClosedControlLoop

g)	Valid for packet switched traffic.

h)	5GS.

* * * End of Changes * * * *