3GPP TSG-SA5 Meeting #161	S5-252925

Fukuoka, Japan, 19 - 23 May 2025

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 28.552 | CR | 0710 | rev | 1 | Current version: | 19.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Rel-19 CR TS 28.552 Add new filter for throughput measurements for RedCap service |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | China Unicom, ZTE |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S5 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_RedCap_OAM |  |  |  |  |  | Date: |  |  | 2025-04-27 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | Solution for the enhancement on the existing throughput measurements to support to monitoring the performance of RedCap service is investigated in clause 5.3.3 of TR 28.876 and the related normative work are recommended. This contribution proposes to add a new filter for throughput measurements to implement the above recommendation accordingly in TS 28.552. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add new filter for throughput measurements. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The requirements of throughput measurements for RedCap service cannot be met. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.1.1.3.1, 5.1.1.3.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

1st Change

5.1.1.3.1	Average DL UE throughput in gNB

a)	This measurement provides the average UE throughput in downlink. This measurement is intended for data bursts that are large enough to require transmissions to be split across multiple slots. The UE data volume refers to the total volume scheduled for each UE regardless if using only primary- or also supplemental aggregated carriers. The measurement is optionally split into subcounters per QoS level (mapped 5QI or QCI in EN-DC) and subcounters per supported S-NSSAI, and subcounters per PLMN ID, and subcounters per BWP, and is optionally filterable for UE type. In the case of per BWP, the UE data volume refers to the total volume scheduled for each Active BWP with same bandwith except UEs with activated supplemental aggregated carrier(s).

b)	DER(N=1)

c)	This measurement is obtained according to the following formula based on the "ThpVolDl" and "ThpTimeDl" defined below. Separate counters are maintained for each mapped 5QI (or QCI for EN-DC) and for each supported S-NSSAI, and for each PLMN ID , and for each Active BWP.

If UEsTℎpTimeDl>0, UEsTℎpVolDlUEsTℎpTimeDl×1000 [kbit/s]

If UEsTℎpTimeDl=0, 0 [kbit/s]

For small data bursts, where all buffered data is included in one initial HARQ transmission,  , otherwise

| ThpTimeDl | The time to transmit a data burst excluding the data transmitted in the slot when the buffer is emptied. A sample of "ThpTimeDl" for each time the DL buffer for one DataRadioBearer (DRB) is emptied. |
| --- | --- |
|  | The point in time after T2 when data up until the second last piece of data in the transmitted data burst which emptied the RLC SDU available for transmission for the particular DRB was successfully transmitted, as acknowledged by the UE. |
|  | The point in time when the first transmission begins after a RLC SDU becomes available for transmission, where previously no RLC SDUs were available for transmission for the particular DRB. |
|  | The RLC level volume of a data burst, excluding the data transmitted in the slot when the buffer is emptied. A sample for ThpVolDl is the data volume, counted on RLC SDU level, in kbit successfully transmitted (acknowledged by UE) in DL for one DRB during a sample of ThpTimeDl. (It shall exclude the volume of the last piece of data emptying the buffer). |

d)	Each measurement is a real value representing the throughput in kbit per second. The number of measurements is equal to one. If the optional QoS level subcounter and S-NSSAI subcounter and PLMN ID subcounter and BWP subcounter measurements are performed, the number of measurements is equal to the number of mapped 5QIs and the number of supported S-NSSAIs, and the number of PLMN IDs, and the number of Active BWPs.

e)	The measurement name has the form 
DRB.UEThpDl, or optionally DRB.UEThpDl.QOS, where QOS identifies the target quality of service class, and DRB.UEThpDl.SNSSAI, where SNSSAI identifies the S-NSSAI, and DRB.UEThpDl.PLMN, where PLMN identifies the PLMN ID, and DRB.UEThpDl.BWP, where BWP identifies the Active BWP, or DRB.UEThpDl_UEType, or DRB.UEThpDl.QOS_UEtype, or DRB.UEThpDl.SNSSAI_UEType, or DRB.UEThpDl.PLMN_UEType, or DRB.UEThpDl.BWP_UEType, where UEType refers to allowed values related to RedCap UE type of uECellBarredAccess in TS 28.541 clause 4.4.1.

f)	NRCellDU

g)	Valid for packet switched traffic

h)	5GS

i)	One usage of this measurement is for performance assurance within integrity area (user plane connection quality).

2nd Change

5.1.1.3.3	Average UL UE throughput in gNB

a)	This measurement provides the average UE throughput in uplink. This measurement is intended for data bursts that are large enough to require transmissions to be split across multiple slots. The UE data volume refers to the total volume scheduled for each UE regardless if using only primary- or also supplemental aggregated carriers. The measurement is optionally split into subcounters per QoS level (mapped 5QI or QCI in EN-DC) and subcounters per supported S-NSSAI, and subcounters per PLMN ID, and subcounters per BWP, and is optionally filterable for UE type. In the case of per BWP, the UE data volume refers to the total volume scheduled for each Active BWP with same bandwith except UEs with activated supplemental aggregated carrier(s).

B)	DER(N=1)

c)	This measurement is obtained according to the following formula based on the "ThpVolUl" and "ThpTimeUl" defined below. Separate counters are maintained for each mapped 5QI (or QCI for EN-DC) and for each supported S-NSSAI, and for each PLMN ID, and for each Active BWP.

If UEsTℎpTimeUl>0, UEsTℎpVolUlUEsTℎpTimeUl×1000 [kbit/s]

If UEsTℎpTimeUl=0, 0 [kbit/s]

For small data bursts, where all buffered data is included in one initial HARQ transmission otherwise:

| ThpTimeUl | The time to transmit a data burst excluding the data transmitted in the slot when the buffer is emptied. A sample of "ThpTimeUl" for each time the UL buffer for one DataRadioBearer (DRB) is emptied. |
| --- | --- |
|  | The point in time when the data up until the second last piece of data in data burst has been successfully received for a particular DRB |
|  | The point in time when transmission is started for the first data in data burst for a particular DRB. |
|  | The RLC level volume of a data burst, excluding the data transmitted in the slot when the buffer is emptied. A sample for ThpVolUl is the data volume counted on RLC SDU level in kbit received in UL for one DRB during a sample of ThpTimeUl, (It shall exclude the volume of the last piece of data emptying the buffer). |

d)	Each measurement is a real value representing the throughput in kbit per second. The number of measurements is equal to one. If the optional QoS level subcounter and S-NSSAI subcounter and PLMN ID subcounter and BWP subcounter measurements are performed, the number of measurements is equal to the number of mapped 5QIs and the number of supported S-NSSAIs, and the number of PLMN IDs, and the number of Active BWPs.

e)	The measurement name has the form 
DRB.UEThpUl, or optionally DRB.UEThpUl.QOS, where QOS identifies the target quality of service class  and DRB.UEThpUl.SNSSAI, where SNSSAI identifies the S-NSSAI, and DRB.UEThpUl.PLMN, where PLMN identifies the PLMN ID, and DRB.UEThpUl.BWP, where BWP identifies the Active BWP, or DRB.UEThpUl_UEType, or DRB.UEThpUl.QOS_UEType, or DRB.UEThpUl.SNSSAI_UEType or DRB.UEThpUl.PLMN_UEType or DRB.UEThpUl.BWP_UEType, where UEType refers to allowed values related to RedCap UE type of uECellBarredAccess in TS 28.541 clause 4.4.1.

f)	NRCellDU

g)	Valid for packet switched traffic

h)	5GS

i)	One usage of this measurement is for performance assurance within integrity area (user plane connection quality).

End of Change