3GPP TSG-SA5 Meeting #161	S5-252985

# Fukuoka, Japan, 19 - 23 May 2025

# Source:	Ericsson Hungary, Nokia

# Title:		DP Long Running Configuration Reads

# Document for:	   Endorsement

# Agenda Item:	6.19.8.1

# 1	Decision/action requested.

The group is asked to endorse the proposal.

# 2	References

[1] 	3GPP TS 28.532: "Management and orchestration; Generic management services (ProvMnS)

[2]		            3GPP TS 28.572: "Management of planned configurations"

# 3	Rationale

Observation 1: Specifications for long running configuration read request is lacking in the current 3GPP specifications (ProvMnS).

There is a need to support read operations that are potentially long running (many minutes or even hours) and might return big amounts of data. An example of such an operation could be

GET /SubNetwork=SN1?\

scopeType=BASE_ALL&\

filter=//NRCellRelation[attributes[hoAllowed=true]]/attributes

A  big Subnetwork may have 60 million NRCellRelations to process as part of the query.

There are two problems with such operations:

- The operation might time out and fail as HTTP has a built-in time-out usually set to 60 sec.

- The operation might occupy the producer for a long time.

The same issue was included in the Release-19 SBMA SID, but was not handled:

WT-10.2	Study how to allow operations which might take longer than http timeout with progress information.

The problem has been raised again by stakeholders, so it should be handled in Release-20.

# 4	Detailed proposal

Proposals for inclusion in 3GPP  R20 release :

Study and potentially implement a capability for the producer to support long running read operations that may run multiple minutes and return large amounts of data.