3GPP TSG-SA5 Meeting #161	S5-253084

Fukuoka, Japan, 19 - 23 May 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 28.554 | CR | 0233 | rev | 2 | Current version: | 19.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Rel-19 CR TS 28.554 Add EE KPI evaluated from network availability performance dimension |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, China Unicom, China Mobile, China Telecom, Orange, Verizon, ATT, Samsung |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA5 |  |  |  |  |  |  |  |  |  |
| Work item code: | Energy_OAM_Ph3 |  |  |  |  |  | Date: |  |  | 2025-05-23 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | Based on the conclusion for multi-dimensional network energy efficiency metrics in TR 28.880, it is proposed to add EE KPI evaluated from network availability performance dimension. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Adding EE KPI evaluated from network availability performance dimension. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  |  |  |  |  |  |  |  |  |  |
| Clauses affected: |  | New clause 6.7.X |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

1st Change

### 6.7.X	Energy efficiency evaluated from network availability

#### 6.7.X.1	Introduction

The KPI is defined from a network availability performance dimension perspective.

#### 6.7.X.2	Energy efficiency of cell based on availability

a) EECell,AvailAvgTimeCU

b) A KPI that shows the energy efficiency of a cell in a gNB based on availability performance. This KPI is obtained by dividing cell availability KPI by the average energy consumption per cell in a gNB over the same observation period. The unit of this KPI is sec/J.

c)

EECell,AvailAvgTimeCU=CellAvailAvgTimeCUECper cell of gNB

d) GNBCUCPFunction

e) The cell availability KPI is defined in clause 6.10.1.1.1. The average energy consumption per cell in a gNB is obtained by considering energy consumption of gNB (ECgNB) for a given time period and dividing it by total number of cells in a gNB that are operational in same time duration/period. ECgNB is defined in clause 6.7.3.4.2.

NOTE:	The cell availability KPI is for the split gNB deployment scenario. The average energy consumption for cell is derived using a weighted average of ECgNB over the number of cells. The weight defined by the consumer of KPI.

End of change