3GPP TSG-SA5 Meeting #161	S5-253036

# Fukuoka, Japan, 19th May 2025 - 23rd May 2025                          revision of S5-252223

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 28.541 | CR | 1526 | rev | 1 | Current version: | 18.11.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | x | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Rel-18 CR 28.541 YANG stage-3 corrections |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson Hungary Ltd |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI18 |  |  |  |  |  | Date: |  |  | 2025-04-24 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | YANG SS is not matching the approved stage 2. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Update YANG code to match existing stage 2. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Stage 2 and Stage 3 mismatch; interoperability problems. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | Forge only |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | Forge MR link: https://forge.3gpp.org/rep/sa5/MnS/-/merge_requests/1705 at commit e486e032a24ffcad83b6c137ba6ceb08fe5c2a9c |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

*** START OF CHANGE 1 ***

*** yang-models/_3gpp-5gc-nrm-QFQoSMonitoringControl.yang ***

<CODE BEGINS>

module _3gpp-5gc-nrm-QFQoSMonitoringControl {

yang-version 1.1;

namespace urn:3gpp:sa5:_3gpp-5gc-nrm-QFQoSMonitoringControl;

prefix qFQMCtrl3gpp;

import _3gpp-common-managed-element { prefix me3gpp; }

import _3gpp-5gc-nrm-smffunction { prefix smf3gpp; }

import _3gpp-common-top { prefix top3gpp; }

import _3gpp-5g-common-yang-types { prefix types5g3gpp; }

organization "3gpp SA5";

contact "https://www.3gpp.org/DynaReport/TSG-WG--S5--officials.htm?Itemid=464";

description "This IOC represents the capabilities and properties for control

of QoS monitoring per QoS flow per UE for URLLC service defined

in 3GPP TS 23.501.

Copyright 2023, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TSDSI,

TTA, TTC). All rights reserved.";

reference "3GPP TS 28.541";

revision 2023-09-24 { reference CR-0916; }

revision 2020-11-08 { reference CR-0412 ; }

revision 2020-08-03 { reference "CR-0321"; }

revision 2020-04-10 { reference "S5-202101";   }

grouping QFPacketDelayThresholdsTypeGrp {

description "Represents the QFPacketDelayThresholdsType";

leaf thresholdDl {

type uint32;

units milliseconds;

description "Downlink threshold";

}

leaf thresholdUl {

type uint32;

units milliseconds;

description "Uplink threshold";

}

leaf thresholdRtt {

type uint32;

units milliseconds;

description "Round trip threshold";

}

}

grouping QFQoSMonitoringControlGrp {

description "Represents the QFQoSMonitoringControl IOC.";

reference "3GPP TS 28.541";

leaf qFQoSMonitoringState {

description "The state of QoS monitoring per QoS flow per UE.";

mandatory true;

type enumeration {

enum ENABLED;

enum DISABLED;

}

}

list qFMonitoredSNSSAIs {

description "The S-NSSAIs for which the QoS monitoring per QoS flow

per UE is to be performed.";

reference "3GPP TS 23.003";

key "sd sst";

uses types5g3gpp:SNssai;}

leaf-list qFMonitored5QIs {

description "The 5QIs for which the QoS monitoring per QoS flow

per UE is to be performed.";

reference "3GPP TS 23.501";

type uint32 {

range "0..255";

}

}

leaf isEventTriggeredQFMonitoringSupported {

description "It indicates whether the event based QoS monitoring

reporting per QoS flow per UE is supported.";

mandatory true;

reference "3GPP TS 29.244";

type boolean;

}

leaf isPeriodicQFMonitoringSupported {

description "It indicates whether the periodic QoS monitoring reporting

per QoS flow per UE is supported.";

mandatory true;

reference "3GPP TS 29.244";

type boolean;

}

leaf isSessionReleasedQFMonitoringSupported {

description "It indicates whether the session release based QoS monitoring

reporting  per QoS flow per UE is supported.";

mandatory true;

reference "3GPP TS 29.244";

type boolean;

}

list qFPacketDelayThresholds {

key "idx";

min-elements 1;

max-elements 1;

description "It specifies the thresholds for reporting the packet delay

between PSA and UE for QoS monitoring per QoS flow per UE.";

leaf idx { type uint32 ; }

uses QFPacketDelayThresholdsTypeGrp;

}

leaf qFMinimumWaitTime {

description "It specifies the minimum waiting time (in seconds) between

two consecutive reports for event triggered QoS monitoring reporting

per QoS flow per UE.";

type uint32;

}

leaf qFMeasurementPeriod {

description "It specifies the period (in seconds) for reporting the

packet delay for QoS monitoring per QoS flow per UE.";

type uint32;

}

list qFMonitoredSatelliteBackhaulCategories {

description "The satellite backhaul categories for which the QoS

monitoring per QoS flow per UE is to be performed.";

key satellitebackhaulcat;

leaf satellitebackhaulcat{

description "Specifies the satellite backhaul.";

reference "3GPP TS 23.501";

type enumeration {

enum DYNAMIC_GEO;

enum DYNAMIC_MEO;

enum DYNAMIC_LEO;

enum DYNAMIC_OTHER_SAT;

}

}

}

}

augment "/me3gpp:ManagedElement/smf3gpp:SMFFunction" {

list QFQoSMonitoringControl {

description "Represents the QFQoSMonitoringControl IOC.";

reference "3GPP TS 28.541";

key id;

uses top3gpp:Top_Grp;

container attributes {

uses QFQoSMonitoringControlGrp;

}

}

}

}

<CODE ENDS>

*** END OF CHANGE 1 ***

*** START OF CHANGE 7 ***

*** yang-models/_3gpp-nr-nrm-danrmanagementfunction.yang ***

<CODE BEGINS>

module _3gpp-nr-nrm-danrmanagementfunction {

yang-version 1.1;

namespace "urn:3gpp:sa5:_3gpp-nr-nrm-danrmanagementfunction";

prefix "danrmanagementfunction3gpp";

import _3gpp-common-top { prefix top3gpp; }

import _3gpp-nr-nrm-gnbcucpfunction { prefix gnbcucp3gpp; }

import _3gpp-common-managed-element { prefix me3gpp; }

organization "3GPP SA5";

contact "https://www.3gpp.org/DynaReport/TSG-WG--S5--officials.htm?Itemid=464";

description "Defines the YANG mapping of the DANRManagementFunction IOC

that is part of the NR Network Resource Model (NRM).

Copyright 2025, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TSDSI,

TTA, TTC). All rights reserved.";

reference "3GPP TS 28.541 5G Network Resource Model (NRM)";

revision 2025-05-06 { reference "CR-1526 CR-1527" ; }  // common for R18, R19

revision 2024-08-19 { reference CR-1314 ; }

revision 2023-09-18 { reference CR-1043 ; }

revision 2020-05-08 { reference S5-203316; }

grouping DANRManagementFunctionGrp {

description "Represents the DANRManagementFunction IOC.";

leaf intrasystemANRManagementSwitch {

description "This attribute determines whether the intra-system ANR

function is activated or deactivated.";

type boolean;

}

leaf intersystemANRManagementSwitch {

description "This attribute determines whether the inter-system ANR

function is activated or deactivated.";

type boolean;

}

}

augment "/me3gpp:ManagedElement/gnbcucp3gpp:GNBCUCPFunction" {

list DANRManagementFunction {

description "This IOC contains attributes to support the D-SON function

of ANR Management (See clause 6.4.1.3 in TS 28.313 .";

key id;

max-elements 1;

uses top3gpp:Top_Grp;

container attributes {

uses DANRManagementFunctionGrp;

}

}

}

}

<CODE ENDS>

*** END OF CHANGE 7 ***

*** START OF CHANGE 8 ***

*** yang-models/_3gpp-nr-nrm-ecmappingrule.yang ***

<CODE BEGINS>

module _3gpp-nr-nrm-ecmappingrule {

yang-version 1.1;

namespace urn:3gpp:sa5:_3gpp-nr-nrm-ecmappingrule;

prefix ecmap3gpp;

import _3gpp-common-top { prefix top3gpp; }

organization "3gpp SA5";

contact "https://www.3gpp.org/DynaReport/TSG-WG--S5--officials.htm?Itemid=464";

description "This IOC represents the unified mapping rule input to support

Energy Cost Index Mapping when Energy Cost Reporting.

Copyright 2025, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TSDSI,

TTA, TTC). All rights reserved.";

reference "3GPP TS 28.541";

revision 2025-05-06 { reference "CR-1526 CR-1527" ; }  // common for R18, R19

revision 2024-08-12 { reference "CR-1336 CR-1337"; }

grouping NRECMappingRuleGrp {

description "Represents the unified mapping rule to support Energy Cost Mapping

when Energy Cost Reporting is supported.  Can be name-contained by SubNetwork

or ManagedElement.  It applies to all ManagedEntity contained by the parent.";

leaf eCMRInputMinValue {

type uint8;

description "The minimum value of to be applied

for mapping from this attribute to the energy cost.";

}

leaf eCMRInputMaxValue {

type uint8;

description "The maximum value of to be applied

for mapping from this attribute to the energy cost.";

}

leaf eCTimeInterval {

type uint8;

description "Time interval (in seconds) that

should be applied for collecting values of mapping

rule attribute to be used for computing the energy cost.";

units seconds;

}

}

grouping NRECMappingRuleSubtree {

description "Helps augmenting NRECMappingRule into multiple places.";

list NRECMappingRule {

description "Represents the unified mapping rule, applicable for the

group of gNBs (within a certain area), for the calculation of the

energy cost (see TS 34.423).

NRECMappingRule can be name-contained by SubNetwork or ManagedElement.

It applies to all ManagedEntity contained by the parent.";

key id;

uses top3gpp:Top_Grp;

min-elements 1;

max-elements 1;

container attributes {

uses NRECMappingRuleGrp;

}

}

}

}

<CODE ENDS>

*** END OF CHANGE 8 ***

*** START OF CHANGE 9 ***

*** yang-models/_3gpp-nr-nrm-ep.yang ***

<CODE BEGINS>

module _3gpp-nr-nrm-ep {

yang-version 1.1;

namespace "urn:3gpp:sa5:_3gpp-nr-nrm-ep";

prefix "ep3gpp";

import _3gpp-common-ep-rp { prefix eprp3gpp; }

import _3gpp-common-managed-element { prefix me3gpp; }

import _3gpp-common-top { prefix top3gpp; }

import _3gpp-nr-nrm-gnbcucpfunction { prefix gnbcucp3gpp; }

import _3gpp-nr-nrm-gnbcuupfunction { prefix gnbcuup3gpp; }

import _3gpp-nr-nrm-gnbdufunction { prefix gnbdu3gpp; }

organization "3GPP SA5";

contact "https://www.3gpp.org/DynaReport/TSG-WG--S5--officials.htm?Itemid=464";

description "Defines the YANG mapping of the NR related endpoint

Information Object Classes (IOCs) that are part of the NR Network

Resource Model (NRM).

Copyright 2025, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TSDSI,

TTA, TTC). All rights reserved.";

reference "3GPP TS 28.541 5G Network Resource Model (NRM)";

revision 2025-05-06 { reference "CR-1526 CR-1527" ; }  // common for R18, R19

revision 2023-09-18 { reference CR-1043 ; }

revision 2022-01-07 { reference CR-0643; }

revision 2021-05-01 { reference CR-0490; }

revision 2021-03-03 { reference CR-0435 ; }

revision 2021-02-17 { reference CR-0470; }

revision 2020-11-17 { reference CR-0410 ; }

revision 2020-03-02 { reference S5-201191; }

revision 2019-06-17 { reference "Initial revision"; }

feature EPClassesUnderGNBCUCPFunction {

description "Endpoint classes shall be contained under GNBCUCPFunction";

}

feature EPClassesUnderGNBCUUPFunction {

description "Endpoint classes shall be contained under GNBCUUPFunction";

}

feature EPClassesUnderGNBDUFunction {

description "Endpoint classes shall be contained under GNBDUFunction";

}

grouping EP_E1Grp {

description "Represents the EP_E1 IOC.";

reference "3GPP TS 28.541, 3GPP TS 38.401";

uses eprp3gpp:EP_Common;

}

grouping EP_F1CSubtree {

list EP_F1C {

description "Represents the local end point of the control plane

interface (F1-C) between the gNB-DU and gNB-CU or gNB-CU-CP.";

reference "3GPP TS 28.541, 3GPP TS 38.470";

key id;

uses top3gpp:Top_Grp;

container attributes {

uses eprp3gpp:EP_Common;

}

}

}

grouping EP_F1USubtree {

list EP_F1U {

description "Represents the local end point of the user plane

interface (F1-U) between the gNB-DU and gNB-CU or gNB-CU-UP.";

reference "3GPP TS 28.541, 3GPP TS 38.470";

key id;

uses top3gpp:Top_Grp;

container attributes {

uses eprp3gpp:EP_Common;

}

}

}

grouping EP_XnCGrp {

description "Represents the EP_XnC IOC.";

reference "3GPP TS 28.541, 3GPP TS 38.420";

uses eprp3gpp:EP_Common;

}

grouping EP_XnUGrp {

description "Represents the EP_XnU IOC.";

reference "3GPP TS 28.541, 3GPP TS 38.420";

uses eprp3gpp:EP_Common;

}

grouping EP_NgCGrp {

description "Represents the EP_NgC IOC.";

reference "3GPP TS 28.541, 3GPP TS 38.470";

uses eprp3gpp:EP_Common;

}

grouping EP_NgUGrp {

description "Represents the EP_NgU IOC.";

reference "3GPP TS 28.541, 3GPP TS 38.470";

uses eprp3gpp:EP_Common;

}

grouping EP_X2CGrp {

description "Represents the EP_X2C IOC.";

reference "3GPP TS 28.541, 3GPP TS 36.423";

uses eprp3gpp:EP_Common;

}

grouping EP_X2UGrp {

description "Represents the EP_X2U IOC.";

reference "3GPP TS 28.541, 3GPP TS 36.425";

uses eprp3gpp:EP_Common;

}

grouping EP_S1UGrp {

description "Represents the EP_S1U IOC.";

reference "3GPP TS 28.541, 3GPP TS 36.410";

uses eprp3gpp:EP_Common;

}

augment "/me3gpp:ManagedElement/gnbcucp3gpp:GNBCUCPFunction" {

if-feature EPClassesUnderGNBCUCPFunction;

list EP_E1 {

description "Represents the local end point of the logical link,

supporting E1 interface between gNB-CU-CP and gNB-CU-UP.";

reference "3GPP TS 28.541, 3GPP TS 38.401";

key id;

uses top3gpp:Top_Grp;

container attributes {

uses EP_E1Grp;

}

}

uses EP_F1CSubtree;

list EP_NgC {

description "Represents the local end point of the control plane

interface (NG-C) between the gNB and AMF.";

reference "3GPP TS 28.541, 3GPP TS 38.470";

key id;

uses top3gpp:Top_Grp;

container attributes {

uses EP_NgCGrp;

}

}

list EP_XnC {

description "Represents the local gNB node end point of the logical

link, supporting Xn application protocols, to a neighbour NG-RAN node

(including gNB and ng-eNB). The Xn Application PDUs are carried over

SCTP/IP/Data link layer/Physical layer stack.";

reference "3GPP TS 28.541, 3GPP TS 38.420 subclause 7";

key id;

uses top3gpp:Top_Grp;

container attributes {

uses EP_XnCGrp;

}

}

list EP_X2C {

description "Represents the local end point of the logical link,

supporting X2-C application protocols used in EN-DC, to a neighbour

eNB or en-gNB node.";

reference "3GPP TS 28.541, 3GPP TS 36.423";

key id;

uses top3gpp:Top_Grp;

container attributes {

uses EP_X2CGrp;

}

}

}

augment "/me3gpp:ManagedElement/gnbcuup3gpp:GNBCUUPFunction" {

if-feature EPClassesUnderGNBCUUPFunction;

list EP_E1 {

description "Represents the local end point of the logical link,

supporting E1 interface between gNB-CU-CP and gNB-CU-UP.";

reference "3GPP TS 28.541, 3GPP TS 38.401";

key id;

uses top3gpp:Top_Grp;

container attributes {

uses EP_E1Grp;

}

}

uses EP_F1USubtree;

list EP_NgU {

description "Represents the local end point of the NG user plane

(NG-U) interface between the gNB and UPF.";

reference "3GPP TS 28.541, 3GPP TS 38.470";

key id;

uses top3gpp:Top_Grp;

container attributes {

uses EP_NgUGrp;

}

}

list EP_XnU {

description "Represents the one end-point of a logical link supporting

the Xn user plane (Xn-U) interface. The Xn-U interface provides

non-guaranteed delivery of user plane PDUs between two NG-RAN nodes.";

reference "3GPP TS 28.541, 3GPP TS 38.420";

key id;

uses top3gpp:Top_Grp;

container attributes {

uses EP_XnUGrp;

}

}

list EP_X2U {

description "Represents the local end-point of a logical link supporting

the X2 user plane (X2-U) interface used in EN-DC.";

reference "3GPP TS 28.541, 3GPP TS 36.425";

key id;

uses top3gpp:Top_Grp;

container attributes {

uses EP_X2UGrp;

}

}

list EP_S1U {

description "Represents the local end point of the logical link,

supporting S1-U interface towards a S-GW node.";

reference "3GPP TS 28.541, 3GPP TS 36.410";

key id;

uses top3gpp:Top_Grp;

container attributes {

uses EP_S1UGrp;

}

}

}

augment "/me3gpp:ManagedElement/gnbdu3gpp:GNBDUFunction" {

if-feature EPClassesUnderGNBDUFunction;

uses EP_F1CSubtree {

refine EP_F1C {

max-elements 1;

}

}

uses EP_F1USubtree;

}

}

<CODE ENDS>

*** END OF CHANGE 9 ***

*** START OF CHANGE 10 ***

*** yang-models/_3gpp-nr-nrm-eutranetwork.yang ***

<CODE BEGINS>

module _3gpp-nr-nrm-eutranetwork {

yang-version 1.1;

namespace "urn:3gpp:sa5:_3gpp-nr-nrm-eutranetwork";

prefix "eutranet3gpp";

import _3gpp-common-managed-element { prefix me3gpp; }

import _3gpp-nr-nrm-gnbcucpfunction { prefix gnbcucp3gpp; }

import _3gpp-common-top { prefix top3gpp; }

organization "3GPP SA5";

contact "https://www.3gpp.org/DynaReport/TSG-WG--S5--officials.htm?Itemid=464";

description "Defines the YANG mapping of the EUtraNetwork Information Object

Class (IOC) that is part of the NR Network Resource Model (NRM).

Copyright 2023, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TSDSI,

TTA, TTC). All rights reserved.";

reference "3GPP TS 28.541 5G Network Resource Model (NRM)";

revision 2025-04-25 { reference "CR-1526 CR-1527" ; } // common for r18, r19

revision 2023-09-18 { reference CR-1043 ; }

revision 2019-06-17 {reference "Initial revision"; }

grouping EUtraNetworkGrp {

description "Represents the EUtraNetwork IOC.

Contains no additiona attributes.";

}

augment /me3gpp:ManagedElement/gnbcucp3gpp:GNBCUCPFunction {

list EUtraNetwork {

description "A subnetwork containing gNB external E-UTRAN entities.";

key id;

uses top3gpp:Top_Grp;

container attributes {

uses EUtraNetworkGrp;

}

}

}

}

<CODE ENDS>

*** END OF CHANGE 10 ***

*** START OF CHANGE 11 ***

*** yang-models/_3gpp-nr-nrm-eutranfrequency.yang ***

<CODE BEGINS>

module _3gpp-nr-nrm-eutranfrequency {

yang-version 1.1;

namespace "urn:3gpp:sa5:_3gpp-nr-nrm-eutranfrequency";

prefix "eutraneteutranfreq3gpp";

import _3gpp-common-managed-element { prefix me3gpp; }

import _3gpp-nr-nrm-gnbcucpfunction { prefix gnbcucp3gpp; }

import _3gpp-nr-nrm-eutranetwork { prefix eutranet3gpp; }

import _3gpp-common-subnetwork { prefix subnet3gpp; }

import _3gpp-common-top { prefix top3gpp; }

organization "3GPP SA5";

contact "https://www.3gpp.org/DynaReport/TSG-WG--S5--officials.htm?Itemid=464";

description "Defines the YANG mapping of the EUtranFrequency Information

Object Class (IOC), that is part of the NR Network Resource Model (NRM).

Copyright 2025, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TSDSI,

TTA, TTC). All rights reserved.";

reference "3GPP TS 28.541 5G Network Resource Model (NRM),

3GPP TS 28.658 (E-UTRAN) Network Resource Model (NRM)";

revision 2025-04-25 { reference "CR-1526 CR-1527" ; } // common for r18, r19

revision 2023-09-18 { reference CR-1043 ; }

revision 2019-10-28 { reference S5-193518 ; }

revision 2019-06-17 { reference "Initial revision"; }

grouping EUtranFrequencyGrp {

description "Represents the EUtranFrequency IOC.";

reference "3GPP TS 28.541";

leaf earfcnDL {

description "Specifies the channel number for the central DL frequency.";

reference "3GPP TS 36.101";

mandatory true;

type uint32 { range "0..262143"; }

}

leaf-list multiBandInfoListEutra {

description "List of additional frequency bands the frequency belongs to.";

config false;

min-elements 0;

type uint16 { range "1..256"; }

}

}

grouping EUtranFrequencyWrapper {

list EUtranFrequency {

description "Represents certain E-UTRAN frequency properties.";

reference "3GPP TS 28.658";

key id;

uses top3gpp:Top_Grp;

container attributes {

uses EUtranFrequencyGrp;

}

}

}

feature ExternalENBFunctionUnderSubNetwork {

description "The ExternalENBFunction shall be contained under SubNetwork";

}

augment "/subnet3gpp:SubNetwork" {

if-feature ExternalENBFunctionUnderSubNetwork ;

uses EUtranFrequencyWrapper ;

}

feature ExternalENBFunctionUnderEUtraNetwork {

description "The ExternalENBFunction shall be contained under EUtraNetwork";

}

augment "/me3gpp:ManagedElement/gnbcucp3gpp:GNBCUCPFunction/"

+ "eutranet3gpp:EUtraNetwork" {

if-feature ExternalENBFunctionUnderEUtraNetwork;

uses EUtranFrequencyWrapper ;

}

}

<CODE ENDS>

*** END OF CHANGE 11 ***

*** START OF CHANGE 12 ***

*** yang-models/_3gpp-nr-nrm-externalamffunction.yang ***

<CODE BEGINS>

module _3gpp-nr-nrm-externalamffunction {

yang-version 1.1;

namespace "urn:3gpp:sa5:_3gpp-nr-nrm-externalamffunction";

prefix "extamf3gpp";

import _3gpp-common-managed-function { prefix mf3gpp; }

import _3gpp-common-subnetwork { prefix subnet3gpp; }

import _3gpp-common-top { prefix top3gpp; }

import _3gpp-common-yang-types { prefix types3gpp; }

organization "3GPP SA5";

contact "https://www.3gpp.org/DynaReport/TSG-WG--S5--officials.htm?Itemid=464";

description "Defines the YANG mapping of the ExternalAMFFunction Information

Object Class (IOC) that is part of the NR Network Resource Model (NRM).

Copyright 2025, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TSDSI,

TTA, TTC). All rights reserved.";

reference "3GPP TS 28.541 5G Network Resource Model (NRM)";

revision 2025-04-25 { reference "CR-1526 CR-1527" ; } // common for r18, r19

revision 2023-09-18 { reference CR-1043 ; }

revision 2019-10-28 { reference S5-193518 ; }

revision 2019-06-17 { reference "Initial revision"; }

grouping ExternalAMFFunctionGrp {

description "Represents the ExternalAMFFunction IOC.";

uses mf3gpp:ManagedFunctionGrp;

list pLMNIdList {

description "List of at most six entries of PLMN Identifiers, but at least

one (the primary PLMN Id).

The PLMN Identifier is composed of a Mobile Country Code (MCC) and a

Mobile Network Code (MNC).";

min-elements 1;

max-elements 6;

key "mcc mnc";

uses types3gpp:PLMNId;

}

container aMFIdentifier {

presence true;

description "An AMF identifier, comprising an AMF Region ID, an AMF Set ID and an AMF Pointer.";

uses types3gpp:AmfIdentifier;

}

}

grouping ExternalAMFFunctionWrapper {

list ExternalAMFFunction {

description "Represents the properties, known by the management

function, of a AMFFunction managed by another management

function.";

reference "3GPP TS 28.541";

key id;

uses top3gpp:Top_Grp;

container attributes {

uses ExternalAMFFunctionGrp;

}

uses mf3gpp:ManagedFunctionContainedClasses;

}

}

augment "/subnet3gpp:SubNetwork" {

uses ExternalAMFFunctionWrapper;

}

}

<CODE ENDS>

*** END OF CHANGE 12 ***

*** START OF CHANGE 13 ***

*** yang-models/_3gpp-nr-nrm-externalenbfunction.yang ***

<CODE BEGINS>

module _3gpp-nr-nrm-externalenbfunction {

yang-version 1.1;

namespace "urn:3gpp:sa5:_3gpp-nr-nrm-externalenbfunction";

prefix "extenb3gpp";

import _3gpp-common-managed-function { prefix mf3gpp; }

import _3gpp-common-subnetwork { prefix subnet3gpp; }

import _3gpp-common-managed-element { prefix me3gpp; }

import _3gpp-nr-nrm-gnbcucpfunction { prefix gnbcucp3gpp; }

import _3gpp-nr-nrm-eutranetwork { prefix eutranet3gpp; }

import _3gpp-nr-nrm-externaleutrancell { prefix exteutrancell3gpp; }

import _3gpp-common-top { prefix top3gpp; }

organization "3GPP SA5";

contact "https://www.3gpp.org/DynaReport/TSG-WG--S5--officials.htm?Itemid=464";

description "Defines the YANG mapping of the ExternalENBFunction

Information Object Class (IOC) that is part of the NR Network Resource

Model (NRM).

Copyright 2025, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TSDSI,

TTA, TTC). All rights reserved.";

reference "3GPP TS 28.541 5G Network Resource Model (NRM),

3GPP TS 28.658 (E-UTRAN) Network Resource Model (NRM)";

revision 2025-04-25 { reference "CR-1526 CR-1527" ; } // common for r18, r19

revision 2023-09-18 { reference CR-1043 ; }

revision 2019-10-28 { reference S5-193518 ; }

revision 2019-06-17 { reference "Initial revision"; }

grouping ExternalENBFunctionGrp {

description "Represets the ExternalENBFunction IOC.";

reference "3GPP TS 28.658";

uses mf3gpp:ManagedFunctionGrp;

leaf eNBId {

description "Unambiguously identifies an eNodeB within a PLMN.";

reference "3GPP TS 36.413, 3GPP TS 36.300";

mandatory true;

type int32 { range "0..268435455"; } // Representing 28 bit eNB ID.

// 18, 20 and 21 bit eNB IDs also

// allowed.

}

}

grouping ExternalENBFunctionWrapper {

list ExternalENBFunction {

description "Represents an external eNB functionality.";

reference "3GPP TS 28.658";

key id;

uses top3gpp:Top_Grp;

container attributes {

uses ExternalENBFunctionGrp;

}

uses mf3gpp:ManagedFunctionContainedClasses;

uses exteutrancell3gpp:ExternalEUtranCellFDDSubtree;

uses exteutrancell3gpp:ExternalEUtranCellTDDSubtree;

}

}

feature ExternalENBFunctionUnderSubNetwork {

description "The ExternalENBFunction shall be contained under

SubNetwork";

}

augment "/subnet3gpp:SubNetwork" {

if-feature ExternalENBFunctionUnderSubNetwork ;

uses ExternalENBFunctionWrapper;

}

feature ExternalENBFunctionUnderEUtraNetwork {

description "The ExternalENBFunction shall be contained under

EUtraNetwork";

}

augment "/me3gpp:ManagedElement/gnbcucp3gpp:GNBCUCPFunction/"

+ "eutranet3gpp:EUtraNetwork" {

if-feature ExternalENBFunctionUnderEUtraNetwork;

uses ExternalENBFunctionWrapper;

}

}

<CODE ENDS>

*** END OF CHANGE 13 ***

*** START OF CHANGE 14 ***

*** yang-models/_3gpp-nr-nrm-externaleutrancell.yang ***

<CODE BEGINS>

module _3gpp-nr-nrm-externaleutrancell {

yang-version 1.1;

namespace "urn:3gpp:sa5:_3gpp-nr-nrm-externaleutrancell";

prefix "exteutrancell3gpp";

import _3gpp-common-yang-types { prefix types3gpp; }

import _3gpp-common-managed-function { prefix mf3gpp; }

import _3gpp-common-top { prefix top3gpp; }

organization "3GPP SA5";

contact "https://www.3gpp.org/DynaReport/TSG-WG--S5--officials.htm?Itemid=464";

description "Defines the YANG mapping of the ExternalEUtranCellFDD and

ExternalEUtranCellTDD Information Object Classes (IOCs) that are part

of the NR Network Resource Model (NRM).

Copyright 2025, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TSDSI,

TTA, TTC). All rights reserved.";

reference "3GPP TS 28.541 5G Network Resource Model (NRM),

3GPP TS 28.658 (E-UTRAN) Network Resource Model (NRM)";

revision 2025-04-25 { reference "CR-1526 CR-1527" ; } // common for r18, r19

revision 2023-09-18 { reference CR-1043 ; }

revision 2019-10-28 { reference S5-193518 ; }

revision 2019-06-17 { reference "Initial revision"; }

grouping ExternalEUtranGenericCellGrp {

description "Represents the ExternalEUtranGenericCell IOC.";

reference "3GPP TS 28.658";

uses mf3gpp:ManagedFunctionGrp;

leaf pci {

description "The Physical Cell Identity (PCI) of the cell (for

NM-Centralized, EM-Centralized and Distributed PCI assignment cases).

In the case of NM-Centralized PCI assignment, see 3GPP TS 36.300.";

reference "3GPP TS 36.211";

mandatory true;

type int32 { range "0..503"; }

}

list plmnIdList {

description "List of unique identities for PLMNs. A cell can broadcast

up to 6 PLMN IDs. This is to support the case that one cell can be

used by up to 6 operator's core networks. The PLMN(s) included in this

list will use the same single tracking area code (TAC) and the same

Cell Identity (cellLocalId) for sharing the radio access network

resources. One member of plmnIdList is the primary PLMN ID. A PLMN ID

included in this list cannot be included in the cellAccessInfoList.

The PLMN ID is composed of a Mobile Country Code (MCC) and a Mobile

Network Code (MNC).";

reference "3GPP TS 36.300, 3GPP TS 36.331, 3GPP TS 23.003";

key "mcc mnc";

min-elements 1;

max-elements 6;

uses types3gpp:PLMNId;

}

leaf cellLocalId {

description "Unambiguously identifies a cell within an eNodeB.";

reference "NCI defined in 3GPP TS 38.300";

type int32 {range "0..255"; }

}

leaf eNBId {

description "Unambiguously identifies an eNodeB within a PLMN.";

reference "3GPP TS 36.413, 3GPP TS 36.300";

mandatory true;

type int32 { range "0..268435455"; } // Representing 28 bit eNB ID.

// 18, 20 and 21 bit eNB IDs also

// allowed.

}

}

grouping ExternalEUtranCellFDDGrp {

description "Represents the ExternalEUtranCellFDD IOC.";

reference "3GPP TS 28.658";

uses ExternalEUtranGenericCellGrp;

leaf earfcnDL {

description "The channel number for the central DL frequency.";

reference "3GPP TS 36.101";

mandatory true;

type int32 { range "0..17999 | 46590..262143"; }

}

leaf earfcnUL {

description "The channel number for the central UL frequency. Value 0

means that the UL channel number is N/A for the DL-only bands.";

reference "3GPP TS 36.101";

mandatory true;

type int32 { range "0 | 18000..35999 | 46590..262143"; }

}

}

grouping ExternalEUtranCellTDDGrp {

description "Represents the ExternalEUtranCellTDD IOC.";

reference "3GPP TS 28.658";

uses ExternalEUtranGenericCellGrp;

leaf earfcn {

description "The frequency number for the central frequency.";

reference "3GPP TS 36.104";

mandatory true;

type int32 { range "36000..262143"; }

}

}

grouping ExternalEUtranCellFDDSubtree {

list ExternalEUtranCellFDD {

description "Represents the common properties of external E-UTRAN FDD

cell provided by eNB or NG-RAN FDD cell provided by ng-eNB.";

reference "3GPP TS 28.658";

key id;

uses top3gpp:Top_Grp;

container attributes {

uses ExternalEUtranCellFDDGrp;

}

uses mf3gpp:ManagedFunctionContainedClasses;

}

}

grouping ExternalEUtranCellTDDSubtree {

list ExternalEUtranCellTDD {

description "Represents the common properties of external E-UTRAN cell

TDD provided by eNB or NG-RAN TDD cell provided by ng-eNB.";

reference "3GPP TS 28.658";

key id;

uses top3gpp:Top_Grp;

container attributes {

uses ExternalEUtranCellTDDGrp;

}

uses mf3gpp:ManagedFunctionContainedClasses;

}

}

}

<CODE ENDS>

*** END OF CHANGE 14 ***

*** START OF CHANGE 15 ***

*** yang-models/_3gpp-nr-nrm-externalgnbcucpfunction.yang ***

<CODE BEGINS>

module _3gpp-nr-nrm-externalgnbcucpfunction {

yang-version 1.1;

namespace "urn:3gpp:sa5:_3gpp-nr-nrm-externalgnbcucpfunction";

prefix "extgnbcucp3gpp";

import _3gpp-common-yang-types { prefix types3gpp; }

import _3gpp-common-managed-function { prefix mf3gpp; }

import _3gpp-common-managed-element { prefix me3gpp; }

import _3gpp-nr-nrm-gnbcucpfunction { prefix gnbcucp3gpp; }

import _3gpp-nr-nrm-nrnetwork { prefix nrnet3gpp; }

import _3gpp-common-subnetwork { prefix subnet3gpp; }

import _3gpp-nr-nrm-externalnrcellcu { prefix extnrcellcu3gpp; }

import _3gpp-common-top { prefix top3gpp; }

organization "3GPP SA5";

contact "https://www.3gpp.org/DynaReport/TSG-WG--S5--officials.htm?Itemid=464";

description "Defines the YANG mapping of the ExternalGNBCUCPFunction

Information Object Class (IOC), that is part of the NR Network Resource

Model (NRM).

Copyright 2025, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TSDSI,

TTA, TTC). All rights reserved.";

reference "3GPP TS 28.541 5G Network Resource Model (NRM)";

revision 2025-04-25 { reference "CR-1526 CR-1527" ; } // common for r18, r19

revision 2023-09-18 { reference CR-1043 ; }

revision 2019-10-28 { reference S5-193518 ; }

revision 2019-06-17 { reference "Initial revision"; }

grouping ExternalGNBCUCPFunctionGrp {

description "Represets the ExternalGNBCUCPFunction IOC.";

uses mf3gpp:ManagedFunctionGrp;

leaf gNBId {

description "Identifies a gNB within a PLMN.

The gNB ID is part of the NR Cell Identifier (NCI) of the gNB cells.

See 'gNB Identifier (gNB ID)' of subclause 8.2 of TS 38.300.

See 'Global gNB ID' in subclause 9.3.1.6 of TS 38.413";

mandatory true;

type uint32 { range "0..4294967295"; }

}

leaf gNBIdLength {

description "Indicates the number of bits for encoding the gNB ID.";

reference "'Global gNB ID' in subclause 9.3.1.6 of TS 38.413";

mandatory true;

type int32 { range "22..32"; }

}

list pLMNId {

description "Specifies the PLMN identifier to be used as part of the

global RAN node identity.";

key "mcc mnc";

min-elements 1;

max-elements 1;

uses types3gpp:PLMNId;

}

}

grouping ExternalGNBCUCPFunctionWrapper {

list ExternalGNBCUCPFunction {

description "Represents the properties, known by the management function,

of a GNBCUCPFunction managed by another management function.";

key id;

uses top3gpp:Top_Grp;

container attributes {

uses ExternalGNBCUCPFunctionGrp;

}

uses mf3gpp:ManagedFunctionContainedClasses;

uses extnrcellcu3gpp:ExternalNRCellCUSubtree;

}

}

feature ExternalGNBCUCPFunctionUnderNRNetwork {

description "The ExternalGNBCUCPFunction shall be contained under

NRNetwork";

}

augment "/me3gpp:ManagedElement/gnbcucp3gpp:GNBCUCPFunction/"

+ "nrnet3gpp:NRNetwork" {

if-feature ExternalGNBCUCPFunctionUnderNRNetwork;

uses ExternalGNBCUCPFunctionWrapper;

}

feature ExternalGNBCUCPFunctionUnderSubNetwork {

description "The ExternalGNBCUCPFunction shall be contained under

SubNetwork";

}

augment "/subnet3gpp:SubNetwork" {

if-feature ExternalGNBCUCPFunctionUnderSubNetwork ;

uses ExternalGNBCUCPFunctionWrapper;

}

}

<CODE ENDS>

*** END OF CHANGE 15 ***

*** START OF CHANGE 16 ***

*** yang-models/_3gpp-nr-nrm-externalgnbcuupfunction.yang ***

<CODE BEGINS>

module _3gpp-nr-nrm-externalgnbcuupfunction {

yang-version 1.1;

namespace "urn:3gpp:sa5:_3gpp-nr-nrm-externalgnbcuupfunction";

prefix "extgnbcuup3gpp";

import _3gpp-common-managed-function { prefix mf3gpp; }

import _3gpp-common-managed-element { prefix me3gpp; }

import _3gpp-nr-nrm-gnbcucpfunction { prefix gnbcucp3gpp; }

import _3gpp-nr-nrm-nrnetwork { prefix nrnet3gpp; }

import _3gpp-common-subnetwork { prefix subnet3gpp; }

import _3gpp-common-top { prefix top3gpp; }

organization "3GPP SA5";

contact "https://www.3gpp.org/DynaReport/TSG-WG--S5--officials.htm?Itemid=464";

description "Defines the YANG mapping of the ExternalGNBCUUPFunction

Information Object Class (IOC), that is part of the NR Network

Resource Model (NRM).

Copyright 2023, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TSDSI,

TTA, TTC). All rights reserved.";

reference "3GPP TS 28.541 5G Network Resource Model (NRM)";

revision 2025-04-25 { reference "CR-1526 CR-1527" ; } // common for r18, r19

revision 2023-09-18 { reference CR-1043 ; }

revision 2019-10-28 { reference S5-193518 ; }

revision 2019-06-17 { reference "Initial revision"; }

grouping ExternalGNBCUUPFunctionGrp {

description "Represets the ExternalGNBCUUPFunction IOC.";

reference "3GPP TS 28.541";

uses mf3gpp:ManagedFunctionGrp;

leaf gNBId {

description "Identifies a gNB within a PLMN.

The gNB ID is part of the NR Cell Identifier (NCI) of the gNB cells.

See 'gNB Identifier (gNB ID)' of subclause 8.2 of TS 38.300.

See 'Global gNB ID' in subclause 9.3.1.6 of TS 38.413";

mandatory true;

type uint32 { range "0..4294967295"; }

}

leaf gNBIdLength {

description "Indicates the number of bits for encoding the gNB ID.";

reference "'Global gNB ID' in subclause 9.3.1.6 of TS 38.413";

mandatory true;

type int32 { range "22..32"; }

}

}

grouping ExternalGNBCUUPFunctionSubtree {

list ExternalGNBCUUPFunction {

description "Represents the properties, known by the management function,

of a GNBCUUPFunction managed by another management function.";

key id;

uses top3gpp:Top_Grp;

container attributes {

uses ExternalGNBCUUPFunctionGrp;

}

uses mf3gpp:ManagedFunctionContainedClasses;

}

}

feature ExternalGNBCUUPFunctionUnderNRNetwork {

description "The ExternalGNBCUUPFunction shall be contained under

NRNetwork";

}

augment "/me3gpp:ManagedElement/gnbcucp3gpp:GNBCUCPFunction/"

+ "nrnet3gpp:NRNetwork" {

if-feature ExternalGNBCUUPFunctionUnderNRNetwork;

uses ExternalGNBCUUPFunctionSubtree;

}

feature ExternalGNBCUUPFunctionUnderSubNetwork {

description "The ExternalGNBCUUPFunction shall be contained under

SubNetwork";

}

augment "/subnet3gpp:SubNetwork" {

if-feature ExternalGNBCUUPFunctionUnderSubNetwork ;

uses ExternalGNBCUUPFunctionSubtree;

}

}

<CODE ENDS>

*** END OF CHANGE 16 ***

*** START OF CHANGE 17 ***

*** yang-models/_3gpp-nr-nrm-externalgnbdufunction.yang ***

<CODE BEGINS>

module _3gpp-nr-nrm-externalgnbdufunction {

yang-version 1.1;

namespace "urn:3gpp:sa5:_3gpp-nr-nrm-externalgnbdufunction";

prefix "extgnbdu3gpp";

import _3gpp-common-yang-extensions { prefix yext3gpp; }

import _3gpp-common-managed-function { prefix mf3gpp; }

import _3gpp-common-managed-element { prefix me3gpp; }

import _3gpp-nr-nrm-gnbcucpfunction { prefix gnbcucp3gpp; }

import _3gpp-nr-nrm-nrnetwork { prefix nrnet3gpp; }

import _3gpp-common-subnetwork { prefix subnet3gpp; }

import _3gpp-common-top { prefix top3gpp; }

organization "3GPP SA5";

contact "https://www.3gpp.org/DynaReport/TSG-WG--S5--officials.htm?Itemid=464";

description "Defines the YANG mapping of the ExternalGNBDUFunction

Information Object Class (IOC) that is part of the NR Network Resource

Model (NRM).

Copyright 2025, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TSDSI,

TTA, TTC). All rights reserved.";

reference "3GPP TS 28.541 5G Network Resource Model (NRM)";

revision 2025-04-25 { reference "CR-1526 CR-1527" ; } // common for r18, r19

revision 2023-09-18 { reference CR-1043 ; }

revision 2019-10-28 { reference S5-193518 ; }

revision 2019-06-17 { reference "Initial revision"; }

grouping ExternalGNBDUFunctionGrp {

description "Represets the ExternalGNBDUFunction IOC.";

reference "3GPP TS 28.541";

uses mf3gpp:ManagedFunctionGrp;

leaf gNBId {

description "Identifies a gNB within a PLMN.

The gNB ID is part of the NR Cell Identifier (NCI) of the gNB cells.

See 'gNB Identifier (gNB ID)' of subclause 8.2 of TS 38.300.

See 'Global gNB ID' in subclause 9.3.1.6 of TS 38.413";

mandatory true;

type uint32 { range "0..4294967295"; }

yext3gpp:inVariant;

}

leaf gNBIdLength {

description "Indicates the number of bits for encoding the gNB ID.";

reference "'Global gNB ID' in subclause 9.3.1.6 of TS 38.413";

mandatory true;

type int32 { range "22..32"; }

}

}

grouping ExternalGNBDUFunctionSubtree {

list ExternalGNBDUFunction {

description "Represents the properties, known by the management function,

of a GNBDUFunction managed by another management function.";

key id;

uses top3gpp:Top_Grp;

container attributes {

uses ExternalGNBDUFunctionGrp;

}

uses mf3gpp:ManagedFunctionContainedClasses;

}

}

feature ExternalGNBDUFunctionUnderNRNetwork {

description "The ExternalGNBDUFunction shall be contained under

NRNetwork";

}

augment "/me3gpp:ManagedElement/gnbcucp3gpp:GNBCUCPFunction/"

+ "nrnet3gpp:NRNetwork" {

if-feature ExternalGNBDUFunctionUnderNRNetwork;

uses ExternalGNBDUFunctionSubtree;

}

feature ExternalGNBDUFunctionUnderSubNetwork {

description "The ExternalGNBDUFunction shall be contained under

SubNetwork";

}

augment "/subnet3gpp:SubNetwork" {

if-feature ExternalGNBDUFunctionUnderSubNetwork ;

uses ExternalGNBDUFunctionSubtree;

}

}

<CODE ENDS>

*** END OF CHANGE 17 ***

*** START OF CHANGE 18 ***

*** yang-models/_3gpp-nr-nrm-externalnrcellcu.yang ***

<CODE BEGINS>

module _3gpp-nr-nrm-externalnrcellcu {

yang-version 1.1;

namespace "urn:3gpp:sa5:_3gpp-nr-nrm-externalnrcellcu";

prefix "extnrcellcu3gpp";

import _3gpp-common-yang-types { prefix types3gpp; }

import _3gpp-common-managed-function { prefix mf3gpp; }

import _3gpp-common-top { prefix top3gpp; }

organization "3GPP SA5";

contact "https://www.3gpp.org/DynaReport/TSG-WG--S5--officials.htm?Itemid=464";

description "Defines the YANG mapping of the ExternalNRCellCU Information

Object Class (IOC), that is part of the NR Network Resource Model (NRM).

Copyright 2025, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TSDSI,

TTA, TTC). All rights reserved.";

reference "3GPP TS 28.541 5G Network Resource Model (NRM)";

revision 2025-04-25 { reference "CR-1526 CR-1527" ; } // common for r18, r19

revision 2023-09-18 { reference CR-1043 ; }

revision 2019-10-28 { reference S5-193518 ; }

revision 2019-06-17 { reference "Initial revision"; }

grouping ExternalNRCellCUGrp {

description "Represents the ExternalNRCellCU IOC.";

uses mf3gpp:ManagedFunctionGrp;

leaf cellLocalId {

description "It identifies a NR cell of a gNB.

It, together with the gNB Identifier (using gNBId of the parent

GNBCUCPFunction or GNBDUFunction or OperatorDU (for MOCN network

sharing scenario) or ExternalCUCPFunction), identifies a NR cell within

a PLMN. This is the NR Cell Identity(NCI). See subclause 8.2 of TS38.300.

The NCI can be constructed by encoding the gNB Identifier using gNBId

(of the parent GNBCUCPFunction or GNBDUFunction or OperatorDU

(for MOCN network sharing scenario) or ExternalCUCPFunction) and

cellLocalId where the gNB Identifier field is of length specified by

gNBIdLength (of the parent GNBCUCPFunction or GNBDUFunction or

ExternalCUCPFunction). See 'Global gNB ID' in subclause 9.3.1.6 of

TS 38.413.

The NR Cell Global identifier (NCGI) is constructed from the

PLMN identity the cell belongs to and the NR Cell Identifier (NCI) of

the cell.

See relation between NCI and NCGI subclause 8.2 of TS 38.300 ";

mandatory true;

type int32;

}

leaf nRPCI {

description "This holds the Physical Cell Identity (PCI) of the NR cell.

allowedValues:

See 3GPP TS 36.211 subclause 6.11 for legal values of pci.";

mandatory true;

type int64;

}

list pLMNIdList {

description "Defines which PLMNs that are assumed to be served by the

NR cell in another gNB CU-CP. This list is either updated by the

managed element itself (e.g. due to ANR, signalling over Xn, etc.) or

by consumer over the standard interface.";

key "mcc mnc";

min-elements 1;

max-elements 12;

uses types3gpp:PLMNId;

}

leaf nRFrequencyRef {

description "Reference to corresponding NRFrequency instance.";

mandatory true;

type types3gpp:DistinguishedName;

}

}

grouping ExternalNRCellCUSubtree {

list ExternalNRCellCU {

description "This IOC contains necessary attributes for inter-system

and intra-system handover. It also contains a subset of the attributes

of related IOCs controlled by Management Service Provider.";

key id;

uses top3gpp:Top_Grp;

container attributes {

uses ExternalNRCellCUGrp;

}

uses mf3gpp:ManagedFunctionContainedClasses;

}

}

}

<CODE ENDS>

*** END OF CHANGE 18 ***

*** START OF CHANGE 19 ***

*** yang-models/_3gpp-nr-nrm-externalservinggwfunction.yang ***

<CODE BEGINS>

module _3gpp-nr-nrm-externalservinggwfunction {

yang-version 1.1;

namespace "urn:3gpp:sa5:_3gpp-nr-nrm-externalservinggwfunction";

prefix "extservgw3gpp";

import _3gpp-common-managed-function { prefix mf3gpp; }

//import _3gpp-common-subnetwork { prefix subnet3gpp; }

import _3gpp-common-top { prefix top3gpp; }

organization "3GPP SA5";

description "Defines the YANG mapping of the ExternalServingGWFunction

Information Object Class (IOC) that is part of the NR Network Resource

Model (NRM).

Copyright 2023, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TSDSI,

TTA, TTC). All rights reserved.";

reference "3GPP TS 28.541 5G Network Resource Model (NRM)";

revision 2025-04-25 { reference "CR-1526 CR-1527" ; } // common for r18, r19

revision 2023-09-18 { reference CR-1043 ; }

revision 2019-10-28 { reference S5-193518 ; }

revision 2019-06-17 {

description "Initial revision";

}

grouping ExternalServingGWFunctionGrp {

description "Represents the ExternalServingGWFunction IOC.";

reference "3GPP TS 28.541";

uses mf3gpp:ManagedFunctionGrp;

}

grouping ExternalServingGWFunctionWrapper {

list ExternalServingGWFunction {

description "Represents the properties, known by the management

function, of a ServingGWFunction managed by another management

function.";

reference "3GPP TS 28.658";

key id;

uses top3gpp:Top_Grp;

container attributes {

uses ExternalServingGWFunctionGrp;

}

uses mf3gpp:ManagedFunctionContainedClasses;

}

}

/*augment "/subnet3gpp:SubNetwork" {

if-feature subnet3gpp:ExternalsUnderSubNetwork ;

uses ExternalServingGWFunctionWrapper;

}

augment "/eutranet3gpp:EUtraNetwork" {

if-feature eutranet3gpp:ExternalsUnderEUtraNetwork;

uses ExternalServingGWFunctionWrapper;

}*/

}

<CODE ENDS>

*** END OF CHANGE 19 ***

*** START OF CHANGE 20 ***

*** yang-models/_3gpp-nr-nrm-externalupffunction.yang ***

<CODE BEGINS>

module _3gpp-nr-nrm-externalupffunction {

yang-version 1.1;

namespace "urn:3gpp:sa5:_3gpp-nr-nrm-externalupffunction";

prefix "extupf3gpp";

import _3gpp-common-managed-function { prefix mf3gpp; }

/*import _3gpp-common-subnetwork { prefix subnet3gpp; }

import _3gpp-nr-nrm-nrnetwork { prefix nrnet3gpp; }*/

import _3gpp-common-top { prefix top3gpp; }

organization "3GPP SA5";

contact "https://www.3gpp.org/DynaReport/TSG-WG--S5--officials.htm?Itemid=464";

description "Defines the YANG mapping of the ExternalUPFFunction Information

Object Class (IOC) that is part of the NR Network Resource Model (NRM).

Copyright 2025, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TSDSI,

TTA, TTC). All rights reserved.";

reference "3GPP TS 28.541 5G Network Resource Model (NRM)";

revision 2025-04-25 { reference "CR-1526 CR-1527" ; } // common for r18, r19

revision 2023-09-18 { reference CR-1043 ; }

revision 2019-10-28 { reference S5-193518 ; }

revision 2019-06-17 {

description "Initial revision";

}

grouping ExternalUPFFunctionGrp {

description "Represents the ExternalUPFFunction IOC.";

reference "3GPP TS 28.541";

uses mf3gpp:ManagedFunctionGrp;

}

grouping ExternalUPFFunctionWrapper {

list ExternalUPFFunction {

description "Represents the properties, known by the management

function, of a UPFFunction managed by another management

function.";

reference "3GPP TS 28.541";

key id;

uses top3gpp:Top_Grp;

container attributes {

uses ExternalUPFFunctionGrp;

}

uses mf3gpp:ManagedFunctionContainedClasses;

}

}

/*augment "/subnet3gpp:SubNetwork" {

if-feature subnet3gpp:ExternalsUnderSubNetwork ;

uses ExternalUPFFunctionWrapper;

}

augment "/nrnet3gpp:NRNetwork" {

if-feature nrnet3gpp:ExternalsUnderNRNetwork;

uses ExternalUPFFunctionWrapper;

}*/

}

<CODE ENDS>

*** END OF CHANGE 20 ***

*** START OF CHANGE 21 ***

*** yang-models/_3gpp-nr-nrm-nrfrequency.yang ***

<CODE BEGINS>

module _3gpp-nr-nrm-nrfrequency {

yang-version 1.1;

namespace "urn:3gpp:sa5:_3gpp-nr-nrm-nrfrequency";

prefix "nrfreq3gpp";

import _3gpp-common-managed-element { prefix me3gpp; }

import _3gpp-nr-nrm-gnbcucpfunction { prefix gnbcucp3gpp; }

import _3gpp-nr-nrm-nrnetwork { prefix nrnet3gpp; }

import _3gpp-common-subnetwork { prefix subnet3gpp; }

import _3gpp-common-top { prefix top3gpp; }

organization "3GPP SA5";

contact "https://www.3gpp.org/DynaReport/TSG-WG--S5--officials.htm?Itemid=464";

description "Defines the YANG mapping of the NRFrequency Information Object

Class (IOC) that is part of the NR Network Resource Model (NRM).

Copyright 2025, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TSDSI,

TTA, TTC). All rights reserved.";

reference "3GPP TS 28.541 5G Network Resource Model (NRM)";

revision 2025-04-25 { reference CR-1526 ; }

revision 2023-09-18 { reference CR-1043 ; }

revision 2019-10-28 { reference S5-193518 ; }

revision 2019-06-17 { reference "Initial version"; }

grouping NRFrequencyGrp {

description "Represents the NRFrequency IOC.";

leaf absoluteFrequencySSB {

description "The absolute frequency applicable for a downlink NR carrier

frequency associated with the SSB, in terms of NR-ARFCN.";

mandatory true;

type uint32 { range "0.. 3279165"; }

}

leaf ssBSubCarrierSpacing {

description "Sub-carrier spacing of the SSB.

Note that the allowed values of SSB used for representing data,

by e.g. a BWP, are: 15, 30, 60 and 120 in units of kHz.";

mandatory true;

type uint8 { range "15 | 30 | 60 | 120 | 240"; }

units "kHz";

}

leaf multiFrequencyBandListNR {

description "List of additional frequency bands the frequency belongs to.

The list is automatically set by the gNB.";

mandatory true;

config false;

type uint16 { range "1..256"; }

}

}

grouping NRFrequencyWrapper {

list NRFrequency {

description "Represents certain NR frequency properties.";

key id;

uses top3gpp:Top_Grp;

container attributes {

uses NRFrequencyGrp;

}

}

}

feature NRFrequencyUnderNRNetwork {

description "NRFrequency shall be contained under NRNetwork";

}

augment "/me3gpp:ManagedElement/gnbcucp3gpp:GNBCUCPFunction/"

+ "nrnet3gpp:NRNetwork" {

if-feature NRFrequencyUnderNRNetwork;

uses NRFrequencyWrapper;

}

feature NRFrequencyUnderSubNetwork {

description "NRFrequency shall be contained under SubNetwork";

}

augment "/subnet3gpp:SubNetwork" {

if-feature NRFrequencyUnderSubNetwork;

uses NRFrequencyWrapper;

}

}

<CODE ENDS>

*** END OF CHANGE 21 ***

*** START OF CHANGE 22 ***

*** yang-models/_3gpp-nr-nrm-nrnetwork.yang ***

<CODE BEGINS>

module _3gpp-nr-nrm-nrnetwork {

yang-version 1.1;

namespace "urn:3gpp:sa5:_3gpp-nr-nrm-nrnetwork";

prefix "nrnet3gpp";

import _3gpp-common-managed-element { prefix me3gpp; }

import _3gpp-nr-nrm-gnbcucpfunction { prefix gnbcucp3gpp; }

import _3gpp-common-top { prefix top3gpp; }

organization "3GPP SA5";

contact "https://www.3gpp.org/DynaReport/TSG-WG--S5--officials.htm?Itemid=464";

description "Defines the YANG mapping of the NRNetwork Information Object

Class (IOC) that is part of the NR Network Resource Model (NRM).

Copyright 2025, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TSDSI,

TTA, TTC). All rights reserved.";

reference "3GPP TS 28.541 5G Network Resource Model (NRM)";

revision 2025-04-25 { reference "CR-1526 CR-1527" ; } // common r17, r18

revision 2023-09-18 { reference CR-1043 ; }

revision 2019-06-17 { reference "Initial revision"; }

grouping NRNetworkGrp {

description "Represents the NRNetwork IOC.

Contains no additiona attributes.";

}

augment /me3gpp:ManagedElement/gnbcucp3gpp:GNBCUCPFunction {

list NRNetwork {

description "Represents a set of NR frequencies and external NR entities

(e.g. ExternalGNBCUCPFunction). This is used to differentiate these

entities and frequencies from those in EUTRAN.";

key id;

uses top3gpp:Top_Grp;

container attributes {

uses NRNetworkGrp;

}

}

}

}

<CODE ENDS>

*** END OF CHANGE 22 ***

*** START OF CHANGE 23 ***

*** yang-models/_3gpp-nr-nrm-nroperatorcelldu.yang ***

<CODE BEGINS>

module _3gpp-nr-nrm-nroperatorcelldu {

yang-version 1.1;

namespace "urn:3gpp:sa5:_3gpp-nr-nrm-nroperatorcelldu";

prefix "nropcelld3gpp";

import _3gpp-common-top { prefix top3gpp; }

import _3gpp-common-managed-element { prefix me3gpp; }

import _3gpp-nr-nrm-operatordu { prefix operdu3gpp; }

import _3gpp-5g-common-yang-types { prefix types5g3gpp; }

import _3gpp-common-yang-types { prefix types3gpp; }

import _3gpp-nr-nrm-gnbdufunction { prefix gnbdu3gpp; }

organization "3GPP SA5";

contact "https://www.3gpp.org/DynaReport/TSG-WG--S5--officials.htm?Itemid=464";

description "Defines the YANG mapping of the OperatorDU Information Object

Class (IOC) that is part of the NR Network Resource Model (NRM).

Copyright 2025, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TSDSI,

TTA, TTC). All rights reserved.";

reference "3GPP TS 28.541 5G Network Resource Model (NRM)";

revision 2025-05-06 { reference "CR-1526 CR-1527" ; }  // common for R18, R19

revision 2024-05-24 { reference CR-1273 ; }

revision 2023-09-18 { reference CR-1043 ; }

revision 2021-10-01 { reference "Initial revision"; }

grouping NROperatorCellDUGrp {

description "Represents the NROperatorCellDU IOC.";

reference "3GPP TS 28.541";

uses operdu3gpp:OperatorDUGrp;

leaf cellLocalId {

description "Identifies an NR cell of a gNB. Together with the

corresponding gNB identifier in forms the NR Cell Identity (NCI).";

reference "NCI in 3GPP TS 38.300";

type int32 { range "0..16383"; }

}

leaf administrativeState  {

description "Administrative state of the NROperatorCellDU. Indicates the

permission to use or prohibition against using the cell, imposed

through the OAM services.";

type types3gpp:AdministrativeState;

default LOCKED;

}

list pLMNInfoList {

description "The PLMNInfoList is a list of PLMNInfo data type. It

defines which PLMNs that can be served by the NR cell, and which

S-NSSAIs that can be supported by the NR cell for corresponding PLMN

in case of network slicing feature is supported. The plMNId of the

first entry of the list is the PLMNId used to construct the nCGI for

the NR cell.";

key "mcc mnc sd sst";

min-elements 1;

ordered-by user;

uses types5g3gpp:PLMNInfo;

}

leaf nRTAC {

description "The common 5GS Tracking Area Code for the PLMNs.";

reference "3GPP TS 23.003, 3GPP TS 38.473";

type types5g3gpp:NRTAC;

}

leaf-list nRCellDURef {

description "Reference to corresponding NRCellDU instance.";

type types3gpp:DistinguishedName;

}

}

augment "/me3gpp:ManagedElement/gnbdu3gpp:GNBDUFunction/operdu3gpp:OperatorDU"

{

list NROperatorCellDU {

description "Contains attributes to support 5G MOCN network sharing.";

reference "3GPP TS 28.541";

key id;

uses top3gpp:Top_Grp;

container attributes {

uses NROperatorCellDUGrp;

}

}

}

}

<CODE ENDS>

*** END OF CHANGE 23 ***

*** START OF CHANGE 24 ***

*** yang-models/_3gpp-nr-nrm-operatordu.yang ***

<CODE BEGINS>

module _3gpp-nr-nrm-operatordu {

yang-version 1.1;

namespace "urn:3gpp:sa5:_3gpp-nr-nrm-operatordu";

prefix "operdu3gpp";

import _3gpp-common-top { prefix top3gpp; }

import _3gpp-common-managed-element { prefix me3gpp; }

import _3gpp-nr-nrm-gnbdufunction {prefix gnbdu3gpp; }

import _3gpp-common-yang-types { prefix types3gpp; }

import _3gpp-nr-nrm-ep { prefix ep3gpp; }

organization "3GPP SA5";

contact "https://www.3gpp.org/DynaReport/TSG-WG--S5--officials.htm?Itemid=464";

description "Defines the YANG mapping of the OperatorDU Information Object

Class (IOC) that is part of the NR Network Resource Model (NRM).

Copyright 2025, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TSDSI,

TTA, TTC). All rights reserved.";

reference "3GPP TS 28.541 5G Network Resource Model (NRM)";

revision 2025-05-06 { reference "CR-1526 CR-1527" ; }  // common for R18, R19

revision 2024-02-24 { reference CR-1218; }

revision 2023-11-14 { reference "CR1136"; }

revision 2023-09-30 { reference "CR1048"; }

revision 2023-09-18 { reference CR-1043 ; }

revision 2021-10-01 { reference "Initial revision"; }

grouping OperatorDUGrp {

description "Represents the OperatorDU IOC.";

reference "3GPP TS 28.541";

leaf gNBId {

type int64 { range "0..4294967295"; }

mandatory true;

description "Identifies a gNB within a PLMN. The gNB Identifier (gNB ID)

is part of the NR Cell Identifier (NCI) of the gNB cells.";

reference "gNB ID in 3GPP TS 38.300, Global gNB ID in 3GPP TS 38.413";

}

leaf gNBIdLength {

type int32 { range "22..32"; }

mandatory true;

description "Indicates the number of bits for encoding the gNB ID.";

reference "gNB ID in 3GPP TS 38.300, Global gNB ID in 3GPP TS 38.413";

}

leaf configurable5QISetRef {

type types3gpp:DistinguishedName;

description "DN of the Configurable5QISet that the OperatorDU

supports (is associated to).";

}

leaf dynamic5QISetRef {

type types3gpp:DistinguishedName;

description "DN of the Dynamic5QISet that the OperatorDU

supports (is associated to).";

}

}

augment "/me3gpp:ManagedElement/gnbdu3gpp:GNBDUFunction" {

list OperatorDU {

description "This IOC contains attributes to support operator specific

gNB-DU level information to support the NG-RAN Multi-Operator Core

Network (NG-RAN MOCN) network sharing with multiple Cell Identity

broadcast feature. An instance of OperatorDU <<IOC>> should be created

and configured for each POP. When configured the attributes override

those in parent GNBDUFunction instance.

The OperatorDU <<IOC>> is only used to support NG-RAN MOCN with

multiple cell identity broadcast feature. If NG-RAN MOCN with multiple

cell identity broadcast feature is not supported, is not used.

For scenarios with an F1 interface supporting multiple PLMN broadcast,

the values of the EP_F1C and EP_F1U attributes contained by different

OperatorDU of the same GNBDUFunction should be same.";

reference "3GPP TS 28.541";

key id;

uses top3gpp:Top_Grp;

container attributes {

uses OperatorDUGrp;

}

uses ep3gpp:EP_F1CSubtree {

refine EP_F1C {

max-elements 1;

}

}

uses ep3gpp:EP_F1USubtree;

}

}

}

<CODE ENDS>

*** END OF CHANGE 24 ***