3GPP TSG-SA5 Meeting #161	S5-253055

# Fukuoka, Japan, 19th May 2025 - 23rd May 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 28.541 | CR | 1548 | rev | 1 | Current version: | 19.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Rel-19 TS 28.541 Correction on wrong attribute property definition |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI19 |  |  |  |  |  | Date: |  |  | 2025-05-09 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | There is a typo for the attribute property isWritable of isOnboardSatellite and onboardSatelliteId in clause 4.3.2.2 as well as it is missing stage 3 yang definition. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Correct stage 2 isWritable typo for isOnboardSatellite and onboardSatelliteId Add missing stage 3 yang definition of isOnboardSatellite and onboardSatelliteId for GNBCUCPFunction. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Wrong attribute property definition causes confusion. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.3.2.2 , Forge |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | Forge MR link: https://forge.3gpp.org/rep/sa5/MnS/-/merge_requests/1774 at commit 7a521607f01bf25a2a60ee02f739b6c192f5e58e |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

1st change

#### 4.3.2.2	Attributes

The GNBCUCPFunction IOC includes attributes inherited from ManagedFunction IOC (defined in TS 28.622[30]) and the following attributes:

| Attribute name | S | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| gNBId | M | T | T | F | T |
| gNBIdLength | M | T | T | F | T |
| gNBCUName | O | T | T | F | T |
| pLMNId | M | T | T | T | T |
| x2BlockList | CM | T | T | F | T |
| x2AllowList | CM | T | T | F | T |
| xnBlockList | M | T | T | F | T |
| xnAllowList | M | T | T | F | T |
| x2HOBlockList | CM | T | T | F | T |
| xnHOBlockList | M | T | T | F | T |
| mappingSetIDBackhaulAddressList | CM | T | T | F | T |
| tceIDMappingInfoList | CM | T | T | F | T |
| dDAPSHOControl | CM | T | T | F | T |
| dCHOControl | CM | T | T | F | T |
| qceIdMappingInfoList | CM | T | T | F | T |
| mdtUserConsentReqList | CM | T | T | F | T |
| mappedCellIdInfoList | CO | T | T | F | T |
| isOnboardSatellite | O | T | F | F | T |
| onboardSatelliteId | O | T | T | F | T |
| Attribute related to role |  |  |  |  |  |
| configurable5QISetRef | CO | T | T | F | T |
| dynamic5QISetRef | CO | T | F | F | T |
| ephemerisInfoSetRef | CO | T | F | F | T |
| nRECMappingRuleRef | CM | T | T | F | T |
| mWABRef | CO | T | T | F | T |

#### 4.3.2.3	Attribute constraints

| Name | Definition |
| --- | --- |
| x2BlockList | Condition: Multi-Radio Dual Connectivity with the EPC (see TS 37.340 [9] clause 4.1.2) is supported. |
| x2AllowList | Condition: Multi-Radio Dual Connectivity with the EPC (see TS 37.340 [9] clause 4.1.2) is supported. |
| x2HOBlockList | Condition: Multi-Radio Dual Connectivity with the EPC (see TS 37.340 [9] clause 4.1.2) is supported. |
| mappingSetIDBackhaulAddressList | Condition: Remote Interference Management function is supported. |
| tceIDMappingInfolist | Condition: MDT Function is supported. |
| dDAPSHOControl | Condition: DAPS is supported. |
| dCHOControl | Condition: CHO is supported. |
| mappedCellIdInfoList | Condition: NTN is supported. |
| configurable5QISetRef | Condition: Configurable5QISet is name contained by SubNetwork or ManagedElement |
| dynamic5QISetRef | Condition: Dynamic5QISet is name contained by SubNetwork or ManagedElement |
| ephemerisInfoSetRef | Condition: NTN is supported. |
| qceIdMappingInfoList | Condition: QMC Function is supported. |
| mdtUserConsentReqList | Condition: MDT Function is supported. |
| nRECMappingRuleRef | Condition: Energy cost reporting feature is supported in non-split NG-RAN deployment scenario. |
| mWABRef | Condition: MWAB feature is supported. |

#### 4.3.2.4	Notifications

The common notifications defined in subclause 4.5 are valid for this IOC, without exceptions or additions.

Next change

*** yang-models/_3gpp-nr-nrm-gnbcucpfunction.yang ***

<CODE BEGINS>

module _3gpp-nr-nrm-gnbcucpfunction {

yang-version 1.1;

namespace "urn:3gpp:sa5:_3gpp-nr-nrm-gnbcucpfunction";

prefix "gnbcucp3gpp";

import _3gpp-common-yang-types { prefix types3gpp; }

import _3gpp-5g-common-yang-types { prefix types5g3gpp; }

import _3gpp-common-yang-extensions { prefix yext3gpp; }

import _3gpp-common-managed-function { prefix mf3gpp; }

import _3gpp-common-managed-element { prefix me3gpp; }

import _3gpp-common-top { prefix top3gpp; }

import _3gpp-5gc-nrm-configurable5qiset { prefix fiveqi3gpp; }

import ietf-inet-types { prefix inet; }

organization "3GPP SA5";

contact "https://www.3gpp.org/DynaReport/TSG-WG--S5--officials.htm?Itemid=464";

description "Defines the YANG mapping of the GNBCUCPFunction Information

Object Class (IOC) that is part of the NR Network Resource Model (NRM).

Copyright 2025, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TSDSI,

TTA, TTC). All rights reserved.";

reference "3GPP TS 28.541 5G Network Resource Model (NRM)";

revision 2025-05-07 { reference "CR-1548"; }

revision 2025-03-25 { reference "CR-1509"; }

revision 2025-01-25 { reference "CR-1442"; }

revision 2024-11-07 { reference "CR-1443"; }

revision 2024-08-19 { reference "CR-1405"; }

revision 2024-08-19 { reference "CR-1337 CR-1352"; }

revision 2024-05-24 { reference CR-1273 ; }

revision 2024-04-04 { reference CR-1139; }

revision 2024-02-24 { reference CR-1218; }

revision 2024-01-12 { reference CR-1138; }

revision 2023-09-18 { reference CR-1043; }

revision 2023-04-26 { reference CR-0916; }

revision 2022-07-28 { reference "CR-0770"; }

revision 2021-11-06 { reference "CR-0611" ; }

revision 2021-11-05 { reference "CR-0609"; }

revision 2020-10-02 { reference CR-0384; }

revision 2020-08-06 { reference "CR-0333"; }

revision 2020-08-03 { reference "CR-0321"; }

revision 2020-06-03 { reference "CR-0286"; }

revision 2020-05-08 { reference S5-203316 ; }

revision 2020-04-28 { reference "0260"; }

revision 2020-02-14 { reference S5-20XXXX ; }

revision 2019-10-28 { reference S5-193518 ; }

revision 2019-06-17 { reference "Initial revision"; }

feature Configurable5QISetUnderGNBCUCPFunction {

description "The Configurable5QISet shall be contained under

GNBCUCPFunction";

}

feature DESManagementFunction {

description "Class representing Distributed SON Energy Saving feature";

}

feature DANRManagementFunction {

description "Class representing D-SON function of ANR Management feature";

}

feature DMROFunction {

description "Class representing D-SON function of MRO feature";

}

grouping BackhaulAddressGrp {

description "This data type represents the properties describing the

backhaul address of gNB. ";

leaf gNBId {

type uint32 ;

mandatory true;

description "It identifies a gNB within a PLMN. The gNB ID is part of

the NR Cell Identifier (NCI) of the gNB cells.";

reference "gNB Identifier (gNB ID) of subclause 8.2 of TS 38.300.

Global gNB ID in subclause 9.3.1.6 of TS 38.413";

}

list tAI {

key tac;

min-elements 1;

max-elements 1;

description "Tracking Area Identity";

reference "subclause 9.3.3.11 in TS 38.413";

uses types3gpp:TaiGrp;

}

}

grouping MappingSetIDBackhaulAddressGrp {

description "This data type represents the properties describing the

mapping relationship between set ID and backhaul address of gNB.";

leaf setId {

type uint32 {

range 0..4194304;  // 22 bits

}

mandatory true;

description "This specifies the set ID of a victim Set (RIM-RS1 Set) or

aggressor Set (RIM-RS2 set). (See subclause 7.4.1.6 in TS 38.211).

allowedValues:

The bit length of the set ID is maximum 22bit.";

}

list backhaulAddress  {

description "The attribute specifies backhaulAddress which is defined as

a datatype.";

min-elements 1;

max-elements 1;

key gNBId;

uses BackhaulAddressGrp;

}

}

grouping TceIDMappingInfoGrp {

description "This data type represents the properties describing the

mapping relationship between TCE ID, PLMN where TCE resides and

IP address of TCE.";

leaf tceIPAddress {

type inet:ip-address;

mandatory true;

description "This attribute indicates IP address of TCE.

(See subclause 4.1.1.9.2 in TS 32.422)";

}

leaf tceID {

type int64;

mandatory true;

description "This attribute indicates TCE Id.

(See subclause 4.1.1.9.2 in TS 32.422)";

}

list pLMNTarget {

description "Indicates the PLMN where TCE resides.

(See subclauses 4.1.1.9.2 and 4.9.2 in TS 32.422)";

min-elements 1;

max-elements 1;

key mnc;

uses types3gpp:PLMNId;

}

}

grouping GNBCUCPFunctionGrp {

description "Represents the GNBCUCPFunction IOC.";

reference "3GPP TS 28.541";

uses mf3gpp:ManagedFunctionGrp;

leaf gNBId {

description "Identifies a gNB within a PLMN. The gNB Identifier (gNB ID)

is part of the NR Cell Identifier (NCI) of the gNB cells.";

reference "gNB ID in 3GPP TS 38.300, Global gNB ID in 3GPP TS 38.413";

mandatory true;

type int64 { range "0..4294967295"; }

}

leaf gNBIdLength {

description "Indicates the number of bits for encoding the gNB ID.";

reference "gNB ID in 3GPP TS 38.300, Global gNB ID in 3GPP TS 38.413";

mandatory true;

type int32 { range "22..32"; }

}

leaf gNBCUName {

description "Identifies the Central Unit of an gNB.";

reference "3GPP TS 38.473";

mandatory true;

type string { length "1..150"; }

}

list pLMNId {

description "The PLMN identifier to be used as part of the global RAN

node identity.";

key "mcc mnc";

min-elements 1;

max-elements 1;

yext3gpp:inVariant;

uses types3gpp:PLMNId;

}

list x2BlockList {

description "List of nodes to which X2 connections are prohibited.";

key idx;

leaf idx {type uint32;}

uses GeNBIdGrp;

}

list x2AllowList {

description "List of nodes to which X2 connections are enforced.";

key idx;

leaf idx {type uint32;}

uses GeNBIdGrp;

}

list xnBlockList {

description "List of nodes to which Xn connections are prohibited.";

key idx;

leaf idx {type uint32;}

uses GgNBIdGrp;

}

list xnAllowList {

description "List of nodes to which X2 connections are enforced.";

key idx;

leaf idx {type uint32;}

uses GgNBIdGrp;

}

list x2HOBlockList {

description "List of nodes to which handovers over X2 are prohibited.";

key idx;

leaf idx {type uint32;}

uses GeNBIdGrp;

}

list xnHOBlockList {

description "List of nodes to which handovers over  Xn are prohibited.";

key idx;

leaf idx {type uint32;}

uses GgNBIdGrp;

}

list mappingSetIDBackhaulAddressList {

description "The attribute specifies a list of mappingSetIDBackhaulAddress

which is defined as a datatype. Which is used to

retrieve the backhaul address of the victim set.";

min-elements 1;

key idx;

leaf idx {type uint32; }

uses MappingSetIDBackhaulAddressGrp;

}

list tceIDMappingInfoList {

description "This attribute includes a list of TCE ID, PLMN where TCE

resides and the corresponding TCE IP address. It is used in Logged MDT

case to provide the information to the gNodeB or GNBCUCPFunction to

get the corresponding TCE IP address when there is an MDT log

received from the UE.";

min-elements 1;

key idx;

leaf idx { type uint32; }

uses TceIDMappingInfoGrp;

}

leaf dDAPSHOControl {

type boolean;

description "This attribute determines whether the DAPS handover function

is enabled or disabled.";

}

leaf dCHOControl {

type boolean;

description "This attribute determines whether the CHO function is

enabled or disabled.";

}

leaf dLTMControl {

type boolean;

default false;

description "This attribute determines whether the distributed SON

function LTM Cell Switch is enabled or disabled.";

}

list qceIdMappingInfoList {

description "List of the mapping relationship between QoE collection entity

identity, PLMN where QoE collection entity resides, and the IP address of

the QoE collection entity.";

key idx;

min-elements 1;

uses QceIdMappingInfoGrp;

leaf idx { type string; }

}

leaf-list mdtUserConsentReqList {

type enumeration {

enum M1;

enum M2;

enum M3;

enum M4;

enum M5;

enum M6;

enum M7;

enum M8;

enum M9;

enum MDT_UE_LOCATION;

}

description "represents a list of MDT measurement names that are

subject to user consent at MDT activation.

Any MDT measurement, whose name is not specified in this list, is not

subject to user consent at MDT activation.";

}

list mappedCellIdInfoList  {

description "This attribute provides the list of mapping between

geographical location and Mapped Cell ID.";

key idx;

leaf idx { type uint32; }

leaf isOnboardSatellite {

type boolean;

default false;

description "This attribute indicates whether the function is

on board the satellite.";

}

leaf onboardSatelliteId {

type string;

config false;

description "This attribute indicates the onboard satellite Id.

It shall be formatted as a fixed 5-digit string, padding with

leading digits '0' to complete a 5-digit length.";

}

}

leaf configurable5QISetRef {

type types3gpp:DistinguishedName;

description "DN of the Configurable5QISet that the GNBCUCPFunction

supports (is associated to).";

}

leaf dynamic5QISetRef {

type types3gpp:DistinguishedName;

description "DN of the Dynamic5QISet that the GNBCUCPFunction supports

(is associated to).";

}

leaf ephemerisInfoSetRef {

type types3gpp:DistinguishedName;

description "This is the DN of EphemerisInfoSet.";

}

leaf nRECMappingRuleRef {

type types3gpp:DistinguishedName;

description "DN of a NRECMappingRule.  An empty value indicates the

NRECMappingRule contained by parent, e.g. ManagedElement or

SubNetwork, applies.";

}

leaf mWABRef {

type types3gpp:DistinguishedName;

}

}

grouping QceIdMappingInfoGrp {

leaf qoECollectionEntityAddress {

type inet:ip-address;

description "Specifies the address to which the QMC reports shall be

transferred.  Ipv4 or Ipv6 address may be used.";

}

leaf qoECollectionEntityIdentity {

type string;

description "Specifies the unique identity to which the QMC reports

shall be transferred.";

}

list pLMNTarget {

description "The PLMN identifier where QoE collection entity

resides. ";

key "mcc mnc";

min-elements 1;

max-elements 1;

yext3gpp:inVariant;

uses types3gpp:PLMNId;

}

}

mandatory true;

grouping GgNBIdGrp {

description "Represents the properties of a global gNB ID (GgNBId).";

uses types3gpp:PLMNId;

leaf gnbIdLength {

type int32 {range "22..32";}

mandatory true;

description "This indicates the number of bits for encoding the gNB ID.

See 'Global gNB ID' in subclause 9.3.1.6 of TS 38.413";

}

leaf gnbId {

description "Identifies a gNB within a PLMN. The gNB Identifier (gNB ID)

is part of the NR Cell Identifier (NCI) of the gNB cells.";

reference "gNB ID in 3GPP TS 38.300, Global gNB ID in 3GPP TS 38.413";

mandatory true;

type int64 { range "0..4294967295"; }

}

}

grouping GeNBIdGrp {

description "Represents the properties of a global eNB ID (GeNBId).";

uses types3gpp:PLMNId;

leaf enbId {

description "It identifies an eNB within a PLMN. The eNB ID is part of

the E-UTRAN Cell Global Identifier (ECGI) of the eNB cells.";

reference "eNB ID in 3GPP TS 36.300. Global eNB ID in 3GPP TS 36.413.";

mandatory true;

type int32 {range "0..4194303";}

}

}

augment "/me3gpp:ManagedElement" {

list GNBCUCPFunction {

description "Represents the logical function CU-CP of gNB and en-gNB.";

reference "3GPP TS 28.541";

key id;

uses top3gpp:Top_Grp;

container attributes {

uses GNBCUCPFunctionGrp;

}

uses mf3gpp:ManagedFunctionContainedClasses;

uses fiveqi3gpp:Configurable5QISetSubtree {

if-feature Configurable5QISetUnderGNBCUCPFunction;

}

}

}

}

<CODE ENDS>

Next change

*** yang-models/_3gpp-nr-nrm-gnbdufunction.yang ***

<CODE BEGINS>

module _3gpp-nr-nrm-gnbdufunction {

yang-version 1.1;

namespace "urn:3gpp:sa5:_3gpp-nr-nrm-gnbdufunction";

prefix "gnbdu3gpp";

import _3gpp-common-managed-function { prefix mf3gpp; }

import _3gpp-common-managed-element { prefix me3gpp; }

import _3gpp-common-top { prefix top3gpp; }

import _3gpp-common-yang-types { prefix types3gpp; }

import _3gpp-common-yang-extensions { prefix yext3gpp; }

import _3gpp-5gc-nrm-configurable5qiset { prefix fiveqi3gpp; }

organization "3GPP SA5";

contact "https://www.3gpp.org/DynaReport/TSG-WG--S5--officials.htm?Itemid=464";

description "Defines the YANG mapping of the GNBDUFunction Information

Object Class (IOC) that is part of the NR Network Resource Model (NRM).

Copyright 2025, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TSDSI,

TTA, TTC). All rights reserved.";

reference "3GPP TS 28.541 5G Network Resource Model (NRM)";

revision 2025-05-07 { reference CR-1548; }

revision 2025-02-05 { reference CR-1441; }

revision 2024-02-24 { reference CR-1218; }

revision 2023-09-18 { reference CR-1043 ; }

revision 2023-04-26 { reference CR-0916; }

revision 2022-11-02 { reference "CR-0753"; }

revision 2022-07-28 { reference "CR-0770"; }

revision 2021-10-28 { reference CR-0607 ; }

revision 2021-04-30 { reference CR-0490 ; }

revision 2020-10-02 { reference CR-0384 ; }

revision 2020-03-12 { reference "SP-200233 S5-201547" ; }

revision 2020-02-14 { reference S5-20XXXX ; }

revision 2019-10-28 { reference S5-193518 ; }

revision 2019-08-21 {reference "Initial revision."; }

feature Configurable5QISetUnderGNBDUFunction {

description "The Configurable5QISet shall be contained under

GNBDUFunction";

}

feature DRACHOptimizationFunction {

description "Class representing D-SON function of RACH optimization

feature";

}

grouping RimRSReportInfoGrp {

description "This  data type defines necessary reporting information

derived from the detected RIM-RS, including

1)  The detected set ID;

2)  Propagation delay in number of OFDM symbols

3)  Functionality of the RS (RS-1 or RS-2, Enough or Not enough

mitigation for RS-1).

RS-1 is equivalent to RIM-RS type 1 (see 38.211, subclause 7.4.1.6).

RS-2 is equivalent to RIM-RS type 2 (see 38.211, subclause 7.4.1.6).

Enough mitigation for RS-1 means 'Enough' / 'Not enough' indication

functionality is enabled for RIM RS-1 and RIM-RS type 1 is used to

indicate 'enough mitigation' functionality.

Not enough mitigation for RS-1 means 'Enough' / 'Not enough' indication

functionality is enabled for RIM RS-1 and RIM-RS type 1 is used to

indicate 'Not enough mitigation' functionality.";

leaf detectedSetID {

type uint32 ;

description "Set ID of the detected RIM-RS

allowedValues: 0,1...max{totalnrofSetIdofRS1, totalnrofSetIdofRS2}";

}

leaf propagationDelay {

type uint32 ;

must '. <= ../../maxPropagationDelay' {

error-message "allowedValues: 0, 1.. maxPropagationDelay";

}

description "This attribute indicates the propagation delay of the

detected RIM-RS, in number of OFDM symbol.";

}

leaf functionalityOfRIMRS {

type enumeration {

enum RS1;

enum RS2;

enum RS1_FOR_ENOUGH_MITIGATION;

enum RS1_FOR_NOT_ENOUGH_MITIGATION;

}

mandatory true;

description "Indicates the functionality of the detected RIM-RS.

If the indication of enableEnoughNotEnoughIndication is 'enabled',

valid values are {RS2, RS1forEnoughMitigation,

RS1forNotEnoughMitigation};

If the indication of enableEnoughNotEnoughIndication is 'disabled',

valid values are {RS1, RS2}.

RS1forEnoughMitigation means RIM-RS type 1 is used to indicate

'enough mitigation' functionality.

RS1forNotEnoughMitigation means RIM-RS type 1 is used to indicate

'Not enough mitigation' functionality.";

}

}

grouping RimRSReportConfGrp {

description "Defines RIM-RS reporting configuration";

leaf reportIndicator {

type types3gpp:EnabledDisabled;

default DISABLED;

description "Used to enable or disable the RS report on a gNB.

If the indication is 'enable', the gNB starts to periodically report

necessary information derived from the detected RIM-RS to OAM.

If the indication is 'disable', the gNB stops reporting.";

}

leaf reportInterval {

type uint32;

mandatory true;

units ms;

description "Used to define reporting interval of a gNB in ms.";

}

leaf nrofRIMRSReportInfo {

type uint32;

mandatory true;

description "Used to define the maximum number of RIMRSReportInfo in

a single report.";

}

leaf isOnboardSatellite {

type boolean;

mandatory false;

default false;

description "This attribute indicates whether the function is

on board the satellite.";

}

leaf onboardSatelliteId {

type string;

description "This attribute indicates the onboard satellite Id.

It shall be formatted as a fixed 5-digit string, padding with

leading digits “0” to complete a 5-digit length.";

}

leaf maxPropagationDelay {

type uint32 {

range "0..327679";

}

mandatory true;

description "Used to define the maximum reported OFDM symbol number for

the propagation delay of the detected RIM-RS in each RIMRSReportInfo.

allowedValues: 0, 1..20**2*maxNrofSymbols-1, where maxNrofSymbols=14.";

}

list RimRSReportInfoList {

key detectedSetID;

description "Represents a list (the length of the list is

nrofRIMRSReportInfo) of necessary information derived from the

detected RIM-RS.";

uses RimRSReportInfoGrp;

}

}

grouping GNBDUFunctionGrp {

description "Represents the GNBDUFunction IOC.";

reference "3GPP TS 28.541";

uses mf3gpp:ManagedFunctionGrp;

leaf gNBId {

type int64 { range "0..4294967295"; }

mandatory true;

description "Identifies a gNB within a PLMN. The gNB Identifier (gNB ID)

is part of the NR Cell Identifier (NCI) of the gNB cells.";

reference "gNB ID in 3GPP TS 38.300, Global gNB ID in 3GPP TS 38.413";

}

leaf gNBIdLength {

type int32 { range "22..32"; }

mandatory true;

description "Indicates the number of bits for encoding the gNB ID.";

reference "gNB ID in 3GPP TS 38.300, Global gNB ID in 3GPP TS 38.413";

}

leaf gNBDUId {

type int64 { range "0..68719476735"; }

mandatory true;

description "Uniquely identifies the DU at least within a gNB.";

reference "3GPP TS 38.473";

}

leaf gNBDUName {

type string { length "1..150"; }

description "Identifies the Distributed Unit of an NR node";

reference "3GPP TS 38.473";

}

list rimRSReportConf {

key reportInterval;

config false;

min-elements 1;

max-elements 1;

yext3gpp:inVariant;

description "Used to configure gNBs to report the all necessary

information derived from the detected RIM-RS to OAM.";

uses RimRSReportConfGrp;

}

leaf configurable5QISetRef {

type types3gpp:DistinguishedName;

description "DN of the Configurable5QISet that the GNBDUFunction supports

(is associated to).";

}

leaf dynamic5QISetRef {

type types3gpp:DistinguishedName;

description "DN of the Dynamic5QISet that the GNBDUFunction supports

(is associated to).";

}

leaf nRECMappingRuleRef {

type types3gpp:DistinguishedName;

description "DN of a NRECMappingRule.  An empty value indicates the

NRECMappingRule contained by parent, e.g. ManagedElement or

SubNetwork, applies.";

}

}

augment "/me3gpp:ManagedElement" {

list GNBDUFunction {

key id;

description "Represents the logical function DU of gNB or en-gNB.";

reference "3GPP TS 28.541";

uses top3gpp:Top_Grp;

container attributes {

uses GNBDUFunctionGrp;

}

uses mf3gpp:ManagedFunctionContainedClasses;

uses fiveqi3gpp:Configurable5QISetSubtree {

if-feature Configurable5QISetUnderGNBDUFunction;

}

}

}

}

<CODE ENDS>