3GPP TSG-SA5 Meeting #161	S5-252300

# Fukuoka, Japan, 19th May 2025 - 23rd May 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 28.622 | CR | 0540 | rev | 2 | Current version: | 19.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Rel-19 CR TS 28.622 Update Attribute definition for TimeWindow |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI19 |  |  |  |  |  | Date: |  |  | 2025-05-06 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | The Timewindow is defined as follows: - When startTime and endTime is present or configured by MnS consumer, the time window starts when startTime is reached and ends when endTime is reached. - When only the startTime attribute is present or configured by MnS consumer, the time window starts when startTime is reached and runs until deletion of the managed object instance including this timeWindow. - When only the endTime attribute is present or configured by MnS consumer, the time window starts when the managed object instance including this TimeWindow is created and ends when endTime is reached. It implies both startTime and endTime are mandatory attributes to be supported by MnS producer. However, the Timewindow is defined as choice for different run time conditions instead of design time conditions. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Update Timewindow as an <<dataType>> includes two mandatory attributes startTime and endTime. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Incorrect definition for Timewindow |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.3.48, 4.3.48.1, 4.3.48.2, 4.3.48.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

1st Change

4.3.48	TimeWindow <<dataType>>

4.3.48.1	Definition

This dataType defines a time window.

It is a dataType between the control parameters required to define the time window as follows:

When startTime and endTime is present, the time window starts when startTime is reached and ends when endTime is reached.

When only the startTime attribute is present, the time window starts when startTime is reached and runs until deletion of the managed object instance including this timeWindow.

When only the endTime attribute is present, the time window starts when the managed object instance including this TimeWindow is created and ends when endTime is reached.

4.3.48.2	Attributes

| Attribute name | S | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| startTime | M | T | T | T | T |
| endTime | M | T | T | T | T |

4.3.48.3	Attribute constraints

None

4.3.48.4	Notifications

The clause 4.5 of the <<IOC>> using this <<dataType>> as one of its attributes, shall be applicable.

End of Changes