3GPP TSG-SA5 Meeting #161	S5-252304

# Fukuoka, Japan, 19th May 2025 - 23rd May 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 28.552 | CR | 0706 | rev | - | Current version: | 19.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Rel-19 CR TS 28.552 Update number of active UEs related measurements |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | China Telecom Corporation Ltd. |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S5 |  |  |  |  |  |  |  |  |  |
| Work item code: | PM_KPI_5G_Ph4 |  |  |  |  |  | Date: |  |  | 2025-05-06 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | Monitoring the number of active UEs in RRC_CONNECTED state and RRC_INACTIVE state can help to estimate cell congestion and better configure SDT threshold parameters. It is therefore necessary to update the number of active UEs in different RRC states. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Update number of active UEs related measurements： - Mean number of Active UEs in the DL per cell - Max number of Active UEs in the DL per cell - Mean number of Active UEs in the UL per cell - Max number of Active UEs in the UL per cell |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Cannot monitor the number of active UEs in different RRC states. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.1.1.23 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

1st change

5.1.1.23	Number of Active Ues

5.1.1.23.1	Mean number of Active UEs in the DL per cell

a)	This measurement provides the mean number of active DRBs for UEs in an NRCellDU. The measurement is calculated per PLMN ID and per QoS level (mapped 5QI or/and QCI in NR option 3) and per supported S-NSSAI and per RRC state (mapped RRC_CONNECTED and RRC_INACTIVE).

b)	DER (n=1).

c)	This measurement is obtained by aggregating the measurement " Mean number of Active UEs in the DL per DRB per cell " (see clause 4.2.1.3.2 in TS 38.314 [29]). The measurement is performed per PLMN ID and per QoS level (mapped 5QI or/and QCI in NR option 3) and per supported S-NSSAI and per RRC state.

d)	Each measurement is a single integer value. The number of measurements is equal to the number of PLMNs multiplied by the number of QoS levels or multiplied by the number of supported S-NSSAIs or multiplied by the number of RRC state.

[Total No. of measurement instances] x [No. of filter values for all measurements] (DL and UL) ≤ 100.

e)	The measurement name has the form DRB.MeanActiveUeDl_Filter, 
Where filter is a combination of PLMN ID and QoS level and S-NSSAI and RRC state.

Where PLMN ID represents the PLMN ID, QoS representes the mapped 5QI or/and QCI level, and SNSSAI represents S-NSSAI, and RRC state represents the RRC state.

f)	NRCellDU.

g)	Valid for packet switched traffic.

h)	5GS.

i)	One usage of this measurement is for performance assurance within integrity area (user plane connection quality).

5.1.1.23.2	Max number of Active UEs in the DL per cell

a)	This measurement provides the max number of active DRBs for UEs in an NRCellDU.  The measurement is calculated per PLMN ID and per QoS level (mapped 5QI or/and QCI in NR option 3) and per supported S-NSSAI and per RRC state (mapped RRC_CONNECTED and RRC_INACTIVE).

b)	DER (n=1).

c)	This measurement is defined according to measurement " Max number of Active UEs in the DL per DRB per cell " (see clause 4.2.1.3.3 in TS 38.314 [29]). The measurement is performed per PLMN ID and per QoS level (mapped 5QI or/and QCI in NR option 3) and per supported S-NSSAI and per RRC state.

d)	Each measurement is a single integer value. The number of measurements is equal to the number of PLMNs multiplied by the number of QoS levels or multiplied by the number of supported S-NSSAIs or multiplied by the number of RRC state.

[Total No. of measurement instances] x [No. of filter values for all measurements] (DL and UL) ≤ 100.

e)	The measurement name has the form DRB.MaxActiveUeDl_Filter, 
Where filter is a combination of PLMN ID and QoS level and S-NSSAI and RRC state.

Where PLMN ID represents the PLMN ID, QoS representes the mapped 5QI or/and QCI level, and SNSSAI represents S-NSSAI, and RRC state represents the RRC state.

f)	NRCellDU.

g)	Valid for packet switched traffic.

h)	5GS.

i)	One usage of this measurement is for performance assurance within integrity area (user plane connection quality).

5.1.1.23.3	Mean number of Active UEs in the UL per cell

a)	This measurement provides the mean number of active DRBs for UEs in an NRCellDU.  The measurement is calculated per PLMN ID and per QoS level (mapped 5QI or/and QCI in NR option 3) and per supported S-NSSAI and per RRC state (mapped RRC_CONNECTED and RRC_INACTIVE).

b)	DER (n=1)

c)	This measurement is obtained by aggregating the measurement " Mean number of Active UEs in the UL per DRB per cell " (see clause 4.2.1.3.4 in TS 38.314 [29]). The measurement is performed per PLMN ID and per QoS level (mapped 5QI or/and QCI in NR option 3) and per supported S-NSSAI and per RRC state.

d)	Each measurement is a single integer value. The number of measurements is equal to the number of PLMNs multiplied by the number of QoS levels or multiplied by the number of supported S-NSSAIs or multiplied by the number of RRC state.

[Total No. of measurement instances] x [No. of filter values for all measurements] (DL and UL) ≤ 100.

e)	The measurement name has the form DRB.MeanActiveUeUl_Filter, 
Where filter is a combination of PLMN ID and QoS level and S-NSSAI and RRC state.

Where PLMN ID represents the PLMN ID, QoS representes the mapped 5QI or/and QCI level, and SNSSAI represents S-NSSAI, and RRC state represents the RRC state.

f)	NRCellDU.

g)	Valid for packet switched traffic.

h)	5GS.

i)	One usage of this measurement is for performance assurance within integrity area (user plane connection quality).

5.1.1.23.4	Max number of Active UEs in the UL per cell

a)	This measurement provides the max number of active DRBs for UEs in an NRCellDU.  The measurement is optionally split into subcounters per QoS level (mapped 5QI or/and QCI in NR option 3), subcounters per S-NSSAI and subcounters per PLMN ID and subcounters per RRC state (mapped RRC_CONNECTED and RRC_INACTIVE).

b)	DER (n=1)

c)	This measurement is defined by the measurement " Max number of Active UEs in the UL per DRB per cell " (see clause 4.2.1.3.5 in TS 38.314 [29]). Separate counters are optionally maintained for each mapped 5QI (or/and QCI for option 3) and for each S-NSSAI and for each PLMN ID and for each RRC state.

d)	The number of measurements is equal to one. If the optional QoS level measurement is perfomed, the number of measurements is equal to the number of mapped 5QIs (or/and number of QCI for option 3), and the number of S-NSSAIs, and the number of RRC state.

e)	The measurement name has the form DRB.MaxActiveUeUl, 
DRB.MaxActiveUeUl.QOS where QOS identifies the target quality of service class, and
DRB.MaxActiveUeUl.SNSSAI, where SNSSAI identifies the S-NSSAI and DRB.MaxActiveUeUl.PLMN, where PLMN identifies the PLMN ID, and DRB.MaxActiveUeUl.RRCstate, where RRCstate identifies the RRC state.

f)	NRCellDU.

g)	Valid for packet switched traffic.

h)	5GS.

i)	One usage of this measurement is for performance assurance within integrity area (user plane connection quality).

2nd change

A.60	Monitoring of the number of active UEs in NG-RAN

The number of the active UEs per direction in each cell is a valuable measurement for operators to know how many DRBs are running with buffered data per cell and QoS or S-NSSAI basis. For multi-operator RAN sharing scenario, PLMN basis is needed, too. This kind of information can help operators to tune the admission control parameters for the cell and to estimate load in neighbour cells, to ensure that the UEs admitted achieve the target QoS and that capacity is not over-estimated when distributing load between cells and gNBs.

The evolution of Small Data Transmission (SDT) allows data and/or signalling to be transmitted in each direction not only in the RRC_CONNECTED state, but also in the RRC_INACTIVE state. However, the number of active UEs in RRC_INACTIVE state has less impact on cell congestion than the number of active UEs in RRC_ CONNECTED state. When estimating cell congestion, reducing the weighting factor for the number of active UEs in RRC_INACTIVE state will help to make the judgement more accurate. The number of active UEs in different RRC states can therefore help operators to estimate cell congestion and also to improve configuration threshold parameters such as sdt-DataVolumeThreshold.

End of change