3GPP TSG-SA5 Meeting #161	S5-252396

Fukuoka, Japan, 19 - 23 May 2025

Source:	Samsung

Title:	Solution for triggered CCL

Document for:	Approval

Agenda item:	6.19.4.1

Spec:	3GPP TS 28.567

Version:	0.2.0

Work Item:	CCL

Comments

This contribution proposes the solution for the use case of CCL instantiation based on conditions.

Proposed Changes

* * * First Change * * * *

## 6.2	Class diagram

### 6.2.1	Relationships

Figure 6.2.1-1: Relations for common information models for AI/ML management

Editor’s Note: The handling of Goal, targets or objectives for the general closed control loops is FFS

Figure 6.2.1-x: DynamicCCLTrigger NRM fragment

### 6.2.2	Inheritance

Figure 6.2.2-1: Inheritance Hierarchy for Closed Control Loops

* * * Next Change * * * *

##### 6.3.x	DynamicCCLTrigger

6.3.x.1	Definition

This defines dynamic closed control loop invocation and action criteria/conditions that can be configured by the consumer.

6.3.x.2	Attributes

The DynamicCCLTrigger IOC includes attributes inherited from Top IOC (defined TS 28.622[5]) and the following attributes:

| Attribute name | S | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| cLInitaiationTrigger | M | T | T | F | T |
| cCLActionTrigger | M | T | T | F | T |
| cCLCompositionTrigger | M | T | T | F | T |
| cCLRef | CM | T | T | F | T |
| isCCLRequired | CM | T | F | F | T |
| targetBehavior | CM | T | T | F | T |

6.3.x.3	Attribute constraints

| Name | Definition |
| --- | --- |
| cCLRef | Condition: when the cCLActionTrigger attribute is supported |
| isCCLRequired | Condition: when the cCLInitaiationTrigger attribute is supported |
| targetBehavior | Condition: when the cCCLActionTrigger attribute is supported |

6.3.x.4	Notifications

The common notifications defined in subclause 4.1.2.5 are valid for this IOC, without exceptions or additions.

##### 6.3.y	CCLTrigger <<datatype>>

6.3.y.1	Definition

This defines dynamic CCL initiation criteria/conditions that can be configured by the consumer. When the crietria/condition is met the producre can instantiate a CCL.

6.3.y.2	Attributes

| Attribute name | S | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| performanceBasedCriteria | M | T | T | F | T |
| provisioningBasedCriteria | M | T | T | F | T |
| faultBasedCriteria | M | T | T | F | T |

6.3.y.3	Attribute constraints

None

6.3.y.4	Notifications

The common notifications defined in subclause 4.1.2.5 are valid for this IOC, without exceptions or additions.

##### 6.3.z	FaultBasedCriteria <<datatype>>

6.3.z.1	Definition

This will define various fault related informtion that need to be monitored by the producer to see if an CCL is to be initiated.

6.3.z.2	Attributes

| Attribute name | S | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| targetNode | M | T | T | F | T |
| alarmSeverityThreshold | M | T | T | F | T |
| alarmTypeThreshold | M | T | T | F | T |

6.3.z.3	Attribute constraints

None

6.3.z.4	Notifications

The common notifications defined in subclause 4.1.2.5 are valid for this IOC, without exceptions or additions.

* * * Next Change * * * *

## 6.4	Attribute definitions

### 6.4.1	Attribute properties

Table 6.4.1-1

| Attribute Name | Documentation and Allowed Values | Properties |
| --- | --- | --- |
| scopeType | It indicates the type of scope that represented by the particular scope instance. allowedValues: CCL_MEASUREMENT_SCOPE, CCL_TARGET_SCOPE, CCL_CONTROL_SCOPE, CCL_IMPACT_SCOPE Editor’s Note: The allowed values will be revisited | type: Enum multiplicity: 1..* isOrdered: False isUnique: True defaultValue: None isNullable: False |
| performanceBasedCriteria | This defines the information related with performance measurements and KPIs that need to be monitored by the producer to see if the values have crossed the thresholds defined. When the crietria/condition is met the producre can instantiate an CCL or a CCL may be configured to execute the actions. This is defined as a Jex string supporting the following attributes: - Target Node: The identification of the Managed Object for which the performance is being monitored. - Measurement/KPI Name: Name/identification of the performamce measurement and KPI - Trigger Value: Trigger value of the measurement. The CCL is triggered when the value of the measurement or the KPI exceeds more that the trigger value or when the value decreases below the trigger value. | Type: String multiplicity: * isOrdered: False isUnique: True defaultValue: None isNullable: False |
| provisioningBasedCriteria | This will define various provisioning events that need to be monitored by the producer to see if an CCL is to be initiated or a CCL may be configured to execute the actions. This is defined as a Jex string supporting the following attributes: - Target Node: This can be a particular IOC or a DN. E.g Intent IOC - Provisioning Location: The CCL will be created only when the object created is targeting a specific location. - Provisioning Event (Create, Modify, Delete): The CCL will be created when the given event occur on the given IOC/DN. - Provisioning Time: The CCL will be created only when the given event occur at a specified time. | Type: String multiplicity: * isOrdered: False isUnique: True defaultValue: None isNullable: False |
| targetNode | This defines the node which need to be monitored for the emitted alarms (i.e objectInstance in AlarmInformation). | Type: String multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| alarmSeverityThreshold | This defines the “perceivedSeverity” threshold (i.e threshold for each Severity). If total number of alarms, belonging to particular perceivedSeverity (e.g critical, major etc.), goes beyond the threshold, an CCL will be instantiated. | Type: String multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| alarmTypeThreshold | This defines the “AlarmType” threshold (i.e threshold for each AlarmType). If total number of alarms, belonging to a particular alarmType, goes beyond the threshold, an CCL will be instantiated. | Type: String multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| cCLInitaiationTrigger | This defines dynamic closed control loop invocation criteria/conditions that can be configured by the consumer. When the crietria/condition is met the producre can instantiate a CCL. | Type: CCLTrigger multiplicity: * isOrdered: False isUnique: True defaultValue: None isNullable: False |
| cCLActionTrigger | This defines the criteria/conditions under which the CCL is allowed to take actions. | Type: CCLTrigger multiplicity: * isOrdered: False isUnique: True defaultValue: None isNullable: False |
| cCLRef | This defines the CCL for which the conditions are defined. | Type: Dn multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| isCCLRequired | This defines if the CCL is needed as per the trigger conditions defined. The value TRUE will indicate that the CCL is required. The producre will set this attribute to True if the conditions defined becomes true. | Type: Boolean multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: False isNullable: False |
| targetBehavior | This will define the corresponding behavior of the CCL. The behaviors can be represented by an ENUM to include: - DECISION_ACTIVATION: The CCL executes the recommendations that it derives on to the network. - NOTIFY_RCOMMENDATION: The CCL starts processing input to derive recommendations but without the corresponding actions executed on the network. Instead, the recommendation is notified to the consumer who then considers whether it should be applied or not. - DO_NOTHING: do not do anything. | Type: ENUM multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| cCLCompositionTrigger | This defines the criteria/conditions under which the CCL is desired to be composed. | Type: CCLTrigger multiplicity: * isOrdered: False isUnique: True defaultValue: None isNullable: False |

* * * Next Change * * * *

## A.1.2	CCL inheritance relationships (Figure 6.2.1.1-2)

@startuml

skinparam ClassStereotypeFontStyle normal

skinparam ClassBackgroundColor White

skinparam shadowing false

skinparam monochrome true

hide members

hide circle

class Top << InformationObjectClass >>

class ClosedControlLoop <<InformationObjectClass>>

class CCLReport <<InformationObjectClass>>

class CCLScope <<InformationObjectClass>>

class DynamicCCLTrigger<<InformationObjectClass>>

Top <|-- ClosedControlLoop

Top <|-- CCLScope

Top <|-- CCLReport

Top <|-- DynamicCCLTrigger

@enduml

Source code for Figure 6.2.1.1-2 CCL inheritance relationships

* * * Next Change * * * *

# A.1.x	UML code for Figure 6.2.1-x

@startuml

skinparam ClassStereotypeFontStyle normal

skinparam ClassBackgroundColor White

skinparam shadowing false

skinparam monochrome true

hide members

hide circle

class ManagedEntity <<ProxyClass>>

class DynamicCCLTrigger<<InformationObjectClass>>

ManagedEntity "1" *-- "*" DynamicCCLTrigger: <<names>>

note left of ManagedEntity

Represents the following IOCs:

SubNetwork or

ManagedElement

end note

@enduml

* * * Next Change * * * *

* * * End of Changes * * * *