3GPP TSG-SA5 Meeting #161	S5-252481

Fukuoka, Japan, 19 - 23 May 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 28.552 | CR | 0712 | rev |  | Current version: | 19.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Rel-19 CR TS 28.552 Add the measurement of carbon emission related KPIs for gNB |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA5 |  |  |  |  |  |  |  |  |  |
| Work item code: | Energy_OAM_Ph3 |  |  |  |  |  | Date: |  |  | 2025-05-08 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | Renewable energy usage for gNB is discussed and concluded in TR 28.880. A solution of carbon emission calculation is needed. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Introduce carbon emission calculation for gNB |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The solution is imcomplete. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 2, 5.1.1.19.a, A.n |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

First change

2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	Void.

[3]	3GPP TS 32.404: "Performance Management (PM); Performance measurements - Definitions and template".

[4]	3GPP TS 23.501: "System Architecture for the 5G System".

[5]	IETF RFC 5136: "Defining Network Capacity".

[6]	3GPP TS 38.473: "NG-RAN; F1 Application Protocol (F1AP)".

[7]	3GPP TS 23.502: "Procedures for the 5G System".

[8]	3GPP TS 28.554: "Management and orchestration; 5G end to end Key Performance Indicators (KPI)".

[9]	3GPP TS 32.425: "Performance Management (PM); Performance measurements for Evolved Universal Terrestrial Radio Access Network (E-UTRAN)".

[10]	3GPP TS 32.451: "Key Performance Indicators (KPI) for Evolved Universal Terrestrial Radio Access Network (E-UTRAN); Requirements".

[11]	3GPP TS 38.413: "NG-RAN; NG Application Protocol (NGAP)".

[12]	Void.

[13]	3GPP TS 38.423: "NG-RAN; Xn Application Protocol (XnAP)".

[14]	3GPP TS 29.502: "5G System; Session Management Services; Stage 3".

[15]	Void.

[16]	3GPP TS 29.244: "Technical Specification Group Core Network and Terminals; Interface between the Control Plane and the User Plane Nodes; Stage 3".

[17]	ETSI GS NFV-IFA027 v2.4.1: "Network Functions Virtualisation (NFV); Management and Orchestration; Performance Measurements Specification".

[18]	Void.

[19]	3GPP TS 38.214: "NR; Physical layer procedures for data".

[20]	3GPP TS 38.331: "NR; Radio Resource Control (RRC); Protocol specification".

[21]	3GPP TS 29.518: "5G System; Access and Mobility Management Services; Stage 3".

[22]	3GPP TS 29.413: "Application of the NG Application Protocol (NGAP) to non-3GPP access".

[23]	3GPP TS 29.122: "Technical Specification Group Core Network and Terminals; T8 reference point for Northbound APIs".

[24]	3GPP TS 24.501: "Non-Access-Stratum (NAS) protocol for 5G System (5GS); Stage 3".

[25]	ETSI ES 202 336-12 V1.2.1: "Environmental Engineering (EE); Monitoring and control interface for infrastructure equipment (power, cooling and building environment systems used in telecommunication networks); Part 12: ICT equipment power, energy and environmental parameters monitoring information model".

[26]	3GPP TS 28.541: "Management and orchestration; 5G Network Resource Model (NRM); Stage 2 and stage 3".

[27]	3GPP TS 29.274: "Evolved General Packet Radio Service (GPRS); Tunnelling Protocol for Control plane (GTPv2-C); Stage 3".

[28]	3GPP TS 29.510: "5G System; Network function repository services; Stage 3".

[29]	3GPP TS 38.314: "NR; layer 2 measurements".

[30]	3GPP TS 28.313: "Self-Organizing Networks (SON) for 5G networks".

[31]	3GPP TS 38.415: "NG-RAN; PDU session user plane protocol".

[32]	3GPP TS 38.321: " NR; Medium Access Control (MAC) protocol specification".

[33]	3GPP TS 38.214: "NR; Physical layer procedures for data".

[34]	3GPP TS 38.215: "NR; Physical layer measurements".

[35]	3GPP TS 38.133: "NR; Requirements for support of radio resource management".

[36]	3GPP TS 33.501: "Security architecture and procedures for 5G system".

[37]	3GPP TS 38.304: "NR; User Equipment (UE) procedures in Idle mode and RRC Inactive state".

[38]	Void.

[39]	3GPP TS 29.507: "5G System; Access and Mobility Policy Control Service; Stage 3".

[40]	3GPP TS 29.512: "5G System; Session Management Policy Control Service; Stage 3".

[41]	3GPP TS 29.531: "5G System; Network Slice Selection Services".

[42]	3GPP TS 29.281: "General Packet Radio System (GPRS) Tunnelling Protocol User Plane (GTPv1-U)".

[43]	3GPP TS 29.540: "5G System; SMS Services; Stage 3".

[44]	3GPP TS 29.522: "5G System; Network Exposure Function Northbound APIs; Stage 3".

[45]	3GPP TS 29.541: "5G System; Network Exposure FunctionServices for Non-IP Data Delivery (NIDD); Stage 3".

[46]	3GPP TS 23.503: "Policy and charging control framework for the 5G System (5GS); Stage 2".

[47]	3GPP TS 29.504: "5G System; Unified Data Repository Services; Stage 3".

[48]	3GPP TS 29.554: "5G System; Background Data Transfer Policy Control Service; Stage 3".

[49]	3GPP TS 38.300: "NR and NG-RAN Overall description; Stage-2".

[50]	Void.

[51]	3GPP TS 29.503: "5G System; Unified Data Management Services; Stage 3".

[52]	3GPP TS 23.558: "Architecture for enabling Edge Applications".

[53]	3GPP TS 23.273: "5G System (5GS); Location Services (LCS); Stage 2".

[54]	3GPP TS 29.572: "5G System (5GS); Location Management Services; Stage 3".

[55]	Void

[56]	3GPP TS 38.425: "NG-RAN; NR User plane protocol".

[57]	3GPP TS 36.425: "(E-UTRAN); X2 interface user plane protocol".

[58]	3GPP TS 29.520: "5G System; Network Data Analytics Services; Stage 3".

[59]	3GPP TS 23.288: "Architecture enhancements for 5G System (5GS) to support network data analytics services".

[60]	3GPP TS 37.340: "NR; Multi-connectivity; Overall description; Stage-2".

[61]	Void.

[62]	3GPP TS 28.532: "Management and orchestration; Generic management services".

[63]	3GPP TS 28.318: "Management and Orchestration; Network and Service Operations for Energy Utilities (NSOEU)".

[64]	3GPP TS 38.213: "NR;Physical layer procedures for control".

[65]	3GPP TS 28.622: "Generic Network Resource Model (NRM) Integration Reference Point (IRP); Information Service (IS)".

[66]	3GPP TS 38.401: "NG-RAN; Architecture description".

[67]	3GPP TS 24.193: "Access Traffic Steering, Switching and Splitting (ATSSS); Stage 3".

[68]	3GPP TS 23.527: "5G System; Restoration procedures ".

[69]	3GPP TS 32.426: "	Telecommunication management; Performance Management (PM); Performance measurements Evolved Packet Core (EPC) network".

[70]	ITU-T Y.1540: "Internet protocol data communication service - IP packet transfer and availability performance parameters".

[m]	ETSI GS OEU 020: "Operational energy Efficiency for Users (OEU); Carbon equivalent Intensity measurement; Operational infrastructures; Global KPIs; Global KPIs for ICT Sites".

Next change

5.1.m	Performance measurements for renewable energy

5.1.m.1	Measurements related to carbon emission

5.1.m.1.1	Carbon Emission

a)	This measurement provides the Carbon Emission of a gNB (CEgNB) over a time period. The Carbon Emission of a gNB (CEgNB) over a time period can be defined as a proportion of the Carbon Emission of the operator site (CESite) where the gNB resides, the proportion being given by the ratio of the Energy Consumption of the gNB (ECgNB) to the total Energy Consumption of the operator site during the same observation period. The result of carbon emission divided by energy consumption, which represents the quantity of CO2 equivalent emission per unit of energy consumption, is named as carbon intensity.

b)	SI

c)	This measurement is obtained as:

CEgNB=CEsite*ECgNBECsite

, where:

CESite is the Carbon Emission of the operator site where the gNB resides. In case of multiple types of energy sources, CESite can be obtained by summing up, for each type of energy source i used to power the operator site, the energy consumed using this type of energy source multiplied by the Carbon Emission Factor of the type of energy source i. The Carbon Emission Factor is equivalent to the standard emission factor from annexes A.n. Its unit is kg CO2eq;

ECgNB is the Energy Consumption of the gNB; it corresponds to PEE.Energy as defined in clause 5.1.1.19.3. Its unit is kWh;

ECSite represents the total Energy Consumption of the operator site where the gNB resides. Its unit is kWh.

CEsiteECsite indicates the carbon intensity, i.e., quantity of CO2 equivalent emission per unit of energy consumption. Its unit is kg CO2eq/kWh.

NOTE: Either CESite and ECSite, or CEsiteECsite are provided as input from the operator.

d)	Its unit is kg CO2eq.

e)	RNE.CarbonEmission.

f)	ManagedElement

g)	Valid for packet switching

h)	5GS

i)	One usage of this measurement is for monitoring the carbon emission of a gNB during a time period.

5.1.m.1.2	Carbon Emission Efficiency

a)	This measurement provides mobile network data carbon emission efficiency in operational NG-RAN. Carbon Emission (CE) divided by Data Volume (DV) of the considered network elements.

b)	SI

c)	This measurement is obtained as:

CEEgNB=CEgNBgNBDRB.PdcpSduUolumeUl+DRB.PdcpSduVolumeDl

, where:

CEgNB is the Carbon Emission of the gNB as defined in clause 5.1.m.1.1. Its unit is kg CO2eq;

The Data Volume (in kbits) is obtained by measuring amount of DL/UL PDCP SDU bits of the considered network elements over the measurement period.

d)	Its unit is kg CO2eq/kbits.

e)	RNE.CarbonEmissionEfficiency.

f)	ManagedElement

g)	Valid for packet switching

h)	5GS

i)	One usage of this measurement is for monitoring the carbon emission efficiency of a gNB during a time period.

5.1.m.2	Measurements related to renewable energy usage

5.1.m.2.1	Ratio of renewable energy

a)	This measurement provides the ratio of renewable enegy for a gNB. The ratio of renewable enegy for a gNB (RREgNB) is equal to the ratio of renewable enegy of the operator site (RRESite) where the gNB resides, which is defined as the ratio of the Energy Consumption of the operator site coming from renewable energy sources to the total Energy Consumption of the operator site.

b)	SI

c)	This measurement is obtained as:

RREgNB=ECsite, RenECsite

, where:

ECSite,Ren is the Energy Consumption of the operator site coming from renewable energy sources only. Its unit is kWh;

ECSite is the total Energy Consumption of the operator site. Its unit is kWh

d)	0 ≤ RREgNB ≤ 1.

e)	RNE.RenewableRatio.

f)	ManagedElement

g)	Valid for packet switching

h)	5GS

i)	One usage of this measurement is for monitoring the ratio of renewable enegy of a gNB during a time period.

Next change

A.n	Monitoring of renewable energy information in NG-RAN

A.n.1	Carbon Emission Factors

Tables from annexes A.3 and A.4 of ETSI GS OEU 020 [m] providing standard emission factors are partially copy/pasted here below for information:

Figure A.n.1-1: ETSI GS OEU 020 [m] Standard emission factor for fuel combustion

Figure A.n.1-2: ETSI GS OEU 020 [m] Standard emission factor for local renewable electricity production

A.n.2	Types of energy related information

Energy information related to the measurement of carbon emission can be classified in 3 types:

- Energy related information about the different types of energy source used by the MNO. Such information can generally be obtained from Standards Development Organizations (SDO) or government agencies or regulation bodies, etc. This information mainly includes:

- CEFi: Carbon Emission Factor for the type of energy source i

It is the responsibility of each MNO to collect this information by its own means. How the MNO obtains this information is not in the scope of the present document. An example of this information is the Standard Emission Factors given by ETSI GS OEU 020 [m], as depicted in the left-most column in the diagram below;

- Energy related information about energy consumption at operator site level, per type of energy source and indication about renewable / non-renewable energy type of energy source – see middle column. This information mainly includes:

- ECi: Energy Consumption corresponding to the type of energy source i used to power the operator site,

- ECSite: total Energy Consumption of the operator site, which can be calculated by summing up all ECi for a given site,

- CESite: Carbon Emission of the operator site, which can be calculated from CEFi and ECi,

- ECSite,Ren: Energy Consumption of the operator site coming from renewable energy sources only,

- REFSite: Renewable Energy Factor of the operator site, which can be calculated from ECSite and ECSite,Ren

This information may typically be managed by the MNO Energy Management System. How the MNO Element Management System obtains this information is not in the scope of 3GPP

- Energy related information relative to the 3GPP system – see right-most column in the diagram below. In this version of the document, this information focuses on non-split gNB:

- ECNonSplit gNB: Energy Consumption, which is collected by the 3GPP management system,

- CENonSplitgNB: Carbon Emission, which can be calculated from ECNonSplit gNB, ECSite and CESite,

- REFNonSplitgNB: Renewable Energy Factor, which can be obtained from REFSite,

This information is to be managed by the 3GPP management system.

Figure A.n.2-1: Examples of three types of renewable energy related information

End of change