3GPP TSG-SA5 Meeting #161	S5-252484

Fukuoka, Japan, 19 - 23 May 2025

Source:	Huawei, China Mobile

Title:	Rel-19 pCR TS 28.561 Add attributes for NDT NRMs

Document for:	Approval

Agenda item:	6.19.5.1

Spec:	3GPP TS 28.561

Version:	0.2.0

Work Item:	NDT

Comments

This contribution is proposed to add attributes for IOC NDTFunction, NDTJob, NDTReport.

Proposed Changes

* * * First Change * * * *

6.2.1.3	Class definition

Editor’s Note: The attributes of IOCs below are FFS.

6.2.1.3.1	NDTFunction <<InformationObjectClass>>

6.2.1.3.1.1	Definition

This IOC represents the properties of an NDT Function.

The attribute "supportedNdtCapabilities" indicates the capabilities that are support by the NDTFunction instance.

The attribute "ndtType" indicates the domain type that is support by the NDTFunction instance.

6.2.1.3.1.2	Attributes

The NDTFunction IOC includes attributes inherited from Top IOC (defined in 3GPP TS 28.622 [7]) and the following attributes.

Table 6.2.1.2.1.2-x

| Attribute Name | Support Qualifier | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| supportedNdtCapabilities | M | T | F | F | T |
| ndtType | M | T | F | F | T |

6.2.1.3.2	NDTJob <<InformationObjectClass>>

6.2.1.3.2.1	Definition

This IOC represents the properties of an NDT job demand created by an MnS consumer.

The attribute "ndtJobType" indicates the type of NDT capability that is asked by the NDT MnS consumer.

The attribute "ndtJobStatus" indicates administration status of an NDT job.

The attribute "ndtJobBehaviour" indicates the input that is defined by MnS consumer and doesn’t happen yet in actual network and will be simulated/emulated by NDT. The behaviour can be configurations for verification. The configuration can either be network configurations or automation functionality configurations, such as RAN ES functionality provided by SON. The behaviour can also indicate a network event, issue, or scenario that defined by MnS consumer and will be injected to NDT. One typical network failure event is signalling storm.

The attribute "ndtJobTime" is a time window which indicates if the simulation/emulation is for the past, present, or future.

The attribute "ndtJobExecutionRequirements" represents the performance requirements for NDT modelling, e.g., maximum run time for each simulation/emulation task, precision, etc.

6.2.1.3.2.2	Attributes

The NDTJob IOC includes attributes inherited from Top IOC (defined in 3GPP TS 28.622 [7]) and the following attributes.

Table 6.2.1.3.2.2-1

| Attribute Name | Support Qualifier | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| ndtJobType | M | T | T | F | T |
| ndtJobStatus | M | T | T | F | T |
| ndtJobBehaviour | M | T | T | F | T |
| ndtJobTime | M | T | T | F | T |
| collectedDataNameList | M | T | T | F | T |
| ndtJobExecustionRequirements | O | T | T | F | T |
| Attribute related roles |  |  |  |  |  |
| ndtFunctionRef | M | T | T | F | T |
| ndtReportRefList | M | T | T | F | T |

6.2.1.3.3	NDTReport <<InformationObjectClass>>

6.2.1.3.3.1	Definition

This IOC represents the properties of an NDT report corresponding to an NDT job.

The attribute "dataResults" is specified performance metrics and/or alarm data/CM data that are collected and reported by NDT after the behaviour is modelled in NDT.

6.2.1.3.3.2	Attributes

The NDTReport IOC includes attributes inherited from Top IOC (defined in 3GPP TS 28.622 [7]) and the following attributes.

Table 6.2.1.3.3.2-1

| Attribute Name | Support Qualifier | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| dataResults | M | T | T | F | T |
| Attribute related roles |  |  |  |  |  |
| ndtJobRef | M | T | T | F | T |
| ndtFunctionRef | M | T | T | F | T |

6.2.1.3.a	Behaviour <<dataType>>

6.2.1.3.a.1	Definition

The Behaviour<<dataType>> represents the network behaviour that aims to be configured to NDT.

6.2.1.3.a.2	Attributes

The Behaviour includes the following attributes.

Table 6.2.1.3.a 3.2-1

| Attribute Name | Support Qualifier | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| object | M | T | T | F | F |
| configurationList | O | T | T | F | F |
| perfMetricsList | O | T | T | F | F |

6.2.1.3.b	NdtJobExecutionReqs <<dataType>>

6.2.1.3.b.1	Definition

The NdtJobExecutionReqs <<dataType>> represents the execution requirements for NDT job.

6.2.1.3.b.2	Attributes

The NdtJobExecutionReqs includes the following attributes.

Table 6.2.1.3.b.2-1

| Attribute Name | Support Qualifier | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| maxRuntime | O | T | T | F | F |
| precision | O | T | T | F | F |

6.2.1.3.c	NdtDataOutput <<dataType>>

6.2.1.3.c.1	Definition

The NdtDataOutput <<dataType>> represents data reported by NDT as requested in NDTJob.

6.2.1.3.c.2	Attributes

The NdtDataOutput includes the following attributes.

Table 6.2.1.3.c 3.2-1

| Attribute Name | Support Qualifier | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| object | M | T | T | F | F |
| ndtDataList | M | T | T | F | F |

* * * Next Change * * * *

6.3	Attribute definitions

6.3.1	Attribute properties

Table 6.3.1-1

| Attribute Name | Documentation and Allowed Values | Properties |
| --- | --- | --- |
| ndtJobRef | It indicates an DN of a NDTJob Instance. allowedValues: N/A | type: DN multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| ndtFunctionRef | It indicates an DN of a NDTFunction Instance. allowedValues: N/A | type: DN multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| ndtReportRefList | It indicates a list of DN for NDTReport Instances. allowedValues: N/A | type: DN multiplicity: * isOrdered: False isUnique: True defaultValue: None isNullable: False |
| supportedNdtCapabilities | It indicates the different capabilities which the NDTFunction can support. Candidate values include: “HIGH-RISK_OPERATION_EVALUATION”, “FAILURE_EVENTS_EVALUATION”, “NETWORK_ISSUE_INDUCEMENT”, “EVENTS-IMPACTS_VERIFICATION”, “NETWORK_CONFIGURATION_VERIFICATION”, “AUTOMATION_CONFIGURATION_VERIFICATION”, “ML-TRAINING_DATA_GENERATION”, “USER_EXPERIENCE_DATA_GENERATION” The meaning of these values is as described in clause 5. The detailed implementation is out of scope of the presenting document. | type: String multiplicity: 1..* isOrdered: False isUnique: True defaultValue: None isNullable: False |
| ndtType | It indicates the domain type that is support by the NDTFunction. allowedValues: “RAN_DOMAIN_NDT”, “CN_DOMAIN_NDT”, “CROSS_DOMAIN_NDT” | type: ENUM multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| ndtJobType | It indicates the type of simulation/emulation that is desired to be executed. allowedValues: “HIGH-RISK_OPERATION_EVALUATION”, “FAILURE_EVENTS_EVALUATION”, “NETWORK_ISSUE_INDUCEMENT”, “EVENTS-IMPACTS_VERIFICATION”, “NETWORK_CONFIGURATION_VERIFICATION”, “AUTOMATION_CONFIGURATION_VERIFICATION”, “ML-TRAINING_DATA_GENERATION”, “USER_EXPERIENCE_DATA_GENERATION” The meaning of these values is as described in clause 5. The detailed implementation is out of scope of the presenting document. | type: ENUM multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| ndtJobStatus | It represents the administration status of an NDT job. allowedValues: “ACTIVE”, “INACTIVE” | type: ENUM multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| ndtJobBehaviour | The input that is defined by MnS consumer and doesn’t happen yet in actual network and will be simulated/emulated by NDT. | type: Behaviour multiplicity: 1..* isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| object | The managed object instance which is simulated/emulated in NDT. | type: DN multiplicity: 1 isOrdered: N/A isUnique: True defaultValue: None isNullable: False |
| configurationList | It describes the attribute name and value for a certain MOI modelled by NDT. The value is different from the one configured to the same MOI in actual network. The content of the attribute is a list of attributeName- attributeValue pairs. | type: AttributeValuePair multiplicity: 1..* isOrdered: False isUnique: True defaultValue: None isNullable: False |
| collectedDataNameList | The name of data (defined in TS 28.552/28.554) that is requested by MnS consumer to be reported by NDT after the particular network behaviour is modelled. | type: string multiplicity: 1..* isOrdered: False isUnique: True defaultValue: None isNullable: False |
| perfMetricsList | It describes the attribute name and value for certain PM or KPI (defined in TS 28.552/28.554) related to the event modelled in NDT. | type: AttributeValuePair multiplicity: 1..* isOrdered: False isUnique: True defaultValue: None isNullable: False |
| ndtJobExecustionRequirements | It describes the performance requirements for NDT modelling, e.g., maximum run time for each simulation/emulation job, precision, etc | type: NdtJobExecutionReqs multiplicity: 1 isOrdered: N/A isUnique: True defaultValue: None isNullable: False |
| maxRuntime | Maximum run time for each simulation task executed in NDT | type: Integer multiplicity: 1 isOrdered: N/A isUnique: True defaultValue: None isNullable: False |
| precision | The expected proximity between the network simulation results and the actual network execution outcome | type: Integer multiplicity: 1 isOrdered: N/A isUnique: True defaultValue: None isNullable: False |
| dataResults | The data reported by NDT as requested in NDTJob | type: NdtDataOutput multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| ndtDataList | It describes the performance metrics list and/or configuration list. The content of the attribute is a list of attributeName- attributeValue pairs. | type: AttributeValuePair multiplicity: 1..* isOrdered: False isUnique: True defaultValue: None isNullable: False |

* * * End of Changes * * * *