3GPP TSG-SA5 Meeting #161	S5-252660

Fukuoka, Japan, 19 - 23 May 2025

# Source:	Nokia

# Title:	Discussion paper on CCF generating access token for external MnS consumers

# Document for:	Endorsement

Agenda Item:	6.19.21.1

# 1	Decision/action requested

In this box give a very clear / short /concise statement of what is wanted.

# 2	References

(Reference - in list form - should be made to previous related SA5/3GPP/etc. documents.)

[1] 3GPP TS 33.122: "Security Aspects of Common API Framework for 3GPP Northbound APIs".

[2] IETF RFC 6749: "The OAuth 2.0 Authorization Framework".

[2] 3GPP TS 28.579, "Management services exposure to external consumers through CAPIF".

# 3	Rationale

## 3.1 Background

Clause 6.5.2.3 of TS 33.122[1] states the following regarding the security procedures over the CAPIF-2e reference point:

“As per OAuth 2.0 [4], the CAPIF core function shall perform the functionality of the authorization server and provide the token endpoint, the API invoker shall perform the function of the client functionality, while the API exposing function shall perform the resource server functions. The API invoker client (client endpoint) shall be registered as a confidential client type with an authorization grant type of 'client credentials'. The authorization shall be previously arranged in the CAPIF core function. The access token shall follow the profile described in annex C.

NOTE: How the authorization is pre-arranged (pre-configured) with the CAPIF core function is out of scope of the present document”

Figure 3.1-1: CAPIF-2e interface authentication and protection using Access Tokens (see clause 6.5.2.3 of TS 33.122)

## 3.2 Observations

From clause 3.1, the following observations can be made regarding the authorization of the external MnS consumers at the CCF:

Observation 1: What authorization information for a given external MnS consumer shall be configured at the CCF?

Observation 2: How to configure the authorization information of the external MnS consumers at the CCF?

Observation 3: How to ensure that the authorization information at the CCF is correlated with the MSAC authorization information configured for a given external MnS consumer within the management system?

Observation 4: If observation 1,2,3 is possible, how can an access token generated by the CCF be understood by the authentication and authorization MnS producer or the MnS producers?

NOTE: OAUTH2.0[2] framework does not support interworking between two authorization servers, which implies only one authorization server (i.e., either within the CCF or the management system) can generate the access token for the external MnS consumers.

## 3.3 Way forward

As a way forward, pCR S5-25XXX submitted in this meeting provides a solution that addresses all the four observations in clause 3.2. In the proposed solution, the CCF does not generate the access token for the external MnS consumers but rather forwards all the authorization requests (e.g., the API invoker onboarding and access token requests for service API invocations) from the external MnS consumer to the authentication and authorization MnS producer as shown in Figure 3.3-1. Further, the authentication and authorization MnS producer is in charge of the CAPIF enrolment process for a given external MnS consumer.

Figure 3.3-1: Authorization of external consumers at the CCF to consume management services

### 3.3.1 Implications from the way forward

- The proposed solution requires that a new CAPIF interface is defined to enable the interaction between the CCF and an external authorization server (in this case the authentication and authorization MnS producer) in situations where the CCF isn’t able to generate the access token directly.

# 4	Detailed proposal

The group is asked to do the following:

- Endorse the solution captured by pCR S5-25XXXX submitted to this meeting

- Send an LS to SA6 to request for the definition of a new CAPIF interface to enable the CCF to interact with external authorization servers.