3GPP TSG-SA5 Meeting #161	S5-252686

Fukuoka, Japan, 19 - 23 May 2025

# Source:	NEC, Intel

# Title:	Discussion Paper: Introduction of management and orchestration reference arc architecture in support of SBMA

# Document for:	Discussion & Endorsement

Agenda Item:	6.1

# 1	Decision/action requested

The group is asked to discuss and endorse the proposals.

# 2	References

[1]	3GPP TS 28.533 Management and orchestration; Architecture framework

[2]	3GPP TS 28.105 Management and orchestration; Artificial Intelligence/ Machine Learning (AI/ML) management

[3]	3GPP TS 28.104 Management and orchestration; Management Data Analytics (MDA)

# 3	Rationale

Starting from Rel-15 in the 5G time frame, 3GPP SA5 adopted the Service-Based Management Architecture (SBMA) as the foundational approach for specifying network management and orchestration services and capabilities in 3GPP systems [1]. SBMA was introduced to support the flexibility and scalability required in 5G networks. It enables the specification of management services (MnSs) independently of any predefined management functions (MnFs), in contrast to earlier architectures that relied on explicitly defined functional entities.

While SBMA offers strong support for modular service definition and architectural flexibility, over successive releases, SA5 specifications have increasingly focused on MnSs while progressively moving away from defining or specifying the corresponding MnFs. As a result, the role and identity of MnS producers have become unclear. This gap has contributed to confusion and misalignment across 3GPP Working Groups (WGs), particularly as network intelligence, automation, and AI/ML capabilities are introduced into the 5G Core, RAN, and management domains.

In SBMA, network management is delivered through MnSs that expose specific management capabilities. These services are produced and consumed by logical entities known as Management Functions (MnFs), which may act as MnS producers, consumers, or both. However, in practice, SA5 has primarily defined MnSs without explicitly specifying the associated MnFs.

This has led other WGs, such as SA2 and RAN, to define their own management-related functions to address perceived architectural gaps. In doing so, some have introduced functional components that overlap with the scope traditionally covered by SA5, resulting in duplication, inconsistencies, and architectural drift.

The issue is further aggravated by developments in 6G, where several WGs are independently introducing new functional entities to support management and orchestration without alignment to SBMA or coordination with SA5. Without a corrective step, this architectural fragmentation will become more entrenched and difficult to reconcile.

The ambiguity in identifying MnS producers has made cross-WG alignment challenging. For example, as SA2 and RAN pursue AI/ML, automation, and orchestration enhancements, they have encountered unclear ownership of management-related functions. In response, they have defined their own entities—unintentionally stepping into the remit of SA5.

Moreover, whereas the 5G Core and RAN continue to maintain clearly defined and specified Network Functions (NFs), the diminishing visibility of MnFs within SA5 has widened the disconnect. This complicates the integration of management services with operational architectures and limits the ability of other WGs and external SDOs to interpret or extend SA5-defined services.

This issue is further underscored by the fact that other 3GPP Working Groups, such as SA2 and RAN3, rely on well-defined architectures to structure their domains, for example, the 5GC architecture in SA2 and the NG-RAN architecture in RAN3. These architectures serve as foundational references that facilitate coordination, clarity, and extensibility. In contrast, if SA5 focuses solely on defining Management Services (MnSs) without describing the management architecture in which they operate, it creates a gap that makes it harder for other WGs to understand how these services fit into the broader system. A clearly defined management architecture from SA5 would enhance interoperability, avoid misinterpretation, and ensure architectural coherence across 3GPP specifications.

SA5 specifications already refer to logical Management Functions (MnFs) in several areas, for example, in the management of AI/ML as described in TS 28.105 [2], and in Management Data Analytics (MDA) as specified in TS 28.104 [3], where the relevant function is the Management Data Analytics Function (MDAF). However, these MnFs are only implicitly referenced and not formally represented within an overarching architectural or management framework. This lack of explicit definition makes their roles and interactions across the management plane less visible and less coherent. Consequently, there is a need for SA5 to introduce a complementary management architecture that explicitly identifies and organizes all relevant MnFs, enabling greater clarity, better alignment across specifications, and a more structured foundation for advancing intelligent and automated network management in 5G and beyond.

To address the identified issues, it is proposed that SA5 introduce a complementary management framework that explicitly defines the logical Management Functions (MnFs) responsible for producing and consuming Management Services (MnSs). This architectural enhancement complements the existing Service-Based Management Architecture (SBMA) specified in 3GPP TS 28.533 [1], rather than replacing or contradicting it. It reinforces SBMA by clarifying the structure, role, and responsibilities of management entities, thereby supporting improved cross-WG alignment, reducing duplication, and ensuring broader architectural consistency. The proposed framework also enables an integrated view in which MnFs interact with 5G Core (5GC) Network Functions (NFs), User Equipment (UE), and RAN components via both Service-Based and legacy management interfaces.

# 4	Detailed proposal

To address the challenges outlined above and reinforce SA5’s leadership in network management, we propose the following:

Proposal #1: Define a complementary management framework
Introduce a complementary management and orchestration reference architecture that augments the Service-Based Management Architecture (SBMA) by:

- Explicitly defining logical Management Functions (MnFs) responsible for producing and consuming Management Services (MnSs), and specifying their roles.

- Mapping MnF functional roles to the MnSs they produce and/or consume (e.g., ML training, management data analytics, intent driven management, etc.).

- Enabling clear traceability from each MnS to its producer(s), facilitating architectural clarity.

- Maintaining MnFs as logical entities, decoupled from any specific implementation or deployment model.

- Supporting seamless interaction between MnFs and other entities such as 5GC NFs, UE, and RAN components, via both service-based and legacy (i.e., Type-1) management interfaces.

Proposal #2: Communicate and align the management and orchestration reference architecture across 3GPP WGs

- Engage with SA2, RAN, and other WGs to present and align on the proposed complementary management and orchestration reference architecture.

- Ensure architectural consistency and coordination across WGs, especially in ongoing and future 6G-related activities.

- Prevent further fragmentation and overlap in the specification of management-related functions.