3GPP TSG-SA5 Meeting #161	S5-252855

Fukuoka, Japan, 19 - 23 May 2025	revision of S5-252366

Source:	Nokia

Title:	Pseudo-CR on Procedure for CCL decision escalation

Document for:	Approval

Agenda item:	6.19.4.1

Spec:	3GPP TS28.567

Version:	0.2.0

Work Item:	Closed Control Loop Management

Comments

This pCR is to add the procedure for CCL decision escalation

Proposed Changes

* * * First Change * * * *

# 7	Procedures

## 7.A	CCL decision escalation

To enable escalation, there has to be entities to which decision can be escalated, called escalation recipients. These are mainly closed control loops but other decision makers, e.g. AIML inference functions could be used as escalation recipients. The CCL which wishes to escalate a decision is named an escalator CCL.

To enable escalation, each CCL contains an attribute identifying an entity acting as an escalation recipient to which a decision is escalated. The CCL also contains an attribute for defining the condition that triggers the escalation. For example, the CCL may trigger escalation when its level of confidence in the derived decision is below some threshold, in which case the confidence threshold is the condition for triggering the escalation. The confidence threshold attribute enables the CCL to autonomously make decisions for each situation and context based on its computed confidence level in the given situation. If the confidence level is lower than the confidence threshold the decision is escalated otherwise the decision is executed.

Figure 7.A-1: Procedure and interactions for CCL decision escalation

Step 0. The escalator CCL and the escalation recipient are composed, configured and instantiated.

Step 1. The MnS consumer configure the escalator CCL with information about the conditions under which to escalate and when to escalate to (the escalation recipient ). The escalator CCL can the trigger escalation either on its own or based on extra information form the Mns consumer.

Step 2. The escalator CCL executes analysis and decision making for a scenario. If the escalator CCL is confident with its decision it executes as normal

Step 3. The escalator CCL detect the need to escalate, e.g., for the case where it is not confident with its decision, the lack of confidence is the indicator of a scenario that should be escalated.

Step 4. When a CCL requires an escalation, escalator CCL instantiates a request for escalation on the escalation recipient.. The escalation request includes:

-	An attribute for proposed CM change as a plan containing the configuration management changes that has been proposed by the escalator CCL.

-	An attribute for the context and conditions describing decision constraints observed by the escalating CCL in making the decision(s).

Step 5. Based on the information in the escalation request, the capabilities of the escalation recipient as well as its observations of the evaluated network state, the escalation recipient decides whether it can undertake the escalation or not.

Step 6. The escalation recipient can notify (send an acceptane of) the escalation request.

Step 7. For an accepted escalation request, the escalation recipient derives an outcomes for the request

Step 8. The escalation recipient provides the outcomes to the escalator CCL, by writing it into an escalation outcome attribute on the escalation recipient. The outcome may then 	be written into an equivalent attribute on the escalator CCL, i.e., the escalator CCL contains an attribute for the escalation outcome to which the escalation recipient writes its computed escalation outcome. The escalation recipient contains an attribute for an escalation outcomes report in which it writes the derived outcomes for each corresponding escalation request. This can then be notified to the escalator CCL which subsequently reads it to obtain the recommendations.

The escalation outcome indicates whether the escalator CCL should take any action and what that action is. Accordingly, it contains an ENUM attribute to indicate what should be done by the escalator CCL, with the values:

-	"DONOTHING"- indicating that the escalator CCL does not need to take any action, i.e. the escalation recipient is addressing the scenario.

-	"APPLYACTION"- indicating that the escalator CCL should apply a specific set of actions proposed by the escalation recipient. The action is written into a proposed-actions attribute of the escalation outcome, which is the type plan according to TS 28.872[X].

-	"APPLYGUIDANCE"- indicating that the escalator CCL should compute a new CM change based on the guidance from the escalation recipient. The guidance is written into the proposed-actions attribute.

* * * Next Change * * * *

Annex X (informative):
UML code for procedure diagrams

# X.1		UML code for CCL coordination procedure diagrams

This annex contains the PlantUML source code for the procedure diagrams in clause 7 of the present document.

## X.1.1	CCL decision escalation procedure (Figure 7.A-1)

@startuml avoidance of potential action-execution-time conflicts - Information on detected conflict

skinparam Shadowing false

autonumber

skinparam monochrome true

participant "CCL MnS Consumer" as MNSCS

participant "CCL (Escalator CCL)" as ESCCL

participant "Escalation Recipient\n (e.g. another CCL or CCL Coordination Entity)" as ESCRP

Note over MNSCS, ESCRP: Compose, configure and instantiate the Escalator CCL and Escalation Recipient.

MNSCS -> ESCCL: configure or reconfigure Escalator CCL\n with when and where to escalate

Note over MNSCS,ESCCL: Trigger CCL execution

ESCCL -> ESCCL: Derive analysis and decision for a scenario

ESCCL -> ESCCL: detect need to escalate the scenario

ESCCL -> ESCRP: Request escalation for the scenario

ESCRP -> ESCRP: Decide whether to accept\n escalated request.

ESCRP -> ESCCL: Notify  acceptance of escalated request.

ESCRP -> ESCRP: Derive analysis and decision\n for a escalated scenario

ESCRP -> ESCCL: Notify  Escalator CCL of\n escalation outcome for the scenario.

@enduml

PlantUML source code for Figure 7.X-1 CCL NRM fragment

* * * End of Changes * * * *