3GPP TSG-SA5 Meeting #161	S5-252960

# Fukuoka, Japan, 19th May 2025 - 23rd May 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 32.130 | CR | 0055 | rev | 1 | Current version: | 19.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Rel-19 CR TS 32.130 Add management workflows for requirements for management of INS |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | China Unicom |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA5 |  |  |  |  |  |  |  |  |  |
| Work item code: | NetShare_OAM_Ph3 |  |  |  |  |  | Date: |  |  | 2025-05-08 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | During latest meetings, concepts and requirements of INS have been approved. To clearly demonstrate the process of management of INS and align with other scenarios, management workflows are supposed to be introduced. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add management workflows for requirements for management of INS |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The process of management of INS is not clear. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | New clause |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

First change

X	Management workflow for requirements for the management for Indirect Network Sharing scenario

This clause describes the workflows for the management for Indirect Network Sharing scenario.

In this workflow, the radio access network (i.e. one or multiple shared NG-RAN NE(s)) is shared between one MOP (MOP A identified by PLMN#1) and one POP (POP B identified by PLMN#2). MOP MnS consumer and MOP MnS producer for the management of shared NG-RAN NE(s) belong to MOP, and POP MnS consumer and POP MnS producer for the management of shared NG-RAN NE(s) belong to POP

For the REQ-INS-Cfg-CON-1:

MOP MnS consumer determines the individual EP_NgC MOI and EP_NgU MOI (see the attributes of NgC and NgU in TS 28.541[8]) for POP, and requests MnS producer to create and configure EP_NgC MOI and EP_NgU MOI for POP.

MOP MnS producer creates and configures the EP_NgC MOI and EP_NgU MOI for POP based on the requests from POP MnS consumer. The EP_NgC MOI and EP_NgU MOI are name containe by same GNBCUCPFunction MOI and GNBCUUPFunction MOI which is shared for different POPs.

For the REQ-INS-Cfg-CON-2:

MOP MnS consumer determines the attribute "PLMNInfoList" in NRCellDU MOI and NRCellCU MOI (see the attribute definition in TS 28.541[8]), which includes the PLMN#1 and PLMN#2, and requests MOP MnS producer to configure NRCellDU MOI and NRCellCU MOI with attribute "PLMNInfoList".

MOP MnS producer configures the NG-RAN NE(s) (i.e. subtree of ManagedElement MOI) based on the requests from MnS consumer, including configuring the NRCellDU MOI and NRCellCU MOIwith attribute "PLMNInfoList" to include PLMN#1 and PLMN#2.

For the REQ-INS-Cfg-CON-3:

MOP MnS producer determines the attribute "interPlmnFqdn" in AMFFunction MOI, SMFFunction MOI and UDMFunction MOI(see the attribute definition in TS 28.541[8]), which make them discoverable by POP, and MOP MnS producer to configure AMFFunction MOI, SMFFunction MOI and UDMFunction MOI with attribute " interPlmnFqdn ".

MOP MnS consumer determines the attribute "PLMNInfoList" in AMFFunction MOI (see the attribute definition in TS 28.541[8]), which includes the PLMN#1 and PLMN#2, and requests MOP MnS producer to configure AMFFunction MOI with attribute "PLMNInfoList".

POP MnS producer determines the attribute "interPlmnFqdn" in AMFFunction MOI, SMFFunction MOI and UDMFunction MOI(see the attribute definition in TS 28.541[8]), which make them discoverable by MOP, and POP MnS producer to configure AMFFunction MOI, SMFFunction MOI and UDMFunction MOI with attribute " interPlmnFqdn ".

End of change