3GPP TSG-SA5 Meeting #162	S5-253227

# Stor-Göteborg, Sweden, 25th Aug 2025 - 29th Aug 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 32.160 | CR | 0075 | rev | - | Current version: | 19.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Rel-19 CR 32.160 Clarify YANG mapping rules |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson Hungary Ltd |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S5 |  |  |  |  |  |  |  |  |  |
| Work item code: | SBMA_Ph3 |  |  |  |  |  | Date: |  |  | 2025-07-23 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | The design patterns for class containment are not well documented and don't reflect current best practice The rules for naming new YANG modules are not documented |  |  |  |  |  |  |  |  |
| Summary of change: |  | Update documentation for YANG containment and module naming The deprecated notification notifyChangedAlarm is substituted with the replacement notifyChangedAlarmGeneral. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Inconsistent YANG code, using different design patterns for the same problem. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.3, 6.2.1.2, 6.2.6, 6.2.6.1, 6.2.6.2, 6.2.6.2.1, 6.2.6.2.2, 6.2.6.2.3, 6.2.6.2.w, 6.2.6.2.x, 6.2.6.2.y, 6.2.6.2.z, 6.2.7, 6.2.8, 6.2.9 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

First change

## 5.3	Template for Management service operations and notifications

Y4	Overview

Yb	Management service name

Management service name should be replaced with the name of the Management Service (MnS).

"b" represents a number, starting at 1 and increasing by 1 with each new definition of a Management Service.

Yb.1	Operations and notifications

Yb.1.a	Operation OperationName

OperationName is the name of the operation followed by a qualifier indicating whether the operation is Mandatory (M), Optional (O), Conditional-Mandatory (CM), Conditional-Optional (CO), or SS-Conditional (C).

"a" represents a number, starting at 1 and increasing by 1 with each new definition of an operation.

Yb.1.a.1	Definition

Yb.1.a.1.1	Description

This subclause shall be written in natural language.

Operations have a lifecycleStatus property as defined by [3] clause 5.2.A. If and only if the lifecycleStatus is not current (its default value), that shall be indicated in this subclause.

Information on traceability back to one or more requirements supported by this operation should also be defined here, in the following form:

| Referenced TS | Requirement label | Comment |
| --- | --- | --- |
| 3GPP TS 32.xyz [xy] | REQ-SM-CON-23 | Optional clarification |
| 3GPP TS 32.xyz [xy] | REQ-SM-FUN-11 | Optional clarification |

Yb.1.a.1.2	Pre-condition

A pre-condition is a collection of assertions joined by AND, OR, and NOT logical operators. The pre-condition shall be true before the operation is invoked. An example is given here below:

notificationCategoriesNotAllSubscribed OR notificationCategoriesParameterAbsentAndNotAllSubscribed

Each assertion is defined by a pair (propertyName, propertyDefinition). All assertions constituting the pre-condition are provided in a table. An example of such a table is given here below:

| Assertion Name | Definition |
| --- | --- |
| notificationCategoriesNotAllSubscribed | At least one notificationCategory identified in the notificationCategories input parameter is supported by an MnS producer and is not a member of the ntfNotificationCategorySet attribute of an NtfSubscription which is involved in a subscription relationship with the NtfSubscriber identified by the managerReference input parameter. |
| notificationCategoriesParameterAbsentAndNotAllSubscribed | The notificationCategories input parameter is absent and at least one notificationCategory supported by MnS producer is not a member of the ntfNotificationCategorySet attribute of an ntfSsubscription which is involved in a subscription relationship with the NtfSubscriber identified by the managerReference input parameter. |

Yb.1.a.1.3	Post-condition

A post-condition is a collection of assertions joined by AND, OR, and NOT logical operators. The post-condition shall be true after the completion of the operation. When nothing is said in a post-condition regarding an information entity, the assumption is that this information entity has not changed compared to what is stated in the
pre-condition. An example is given here below:

subscriptionDeleted OR allSubscriptionDeleted

Each assertion is defined by a pair (propertyName, propertyDefinition). All assertions constituting the post-condition shall be provided in a table. An example of such a table is given here below:

| Assertion Name | Definition |
| --- | --- |
| subscriptionDeleted | The ntfSubscription identified by subscriptionId input parameter is no more involved in a subscription relationship with the ntfSubscriber identified by the managerReference input parameter and has been deleted. If this ntfSubscriber has no more ntfSubscription, it is deleted as well. |
| allSubscriptionDeleted | In the case subscriptionId input parameter was absent, the ntfSubscriber identified by the managerReference input parameter is no more involved in any subscription relationship and is deleted, the corresponding ntfSubscription have been deleted as well. |

Yb.1.a.1.4	Exceptions

List of exceptions that can be raised by the operation. Each element shall be a tuple (exceptionName, condition, ReturnedInformation, exitState).

Yb.1.a.1.4.c		exceptionName

ExceptionName is the name of an exception.

"c" represents a number, starting at 1 and increasing by 1 with each new definition of an exception.

This information shall be provided in a table. An example of such a table is given here below:

| Exception Name | Definition |
| --- | --- |
| ope_failed_existing_subscription | Condition: (notificationCategoriesNotAllSubscribed OR notificationCategoriesParameterAbsentAndNotAllSubscribed) not verified. Returned information: output parameter status is set to OperationFailedExistingSubscription. Exit state: Entry State. |

NOTE: An example of an exception can be a situation where an operation is raised and the required information between a consumer and producer cannot be conveyed via the input and output parameters.

Yb.1.a.2	Input parameters

List of input parameters of the operation. Each element contains theParameter Name, Support Qualifier, Documentation and Allowed Values and Properties. Legal Values for the Support Qualifier are: Mandatory (M), Optional (O), Conditional-Mandatory (CM), Conditional-Optional (CO), or SS-Conditional (C).

Properties shall include type and multiplicity. If multiplicity allows multiple values the properties isOrdered and isUnique shall also be included, if multiplicity is not greater than 1 isOrdered and isUnique shall be absent. The individual properties shall follow the same rules as attribute properties, see clause 5.2.

This information shall be provided in a table. An example of such a table is given here below:

| Parameter Name | S | Documentation and Allowed Values | Properties |
| --- | --- | --- | --- |
| eventIdList | M | One or more event identifiers | Type: DN multiplicity: 0..* isOrdered: False isUnique: True |

NOTE:	In the case where the Allowed Values can be enumerated, each element is a pair (Allowed Value Name, Allowed Value Semantics), unless an Allowed Value Semantics applies to several values in which case the definition can be provided only once.

Yb.1.a.3	Output parameters

List of output parameters of the operation. Each element contains the Parameter Name, Support Qualifier, Documentation and Allowed Values and Properties. Legal Values for the Support Qualifier are: Mandatory (M), Optional (O), Conditional-Mandatory (CM), Conditional-Optional (CO), or SS-Conditional (C).

Properties shall include type and multiplicity. If multiplicity allows multiple values the properties isOrdered and isUnique shall also be included, if multiplicity is not greater than 1 isOrdered and isUnique shall be absent. The individual properties shall follow the same rules as attribute properties, see clause 5.2.

This information shall be provided in a table. An example of such a table is given here below:

| Parameter Name | S | Documentation and Allowed Values | Properties |
| --- | --- | --- | --- |
| eventTime | M | The parameter carries the - alarmRaisedTime in case notificationType carries notifyNewAlarm, - alarmChangedTime in case notificationType carries notifyChangedAlarmGeneral, alarmClearedTime in case notificationType carries notifyClearedAlarm. E.g. AlarmRecord.alarmRaisedTime | Type: DateTime multiplicity: 1 |

NOTE: 	Information Type qualifies the parameter of Parameter Name. In the case where the Legal Values can be enumerated, each element is a pair (Legal Value Name, Legal Value Semantics), unless a Legal Value Semantics applies to several values in which case the definition can be provided only once. When the Legal Values cannot be enumerated, the list of Legal Values is defined by a single definition.

This table shall also include a special parameter ’status’ to indicate the completion status of the operation (success, partial success, failure reason etc.).

Yb.1.a.4	Result

Yb.1.a.4,1	Error messages

This subclause presents error messages in case the operation is not successful.

This subclause does not need to be present when there are no error messages to define.

Yb.1.a.4,2	Constraints

This subclause presents constraints for the operation or its parameters.

This subclause does not need to be present when there are no constraints to define.

Yb.1.a	Notification NotificationName

NotificationName shall be the name of the notification followed by a qualifier indicating whether the notification is Mandatory (M), Optional (O), Conditional-Mandatory (CM), Conditional-Optional (CO) or SS-Conditional (C).

"a" represents a number, starting at 1 and increasing by 1 with each new definition of a notification.

Yb.1.a.1	Definition

This subclause shall be written in natural language.

Notifications have a lifecycleStatus property as defined by [3] clause 5.2.A. If and only if the lifecycleStatus is not current (its default value), that shall be indicated in this subclause.

Information on traceability back to one or more requirements supported by this notification should also be defined here, in the following form:

| Referenced TS | Requirement label | Comment |
| --- | --- | --- |
| 3GPP TS 32.xyz [xy] | REQ-SM-CON-23 | Optional clarification |
| 3GPP TS 32.xyz [xy] | REQ-SM-FUN-11 | Optional clarification |

Yb.1.a.2	Input parameters

List of input parameters of the notification. Each element contains the Parameter Name, Support Qualifier, Documentation and Allowed Values and Properties. The Support Qualifier indicates whether the attribute is Mandatory (M), Optional (O), Conditional-Mandatory (CM), Conditional-Optional (CO), or SS-Conditional (C).

Properties shall include type and multiplicity. If multiplicity allows multiple values the properties isOrdered and isUnique shall also be included, if multiplicity is not greater than 1 isOrdered and isUnique shall be absent. The individual properties shall follow the same rules as attribute properties, see clause 5.2.

This information shall be provided in a table. An example of such a table is given here below:

| Parameter Name | S | Documentation and Allowed Values | Properties |
| --- | --- | --- | --- |
| managerReference | M | It specifies the reference of the consumer to which notifications shall be sent. E.g. ntfSubscriber.ntfManagerReference | Type: String multiplicity: 0..* isOrdered: False isUnique: True |
| alarmType | M | AlarmInformation.eventType "Communications Alarm": a communication error alarm. "Processing Error Alarm": a processing error alarm. "Environmental Alarm": an environmental violation alarm. "Quality Of Service Alarm": a quality of service violation alarm. "Equipment Alarm": an alarm related to equipment malfunction. | Type: ENUM multiplicity: 0..1 |

NOTE:	In the case where the Allowed Values can be enumerated, each element is a pair (Allowed Value Name, Allowed Value Semantics), unless an Allowed Value Semantics applies to several values in which case the definition can be provided only once.

Yb.1.a.3	Triggering event

The triggering event for the notification to be sent is the transition from the information state defined by the "from state" subclause to the information state defined by the "to state" subclause.

Yb.1.a.3.1		From state

This subclause is a collection of assertions joined by AND, OR, and NOT logical operators. An example is given here below:

alarmMatched AND alarmInformationNotCleared

Each assertion is defined by a pair (propertyName, propertyDefinition). All assertions constituting the state "from state" are provided in a table. An example of such a table is given here below:

| Assertion Name | Definition |
| --- | --- |
| alarmMatched | The matching-criteria-attributes of the newly generated network alarm has values that are identical (matches) with ones in one AlarmInformation in AlarmList. |
| alarmInformationNotCleared | The perceivedSeverity of the newly generated network alarm is not Cleared. |

Yb.1.a.3.2		To state

This subclause contains a collection of assertions joined by AND, OR and NOT logical operators. When nothing is said in a to-state regarding an information entity, the assumption is that this information entity has not changed compared to what is stated in the from-state. An example is given here below:

resetAcknowledgementInformation AND perceivedSeverityUpdated

Each assertion is defined by a pair (propertyName, propertyDefinition). All assertions constituting the state "to state" are provided in a table. An example of such a table is given here below:

| Assertion Name | Definition |
| --- | --- |
| resetAcknowledgementInformation | The matched AlarmInformation identified in inv_alarmMatched in pre-condition has been updated according to the following rule: ackTime, ackUserId and ackSystemId are updated to contain no information; ackState is updated to "unacknowledged". |
| perceivedSeverityUpdated | The perceivedSeverity attribute of matched AlarmInformation identified in inv_alarmMatched in pre-condition has been updated. |

Yb.2	Managed information

Nextt change

#### 6.2.1.2	Unique YANG Module names

The names of 3GPP YANG modules shall start with the "_3gpp" prefix.

After the prefix the next part of the module name shall indicate the 3GPP specification and optionally the management area that is modeled. The last part of module name shall indicate the specific function that is modeled.

_3gpp-<specification-area>-<function>

Example:

_3gpp-common-managed-element

_3gpp-5gc-nrm-udmfunction

_3gpp-stm-nrm-stmfunction

Within one specification/functional area the middle part of the module name shall be used consistently. When chosing a new module name the YANG code moderator should be consulted.

Next change

### 6.2.6	Name containment

#### 6.2.6.1	Introduction

Reference [3] clause 5.2.4 - Composite aggregation association relationship

Example model: The classes ParentClass and LocalChildClass are defined in the YANG module _3gpp-ParentClass. ParentClass name-contains LocalChildClass. Another YANG module (_3gpp-ChildClass) defines classes ChildClass1 and ChildClass2. ParentClass name-contains ChildClass1 and ChildClass2.

As on Stage 2 all name-containment is optional, an if-feature statement should be added under “list”, “uses” or “augment” statements modeling name-containment.  However, if a YANG module models only a single containment relationship, which is modeled by an augment statement, the if-feature statement is not needed, as the optionality is modeled with the implementation or the non-implementation of the module.

The YANG feature should be named <Child>Under<ParentIocName>  . The <Child> section is usually not the name of a specific class, but some name identifying a collection of child classes. The feature statement should be placed in the YANG module where it is used.

Even if a containment relationship (and the contained IOC) is marked as not supported by the YANG feature, any imported but not implemented YANG modules still need to be present in the product with a conformance statement import-only.(See RFC 8525 [19] conformance-type indicated either by leaf conformance-type or by placing the module under the import-only-module list).. This should not be a problem for implementers as real implementation is not needed, only the YANG files need to be present.

#### 6.2.6.2	YANG mapping

##### 6.2.6.2.1	General

The containment of classes defined in the same YANG module is mapped as embedded "lists".

Containment of classes defined in different YANG modules can be mapped in one of two ways: using the augment or the uses statements.

##### 6.2.6.2.2	Void

##### 6.2.6.2.3	Void

##### 6.2.6.2.w	Parent and child classes in the same YANG module

If the ParentClass and ChildClass are defined in the same YANG module the ChildClass shall be placed inside the list representing the ParentClass after its attribute container.

// Local class containment

module _3gpp-ParentClass {

feature LocalChildClassUnderParentClass {

description “Indicates that LocalChildClass is contained under ParentClass”;

}

grouping ChildClassGrp {

// ChildClass attributes

}

grouping ParentClassGrp {

// ParentClass attributes

}

list ParentClass {

key id;

leaf id {}  // usually defined by uses top3gpp:Top_Grp;

attributes {

use ParentClassGrp;

}

list LocalChildClass {

if-feature LocalChildClassUnderParentClass;

key id;

uses top3gpp:Top_Grp;

attributes {

uses LocalChildClassGrp;

}

}

}

}

##### 6.2.6.2.x	Parent and child classes in different YANG modules – grouping/uses based

If the ParentClass and ChildClass are defined in different YANG modules the ChildClass shall be placed inside a grouping statement named "ChildClassTreeGrp". (It is called a "tree", as the grouping itself may contain further child classes effectively a subtree of classes.) The ParentClass shall use the "uses" statement to include the ChildClassTreeGrp.

// Grouping/uses based class containment

module _3gpp-ParentClass {

import _3gpp-ChildClass { prefix yyy3gpp; }

feature ChildClassUnderParentClass {

description "Indicates that ChildClass shall be contained

under ParentClass";

}

grouping ParentClassGrp {

// ParentClass attributes

}

list ParentClass {

key id;

uses top3gpp:Top_Grp;

attributes {

use ParentClassGrp;

}

uses yyy3gpp: ChildClassSubtree {

if-feature ChildClassUnderParentClass;

}

}

}

module _3gpp-ChildClass {

grouping ChildClassGrp {

// ChildClassGrp attributes

}

grouping ChildClassSubtree {

list ChildClass {

key id;

uses top3gpp:Top_Grp;

attributes {

uses ChildClassGrp;

}

}

}

}

Multiple parent classes may contain the ChildClass with the "uses" statement.

##### 6.2.6.2.y	Parent and child classes in different YANG modules – augment based

If the ChildClass can be contained only by a single or a small number of paths (<=2) then it is possible to use the augment statement to specify class containment. The "paths" correspond to the "target" argument of the augment statement.

A "path" in this context means the full list of classes and their containment relationships from the root information object class (e.g. ManagedElement or SubNetwork) to the ChildClass. A "path" is the list of classes in the distinguished name leading from the root class to the childclass (without considering the id values).

If the number of containment paths is uncertain, augment-based containment should not be used.

Containment by augment is simpler than uses/grouping because it does not involve modifying the ParentClass. However, if the number of containing paths is big or uncertain, the uses/grouping-based solution is preferred. In case the containment needs to be modified later, maintaining the correct augment paths is difficult.

// Augment based class containment

module _3gpp-ParentClass {

grouping ParentClassGrp {

// ParentClass attributes

}

list ParentClass {

key id;

uses top3gpp:Top_Grp;

attributes {

use ParentClassGrp;

}

}

}

module _3gpp-ChildClass {

import _3gpp-ParentClass { prefix xx3gpp;}

feature ChildClassUnderParentClass {

description “Indicates that ChildClass is contained under

ParentClass”;

}

grouping ChildClassGrp {

// ChildClassGrp attributes

}

augment /xx3gpp:ParentClass {

// this path might be longer e.g.

// /me3gpp:ManagedElement/gnbcucp3gpp:GNBCUCPFunction/eutranet3gpp:EUtraNetwork

if-feature ChildClassUnderParentClass;

list ChildClass {

key id;

uses top3gpp:Top_Grp;

attributes {

uses ChildClass1Grp;

}

}

}

}

It is possible to augment a ChildClass into multiple ParentClasses, however adding more than two augment statements is not preferred. Note, a separate augment statement is needed for each containing "path" even if the immediately containing class is the same.

augment /SubNetwork/ManagedElement/ParentClass

and

augment /ManagedElement/ParentClass

If a ChildClass is augmented into multiple containing "paths" the ChildClass shall be placed inside a grouping statement named "ChildClassTreeGrp" just as for grouping/uses based containment in clause 6.2.6.2.x. The ChildClassTreeGrp should be used in each augment statement.

augment /xx3gpp:ParentClass {

if-feature ChildClassUnderParentClass;

uses ChildClassTreeGrp;

}

##### 6.2.6.2.z	Optional containment

On Stage 2 all name-containment is optional. This is modeled as an if-feature statement that shall be added under the “list”, “uses” or “augment” statements modeling name-containment. However, if a YANG module uses a single augment statement to model containment, the if-feature statement is not needed and shall not be used, as the optionality is modeled with the implementation or the non-implementation of the ChildClass' module.

The YANG feature should be named <ChildIocName>Under<ParentIocName>. The feature statement should be placed in the YANG module where it is used. In special cases it is possible and advantageous to have a common feature e.g. GroupOfChildClassesUnderParentClass.

Even if a containment relationship (and the contained IOC) is marked as not supported by the YANG feature, any imported but not implemented YANG modules still need to be present in the product with a conformance statement import-only.(See RFC 8525 [19] conformance-type indicated either by leaf conformance-type or by placing the module under the import-only-module list).. This should not be a problem for implementers as real implementation is not needed, only the YANG files need to be present.

### 6.2.7	Recursive containment - reference based solution

The NRM stage 2 definition contains some cases where a class contains itself (so called recursive containment) e.g. SubNetwork, VsDataContainer, ManagedFunctio, NetworkSliceSubnet classes.

Recursive containment for SubNetwork is modeled using YANG schema-mount.

For other classes recursive containment may be modeled

using a pair of "leaf-list" references between the instances of the class. The references shall be named "leaf-list parents {…}" and "leaf-list containedChildren {…}". Note the two reference "leaf-lists" should be defined directly under the "list" defining the class not in its "grouping" because the "path" statements are specific to each class, so the "leaf-lists" must not be inherited.

list ExampleClass {

key id;

leaf id {..}

container attributes {

uses ExampleClassGrp;

leaf-list parents {

description "Reference to all containg ExampleClass instances

in strict order from the root ExampleClass down to the immediate

parent ExampleClass.

If ExampleClasses form a containment hierarchy this is

modeled using references between the child ExampleClass and the parent

ExampleClasses.

This reference MUST NOT be present for the top level ExampleClass and

MUST be present for other ExampleClasses.";

type leafref {

path "../../../ExampleClass/id";

}

}

leaf-list containedChildren{

description "Reference to all directly contained ExampleClass instances.

If ExampleClasses form a containment hierarchy this is

modeled using references between the child ExampleClass and the parent

ExampleClass.";

type leafref {

path "../../../ExampleClass/id";

}

}

}

The following instance data example shows how the reference values specify the ExampleClass hierarchy:

Top level:  exclass=root

| \   +----------------+

|  +--------+          |

|           |          |

Level 1:    exclass=A1  exclass=B1  exclass=C1

| \   +----------------+

|  +--------+          |

|           |          |

Level 2:   exclass=A2   exclass=B2  exclass=C2

| \   +----------------+

|  +--------+          |

|           |          |

Level 3:   exclass=A3   exclass=B3  exclass=C3

Top level: id=root        parents=null         containedChildren= A1,B1,C1

Level 1:   id=A1,(B1,C1)  parents=root         containedChildren = A2,B2,C2

Level 2:   id=A2,(B2,C2)  parents=root,A1      containedChildren = A3,B3,C3

Level 3:   id=A3,(B3,C3)  parents=root,A1,A2   containedChildren = A4,B4

When reading/writing self-contained classes only the last such class instance needs to be specified in the Netconf request as that uniquely identifies the exact instance. The following Netconf request could be used to retrieve all attributes of ExampleClass=root, ExampleClass=A1, ExampleClass=B2, NRFrequency=22

<rpc message-id="101" xmlns="urn:ietf:params:xml:ns:netconf:base:1.0">

<get-config>

<source>

<running/>

</source>

<!-- ExampleClass=root, ExampleClass=A1, ExampleClass=B2, NRFrequency=22 -->

<filter type="subtree"/>

<ExampleClass>

<id>B2</id>

<NRFrequency>

<id>22</>

<attributes/>

</NRFrequency>

</ExampleClass>

</get-config>

</rpc>

There is no need to specify the ancestors ExampleClass=root, ExampleClass=A1 as any ExampleClass can be addressed directly.

### 6.2.8	Multi-root management tree

YANG supports multi-rooted managed models natively; the standardized IETF models have many root "list"/"container" nodes.

### 6.2.9	Alternative containment

Stage 2 models allows multiple different name-containment hierarchies. A particular name-containment hierarchy implemented by a specific vendor/product can be discovered in run-time, by reading the content of the ietf-yang-library and the ietf-yang-schema mount modules.

YANG provides multiple possible methods to model alternative containment hierarchies.

Depending on how containment is modeled either

- YANG features may be used to specify optional containment or

- implementation or not implementation of a YANG module

See clause 6.2.6.

In cases where the number of YANG modules affected by the alternative containment is large (cca. more than 8), the following mapping is proposed (using the optional containment of SubNetwork and ManagedElement as an example):

-	If the ManagedElement is a root class, no further documentation or implementation steps are required.

- 	If the ManagedElement shall be contained under Subnetwork it shall be mounted under the SubNetwork "list" using the YANG schema mount mechanism as described in RFC 8528 [13].

Mounted schemas will appear in Netconf, the CLI and management GUIs as if they were part of a common containment hierarchy.

Yang Schema Mount provides vendor the flexibility of arranging the containment tree in accordance of operator intention, and provides a way for a consumer to discover the actual mount and containment hierarchy in run-time.

End of  changes