# 3GPP TSG SA WG5 Meeting #162	S5- 253359

# Goteborg, Sweden 25 - 29 August 2025

# Source:	Amdocs, AT&T

# Title:	Discussion Paper for Optimizing CHF Messages

# Document for:	Discussion

Agenda Item:	7.2

# Decision/action requested

Discuss options for optimizing messages in the Nchf Service-based Interface (SBI) for CHF.

# References

- 3GPP TS 32.290: "Telecommunication management; Charging management; 5G system; Services, operations and procedures of charging using Service Based Interface (SBI)".

- 3GPP TS 32.291: "Telecommunication management; Charging management; 5G system, charging service; Stage 3".

- 3GPP TS 32.254:  "Telecommunication management; Charging management; Exposure function Northbound Application Program Interfaces (APIs) charging".

- 3GPP TS 32.255:  "Telecommunication management; Charging management; 5G data connectivity domain charging; Stage 2".

- 3GPP TS 32.256:  "Telecommunication management; Charging management; 5G connection and mobility domain charging; Stage 2".

- 3GPP TS 32.257: "Telecommunication management; Charging management; Edge computing domain charging".

- 3GPP TS 32.273:  "Telecommunication management; Charging management; Multimedia Broadcast and Multicast Service (MBMS) charging".

- 3GPP TS 32.277:  "Telecommunication management; Charging management; Proximity-based Services (ProSe) charging".

- 3GPP TS 32.282:  "Charging management; Time-Sensitive Networking (TSN) charging".

- 3GPP TS 29.503: “5G System; Unified Data Management Services; Stage 3”

- 3GPP TR 28.840: "Study on CHF Segmentation

# Rationale

## Background

The current (Rel-19) Nchf_ConvergedCharging interface has 20+ charging information containers to support charging for various network services and Charging Trigger Functions (CTFs).  As new monetizable services are introduced in each network evolution, and the range of functionalities required in the CHF widens, this interface will continue grow in both size and resource-level complexity making it increasingly difficult to manage changes and deploy updates and new interface versions into the network and to handle increasing message payload sizes.

This discussion paper proposes to simplify the Nchf_ConvergedCharging interface for future growth while maintaining support for existing consumers of the Nchf services.

The aim of the proposed approach is to offer a simpler access to CHF services going forward and to maintain backward compatibility for consuming network functions that may wish to continue using the current interface.  The proposed approach relies on the concepts of CHF Segmentation and Charging Domains previously discussed in TR 28.840 [11].

Specific problems with the current Nchf_ConvergedCharging SBI we aim to address:

- Complexity:

A large number of attributes and containers (Information Elements) makes the API more complex and harder to understand. Developers may need to spend more time learning the API. Example: An API with dozens of containers, each with numerous attributes, can be overwhelming.

- Increased Payload Size:

Large containers with many attributes can result in larger payload sizes, which can affect the performance of the API, especially with high volumes of requests.

- Potential for Redundancy:

Grouping attributes into containers can sometimes lead to redundancy, where the same attribute is repeated in multiple containers.

## Proposal Summary

At a high level, this paper proposes to segment the charging information into different resources, each of which would be accessible via a different URI.  One approach for the proposed resource segmentation is to base it in part on the supportedFeatures attribute in Nchf_ConvergedCharging, as defined in TS 32.291 [2], clause 6.1.8.

The supportedFeatures attribute enables a Consumer NF to communicate to the CHF the specific charging functionalities that it requires.  Therefore, segmenting the Nchf charging information along these lines could provide consumers of CHF services the flexibility to address and use only the specific information that is relevant and required for the services that the Consumer NF needs.

The following examples may help to clarify this further:

- The 5MBS_CH feature (23) could be set by the Broadcast Multicast - Service Centre (as CTF) to use only the mBSSessionChargingInformation container necessary for charging 5G multicast-broadcast services.

- The ProSe feature (30) could be set by the 5G Direct Discovery Name Management Function (as CTF) to use only the proSeChargingInformation container necessary for charging 5G proximity services.

- The NSAC_CH feature (28) could be set by the Network Slice Admission Control Function (as CTF) to use only the nSACFChargingInformation container necessary for network slice admission control charging.

- The TSN feature (26) could be set by the Time Sensitive Networking Application Function (as CTF) to use only the tSNChargingInformation container necessary for charging for time sensitive networking services.

- The EdgeComputing feature (18) could be set by the Charging Enablement Function (CEF) as CTF to use only the eASDeploymentChargingInformation container necessary for EAS management charging, based on the Management Services that CEF consumes.

- The EdgeComputing feature (18) could be set by the CEF (as CTF) to use only the edgeInfrastructureUsageChargingInformation container necessary for charging of edge enabling infrastructure resource usage, based on the Management Services that CEF consumes.

- The EdgeComputing feature (18) could also be set by the Edge Enabler Server (as CTF) to use the directEdgeEnablingServiceChargingInformation or exposedEdgeEnablingServiceChargingInformation containers, as needed, for charging edge enabling services.

Note that these examples are illustrative, and this approach may not be suitable or desirable for every charging domain specified in the Nchf_ConvergedCharging supportedFeatures.  But where it is used, this proposal will enable Consumer NFs to limit their charging requests to only the charging information (and resources) that are of specific interest to that Consumer NF.

As new network services and charging nodes are added in future releases, the related charging information segmentation and the specified supportedFeatures can be extended easily to accommodate these new requirements.

# Detailed proposal

Within the 5GC, CHF services are consumed by various NFs acting as a CTF, including but not limited to SMF, AMF, SMSF, NEF, GMLC, TSN AF via the Nchf service-based interface (SBI).

CHF services may also be consumed by CTFs external to 5GC, for example in Edge Computing subsystems or in the Management domain.

The figure below illustrates the logical reference point CHF architecture and information flows for converged charging using Nchf.

Figure 4.1:  Reference Point architecture for CHF (based on TS 23.240 v19.2.0, Figure 4.2.3.2)

NOTE:  (x-)CHF in the figure above refers to inter-CHF interactions involving V-CHF, Business CHF (B-CHF), Consumer CHF (C-CHF), etc.

The current CHF architecture also provides for charging of management domain services from Management Data Analytics Service (MDAS) and the control domain from Network Data Analytics Function (NWDAF), both of which are supported via the Charging Enablement Function (CEF).  These are not shown in the figure above.

## Granularity of Resources in Nudm SBI

To better explain the proposed CHF approach, we consider how the Nudm SBI structures access to its different resources.

The 5GC UDM offers various services and functionality via the Nudm SBI to range of Consumer NFs, which are further segmented based on the specific needs of the service consumer.  For example, the Nudm_SubscriberDataManagement Service is used by Consumer NFs (e.g., AMF, AAnF, SMF, SMSF, GMLC, NEF, 5G DDNMF, NWDAF, DCCF, trusted AF) to retrieve the UE's individual subscription data relevant to the Consumer NF [10].

The table below shows some examples of different subscriber data sets that are accessible using the Nudm_SubscriberDataManagement service:

| Subscriber Data Set | Description | Resource URI |
| --- | --- | --- |
| Slice Selection Subscription Data | UE's subscribed Network Slice Selection Assistance Information | /nudm-sdm/<apiVersion>/{supi}/nssai |
| Access and Mobility Subscription Data | UE's subscribed Access and Mobility Data | /nudm-sdm/<apiVersion>/{supi}/am-data |
| SMF Selection Subscription Data | UE's subscribed SMF Selection Data | /nudm-sdm/<apiVersion>/{supi}/smf-select-data |
| Session Management Subscription Data | UE's session management subscription data | /nudm-sdm/<apiVersion>/{supi}/sm-data |
| SMS Subscription Data | UE's SMS subscription data | /nudm-sdm/<apiVersion>/{supi}/sms-data |
| Identifier Translation | Retrieve UE's SUPI or GPSI | /nudm-sdm/<apiVersion>/{ueId}/id-translation-result |
| User Consent Subscription Data | UE's User Consent subscription data | /nudm-sdm/<apiVersion>/{supi}/uc-data |

## Proposed Enhancement to Nchf SBI

We propose a study that considers a similar approach to Nudm to simplify use of the Nchf_ConvergedCharging SBI for Consumer NFs with requirements for charging for a single service or a limited number of services.  The proposed Nchf interface would enable these Consumer NFs to target their charging requests to only the CHF functionality that is relevant to their specific service(s) and using only the relevant subset of Nchf data.

So, mirroring the UDM example above, the table below provides a possible view of an Nchf interface that implements the proposed approach (please note that not all the charging services are shown):

| Charging | Description | Proposed CHF Resource URI |
| --- | --- | --- |
| Current Charging API | Charging for all services Included for full backward compatibility. | chargingdata/ |
| Northbound Exposure API | Charging for Northbound APIs exposed over T8 and N33 reference points via NEF, as in TS 32.291, clause 7.5. | nef-charging |
| Time Sensitive Networking (TSN) | Charging for TSN, including TSN 5GS bridge configuration & management; Time Sensitive Communications (TSC) and Time Synchronization; TSC traffic | tsn-charging |
| Connection and Mobility | Charging for functionalities supported by the AMF as in TS 32.291, clause 7.4, including: - Registration Management; - Connection Management; - Location Reporting | registration-charging n2-connection-charging loc-report-charging inter-chf-info |
| Proximity-based Services | Charging for proximity services, including Discovery and Direct Communications, via 5G-DDNMF, as in TS 32.291, clause 7.9. | prose-charging |
| Short Message Service (SMS) | Charging for various SMS scenarios involving persons, applications, device triggers, via SMSF as in TS 32.291, clause 7.3. | smsc-charging |
| Network Slice Admission Control | Charging for Network Slice Admission Control, including monitoring and controlling the number of registered UEs and the number of established PDU Sessions per network slice, via NSACF as in TS 32.291, clause 7.15. | nsacf-charging |
| Edge Computing | Charging for edge computing applications and usage of edge infrastructure resources as in TS 32.257. | edge-computing |

This approach also enables the deployment of CHF instances that support specific services that are required by specific Consumer NFs.  For example, deploying a CHF instance specifically for Northbound Exposure API charging, with NEF as the Consumer NF (CTF).

As noted above, the proposed enhancement may not be applicable or useful globally across the entire set of Consumer NFs of the Nchf SBI.  Nor would all implementers wish to follow this approach.

For backward compatibility it is necessary that the existing Nchf chargingdata URI be maintained, and the new Nchf capability be activated via a supportedFeature that is negotiated between CHF and each Consumer NF serving as a CTF. The first row in the above table indicates that the chargingdata resource is maintained as it is in Release 19.

For some of the above refined resources, the HTTP operations would also be simplified. For example, for Post-Event Charging (PEC), there is no need to have chargingdata/{ChargingDataRef}/update and have chargingdata/{ChargingDataRef}/delete custom HTTP operations.

The request and response schema will be extended to include the simplified IEs necessary.

Benefits:

- This approach will enable CHF provider implementations to selectively provide a subset of the converged charging service.

- This approach will provide clarity with regards to the request and response data specific to the resource.

- Charging requests/responses will be smaller compared to the Release 19 version.

- This approach will be backward compatible with Release 19 SBI.

- There would be a one-time cost of refactoring. Subsequent updates would be straightforward.

Drawbacks:

- The OpenAPI specification will be bigger and may impact tooling for automated code generation and documentation pipelines.

- The one-time refactoring effort would require coordinated updates across multiple network functions and test environments, which could delay adoption.