3GPP TSG-SA5 Meeting #162	S5-253366

Goteborg, Sweden, 25 - 29 August 2025

Source:	Huawei

Title:	Pseudo-CR on TR 28.881 Add new issue for enhancement of radio service delivering and assurance scenarios

Document for:	Approval

Agenda item:	6.20.1

Spec:	3GPP TR 28.881

Version:	V0.0.0

Work Item:	FS_IDMS_MN_Ph4

Comments

This contribution proposes to add a new issue for the Enhancement of radio service delivering and assurance scenarios based on SP-250861.

This pCR related to WT-1.2 and WT-6.1.

Proposed Changes

* * * First Change * * * *

2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[X]	3GPP TS 28.312: "Management and orchestration; Intent driven management services for mobile networks".

[Y]	3GPP TS 28.541: "Management and orchestration; 5G Network Resource Model (NRM); Stage 2 and stage 3".

[Z]	3GPP TS 29.572: "5G System; Location Management Services; Stage 3"

* * * Next Change * * * *

4.X Issue#X: Enhancement of radio service delivering and assurance scenarios

4.X.1 Description

In 3GPP TS 28.312 [X], the existing use case and requirements for intent containing an expectation for delivering a radio service is described in clause 5.1.2. The RadioServiceExpectation is defined to represent MnS consumer's expectations for radio service delivering and assurance in the specified area. However, following scenarios are not supported:

-	MnS consumer expresses the radio service delivering and assurance expectation with service reliability information.

-	MnS consumer expresses the radio service delivering and assurance expectation for a specified assurance time duration. Using the concert as an example, MnS consumer expresses the intent indicating a radio service to be delivered and assured for two hours.

-	MnS consumer expresses the radio service delivering and assurance intent expectation for a specified area with explicitly specified scene (e.g. Venue, Airport, Outdoor). Different scenes have different typical traffic characteristics. It is useful for the MnS producer to fulfil the radio service intent with the scene the information. For example, MnS consumer expresses the radio service delivering and assurance intent expectation for area A which is an airport.

-	MnS consumer expresses the radio service delivering and assurance intent expectation for a specified area described in the form of civic address. For example, MnS consumer wants to ensure the radio service targets (dLThptPerUETarget and dLLatencyTarget) for a specific civic address.

4.X.2 Potential requirements

REQ-IDMS_RadioServiceIntent -CON-1: The intent driven MnS producer for radio service should have capabilities enabling the MnS consumer to express service reliability requirements or assurance time duration requirements.

REQ-IDMS_RadioServiceIntent -CON-2: The intent driven MnS producer for radio service should have capabilities enabling the MnS consumer to specify the scene information for the radio service to be delivered and assured in a specified area.

REQ-IDMS_RadioServiceIntent -CON-3: The intent driven MnS producer for radio service should have capabilities enabling the MnS consumer to express radio service delivering and assurance expectations for a specified area described in the form of civic address.

4.X.3 Potential solution

This solution proposes to reuse and enhance the existing RadioServiceExpectation defined in 3GPP TS 28.312 [2].

Enhancement Aspect1: Add following attributes as the ExpectationTargets for the RadioServiceExpectation to support reliability requirements:

- dLReliabilityTarget, it represents the DL reliability target for the radio service that the intent expectation is applied. The detailed definition for dLReliability see TS 28.541 [Y]. The type is Real.

- ulReliabilityTarget, it represents the UL reliability target for the radio service that the intent expectation is applied. The detailed definition for uLReliability see TS 28.541 [Y]. The type is Real.

Enhancement Aspect2: Add following attributes as the ObjectContexts for the RadioServiceExpectation to support scene information and civic address information:

- SceneTypeContext, it describes the scene for the coverage areas for the Radio Service that the intent expectation. The type is string.

- CivicAddressContext, the coverage areas for the Radio Service that the intent expectation is applied in the form of civic address. The detailed definition for civic address reuses the CivicAddress defined in clause 6.1.6.2.14 in TS 29.572 [Z].

Enhancement Aspect3: Add following attributes as the Expectation Contexts for the RadioServiceExpectation to support assurance duration information:

-	AssuranceDurationContext, it describes the time duration at which the radio service instance is scheduled to be available and guaranteed. The type is integer and unit is hour.

4.X.4 Evaluation of potential solutions

TBD

* * * End of Changes * * * *