3GPP TSG-SA5 Meeting #162	S5-253422

Goteborg, Sweden, 25 - 29 August 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 28.105 | CR | DraftCR | rev | - | Current version: | 19.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Input to DraftCR Rel-19 TS 28.105 Add Procedure for ML Model LCM |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ZTE Corporation |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA5 |  |  |  |  |  |  |  |  |  |
| Work item code: | AIML_MGT_Ph2 |  |  |  |  |  | Date: |  |  | 2025-08-01 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | TS 28.105 currently omits the generic life-cycle procedures for ML models—training, testing, deployment, and inference. This contribution fills that gap by adding clear, end-to-end procedures that significantly enhance readability. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add procedures for trianing, testing, depolyment, and inference of ML model. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Relevant procedures are missing. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7.x (new), A.x(new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | No impact on Stage 3 |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

Start of the First Change

7.x	Generic Procedure for AI/ML management

7.x.1	Procedure for Consumer Initiated ML Model Training

Figure 7.x.1-1: Procedure for Consumer Initiated ML Model Training

Step 1. MnS Consumer sends a request to create an MLTrainingRequest MOI (see createMOI operation defined in TS 28.532 [11]) to MnS Producer with the training information, where the MLTrainingFunction acts as the MnS Producer for ML Training. The detailed training information sees attributes of the concrete MLTrainingRequest IOC defined in clause 7.3a.

Step 2. Based on the received request, MnS Producer creates the concrete MLTrainingRequest MOI (i.e. instance of MLTrainingRequest IOC) and configure the new created MLTrainingRequest MOI with the received training information.

Step 3.	MnS Producer sends a response (see createMOI operation defined in TS 28.532[11]) to MnS Consumer.

Step 4. MnS Producer creates the concrete MLTrainingProcess MOI (i.e. instance of MLTrainingProcess IOC) and MLTrainingReport MOI (i.e. instance of MLTrainingReport IOC), and configure the new created MLTrainingProcess MOI and MLTrainingReport MOI based on ML training process and ML training results, respectively.

Note: The step 2-4 above is an example for triggering ML training and there is no restriction on the sequence.

Step 5. MnS Producer sends a notification to the MnS Consumer for creating MLTrainingProcess MOI and MLTrainingReport MOI.

Step 6.	Based on the created MLTrainingRequest MOI, MnS Producer performs the ML model training for the specific aIMLInferenceName.

Step 7.	MnS Producer notifies the ML training report to the MnS consumer.

7.x.2	Procedure for Consumer Initiated ML Model Testing

Figure 7.x.2-1: Procedure for Consumer Initiated ML Model Testing

Step 1. MnS Consumer sends a request to create an MLTestingRequest MOI (see createMOI operation defined in TS 28.532 [11]) to MnS Producer with the ML model testing information, where the MLTestingFunction or MLTrainingFunction acts as the MnS Producer for ML testing. The detailed ML model testing information sees attributes of the concrete MLTestingRequest IOC defined in clause 7.3a.

Step 2. Based on the received request, the MnS Producer creates the concrete MLTestingRequest MOI (i.e. instance of MLTestingRequest IOC) and configure the new created MLTestingRequest MOI with the received ML model testing information.

Step 3.	MnS Producer sends a response (see createMOI operation defined in TS 28.532[11]) to the MnS Consumer.

Step 4. MnS Producer creates and configures the concrete MLTestingReport MOI (i.e. instance of MLTestingReport IOC).

Step 5. MnS Producer sends a notification to the MnS Consumer for creating MLTestingReport MOI.

Note: The step 2-5 above is an example for triggering ML Model testing and there is no restriction on the sequence.

Step 6.	Based on the created MLTestingRequest MOI, MnS Producer performs the ML model testing for specific ML Model(s).

Step 7.	MnS Producer notifies the ML testing report to the MnS consumer.

7.x.3	Procedure for Consumer Initiated ML Model Deployment

Figure 7.x.1-1: Procedure for Consumer Initiated ML Model Deployment

Step 1. MnS Consumer sends a request to create an MLModelLoadingRequest MOI (see createMOI operation defined in TS 28.532 [11]) to MnS Producer with the ML Model loading information, where the AIMLInferenceFunction acts as the MnS Producer for ML Model Loading. The detailed ML Model Loading information sees attributes of the concrete MLModelLoadingRequest IOC defined in clause 7.3a.

Step 2. Based on the received request, the MnS Producer creates the concrete MLModelLoadingRequest MOI (i.e. instance of MLModelLoadingRequest IOC) and configure the new created MLModelLoadingRequest MOI with the received ML Model loading information.

Step 3.	MnS Producer sends a response (see createMOI operation defined in TS 28.532[11]) to the MnS Consumer.

Step 4. MnS Producer creates the concrete MLModelLoadingProcess MOI (i.e. instance of MLModelLoadingProcess IOC) and configure the new created MLModelLoadingProcess MOI based on ML Model loading process.

Step 5. MnS Producer sends a notification to the MnS Consumer for creating MLModelLoadingProcess MOI.

Note: The step 2-5 above is an example for triggering ML Model loading and there is no restriction on the sequence.

Step 6.	Based on the created MLModelLoadingRequest MOI, MnS Producer performs the ML model loading for the specific ML Model(s).

Step 7.	MnS Producer notifies the ML Model Loading completed notification to the MnS consumer.

7.x.4	Procedure for Consumer Initiated AI/ML Inference based on AIMLInferenceFunction

Figure 7.x.1-1: Procedure for Consumer Initiated AI/ML Inference

Step 1. AI/ML Inference MnS Consumer sends a request to modify the AIMLInferenceFunction MOI (see modifyMOIAttributes operation defined in TS 28.532 [11]) to AI/ML Inference MnS Producer with the inference information, where the AIMLInferenceFunction acts as the AI/ML Inference MnS Producer. The detailed inference information sees attributes of the concrete AIMLInferenceFunction IOC defined in clause 7.3a.

Step 2.	AI/ML Inference MnS Producer sends a response (see modifyMOIAttributes operation defined in TS 28.532[11]) to the AI/ML Inference MnS Consumer.

Step 3. Based on the received request, the AI/ML Inference MnS Producer creates and configures the concrete AIMLInferenceReport MOI (i.e. instance of AIMLInferenceReport IOC).

Step 4. AI/ML Inference MnS Producer sends a notification to the AI/ML Inference MnS Consumer for creating AIMLInferenceReport MOI.

Note: The step 2-4 above is an example for triggering AI/ML Inference and there is no restriction on the sequence.

Step 5.	 AI/ML Inference MnS Producer performs the AI/ML Inference based on the specified ML Model(s).

Step 6.	AI/ML Inference MnS Producer notifies the AI/ML Inference report to the AI/ML Inference MnS consumer.

End of the First Change

Start of the Second Change

Annex A (informative):
PlantUML source code for NRM class diagrams

A.X	Generic Procedure for AI/ML management

A.X.1	Procedure for Consumer Initiated ML Model Training

@startuml

hide footbox

autonumber

participant "ML Training MnS Consumer" #white

participant "ML Training MnS Producer" #white

"ML Training MnS Consumer" -> "ML Training MnS Producer": Request to create an MLTrainingRequest MOI

"ML Training MnS Producer" -> "ML Training MnS Producer": Create and configure MLTrainingRequest MOI

"ML Training MnS Producer" -> "ML Training MnS Consumer": Response for create an MLTrainingRequest MOI

"ML Training MnS Producer" -> "ML Training MnS Producer": Create and configure MLTrainingProcess MOI and MLTrainingReport MOI

"ML Training MnS Producer" -> "ML Training MnS Consumer": Notifiacation for create MLTrainingProcess MOI and MLTrainingReport MOI

"ML Training MnS Producer" -> "ML Training MnS Producer": Perform ML training

"ML Training MnS Producer" -> "ML Training MnS Consumer": Notify the ML Training Report

@enduml

A.X.2	Procedure for Consumer Initiated ML Model Testing

@startuml

hide footbox

autonumber

participant "MnS Consumer for ML Testing" #white

participant "MnS Producer for ML Testing" #white

"MnS Consumer for ML Testing" -> "MnS Producer for ML Testing": Request to create an MLTestingRequest MOI

"MnS Producer for ML Testing" -> "MnS Producer for ML Testing": Create and configure MLTestingRequest MOI

"MnS Producer for ML Testing" -> "MnS Consumer for ML Testing": Response for create an MLTestingRequest MOI

"MnS Producer for ML Testing" -> "MnS Producer for ML Testing": Create and configure MLTestingReport MOI

"MnS Producer for ML Testing" -> "MnS Consumer for ML Testing": Notifiacation for create the MLTestingReport MOI

"MnS Producer for ML Testing" -> "MnS Producer for ML Testing": Perform ML Testing

"MnS Producer for ML Testing" -> "MnS Consumer for ML Testing": Notify the ML Testing Report

@enduml

A.X.3	Procedure for Consumer Initiated ML Model Deployment

@startuml

hide footbox

autonumber

participant "MnS Consumer for ML Model Loading" #white

participant "MnS Producer for ML Model Loading" #white

"MnS Consumer for ML Model Loading" -> "MnS Producer for ML Model Loading": Request to create an MLModelLoadingRequest MOI

"MnS Producer for ML Model Loading" -> "MnS Producer for ML Model Loading": Create and configure MLModelLoadingRequest MOI

"MnS Producer for ML Model Loading" -> "MnS Consumer for ML Model Loading": Response for create an MLModelLoadingRequest MOI

"MnS Producer for ML Model Loading" -> "MnS Producer for ML Model Loading": Create and configure MLModelLoadingProcess MOI

"MnS Producer for ML Model Loading" -> "MnS Consumer for ML Model Loading": Notifiacation for create MLModelLoadingProcess MOI

"MnS Producer for ML Model Loading" -> "MnS Producer for ML Model Loading": Perform ML Model Loading

"MnS Producer for ML Model Loading" -> "MnS Consumer for ML Model Loading": Notification of the ML Model Loading completed

@enduml

A.X.4	Procedure for Consumer Initiated AI/ML Inference

@startuml

hide footbox

autonumber

participant "AI/ML Inference MnS Consumer" #white

participant "AI/ML Inference MnS Producer" #white

"AI/ML Inference MnS Consumer" -> "AI/ML Inference MnS Producer": Request to modify the AIMLInferenceFunction MOI

"AI/ML Inference MnS Producer" -> "AI/ML Inference MnS Consumer": Response for modify the AIMLInferenceFunction MOI

"AI/ML Inference MnS Producer" -> "AI/ML Inference MnS Producer": Create and configure an AIMLInferenceReport MOI

"AI/ML Inference MnS Producer" -> "AI/ML Inference MnS Consumer": Notification for create the AIMLInferenceReport MOI

"AI/ML Inference MnS Producer" -> "AI/ML Inference MnS Producer": Perform AI/MLInference

"AI/ML Inference MnS Producer" -> "AI/ML Inference MnS Consumer": Notify the AI/ML Inference Report

@enduml

End of the Second Change