3GPP TSG-SA5 Meeting #162	S5-253432

Goteborg, Sweden, 25 - 29 August 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 28.540 | CR | 0045 | rev |  | Current version: | 19.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Rel-19 CR TS 28.540 Update requirements for reader location and AIoT service area configurations |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, China Unicom |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA5 |  |  |  |  |  |  |  |  |  |
| Work item code: | AdNRM_Ph3, TEI19 |  |  |  |  |  | Date: |  |  | 2025-08-14 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | RAN3 sent a LS (R3-253803) to SA5 for configuration management supporting Ambient IoT, which specifies “ - The AIOTF is aware of the supported “A-IoT Areas” of a gNB via OAM. - A new A-IoT Area is represented by an A-IoT Area ID - A-IoT Area ID = PLMN ID +NID (optional) + A-IoT Area Code (OCTET STRING (SIZE(3))) - The AIOTF is also aware of the served Reader list of a gNB via OAM. - Each Reader is uniquely identified globally by “Global gNB ID + Reader Index”. - The Reader Index is defined as INTEGER (1..65536, …). - The AIOTF may be aware of the location of reader via OAM configuration. - The details of the Reader Location are out the scope of RAN3. - OAM configures in the AIOTF the mapping relationships among gNBs, readers and A-IoT areas, as needed. ” |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Update requirements for A-IoT - Add description for Ambient IoT management |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Ambient IoT management support is not complete. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.X (new), 5.12.1, 5.12.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

1st Change

4.X	Management of Ambient IoT

As defined in TS 23.369[11], Ambient IoT is a service that can be provided by the 5GS system to support Ambient power-enabled IoT devices that are powered by energy harvesting, being either battery-less or with limited energy storage capability (e.g. using a capacitor) and the energy is provided through the harvesting of radio waves, light, motion, heat, or any other suitable power source. The 5GS System architecture for AIoT includes core network functions, different AIoT Reader architectures and AIoT Devices, where AIoT Reader can be supported by NG-RAN.

Management of Ambient IoT includes the configuration for core network functions and NG-RAN to support Ambient IoT.

Next Change

5.12	Requirements for management of Ambient IoT

5.12.1	Management of Ambient IoT for NG-RAN

REQ-AIOT_NRNRM-1: The 3GPP management system should support the capability to configure NG-RAN to support AIoT.

REQ-AIOT_NRNRM-x: The 3GPP management system should support the capability to configure gNB served A-IoT areas for management of Ambient IoT.

REQ-AIOT_NRNRM-y: The 3GPP management system should support the capability to configure reader location for management of Ambient IoT.

5.12.2	Management of Ambient IoT for 5GC

REQ-AIOT_5GCNRM-1: The 3GPP management system should support the capability to configure 5GC Network Functions to support AIoT.

REQ-AIOT_5GCNRM-x: The 3GPP management system should support the capability to configure gNB served A-IoT areas and reader location for gNB/reader selection.