3GPP TSG-SA5 Meeting #162	S5-253487

Goteborg, Sweden, 25 - 29 August 2025

Source:	NTT DOCOMO

Title:	Pseudo-CR on Rel-20 TR 28.881 Add issue description, requirement and solution for intent decomposition

Document for:	Approval

Agenda item:	6.20.1

Spec:	3GPP TR 28.881

Version:	0.0.0

Work Item:	FS_IDMS_MN_Ph4

Comments

This pCR provides use-case description, requirements and solutions for WT-7 Investigate the ability to trace the decomposition across intent handling functions.

Proposed Changes

* * * First Change * * * *

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[Z]	3GPP TS 28.312: "Management and orchestration; Intent driven management services for mobile networks".

* * * Next Change * * * *

# 4	Issue investigations and potential solutions for new areas

Editor's note: this clause will contain issues and potential solutions related to the new scenarios, requirements and generic capability for intent driven management.

## 4.X Issue#X: Decomposition of an intent of across
intent handling functions

### 4.X.1	Description

Intent decomposition refers to the process by which an intent-driven MnS producer translates an intent into one or more new intents, as described in 3GPP TS.28.312 [Z], clause 4.2.3, and submits them to another intent-driven MnS producer(s). In this case, the former intent-driven MnS producer acts as the consumer of the latter intent driven MnS producer(s), to which the newly generated intents are submitted.

When the intent decomposition is carried out by an intent driven MnS producer, in current specifications, the intent driven MnS consumer does not have any means to control and trace this decomposition into new intents which can end up in intent driven MnS producers of managed domains that are not desired by the original intent owner. This is also important for troubleshooting an intent driven MnS producer, to check whether it is acting as expected.

### 4.X.2	Potential requirements

REQ-Intent_DCMP-1: The intent driven MnS producer should have the capability to allow an MnS consumer to specify whether the decomposition of an intent is allowed or not.

REQ-Intent_DCMP-2: The intent driven MnS producer should have the capability to allow an MnS consumer to receive a report regarding the decomposition of an intent.

### 4.X.3	Potential solutions

#### 4.X.3.1	Potential solution #1

This solution proposes to reuse and enhance the existing generic intent and intent report information models defined in 3GPP TS 28.312 [Z].

The following attributes need to be defined and included in intent IOC in 3GPP TS 28.312 [Z].

- A new attribute intentId which is a unique identifier of the Intent.

- A new attribute isIntentDecompositionAllowed whether MnS consumer allows the intent driven MnS producer to decompose the intent into new intents and submit them.

- A new attribute allowedDecompositionIntentHandlingCapabilityList that describes the list of preferred intentHandlingCapabilityId as described in 3GPP TS 28.312 [Z], clause 6.2.1.4, where decomposed intents can be submitted to by the intent driven MnS producer, if the MnS consumer has such preference.

The following dataTypes and attributes need to be defined in IntentReport IOC in 3GPP TS 28.312 [Z].

- intentDecompositionReport which is a dataType including the list of decomposedIntentInfo which describes the intents that are decomposed from the intent that is reported.

- decomposedIntentInfo is proposed to be a new dataType that describes the decomposed intent information with following attributes:

- intentId describes the unique identifier of the intent that the reported intent is decomposed into.

- - intentHandlingCapabilityId as described in 3GPP TS 28.312[Z], clause 6.2.1.4.

* * * End of Changes * * * *