3GPP TSG-SA5 Meeting #162	S5-253498

Goteborg, Sweden, 25 - 29 August 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 28.622 | CR | 0586 | rev | - | Current version: | 19.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Rel-19 CR TS 28.622 Correction of Job Id |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA5 |  |  |  |  |  |  |  |  |  |
| Work item code: | MADCOL_Ph2 |  |  |  |  |  | Date: |  |  | 2025-08-14 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | Attribute “jobId” is differently used in several IOCs. This needs to be aligned. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Correct in IOCs “TraceJob” and “QMCJob” that attribute “jobId” attribute can be used to associate multiple instances and is NOT the identifier of the MOI which is given by the DN. - Correct in IOCs “Files” and “File” that attribute “jobId” provides an identifier of the set of associated jobs. - Correct in IOC “ManagementDataCollection” that attribute “jobId” is used to correlate the derived MOIs and the parent “ManagementDataCollection” MOI. - Correct the documentation of attribute “jobId” in the attribute properties table |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Misalignment between the usage of attribute “jobId”. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.3.30.1, 4.3.44.1, 4.3.45.1, 4.3.47.1, 4.3.54.1, 4.4.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

First change

### 4.3.30	TraceJob

#### 4.3.30.1	Definition

A TraceJob instance represents the Trace Control and Configuration parameters of a particular Trace Job (see TS 32.421 [29] and TS 32.422 [30] for details). It can be name-contained by SubNetwork, ManagedElement, ManagedFunction. In case of signalling based trace activation, it shall be name-contained by the UDMFunction, see TS 28.541 [48].

To activate Trace Jobs, a MnS consumer has to create TraceJob object instances on the MnS producer. A MnS consumer can activate a Trace Job for another MnS consumer since it is not required the value of traceCollectionEntityIPAddress or traceReportingConsumerUri to be his own.

For the details of Trace Job activation see clauses 4.1.1.1.2 and 4.1.2.1.2 of TS 32.422 [30].

When a MnS consumer wishes to deactivate a Trace Job, the MnS consumer shall delete the corresponding TraceJob instance. For details of management Trace Job deactivation see clauses 4.1.3.8 to 4.1.3.11 and 4.1.4.10 to 4.1.4.13  of TS 32.422 [30].

The attribute traceReference specifies a globally unique ID and identifies a Trace session. One Trace Session may be activated to multiple Network Elements. The traceReference is populated by the consumer that makes the request for a Trace Session, TS 32.422 [30].

The jobId attribute can be used to associate multiple TraceJob instances. For example, it is possible to configure the same jobId value for multiple TraceJob instances required to produce the data (e.g. RSRP values of M1 and RLF reports) for a specific network analysis.

The attribute traceReportingFormat defines the method for reporting the produced measurements. The selectable options are file-based or stream-based reporting. In case of file-based reporting the attribute traceCollectionEntityIPAddress is used to specify the IP address to which the trace records shall be transferred, while in case of stream-based reporting the attribute traceReportingConsumerUri specifies the streaming target.

The mandatory attribute traceTarget determines the target object of the TraceJob. Dependent on the network element to which the Trace Session is activated different types of the target object are possible.

The attribute pLMNTarget defines the PLMN for which sessions shall be selected in the Trace Session in case of management based activation when several PLMNs are supported in the RAN.  The MDT configuration may include area scope defined by network slice, in which case the attribute pLMNTarget is not applicable.

The attribute listOfTraceMetrics allows configuration of which metrics shall be recorded.

The attribute jobType specifies the kind of data to collect. In case of TRACE_ONLY, the configuration parameters of attribute traceConfig shall be applied. In case of IMMEDIATE_MDT_ONLY, LOGGED_MDT_ONLY, RLF_REPORT_ONLY, RCEF_REPORT_ONLY and LOGGED_MBSFN_MDT the configuration parameters of attribute mdtConfig or a subset of these shall be applied. In case of 5GC_UE_LEVEL_MEASUREMENTS_ONLY, the configuration parameters of attribute ueCoreMeasConfig shall be applied. In case of any combination of Trace, Immediate MDT, and 5GC UE level measurements, the configuration parameters of the corresponding attributes, traceConfig, mdtConfig and ueCoreMeasConfig are applicable.

If jobType has the value RRC Report, the attribute rrcReportType shall be present. The rrcReportType allows the tracing of RRC reports.

Creation and deletion of TraceJob instances by MnS consumers is optional; when not supported, the TraceJob instances may be created and deleted by the system or be pre-installed.

Next change

### 4.3.44	Files

#### 4.3.44.1	Definition

This IOC represents a collection of files. It can be name-contained by SubNetwork, ManagedElement, PerfMetricJob or TraceJob. The Files object name-contains File objects, that represent the files of the collection. File collections allow to structure related files under a common root.

Instances of Files are created by MnS producers. They shall be created at latest when the first file of the collection becomes available for retrieval by MnS consumers.

The attributes of Files represent properties of the file collection and not properties of individual files.

When the file retrieval NRM fragment is used together with a data collection job (PerfMetricJob or TraceJob) the following provisions shall apply:

-	The Files object shall be created at the same time as the object representing the data collection job.

-	The attributes jobRef and jobId shall be supported and present in a Files instance. The attribute jobRef identifies the job that the files in the file collection relate to. The attribute jobId provides an identifier of the set of associated jobs.

-	A Files instance shall contain files related to one and only one job.

-	The files produced by one job shall be contained in one and only one Files instance.

-	The job object shall support an attribute with a link to the created Files instance ("_linkToFiles").

-	The attribute "_linkToFiles" shall be returned in the job creation response, if the stage 3 protocol supports returning attributes in an object creation response.

-	The MnS producer decides where to name-contain the Files instance related to a job.

The attribute _linkToFiles allows a MnS consumer to create simple and targeted subscriptions for notifyFileReady and notifyFilePreparationError, or notifyMOICreation, notifyMOIChanges, and notifyMOIDeletion related to File instances created or deleted under the Files instance of a specific job. The subscription needs to scope simply objects one level below the Files object.

In addition, the attribute _linkToFiles allows for simple deployments not relying on notifications for reporting the availability of new files, where the MnS consumer polls regularly for new files under Files.

#### 4.3.44.2	Attributes

The Files IOC includes the attributes inherited from Top IOC (defined in clause 4.3.29) and the following attributes:

| Attribute name | S | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| numberOfFiles | M | T | F | F | F |
| Attributes related to roles |  |  |  |  |  |
| jobRef | CM | T | F | T | F |
| jobId | CM | T | F | T | F |

#### 4.3.44.3	Attribute constraints

| Name | Definition |
| --- | --- |
| jobRef | Condition: This attribute shall be supported when "PerfMetricJob" or "TraceJob" are supported. |
| jobId | Condition: This attribute shall be supported when "PerfMetricJob" or "TraceJob" are supported. |

#### 4.3.44.4	Notifications

The common notifications defined in clause 4.5 are valid for this IOC, without exceptions or additions.

### 4.3.45	File

#### 4.3.45.1	Definition

This IOC represents a file. It is name-contained by Files.

When a file becomes available on a MnS producer for retrieval by a MnS consumer, the MnS producer shall create a File instance representing that file.

The time of creation shall be captured by the MnS producer in the fileReadyTime attribute. The MnS producer shall keep the file at least until the time specified by fileExpirationTime. After that time the MnS producer may delete the File instance. The fileExpirationTime is determined by the MnS producer based on considerations such as available storage space or file retention policies.

The attributes fileSize, fileCompression, fileDataType and fileFormat describe the file properties.

The fileLocation attribute indicates the address where the file can be retrieved. The address includes the file transfer protocol (schema). Allowed file transfer protocols are "sftp", "ftpes" and "https".

The value of fileLocation can be identical to or different from the address of the File instance. The attribute fileContent is provided for retrieving the actual file content. When identifying in the Read request a File instance and specifying only the fileContent attribute be returned, then only the file content shall be returned in the response. Note, as usual, multiple attributes can be specified to be returned, so that the file content together with some or all file meta data attributes can be returned in response to a single request.

In case the fileLocation specifies a location different than the File object location, then the attribute fileContent cannot be used for retrieving the file content. For example, the File object location may be given by

"https://companyA.com/ManagedElement=1/Files=1/File=1

and the value of the fileLocation attribute by

"sftp://companyA.com/datastore/fileName.xml"

In this case the file needs to be retrieved using "sftp" from "sftp://companyA.com/datastore/fileName.xml". Attempts to read the fileContent attribute shall return an error.

When the file retrieval NRM fragment is used together with a data collection job (PerfMetricJob or TraceJob) the following provisions shall apply:

-	The attributes jobRef and jobId shall be supported and present. The attribute jobRef identifies the job that the file is related to. The attribute jobId provides an identifier of the set of associated jobs.

The attributes jobRef and jobId allow to set notification filters in the subscription in such a way that only notifyMOICreation, notifyMOIDeletion and notifyMOIChanges notifications are sent to subscribed MnS consumers if the created or deleted File instance represents data related to jobs the subscribed MnS consumer created or is interested in.

Upon creation of a File instance, a notification of type notifyMOICreation or notifyMOIChanges shall be emitted to subscribed MnS consumers as usual. For the case that the file contains performance metric data (fileDataType is "PERFORMANCE") the MnS producer shall emit either a notification of type notifyMOICreation or notifyMOIChanges or of type notifyFileReady. The MnS consumer selects the notification type he wishes to receive with the subscription created on the MnS producer.

The objectClass and objectInstance parameters in the notification header of notifyFileReady shall identify the new File instance, instead of the related PerfMetricJob, TraceJob, ManagedElement or ManagementNode as described in TS 28.532 [27], clause 11.6.1.1.1 for the case that notifyFileReady is used as part of the file data reporting MnS.

The notification notifyFilePreparationError shall be supported as well by the File object. It shall be sent when an error occurs during the preparation of the file. No notifyFileReady or notifyMOICreation or notifyMOIChanges shall be sent in that case. The objectClass and objectInstance parameters of the notification header shall identify the new File instance representing the corrupted file, instead of the related PerfMetricJob, TraceJob, ManagedElement or ManagementNode as described in 3GPP TS 28.532 [27], clause 11.6.1.1.1 for the case that notifyFilePreparationError is used as part of the file data reporting MnS. When the file is not created at all or deleted, the objectClass and objectInstance parameters of the notification header are populated as described in 3GPP TS 28.532 [27], clause 11.6.1.1.1. Note that to receive notifyFilePreparationError in that case the notification subscription needs to include these objects in its scope.

Next change

### 4.3.47	ManagementDataCollection

#### 4.3.47.1	Definition

This IOC represents a management data collection request job. The requested data is of kind Trace, MDT (Minimization of Drive Test), RLF (Radio Link Failure) report, RCEF (RRC Connection Establishment Failure) report, RRC report, PM (performance measurements), KPI (end-to-end key performance indicators) or a combination of these.

The attribute managementData defines the management data which shall be reported. This may either include a list of data categories or a list of management data identified with their name. For further details see clause 4.3.50.

The targetNodeFilter attribute can be used to target object instance(s) producing the required management data. It is assumed that the consumer may not have detailed knowledge of the network and hence may not identify the exact object instance producing the required management data. In this case consumer can request management data, specified by 3GPP, produced by certain object instance (s) based on a particular location, the domain (CN or RAN) of the object instances, and the handled traffic (CP or UP) of the object instances.

To activate the production of the requested data, a MnS consumer has to create a ManagementDataCollection object instance on the MnS producer.

The production and reporting of the management data can be constrained by conditions such that only when the conditions are satisfied shall management data collection be enabled. For example, a MnS consumer can request to create two ManagementDataCollection instances. One can be configured with high data producing and reporting period on a set of conditions (e.g, to reduce transmission cost when network performance metric is in normal range). Another can be configured with low data producing and reporting period on another set of conditions (e.g. to enable network optimization when network performance metric is in abnormal range).

Editor’s Note: It is currently not possible to construct conditions based on performance metrics. This needs to be enabled before the text in the paragraph above can be approved and published. Furthermore, it needs to be investigated if the ConditionMonitor shall be used or if the conditions should be added to this IOC directly using an attribute.

The MnS producer may derive multiple jobs (PerfMetricJob, TraceJob) from a single ManagementDataCollection job for collecting the required management data. The attribute jobId is used to correlate the derived jobs which will have the same jobId value as the parent ManagementDataCollection job.

If the MnS producer receives the collected data from multiple sources, it will consolidate the data into a set of management data for reporting based on the value of the attribute consolidateOutput.

For consolidation of file-based management data the attribute consolidateOutput controls:

- 	True: the MnS Producer will combine the file output from jobs used to collect the required management data into a single output file as follows:

- 	File is in compressed format, i.e. zip.

-	File shall contain individual output files from each configured job

- 	Each file retains its original filename

-	Each file retains it original content

-	Consolidated filename uses naming convention defined in [27], clause 11.3.2.1.4 with the following:

-	<Type> is a combination of the management data types included in the consolidated output

-	<UniqueIdList> is omitted

-	False: the MnS Producer will not combine the output from jobs used to collect the required management data.  The MnS Consumer will receive separate output from the derived jobs.

Subject to the reporting method, the MnS Consumer may receive file related notifications.  When consolidated output is selected the MnS Producer will create file notifications for the consolidated files.  When consolidated output is not selected, the MnS Producer will create the notification subscriptions on behalf of the MnS Consumer and the MnS Consumer will receive notifications directly from the derived jobs.

The attribute collectionTimeWindow specifies the time window for which the management data should be reported. The attributes startTime and the endTime can be in the past, present or in the future.

The attribute reportingCtrl specifies the method and associated control parameters for reporting the produced management data to MnS consumers. Three methods are available: file-based reporting with selection of the file location by the MnS producer, file-based reporting with selection of the file location by the MnS consumer and stream-based reporting.

The attribute dataScope configures, whether the management data should be reported per S-NSSAI or per 5QI or per PLMN, if applicable.

The attribute processMonitor allows the MnS consumer to monitor the status of the management data collection represented by the object ManagementDataCollection.

The MnS producer indicates in the attribute progressStateInfo the state of the management data collection:

-	NOT_STARTED

- 	RUNNING

- 	CANCELING

and indicates in the attribute resultStateInfo:

-	FINISHED

-	FAILED

-	PARTIALLY_FAILED

-	CANCELLED

Next change

### 4.3.54	QMCJob

#### 4.3.54.1	Definition

The QoE Measurement Collection provides capability for collecting QoE information from:

- UEs which are in the specified area in case of Management Based Activation or

- an individual UE in case of Signalling Based Activation.

The QoE Measurement Collection enables collection of application layer measurements from the UE for specified end user service type. The supported service types are:

-	Streaming services, see TS 26.247 [51].

-	MTSI services, see TS 26.114 [52].

-	VR services, see TS 26.118 [53].

A QMCJob instance represents the job for collecting QoE measurements according to the job parameters. For details of the QoE measurement collection configuration parameters see clause 5 of TS 28.405 [50]. A QMCJob instance can be name-contained by SubNetwork or ManagedElement.

A QMC Job is activated by creating a QMCJob object instance in the MnS producer. For details of Management Based Activation of QoE Measurement Collection see clause 4.5 and for details of Signalling Based Activation of QoE Measurement Collection see clause 4.6 of TS 28.405 [50]. The attributes pLMNTarget is only relevant when Management Based Activation is used and the attribute qoETarget is only relevant when Signalling Based Activation is used. All other attributes are common for both Management Based Activation and Signalling Based Activation.

The areaScope attribute defines the area scope of QoE, which is specified in clause 5.4 of TS 28.405 [50].

When a MnS consumer wishes to deactivate a QMCJob, the MnS consumer shall delete the corresponding QMCJob instance.

NOTE: 	If the reporting is ongoing, when a request to delete a QMCJob instance is received, the reporting does not end. The QMCJob instance is deleted, when the last reporting for the QMC Job expires.

The jobId attribute can be used to associate multiple QMCJob instances. For example, it is possible to configure the same jobId value for multiple QMCJob instances required to produce the data (e.g. Streaming services and MTSI reports) for a specific network analysis.

Next change

## 4.4	Attribute definitions

### 4.4.1	Attribute properties

The following table defines the properties of attributes specified in the present document.

| Attribute Name | Documentation and Allowed Values | Properties |
| --- | --- | --- |
| numberOfFiles | Number of files in a file collection. allowedValues: NA | Type: Integer multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| fileLocation | Location of the file incl. the file transfer protocol, and the file name for the case the file content cannot be retrieved by reading the fileContent attribute. The allowed file transfer protocols are: - sftp - ftpes - https Examples: "sftp://companyA.com/datastore/fileName.xml", "https://companyA.com/ManagedElement=1/Files=1/File=1” allowedValues: NA | Type: Uri multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| fileCompression | Name of the algorithm used for compressing the file. An empty or absent fileCompression parameter indicates the file is not compressed. The MnS producer selects the compression algorithm. It is encouraged to use popular algorithms such as GZIP. allowedValues: N/A | Type: String multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| fileSize | Size of the file. Unit is byte. allowedValues: non-negative integers | Type: Integer multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| fileDataType | Type of the management data stored in the file. AllowedValues: - "PERFORMANCE" - "TRACE" - "ANALYTICS" - "PROPRIETARY" The value "PERFORMANCE" refers to measurements and KPIs. | Type: ENUM multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| fileFormat | Identifier of the XML or ASN.1 schema (incl. its version) used to produce the file content. allowedValues: N/A | Type: String multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| fileReadyTime | Date and time, when the file was closed (the last time) and made available on the MnS producer. The file content will not be changed anymore. allowedValues: N/A | Type: DateTime multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| fileExpirationTime | Date and time after which the file may be deleted. allowedValues: N/A | Type: DateTime multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| fileContent | File content. allowedValues: N/A | Type: String multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| jobMonitor | Provides monitoring for the file download job. The data type of this attribute is the ProcessMonitor as defined in clause 4.3.43 with the specialisations defined in clause 4.3.46.1. allowedValues: N/A | Type: ProcessMonitor multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| cancelJob | Setting this attribute to "TRUE" cancels the file download job. As specified in the definition of ProcessMonitor, cancellation is possible in the "NOT_STARTED" and "RUNNING" state. Setting the attribute to "FALSE" has no observable result. allowedValues: TRUE, FALSE | Type: Boolean multiplicity: 0..1 isOrdered: N/A isUnique: N/A defaultValue: FALSE isNullable: False |
| FileDownloadJob.jobMonitor.resultStateInfo | Provides the following specialisation for the resultStateInfo attribute of the ProcessMonitor data type for the FileDownloadJob. In the event the file download fails, and the status is equal to "FAILED", it provides the reason for the failure. allowedValues for status = "FAILED": - NULL - UNKNOWN - NO_STORAGE - LOW_MEMORY - NO_CONNECTION_TO_REMOTE_SERVER - FILE_NOT_AVAILABLE - DNS_CANNOT_BE_RESOLVED - TIMER_EXPIRED - OTHER The allowed values for "FINISHED" or "CANCELLED" are vendor specific. | Type: String multiplicity: 0..1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| heartbeatNtfPeriod | Periodicity of the heartbeat notification emission. The value of zero has the special meaning of stopping the heartbeat notification emission. Unit is in seconds. AllowedValues: non-negative integers | type: Integer multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: 0 isNullable: False |
| triggerHeartbeatNtf | Setting this attribute to TRUE triggers an immediate additional heartbeat notification emission. Setting the value to FALSE has no observable result. The periodicity of notifyHeartbeat emission is not changed. AllowedValues: TRUE, FALSE | type: Boolean multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: FALSE isNullable: False |
| notificationRecipientAddress | Address of the notification recipient. allowedValues: N/A | type: String multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| notificationTypes | List of notification types. Below is a list of notificationType values that are defined in 3GPP specifications. Other notificationTypes defined by SDOs or enterprises may also be supported. allowedValues: - notifyMOICreation - notifyMOIDeletion - notifyMOIAttributeValueChanges - notifyMOIChanges - notifyEvent - notifyNewAlarm - notifyChangedAlarm - notifyAckStateChanged - notifyComments - notifyCorrelatedNotificationChanged - notifyChangedAlarmGeneral - notifyClearedAlarm - notifyAlarmListRebuilt - notifyPotentialFaultyAlarmList - notifyFileReady - notifyFilePreparationError - notifyThresholdCrossing "notifyPotentialFaultyDataNodeTree" "notifyDataNodeTreeSyncRecommended" | type: ENUM multiplicity: * isOrdered: False isUnique: True defaultValue: None isNullable: False |
| notificationFilter | Filter to be applied to candidate notifications identified by the notificationTypes attribute. Only notifications that pass the filter criteria are forwarded to the notification recipient. All other notifications are discarded. The filter can be applied to any field of a notification. allowedValues: N/A | type: String multiplicity: 0..1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| notificationProtocols | List of protocols supported for notifications. TS 28.532 [27] defines options Restful HTTP and Restful HTTP aligned with VES Other values defined by SDOs or enterprises may also be supported. allowedValues: - HTTP - HTTP_VES_ENCAPS | type: ENUM multiplicity: 1..* isOrdered: False isUnique: True defaultValue: None isNullable: False |
| scope | Scopes (selects) data nodes in an object tree. allowedValues: N/A | type: Scope multiplicity: 0..1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| scopeType | If the optional scopeLevel attribute is not supported or absent, allowed values of scopeType are BASE_ONLY and BASE_ALL. The value BASE_ONLY indicates only the base object is selected. The value BASE_ALL indicates the base object and all of its subordinate objects (incl. the leaf objects) are selected. If the scopeLevel attribute is supported and present, allowed values of scopeType are BASE_NTH_LEVEL and BASE_SUBTREE. The value BASE_NTH_LEVEL indicates all objects on the level, which is specified by the scopeLevel attribute, below the base object are selected. The base object is at scopeLevel zero. The value BASE_SUBTREE indicates the base object and all subordinate objects down to and including the objects on the level, which is specified by the scopeLevel attribute, are selected. The base object is at scopeLevel zero. allowedValues: N/A | type: ENUM multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| scopeLevel | See definition of scopeType attribute. allowedValues: N/A | type: Integer multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| dataNodeSelector |  | The dataNodeSelector attribute allows to select one or more managed object instances, attributes, attribute fields or attribute elements. Its value contains a solution set specific expression for selecting the nodes. allowedValues: N/A | type: String multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| availabilityStatus |  | The availability status provides additional information about the operational state allowedValues: - DEGRADED - DEPENDENCY | Type: AvailabilityStatus multiplicity: * isOrdered: False isUnique: True defaultValue: None isNullable: False |
| lastSequenceNo |  | The sequence number of the last notification that was sent by a "NtfSubscriptionControl" instance. allowedValues: non-negative integers | Type: Integer multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| farEndEntity | The value of this attribute shall be the Distinguished Name of the far end network entity to which the reference point is related. allowedValues: N/A | type: DN multiplicity: 0..1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| linkType | This attribute defines the type of the Link. allowedValues: Signalling, Bearer, OAM&P, Other or multiple combinations of this type. | type: String multiplicity: 0..* isOrdered: False isUnique: True defaultValue: None isNullable: False |
| locationName | The physical location of this entity (e.g. an address). allowedValues: N/A | type: String multiplicity: 0..1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| monitorGranularityPeriod | Granularity period used to monitor performance metrics for threshold crossings. The period is defined in seconds. See Note 5 allowedValues: a multiple of a supported GP of the associated performance metrics | type: Integer multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| reportingPeriods | Reporting periods supported for the associated performance metrics. The period is defined in seconds. allowedValues: Integer with a minimum value of 1 | type: Integer multiplicity: * isOrdered: False isUnique: True defaultValue: None isNullable: False |
| thresholdInfoList | List of threshold infos. | type: ThresholdInfo multiplicity: 1..* isOrdered: False isUnique: True defaultValue: None isNullable: False |
| thresholdValue | Value against which the monitored performance metric is compared at a threshold level in case the hysteresis is zero. allowedValues: float or integer | type: Float or Integer multiplicity: 1 isOrdered: NA isUnique: NA defaultValue: None isNullable: False |
| hysteresis | Hysteresis of a threshold. If this attribute is present the monitored performance metric is not compared against the threshold value as specified by the thresholdValue attribute but against a high and low threshold value given by highThresholdValue- = thresholdValue + hysteresis lowThresholdValue = thresholdValue - hysteresis When going up, the threshold is triggered when the performance metric reaches or crosses the high threshold value. When going down, the threshold is triggered when the performance metric reaches or crosses the low threshold value. A hysteresis may be present only when the monitored performance metric is not of type counter that can go up only. If present for a performance metric of type counter, it shall be ignored. allowedValues: non-negative float or integer | type: Float or Integer multiplicity: 0..1 isOrdered: NA isUnique: NA defaultValue: None isNullable: False |
| thresholdDirection | Direction of a threshold indicating the direction for which a threshold crossing triggers a threshold. When the threshold direction is configured to "UP", the associated treshold is triggered only when the performance metric value is going up upon reaching or crossing the threshold value. The treshold is not triggered, when the performance metric is going down upon reaching or crossing the threshold value. Vice versa, when the threshold direction is configured to "DOWN", the associated treshold is triggered only when the performance metric is going down upon reaching or crossing the threshold value. The treshold is not triggered, when the performance metric is going up upon reaching or crossing the threshold value. When the threshold direction is set to "UP_AND_DOWN" the treshold is active in both direcions. In case a threshold with hysteresis is configured, the threshold direction attribute shall be set to "UP_AND_DOWN". allowedValues: - UP - DOWN - UP_AND_DOWN | type: ENUM multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| objectClass | Class of a managed object instance. allowedValues: N/A | type: String multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| objectInstance | Managed object instance identified by its DN. allowedValues: N/A | type: String multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| objectInstances | List of managed object instances. Each object instance is identified by its DN. allowedValues: N/A | type: DN multiplicity: * isOrdered: False isUnique: True defaultValue: None isNullable: False |
| peeParametersList | This attribute contains the parameter list for the control and monitoring of power, energy and environmental parameters of ManagedFunction instance(s). | type: PeeParameters multiplicity: 0..* isOrdered: False isUnique: True defaultValue: None isNullable: False |
| PeeParameter.siteIdentification | The identification of the site where the ManagedFunction resides. allowedValues: N/A | type: String multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| PeeParameter.siteLatitude | The latitude of the site where the ManagedFunction instance resides, based on World Geodetic System (1984 version) global reference frame (WGS 84). Positive values correspond to the northern hemisphere. This attribute is optional for BTSFunction, RNCFunction , GNBDUFunction and NRSectorCarrier instance(s). allowedValues: -90.0000 to +90.0000 | type: Float multiplicity: 0..1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| PeeParameter.siteLongitude | The longitude of the site where the managedFunction instance resides, based on World Geodetic System (1984 version) global reference frame (WGS 84). Positive values correspond to degrees east of 0 degrees longitude. This attribute is optional for BTSFunction, RNCFunction, GNBDUFunction and NRSectorCarrier instance(s). allowedValues: -180.0000 to +180.0000 | type: Float multiplicity: 0..1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| PeeParameter.siteAltitude | The altitude of the site where the ManagedFunction instance resides, in unit of meter. This attribute is optional for BTSFunction, RNCFunction, GNBDUFunction and NRSectorCarrier instance(s). | type: Float multiplicity: 0..1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| PeeParameter.siteDescription | An operator defined description of the site where the ManagedFunction instance resides. allowedValues: N/A | type: String multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| PeeParameter.equipmentType | equipmentType: The type of equipment where the ManagedFunction instance resides. allowedValues: see clause 4.4.1 of ETSI ES 202 336-12 [18]. | type: String multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| PeeParameter.environmentType | environmentType: The type of environment where the ManagedFunction instance resides. allowedValues: see clause 4.4.1 of ETSI ES 202 336-12 [18]. | type: String multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| PeeParameter.powerInterface | powerInterface: The type of power. allowedValues: see clause 4.4.1 of ETSI ES 202 336-12 [18]. | type: String multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| priorityLabel | This is a label that consumer would assign a value on a concrete instance of the managed object. The management system takes the value of this attribute into account. The effect of this attribute value to the subject managed entity is not standardized | type: Integer multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| protocolVersion | Versions(s) and additional descriptive information for the protocol(s) used for the associated communication link. Syntax and semantic is not specified. allowedValues: N/A | type: String multiplicity: * isOrdered: False isUnique: True defaultValue: None isNullable: False |
| setOfMcc | Set of Mobile Country Code (MCC). The MCC uniquely identifies the country of domicile of the mobile subscriber. MCC is part of the IMSI (TS 23.003 [5]) This list contains all the MCC values in subordinate object instances to this SubNetwork instance. allowedValues: See clause 2.3 of TS 23.003 [5] for MCC allocation principles. | type: Integer multiplicity: 1..* isOrdered: False isUnique: True defaultValue: None isNullable: False |
| swVersion | The software version of the ManagementNode or ManagedElement (this is used for determining which version of the vendor specific information is valid for the ManagementNode or ManagedElement). allowedValues: N/A | type: String multiplicity: 0..1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| systemDN | Distinguished Name (DN) of a MnSAgent. allowedValues: N/A | type: DN multiplicity: 0..1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| userDefinedState | An operator defined state for operator specific usage. allowedValues: N/A | type: String multiplicity: 0..1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| userLabel | A user-friendly (and user assignable) name of this object. allowedValues: N/A | type: String multiplicity: 0..1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| vendorName | The name of the vendor. allowedValues: N/A | type: String multiplicity: 0..1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| vnfParametersList | This attribute contains the parameter set of the VNF instance(s) corresponding to an NE. The presence of this attribute indicates that the ManagedFunction represented by the MOI is a virtualized function. See Note 3. allowedValues: N/A | type: VnfParameters multiplicity: * isOrdered: False isUnique: True defaultValue: None isNullable: False |
| VnfParameter.vnfInstanceId | vnfInstanceId: VNF instance identifier (vnfInstanceId, see section 9.4.2 of ETSI GS NFV-IFA 008 [16]). A string length of zero for vnfInstanceId means the VNF instance(s) corresponding to the MOI does not exist (e.g. has not been instantiated yet, has already been terminated). See Note 1. allowedValues: N/A | type: string multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| VnfParameter.vnfdId | vnfdId: Identifier of the VNFD on which the VNF instance is based, see section 9.4.2 of [16]. This attribute is optional. Note: the value of this attribute is identical to that of the same attribute in clause 9.4.2 of ETSI GS NFV-IFA 008 [16]. | type: String multiplicity: 0..1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| VnfParameter.flavourId | flavourId: Identifier of the VNF Deployment Flavour applied to this VNF instance, see section 9.4.3 of ETSI GS NFV-IFA 008 [16]. This attribute is optional. Note: the value of this attribute is identical to that of the same attribute in clause 9.4.3 of ETSI GS NFV-IFA 008 [16]. | type: String multiplicity: 0..1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| VnfParameter. autoScalable | autoScalable: Indicator of whether the auto-scaling of this VNF instance is enabled or disabled. The type is Boolean. This attribute is optional. See Note2. | type: Boolean multiplicity: 0..1 isOrdered: N/A isUnique: N/A defaultValue: FALSE isNullable: False |
| vsData | Vendor specific attributes of the type vsDataType. The attribute definitions including constraints (value ranges, data types, etc.) are specified in a vendor specific data format file. allowedValues: -- | type: -- multiplicity: -- isOrdered: -- isUnique: -- defaultValue: -- isNullable: False |
| vsDataFormatVersion | Name of the data format file, including version. allowedValues: N/A | type: String multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| vsDataType | Type of vendor specific data contained by this instance, e.g. relation specific algorithm parameters, cell specific parameters for power control or re-selection or a timer. The type itself is also vendor specific. allowedValues: N/A | type: String multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| supportedPerfMetricGroups | A set of performance metric groups. When this attribute is contained in a managed object it may define performance metrics for this object and all descendant objects. allowedValues: N/A | type: SupportedPerfMetricGroup multiplicity: * isOrdered: False isUnique: True defaultValue: None isNullable: False |
| performanceMetrics | List of performance metrics identified by name allowedValues: Performance metrics include measurements defined in TS 28.552 [20] and KPIs defined in TS 28.554 [28]. For measurements defined in TS 28.552 [20] the name is constructed as bullet e) of the measurement definition with allowed measurement type. For KPIs defined in TS 28.554 [28] the name is defined in the KPI definitions template, see chapter 5 in TS 28.554 [28], as the component designated with a). For non-3GPP specified measurements the name is defined elsewhere. | type: String multiplicity: 1..* isOrdered: False isUnique: True defaultValue: None isNullable: False |
| supportedTraceMetrics | List of trace metrics. When this attribute is contained in a managed object it defines the trace metrics supported for this object and all descendant objects. Trace metrics include trace messages, MDT measurements (Immediate MDT, Logged MDT, Logged MBSFN MDT), RLF, RCEF and RRC reports, see TS 32.422 [30]. Trace metrics are identified with their metric identifier. The metric identifier is constructed as defined in clause 10 of TS 32.422 [30]. For non-3GPP specified trace metrics the name is defined elsewhere. allowedValues: N/A | type: String multiplicity: * isOrdered: False isUnique: True defaultValue: None isNullable: False |
| listOfTraceMetrics | List of trace metrics identified by name. Includes trace messages, MDT measurements (Immediate MDT, Logged MDT, Logged MBSFN MDT), RLF, RCEF and RRC reports, see TS 32.422 [30]. Trace messages are identified with their message identifier. Trace metric identifier is constructed as defined in clause 10 of TS 32.422 [30]. For non-3GPP specified trace metrics the name is defined elsewhere. allowedValues: N/A | type: String multiplicity: * isOrdered: False isUnique: True defaultValue: None isNullable: False |
| rootObjectInstances | List of object instances. Each object instance is identified by its DN and designates the root of a subtree that contains the root object and all descendant objects. | type: DN multiplicity: * isOrdered: False isUnique: True defaultValue: None isNullable: False |
| reportingMethods | List of reporting methods for performance metrics allowedValues: - "FILE_BASED_LOC_SET_BY_PRODUCER", - "FILE_BASED_LOC_SET_BY_CONSUMER", - "STREAM_BASED" | type: ENUM multiplicity: * isOrdered: False isUnique: True defaultValue: None isNullable: False |
| jobRef | Object instance of the PerfMetricJob or TraceJob that produced the file. allowedValues: NA | Type: Dn multiplicity: 0..* isOrdered: False isUnique: True defaultValue: None isNullable: False |
| jobId | Identifier to associate multiple instances of a PerfMetricJob, a TraceJob or a QMCJob or to associate a ManagementDataCollection instance with the derived PerfMetricJob or TraceJob instances. | type: String multiplicity: 0..1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| granularityPeriod | Granularity period used to produce performance metrics. The period is defined in seconds. See Note 4. allowedValues: Integer with a minimum value of 1 | type: Integer multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| granularityPeriods | Granularity periods supported for the production of associated performance metrics. The period is defined in seconds. allowedValues: Integer with a minimum value of 1 | type: Integer multiplicity: * isOrdered: False isUnique: True defaultValue: None isNullable: False |
| reportingCtrl | Selecting the reporting method and defining associated control parameters. | type: ReportingCtrl multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| fileReportingPeriod | For the file-based reporting method this is the time window during which collected measurements are stored into the same file before the file is closed and a new file is opened. The period is defined in minutes. allowedValues: Multiples of granularityPeriod | type: Integer multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| _linkToFiles | Link to a Files object. allowedValues: N/A | type: String multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| streamTarget | The stream target for the stream-based reporting method. allowedValues: N/A | type: String multiplicity: 0..1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| administrativeState | Administrative state of a managed object instance. The administrative state describes the permission to use or prohibition against using the object instance. The adminstrative state is set by the MnS consumer. allowedValues: LOCKED, UNLOCKED. | type: ENUM multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: LOCKED isNullable: False |
| operationalState | Operational state of manged object instance. The operational state describes if an object instance is operable ("ENABLED") or inoperable ("DISABLED"). This state is set by the object instance or the MnS producer and is hence READ-ONLY. allowedValues: ENABLED, DISABLED. | type: ENUM multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: DISABLED isNullable: False |
| jobType | It specifies whether the TraceJob represents only MDT, Trace, RLF, RCEF, RRC or 5GC UE level measurements job, or a combined job. It also defines the MDT mode. See the clause 5.9a of TS 32.422 [30] for additional details on the allowed values. | type: ENUM multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: TRACE_ONLY isNullable: False |
| rrcReportType | Specifies the RRC reports requested, see 3GPP TS 38.331 [38]. allowed values: RLF_REPORT, RCEF_REPORT, SHR, SPR, MHI, or RA_REPORT. | type: ENUM multiplicity: 0..* isOrdered: False isUnique: True defaultValue: None isNullable: False |
| traceConfig | The set of parameters specific for trace configuration. | type: TraceConfig multiplicity: 0..1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| mdtConfig | The set of parameters specific for MDT configuration. | type: MdtConfig multiplicity: 0..1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| immediateMdtConfig | The set of parameters specific for Immediate MDT configuration. | type: ImmediateMdtConfig multiplicity: 0..1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| loggedMdtConfig | The set of parameters specific for Logged MDT and Logged MBSFN MDT configuration. | type: LoggedMdtConfig multiplicity: 0..1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| listOfInterfaces | It specifies the interfaces that need to be traced. The attribute is applicable only for Trace. See the clause 5.5 of TS 32.422 [30] for additional details on the allowed values. | type: ENUM multiplicity: * isOrdered: False isUnique: True defaultValue: None isNullable: False |
| listOfNeTypes | It specifies the network element types where the trace should be activated. The attribute is applicable only for Trace with Signalling Based Trace activation. See the clause 5.4 of TS 32.422 [30] for additional details on the allowed values. | type: ENUM multiplicity: * isOrdered: False isUnique: True defaultValue: None isNullable: False |
| pLMNTarget | It specifies which PLMN that the subscriber of the session to be recorded uses as selected PLMN. | type: PlmnId multiplicity: 0..1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| traceReportingConsumerUri | It specifies the Uniform Resource Identifier (URI) of the Streaming Trace data reporting MnS consumer (a.k.a. streaming target). See the clause 5.9 c of TS 32.422 [30] for additional details on the allowed values. | type: String multiplicity: 0..1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| traceCollectionEntityIPAddress | It specifies the address of the Trace Collection Entity when the attribute traceReportingFormat is configured for the file-based reporting. The attribute is applicable for both Trace and MDT. See the clause 5.9 of TS 32.422 [30] for additional details on the allowed values. | type: IpAddress multiplicity: 0..1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| traceDepth | It specifies the trace depth. The attribute is applicable only for Trace. See the clause 5.3 of 3GPP TS 32.422 [30] for additional details on the allowed values. | type: ENUM multiplicity: 0..1 isOrdered: N/A isUnique: N/A defaultValue: MAXIMUM isNullable: False |
| traceReference | A globally unique identifier, which uniquely identifies the Trace Session that is created by the TraceJob. In case of shared network, it is the MCC and MNC of the Participating Operator that request the trace session that shall be provided. The attribute is applicable for both Trace and MDT. See the clause 5.6 of 3GPP TS 32.422 [30] for additional details on the allowed values. | type: TraceReference multiplicity: 0..1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| traceReportingFormat | It specifies the trace reporting format - streaming trace reporting or file-based trace reporting. AllowedValues: FILE-BASED, STREAMING | type: ENUM multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: FILE-BASED isNullable: False |
| traceTarget | It specifies the target object of the Trace and MDT. The attribute is applicable for both Trace and MDT. This attribute consists the traceTargetType and traceTargetValueList In case of management based Immediate MDT, RLF reporting, RCEF reporting or RRC reporting, the traceTarget attribute shall be null value. | type: TraceTarget multiplicity: 0..1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| traceTargetType | It specifies the target object type of the Trace, MDT and 5GC UE level measurements collection. The attribute is applicable for Trace, MDT, and 5GC UE level measurements collection. The traceTargetType shall be "PUBLIC_ID" in case of a Management Based Activation is done to an SCSCFFunction (Serving Call Session Control Function) or PCSCFFunction (Proxy Call Session Control Function) (TS 28.705[44]). The traceTargetType shall be "UTRAN_CELL" only in case of the UTRAN cell traffic trace function. The traceTargetType shall be "E-UTRAN_CELL" only in case of E-UTRAN cell traffic trace function. The traceTargetType shall be "NG-RAN_CELL" only in case of NR cell traffic trace function. The traceTargetType shall be either "IMSI", "IMEI" or "IMEISV" if the Trace Session is activated to any of the following ManagedEntity(ies): - HSSFunction (Home Subscriber Server) (TS 28.705 [44]) - MscServerFunction (Mobile Switching Centre Server) (TS 28.702 [45]) - SgsnFunction (Serving GPRS Support Node) (TS 28.702[45]) - GgsnFunction (Gateway GPRS Support Node) (TS 28.702[45]) - BmscFunction (Broadcast Multicast Service Centre) (TS 28.702[45]) - RncFunction (Radio Network Controller) (TS 28.652[46]) - MmeFunction (Mobility Management Entity) (TS 28.708[47]) - ServingGWFunction (Serving Gateway) (TS 28.708[47]) - PGWFunction (PDN Gateway) (TS 28.708[47]). The traceTargetType shall be either “SUPI” or “IMEISV” if the Trace Session is activated to any of the following ManagedEntity(ies) (TS 28.541[48]): - AFFunction - AMFFunction - AUSFunction - NEFFunction - NRFFunction - NSSFFunction - PCFFunction - SMFFunction - UPFFunction - UDMFunction In case of signalling based MDT, the traceTargetType attribute shall be able to carry "PUBLIC_ID", "IMSI", "IMEI", "IMEISV)" or "SUPI". In case of management based Logged MDT, the traceTarget attribute shall carry an "eNB" or a "gNB" or an "RNC". The Logged MDT should be initiated on the specified eNB/gNB/RNC in traceTarget. In case of signalling based 5GC UE level measurements collection, the traceTargetType attribute shall be able to carry "IMEISV" or "SUPI". In case of management based 5GC UE level measurements collection, the traceTargetType attribute shall be able to carry the corresponding Measured UE Identifier as defined by the bullet g) of the 5GC UE level measurements (see TS 28.558 [57]) when the TraceJob is created at the subject ManagedEntity. allowedValues: PUBLIC_ID, IMSI, IMEI, IMEISV, SUPI, ENB, GNB, RNC, UTRAN_CELL, EUTRAN_CELL, NGRAN_CELL, N4_SESSION_ID. | type: ENUM multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| traceTargetValueList | It specifies the ID value(s) of the target object type defined by traceTargetType | type: String multiplicity: * isOrdered: False isUnique: True defaultValue: N/A isNullable: False |
| triggeringEvents | It specifies the triggering event parameter of the trace session. The attribute is applicable only for Trace. See the clause 5.1 of 3GPP TS 32.422 [30] for additional details on the allowed values. | type: ENUM multiplicity: 0..1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| anonymizationOfMdtData | It specifies the level of anonymization of MDT data. This attribute is only applicable for management based activation. See the clause 5.10.12 of 3GPP TS 32.422 [30] for additional details on the allowed values. | type: ENUM multiplicity: 0..1 isOrdered: N/A isUnique: N/A defaultValue: NO_IDENTITY isNullable: False |
| areaConfigurationForNeighCell | It specifies the area for which UE is requested to perform measurement logging for neighbour cells which have list of frequencies. If it is not configured, the UE shall perform measurement logging for all the neighbour cells. Applicable only to NR Logged MDT. See the clause 5.10.26 of 3GPP TS 32.422 [30] for additional details on the allowed values. | type: AreaConfig multiplicity:* isOrdered: False isUnique: True defaultValue: None isNullable: False |
| areaScope | It specifies the area where data shall be collected. | type: AreaScope multiplicity: 0..1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| collectionPeriodRRMLTE | It specifies the collection period for collecting RRM configured measurement samples for M3 in LTE. The attribute is applicable only for Immediate MDT. See the clause 5.10.20 of 3GPP TS 32.422 [30] for additional details on the allowed values. | type: ENUM multiplicity: 0..1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| collectionPeriodRRMUMTS | It specifies the collection period for collecting RRM configured measurement samples for M3, M4, M5 in UMTS. The attribute is applicable only for Immediate MDT. See the clause 5.10.21 of 3GPP TS 32.422 [30] for additional details on the allowed values. | type: ENUM multiplicity: 0..1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| eventListForEventTriggeredMeasurement | It specifies event types for event triggered measurement in the case of logged NR MDT. Each trace session may configure at most one event. The UE shall perform logging of measurements only upon certain condition being fulfilled: - Out of coverage. - A2 event. See the clause 5.10.28 of 3GPP TS 32.422 [30] for additional details on the allowed values. | type: ENUM multiplicity: 0..1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| eventThreshold | It specifies the threshold which should trigger the reporting in case A2 event reporting in LTE and NR or 1F/1l event in UMTS. The attribute is applicable only for Immediate MDT and when reportingTrigger is configured for A2 event in LTE and NR or 1F event or 1l event in UMTS. See the clauses 5.10.7 and 5.10.7a of 3GPP TS 32.422 [30] for additional details on the allowed values. | type: Integer multiplicity: 0..1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| listOfMeasurements | It specifies the UE measurements that shall be collected in an Immediate MDT job. The attribute is applicable only for Immediate MDT. See the clause 5.10.3 of 3GPP TS 32.422 [30] for additional details on the allowed values. | type: ENUM multiplicity: 0..1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| loggingDuration | It specifies how long the MDT configuration is valid at the UE in case of Logged MDT. The attribute is applicable only for Logged MDT and Logged MBSFN MDT. See the clause 5.10.9 of 3GPP TS 32.422 [30] for additional details on the allowed values. | type: ENUM multiplicity: 0..1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| loggingInterval | It specifies the periodicity for Logged MDT. The attribute is applicable only for Logged MDT and Logged MBSFN MDT. See the clause 5.10.8 of 3GPP TS 32.422 [30] for additional details on the allowed values. | type: ENUM multiplicity: 0..1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| eventThresholdL1 | It specifies the threshold which should trigger the reporting in case of event based reporting of logged NR MDT. The attribute is applicable only for Logged MDT and when reportType is configured for event triggered reporting and when eventListEventForTriggeredMeasurement is configured for L1 event. See the clause 5.10.36 of TS 32.422 [30] for additional details on the allowed values. | type: Integer multiplicity: 0..1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| hysteresisL1 | It specifies the hysteresis used within the entry and leave condition of the L1 event based reporting of logged NR MDT. The attribute is applicable only for Logged MDT, when reportType is configured for event triggered reporting and when eventListForEventTriggeredMeasurement is configured for L1 event. See the clause 5.10.37 of TS 32.422 [30] for additional details on the allowed values. | type: Integer multiplicity: 0..1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| timeToTriggerL1 | It specifies the threshold which should trigger the reporting in case of event based reporting of logged NR MDT. The attribute is applicable only for Logged MDT, when reportType is configured for event triggered reporting and when eventListForEventTriggeredMeasurement is configured for L1 event. See the clauses 5.10.38 of TS 32.422 [30] for additional details on the allowed values. | type: ENUM multiplicity: 0..1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| mbsfnAreaList | The MBSFN Area consists of a MBSFN Area ID and Carrier Frequency (EARFCN). The target MBSFN area List can have up to 8 entries. This parameter is applicable only if the job type is Logged MBSFN MDT. See the clause 5.10.25 of TS 32.422 [30] for additional details on the allowed values. | type: MbsfnArea multiplicity: 0..8 isOrdered: False isUnique: True defaultValue: None isNullable: False |
| measurementPeriodLTE | It specifies the collection period for the Data Volume (M4) and Scheduled IP throughput measurements (M5) for LTE MDT taken by the eNB. The attribute is applicable only for Immediate MDT. See the clause 5.10.23 of TS 32.422 [30] for additional details on the allowed values. | type: ENUM multiplicity: 0..1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| measurementPeriodM6LTE | It specifies the collection period for the Packet Delay measurement (M6) for MDT taken by the eNB. The attribute is applicable only for Immediate MDT. See the clause 5.10.32 of TS 32.422 [30] for additional details on the allowed values. | type: ENUM multiplicity: 0..1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| collectionPeriodM7LTE | It specifies the collection period for the Packet Loss Rate measurement (M7) for LTE MDT taken by the eNB. The attribute is applicable only for Immediate MDT. See the clause 5.10.33 of TS 32.422 [30] for additional details on the allowed values. | type: ENUM multiplicity: 0..1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| measurementPeriodUMTS | It specifies the collection period for the Data Volume (M6) and Throughput measurements (M7) for UMTS MDT taken by RNC. The attribute is applicable only for Immediate MDT. See the clause 5.10.22 of TS 32.422 [30] for additional details on the allowed values. | type: ENUM multiplicity: 0..1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| collectionPeriodRRMNR | It specifies the collection period for collecting RRM configured measurement samples for M4, M5 in NR. The attribute is applicable only for Immediate MDT. See the clause 5.10.30 of TS 32.422 [30] for additional details on the allowed values. | type: ENUM multiplicity: 0..1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| collectionPeriodM6NR | It specifies the collection period for the Packet Delay measurement (M6) for NR MDT taken by the gNB. The attribute is applicable only for Immediate MDT. See the clause 5.10.34 of TS 32.422 [30] for additional details on the allowed values. | type: ENUM multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| collectionPeriodM7NR | It specifies the collection period for the Packet Loss Rate measurement (M7) for NR MDT taken by the gNB. The attribute is applicable only for Immediate MDT. See the clause 5.10.35 of TS 32.422 [30] for additional details on the allowed values. | type: ENUM multiplicity: 0..1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: True |
| beamLevelMeasurement | This indicates whether the NR M1 beam level measurements shall be included or not. See the clause 5.10.40 of TS 32.422 [30] for additional details. The default value is "FALSE". allowedValues: TRUE, FALSE | type: Boolean multiplicity: 0..1 isOrdered: N/A isUnique: N/A defaultValue: FALSE isNullable: False |
| eventThresholdUphUMTS | It specifies the threshold which should trigger the reporting in case of event-triggered periodic reporting for M4 (UE power headroom measurement) in UMTS. See the clause 5.10.39 of TS 32.422 [30] for additional details on the allowed values. | type: Integer multiplicity: 0..1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| measurementQuantity | It specifies the measurements that are collected in an MDT job for a UMTS MDT configured for event triggered reporting. See the clause 5.10.15 of TS 32.422 [30] for additional details on the allowed values. | type: ENUM multiplicity: 0..1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| plmnList | It indicates the PLMNs where measurement collection, status indication and log reporting are allowed. See the clause 5.10.24 of TS 32.422 [30] for additional details on the allowed values. | type: PlmnId multiplicity: 0..16 isOrdered: False isUnique: True defaultValue: None isNullable: False |
| positioningMethod | It specifies what positioning method should be used in the MDT job. See the clause 5.10.19 of TS 32.422 [30] for additional details on the allowed values. | type: Integer multiplicity: 0..1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| reportAmount | It specifies the number of measurement reports that shall be taken for periodic reporting while the UE is in connected mode. The attribute is applicable only for Immediate MDT and when reportingTrigger is configured for periodical measurements. See the clause 5.10.6 of TS 32.422 [30] for additional details on the allowed values. | type: ENUM multiplicity: 0..1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| reportAmountM1LTE | It specifies the number of measurement reports that shall be taken for periodic reporting while the UE is in connected mode. The attribute is applicable only for Immediate MDT and combined Trace and Immediate MDT and when reportingTrigger is configured for periodical measurements and applicable only for LTE. See the clause 5.10.6 of TS 32.422 [30] for additional details on the allowed values. | type: ENUM multiplicity: 0..1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| reportAmountM4LTE | It specifies the number of measurement reports that shall be taken for periodic reporting while the UE is in connected mode. The attribute is applicable only for Immediate MDT and combined Trace and Immediate MDT and when reportingTrigger is configured for periodical measurements and applicable only for LTE. See the clause 5.10.6 of TS 32.422 [30] for additional details on the allowed values. | type: ENUM multiplicity: 0..1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| reportAmountM5LTE | It specifies the number of measurement reports that shall be taken for periodic reporting while the UE is in connected mode. The attribute is applicable only for Immediate MDT and combined Trace and Immediate MDT and when reportingTrigger is configured for periodical measurements and applicable only for LTE. See the clause 5.10.6 of TS 32.422 [30] for additional details on the allowed values. | type: ENUM multiplicity: 0..1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| reportAmountM6LTE | It specifies the number of measurement reports that shall be taken for periodic reporting while the UE is in connected mode. The attribute is applicable only for Immediate MDT and combined Trace and Immediate MDT and when reportingTrigger is configured for periodical measurements and applicable only for LTE. See the clause 5.10.6 of TS 32.422 [30] for additional details on the allowed values. | type: ENUM multiplicity: 0..1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| reportAmountM7LTE | It specifies the number of measurement reports that shall be taken for periodic reporting while the UE is in connected mode. The attribute is applicable only for Immediate MDT and combined Trace and Immediate MDT and when reportingTrigger is configured for periodical measurements and applicable only for LTE. See the clause 5.10.6 of TS 32.422 [30] for additional details on the allowed values. | type: ENUM multiplicity: 0..1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| reportAmountM1NR | It specifies the number of measurement reports that shall be taken for periodic reporting while the UE is in connected mode. The attribute is applicable only for Immediate MDT and combined Trace and Immediate MDT and when reportingTrigger is configured for periodical measurements and applicable only for NR. In case this attribute is not used, it carries a null semantic. See the clause 5.10.6 of TS 32.422 [30] for additional details on the allowed values. | type: ENUM multiplicity: 0..1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| reportAmountM4NR | It specifies the number of measurement reports that shall be taken for periodic reporting while the UE is in connected mode. The attribute is applicable only for Immediate MDT and combined Trace and Immediate MDT and when reportingTrigger is configured for periodical measurements and applicable only for NR. See the clause 5.10.6 of TS 32.422 [30] for additional details on the allowed values. | type: ENUM multiplicity: 0..1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| reportAmountM5NR | It specifies the number of measurement reports that shall be taken for periodic reporting while the UE is in connected mode. The attribute is applicable only for Immediate MDT and combined Trace and Immediate MDT and when reportingTrigger is configured for periodical measurements and applicable only for NR. See the clause 5.10.6 of TS 32.422 [30] for additional details on the allowed values. | type: ENUM multiplicity: 0..1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| reportAmountM6NR | It specifies the number of measurement reports that shall be taken for periodic reporting while the UE is in connected mode. The attribute is applicable only for Immediate MDT and combined Trace and Immediate MDT and when reportingTrigger is configured for periodical measurements and applicable only for NR. See the clause 5.10.6 of TS 32.422 [30] for additional details on the allowed values. | type: ENUM multiplicity: 0..1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| reportAmountM7NR | It specifies the number of measurement reports that shall be taken for periodic reporting while the UE is in connected mode. The attribute is applicable only for Immediate MDT and combined Trace and Immediate MDT and when reportingTrigger is configured for periodical measurements and applicable only for NR. See the clause 5.10.6 of TS 32.422 [30] for additional details on the allowed values. | type: ENUM multiplicity: 0..1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| reportingTrigger | It specifies whether periodic or event based measurements should be collected. The attribute is applicable only for Immediate MDT and when the listOfMeasurements is configured for M1 (for UMTS, LTE and NR) or M2 (only for UMTS). See the clause 5.10.4 of TS 32.422 [30] for additional details on the allowed values. | type: ENUM multiplicity: 0..1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| reportInterval | It specifies the interval between the periodical measurements that shall be taken when the UE is in connected mode. The attribute is applicable only for Immediate MDT and when reportingTrigger is configured for periodical measurements. See the clause 5.10.5 of TS 32.422 [30] for additional details on the allowed values. | type: ENUM multiplicity: 0..1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| reportType | It specifies report type for logged NR MDT as: - periodical. - event triggered. See the clause 5.10.27 of TS 32.422 [30] for additional details on the allowed values. | type: ENUM multiplicity: 0..1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| sensorInformation | It specifies which sensor information shall be included in logged NR MDT and immediate NR MDT measurement if they are available. The following sensor measurement can be included or excluded for the UE: - Barometric pressure. - UE speed. - UE orientation. See the clause 5.10.29 of 3GPP TS 32.422 [30] for additional details on the allowed values. | type: ENUM multiplicity:* isOrdered: False isUnique: True defaultValue: None isNullable: False |
| traceCollectionEntityId | It specifies the TCE Id which is sent to the UE in Logged MDT. See the clause 5.10.11 of 3GPP TS 32.422 [30] for additional details on the allowed values. | type: Integer multiplicity: 0..1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| mcc | Mobile Country Code allowedValues: As defined by the data type | type: Mcc multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| mnc | Mobile Network allowedValues: As defined by the data type | type: Mnc multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| traceId | An identifier, which identifies the Trace (together with MCC and MNC). This is a 3 byte Octet String. See the clause 5.6 of 3GPP TS 32.422 [30] for additional details on the allowed values. | type: String multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| freqInfo | It specifies the carrier frequency and bands used in a cell. | type: FreqInfo multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| arfcn | RF Reference Frequency as defined in TS 38.104 [35], clause 5.4.2.1. The frequency provided identifies the absolute frequency position of the reference resource block (Common RB 0) of the carrier. Its lowest subcarrier is also known as Point A. allowedValues: 0, 1, …,3279165 | type: Integer multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| freqBands | List of NR frequency operating bands. Primary NR Operating Band as defined in TS 38.104 [35], clause 5.4.2.3. The value 1 corresponds to n1, value 2 corresponds to NR operating band n2, etc. allowedValues: 1, 2, …,1024 | type: Integer multiplicity: 1..* isOrdered: False isUnique: True defaultValue: None isNullable: False |
| pciList | List of neighbour cells subject for MDT scope. allowedValues: 0, 1, …,1007 | type: Integer multiplicity: 1..32 isOrdered: False isUnique: True defaultValue: None isNullable: False |
| tac | Tracking Area Code allowedValues: As defined by the data type | type: Tac multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| utraCellIdList | List of UTRAN cells identified by UTRAN CGI allowedValues: As defined by the data type | type: UtraCellId multiplicity: 1..32 isOrdered: False isUnique: True defaultValue: None isNullable: False |
| eutraCellIdList | List of E-UTRAN cells identified by E-UTRAN-CGI allowedValues: As defined by the data type | type: EutraCellId multiplicity: 1..32 isOrdered: False isUnique: True defaultValue: None isNullable: False |
| nrCellIdList | List of NR cells identified by NG-RAN CGI allowedValues: As defined by the data type | type: NrCellId multiplicity: 1..32 isOrdered: False isUnique: True defaultValue: None isNullable: False |
| tacList | Tracking Area Code list allowedValues: As defined by the data type | type: Tac multiplicity: 1..8 isOrdered: False isUnique: True defaultValue: None isNullable: False |
| taiList | Tracking Area Identity list allowedValues: As defined by the data type | type: Tai multiplicity: 1..8 isOrdered: False isUnique: True defaultValue: None isNullable: False |
| mbsfnAreaId | MBSFN Area Identifier AllowedValues: 1, 2, … | type: Integer multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| earfcn | Carrier Frequency AllowedValues: 1, 2, … | type: Integer multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| mnsLabel | Human-readable name of management service. | type: String multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| mnsType | Type of management service. allowedValues: ProvMnS, FaultSupervisionMnS, StreamingDataReportingMnS, FileDataReportingMnS | type: ENUM multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| mnsVersion | Version of management service. | type: String multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| mnsAddress | Addressing information for Management Service operations. | type: String multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| ProcessMonitor.id | Id of the process. It is unique within a single multivalue attribute of type ProcessMonitor. | Type: String multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| ProcessMonitor.status | This attribute represents the status of the associated process, whether it fails, succeeds etc. It does not represent the returned values of a successfully finished process. allowedValues: - NOT_STARTED - RUNNING - CANCELLING - FINISHED - FAILED - PARTIALLY_FAILED - CANCELLED | Type: ENUM multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| ProcessMonitor.progressPercentage | Progress of the process as percentage. Allowed values: integer between 0 and 100 | Type: Integer multiplicity: 0..1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| ProcessMonitor.progressStateInfo | Additional textual qualification of the states "NOT_STARTED", "CANCELLING" and "RUNNING". For specific processes, specific well-defined strings (e.g. string patterns or enums) may be defined as a specialisation. allowedValues: N/A | Type: String multiplicity: 0..* isOrdered: True isUnique: False defaultValue: None isNullable: False |
| ProcessMonitor.resultStateInfo | Additional textual qualification of the states "FINISHED", "FAILED", "PARTIALLY_FAILED and "CANCELLED". For example, in the "FAILED" or "PARTIALLY_FAILED" state this attribute may be used to provide error reasons. This attribute shall not be used to make the outcome of the process available for retrieval, if any. For this purpose, dedicated attributes shall be specified when specifying the representation of a specific process. For specific processes, specific well-defined strings (e.g. string patterns or enums) may be defined as a specialisation. allowedValues: N/A | Type: String multiplicity: 0..1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| ProcessMonitor.startTime | Start time of the associated process, i.e. the time when the status changed from "NOT_STARTED" to "RUNNING". allowedValues: N/A | Type: DateTime multiplicity: 0..1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| ProcessMonitor.endTime | Date and time when status changed to SUCCESS, CANCELLED, FAILED or PARTIALLY_FAILED. If the time is in the future, it is the estimated time the process will end. allowedValues: N/A | Type: DateTime multiplicity: 0..1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| ProcessMonitor.timer | Time until the associated process is automatically cancelled. If set, the system decreases the timer with time. When it reaches zero the cancellation of the associated process is initiated by the MnS_Producer. If not set, there is no time limit for the process. Once the timer is set, the consumer cannot change it anymore. If the consumer has not set the timer the MnS Producer may set it. Unit is minutes. allowedValues: Positive integers | Type: Integer multiplicity: 0..1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| mnsScope | This attribute defines the information about the management scope of the Management Service. The management scope is used to represent the set of managed object instances that can be accessed using the Management Service. | type: MnsScope multiplicity: 1..* isOrdered: False isUnique: True defaultValue: None isNullable: False |
| MnsScope. objectInstanceList | This attribute describes list of DNs for the managed object instances that can be accessed using the Management Service. If a complete SubNetwork can be accessed using the Management Service, this attribute may contain the DN of the SubNetwork instead of the DNs of the individual managed entities within the SubNetwork. If a complete ManagedElement can be accessed using the Management Service, this attribute may contain the DN of the ManagedElement instead of the DNs of the individual managed entities within the ManagedElement. | Type: DN multiplicity: 1..* isOrdered: False isUnique: True defaultValue: None isNullable: False |
| MnsScope.geoAreaList | This attribute describes geographical areas for the managed object instances that can be accessed using the Management Service. | Type: GeoArea multiplicity: 1..* isOrdered: False isUnique: True defaultValue: None isNullable: False |
| MnsScope.taiList | This attribute describes the list of Tracking Area Identities (TAI) for the managed object instances that can be accessed using the Management Service. | Type: Tai multiplicity: 1..* isOrdered: False isUnique: True defaultValue: None isNullable: False |
| mnsCapability | It describes the types of management capabilities of the MnS instance provided by the MnS producer. allowedValues: - NR_PROVISIONING - 5GC_PROVISIONING - NETWORK_SLICING_PROVISIONING - EDGE_COMPUTING_PROVISIONING - PERFORMANCE_METRIC_COLLECTION_CONTROL - PERFORMANCE_METRIC_DATA_REPORT - PERFORMANCE_METRIC_THRESHOLD_MONITOR_CONTROL - PERFORMANCE_METRIC_THRESHOLD_NOTIFICATION - FAULT_CONTROL - FAULT_NOTIFICATION - TRACE_MDT_DATA_COLLECTION_CONTROL - TRACE_MDT_DATA_REPORT - QOE_DATA_COLLECTION_CONTROL - QOE_DATA_REPORT - FILE_RETRIEVAL - FILE_DOWNLOAD - SUBSCRIPTION_CONTROL - HEARTBEAT_CONTROL - HEARTBEAT_NOTIFICATION - ML_MODEL_MANAGEMENT - MANAGEMENT DATA ANALYTIC - RANSC_MANAGEMENT - SON_POLICY - COMMUNICATION_SERVICE_ASSURANCE_CONTROL - INTENT_DRIVEN_MANAGEMENT - ML_MODEL_MANAGEMENT - MNS_REGISTRY_AND_DISCOVERY - MNS_ACCESS_CONTROL_MANAGEMENT - DSO_RAPID_RECOVERY_AND_THRESHOLD MONITORING The detailed description for above enum values see Annex F in TS 28.533 [32]. Note: vendor extension values are allowed for the attribute “mnsCapability”. | Type: Enum multiplicity: 0..* isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| managementData | This attribute defines the list of management data that are requested. | Type: ManagementData multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| mgtDataCategory | This attributes defines the type of management data that are requested. Allowed values for data category are COVERAGE, CAPACITY, ENERGY_EFFICIENCY, MOBILITY, ACCESSIBILITY. The data categories will map to certain measurement families defined in TS 28.552 [20], see below. In addition to the below mappings, MnS producer may map the provided categories to any additional proprietary management data, as appropriate. The COVERAGE category will map to measurement families of MR (measurements related to Measurement Report) and L1M (measurements related to Layer 1 Measurement). The CAPACITY category will map to measurement family RRU (measurements related to Radio Resource Utilization). The ENERGY_EFFICIENCY category will map to measurement family PEE (measurements related to Power, Energy and Environment). The MOBILITY category will map to measurement family MM (measurements related to Mobility Management). The ACCESSIBILITY category will map to measurement family CE (measurements related to Connection Establishment). Allowed values: COVERAGE, CAPACITY, SERVICE EXPERIENCE, TRACE, ENERGY EFFICIENCY, MOBILITY, ACCESSIBILITY See NOTE 7. | type: ENUM multiplicity: * isOrdered: False isUnique: True defaultValue: None isNullable: False |
| mgtDataName | A list of management data identified by name. allowedValues: The list may include metrics or set of metrics defined in TS 28.552 [20], TS 28.554 [28] and TS 32.422 [30]. For performance measurements defined in TS 28.552 [20] the name is constructed as the bullet e) of measurement definition with allowed measurement type. For trace metrics (including trace messages, MDT measurements (Immediate MDT, Logged MDT, Logged MBSFN MDT), RRC, RLF and RCEF reports) defined in TS 32.422 [30], the name (metric identifier) is defined in clause 10 of TS 32.422 [30]. For non-3GPP specified managment data the name is defined elsewhere. | type: String multiplicity: * isOrdered: False isUnique: True defaultValue: None isNullable: False |
| consolidateOutput | Indicates whether the management data collection output will be consolidated into a single file per reporting period. | type: Boolean multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| targetNodeFilter | Set of information to target the Object Instance to collect the management data from. | type: NodeFilter multiplicity: * isOrdered: False isUnique: True defaultValue: No isNullable: False |
| areaOfInterest | It specifies a location(s) from where the management data shall be collected. | type: AreaOfInterest multiplicity: * isOrdered: False isUnique: True defaultValue: No isNullable: False |
| geoAreaToCellMapping | It specifies the geographical area from where the management data shall be collected and the mapping to cells. allowedValues: N/A | type: GeoAreaToCellMapping multiplicity: * isOrdered: False isUnique: True defaultValue: None isNullable: False |
| geoPolygon | It specifies the geographical area with a polygon. The polygon is specified by its corners. allowedValues: N/A | type: GeoCoordinate multiplicity: 1..* isOrdered: True isUnique: True defaultValue: None isNullable: True |
| geoArea | It specifies the geographical area using the coordinates of the corners of a polygon. allowedValues: N/A | type: GeoArea multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| latitude | Latitude based on World Geodetic System (1984 version) global reference frame (WGS 84). Positive values correspond to the northern hemisphere. AllowedValues: -90.0000, …+90.0000 | type: float multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| longitude | Longitude based on World Geodetic System (1984 version) global reference frame (WGS 84). Positive values correspond to degrees east of 0 degrees longitude. AllowedValues: -180.0000, … +180.0000 | type: float multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| altitude | It is the vertical distance between the point of interest from the mean sea level measured in metres. | type: Float multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| associationThreshold | It specifies the threshold of coverage area in percentage whether a cell belongs to the geographical area or not. If this attribute is absent, the location of the base station antenna determines whether a cell belongs to the geographical area or not. Allowed values: 1,…,100 | type: Integer multiplicity: 0..1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| networkDomain | It specifies the network domain of the target node. This will also result in collecting appropriate management data from the nodes belonging to the specified domain. Allowed Values: CN, RAN | type: ENUM multiplicity: 0..1 isOrdered: N/A isUnique: N/A defaultValue: N/A isNullable: False |
| cpUpType | It specifies the traffic type of the target node. This will also result in collecting appropriate management data from the nodes handling the specified traffic (e.g AMF for CP and UPF for UP). Allowed Values: CP, UP | type: ENUM multiplicity: 0..1 isOrdered: N/A isUnique: N/A defaultValue: N/A isNullable: False |
| sst | It specifies the slice service type (SST) of which the slice subnet should be targeted. Please refer to TS 23.501 [22]. | type: Integer multiplicity: 0..1 isOrdered: N/A isUnique: N/A defaultValue: N/A isNullable: False |
| collectionTimeWindow | Collection time window for which the management data should be reported. | type: TimeWindow multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: N/A isNullable: False |
| startTime | It indicates the time (in "date-time" format) when the management activity shall be started. AllowedValues: N/A. | type: DateTime multiplicity: 0..1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| endTime | It indicates the time (in "date-time" format) when the management activityshall be stopped. AllowedValues: N/A. | type: DateTime multiplicity: 0..1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| timeWindow | Time window for which the configured management activity shall be active. | type: TimeWindow multiplicity: 0..1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| timeIntervals | List of intervals within one day for which the service shall be active. | type: TimeInterval multiplicity: * isOrdered: False isUnique: True defaultValue: None isNullable: False |
| intervalStart | It indicates the time (in "full-time" format) when the service shall be started. Data type "FullTime" defines the time as specified by "full-time" in RFC3339 [54]. AllowedValues: N/A. | type: FullTime multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| intervalEnd | It indicates the time (in "full-time" format) when the service shall be stopped. "FullTime" defines the time as specified by "full-time" in RFC3339 [54]. AllowedValues: N/A. | type: FullTime multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| daysOfWeek | It indicates the days on which the service shall be scheduled in case of weekly repetition. The intervals per day are configured by attribute timeIntervals. AllowedValues: - MONDAY - TUESDAY - WEDNESDAY - THURSDAY - FRIDAY - SATURDAY - SUNDAY | type: ENUM multiplicity: 1..7 isOrdered: False isUnique: True defaultValue: None isNullable: False |
| daysOfMonth | It indicates the days in a month on which the service shall be scheduled in case of monthly repetition. Value 0 presents the last day of the month. The intervals per day are configured by attribute timeIntervals. AllowedValues: 0, 1, …31 | type: Integer multiplicity: * isOrdered: False isUnique: True defaultValue: None isNullable: False |
| schedulingTimes | It defines the active scheduling times. | type: SchedulingTime multiplicity: 1..* isOrdered: False isUnique: True defaultValue: None isNullable: False |
| schedulerStatus | Switches between TRUE and FALSE depending upon whether the configured time constraints are fulfilled or not. | type: Boolean multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| conditionStatus | Switches between TRUE and FALSE depending upon whether the configured constraints are fulfilled or not. | type: Boolean multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| schedulerRef | Pointer to a Scheduler object. | type: Dn multiplicity: 0..1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| conditionMonitorRef | Pointer to a ConditionMonitor object. | type: Dn multiplicity: 0..1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| condition | Logical expression of one or several condition(s). The actual syntax and capabilities of condition is SS specific. However, each SS should support condition consisting of one or several assertions that may be grouped using the logical operators AND, OR and NOT. Only if the whole expression of condition evaluates TRUE, the attribute conditionStatus will be TRUE. Each assertion is a pointer to a Boolean parameter or a logical expression of attribute existence or attribute value comparison ("equal to X, less than Y" etc.). An empty string is not allowed. allowedValues: N/A | type: String multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| dataScope | It specifies whether the required data is reported per S-NSSAI or per 5QI or per PLMN. Allowed Value: SNSSAI, 5QI, PLMN | type: ENUM multiplicity: 0..1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| serviceType | Specifies an end user service type for QoE measurements. allowedValues: DASH, MTSI, VR | type: ENUM multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| qoECollectionEntityAddress | Specifies the address to which the QMC records shall be transferred. Ipv4 or Ipv6 address(es) may be used. | type: IpAddress multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| qoETarget | Specifies the target object of the QMC in case of signalling based QMC. The qoETarget attribute shall be able to carry "IMSI” or "SUPI". | type: String multiplicity: 0..1 isOrdered:N/A isUnique: N/A defaultValue: None isNullable: False |
| qoEReference | Identifies the QoE measurement collection job in the Managed Elements and in the measurement collection entity. The QoE reference shall be globally unique therefore it is composed as follows: MCC+MNC+QMC ID, where the MCC and MNC are coming with the QMC activation request from the management system to identify one PLMN containing the management system, and QMC ID is a 3 byte Octet String. The QMC ID is generated by the management system or the operator. | type: String multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| sliceScope | Contains a list of S-NSSAIs (Single Network Slice Selection Assistance Information). A Network Slice is identified by S-NSSAI. | type: S-NSSAI multiplicity: * isOrdered: False isUnique: True defaultValue: None isNullable: False |
| sliceIdList | Contains a list of network slices identified by PLMN-Id and S-NSSAI. | type: PLMNInfo multiplicity: 0..16384 isOrdered: False isUnique: True defaultValue: None isNullable: False |
| pLMNId | Identifies a single PLMN. | type: PLMNId multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| sNSSAI | Identifies a single network slice by S-NSSAI (Single Network Slice Selection Assistance Information). | type: S-NSSAI multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| qMCConfigFile | Provides a reference to a file including the parameters for configuration of application layer measurements, known as Container for Application Layer Measurement Configuration | Type: String multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| excessPacketDelayThresholds | Excess packet delay thresholds info for M6 UL measurement. | type: ExcessPacketDelayThresholds multiplicity: 0..255 isOrdered: False isUnique: True defaultValue: None isNullable: False |
| fiveQIValue | It indicates 5QI value. allowedValues: 0 - 255 | type: Integer multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| excessPacketDelayThresholdValue | Value of excess packet delay threshold for M6 UL measurement. allowedValues: 0.25ms, 0.5ms, 1ms, 2ms, 4ms, 5ms, 10ms, 20ms, 30ms, 40ms, 50ms, 60ms, 70ms, 80ms, 90ms, 100ms, 150ms, 300ms, 500ms, … | type: ENUM multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| mDTAlignmentInformation | This parameter indicates the MDT measurements with which alignment of QoE measurement is required. This parameter is optional and is valid for NR only. | Type: TraceReference multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| availableRANqoEMetrics | This parameter indicates available RAN visible QoE metrics to the gNB. This parameter is optional and is valid for NR only. allowedValues: APP_LAYER_BUFFER_LEVEL_LIST, PLAYOUT_DELAY_FOR_MEDIA_STARTUP | Type: ENUM multiplicity: 0..2 isOrdered: False isUnique: True defaultValue: None isNullable: False |
| dnPrefix | It carries the DN Prefix information or no information. See Annex C of TS 32.300 [13] for one usage of this attribute. allowedValues: N/A | type: DN multiplicity: 0..1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| nPNIdentityList | It defines which NPNs that can be served by the NR cell, and which CAG IDs or NIDs can be supported by the NR cell for corresponding PNI-NPN or SNPN in case of the cell is NPN-only cell. (NPN-Identity referring to TS 38.331 [38]) allowedValues: N/A | type: NpnId multiplicity: 1..* isOrdered: False isUnique: True defaultValue: None isNullable: False |
| cAGIdList | It identifies a CAG list containing up to 256 CAG-identifiers per UE or up to 12 CAG-identifiers per cell, see TS 38.331 [38]. CAG ID is used to combine with PLMN ID to identify a PNI-NPN. CAG ID is a hexadecimal range with size 32 bit. allowedValues: N/A | type: String multiplicity: 0..256 isOrdered: False isUnique: True defaultValue: None isNullable: False |
| nIDList | It identifies a list of NIDs containing up to 16 NIDs, see TS 38.331 [38]. NID is used to combine with PLMN ID to identify an SNPN. NID is a hexadecimal range with size 44 bit. | type: String multiplicity: 0..16 isOrdered: False isUnique: True defaultValue: None isNullable: False |
| nPNTarget | It defines which NPN that the subscriber of the session to be recorded uses as selected NPN. There is maximum one CAG ID present in cAGIdList in case of PNI-NPN or maximum one NID present in nIDList in case of SNPN | type: NpnId multiplicity: 0..1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| ueCoreMeasConfig | The set of parameters specific for 5GC UE level measurements configuration. | type: UECoreMeasConfig multiplicity: 0..1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| ueCoreMeasurements | List of 5GC UE level measurements identified by name. allowedValues: The list may include 5GC UE level measurements defined in TS 28.558 [57], or vendor specific measurements. For 5GC UE level measurements defined in TS 28.558 [57], the name is constructed as the bullet e) of measurement definition with allowed measurement type. For non-3GPP specified 5GC UE level measurements the name is defined elsewhere. | type: String multiplicity: 1..* isOrdered: False isUnique: True defaultValue: None isNullable: False |
| ueCoreMeasGranularityPeriod | Granularity period used to produce 5GC UE level measurements. The period is defined in milliseconds (ms). See Note 8. allowedValues: Integer with a minimum value of 10 | type: Integer multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| nfTypeToMeasure | It indicates the type of NE to produce the 5GC UE level measurements. allowedValues: The NF types represented by the measured object classes as defined by f) of the 5GC UE level measurements specified in TS 28.558 [57]. | type: String multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| processMonitor | This IE indicates the process of the ManagementDataCollection MOI. | Type: ProcessMonitor multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| mBSCommunicationServiceType | This IE indicates for which type of MBS communication service the QoE measurement configuration pertains to. See the clause 4.5.1 of TS 28.405 [50] for additional details. allowedValue: BROADCAST, MULTICAST | type: ENUM multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: False isNullable: False |
| month | It indicates the month in a year. allowedValues: 1, …, 12 | type: DateMonth multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| monthDay | It indicates the day in a month. allowedValues: 1, …, 31 | type: DateMonthDay multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| mNOnly | This indicates whether the MDT configuration is for MN only or not. The value "FALSE" means the MDT configuration is for both MN and SN. The value “TRUE” means the MDT configuration is for MN only. | type: Boolean multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: FALSE isNullable: False |
| externalDataType | Type of external management data as defined by the implementation. Examples: “Electronic Map”, “Camara Data”, “UE path”, “Camera Photo”, “Event Schedule” | Type: String multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| mediaLocation | URI of the media which includes the transfer protocol. Examples: "sftp://companyA.com/datastore/fileName.xml", "https://companyA.com/ManagedElement=1/Files=1/File=1” allowedValues: NA | Type: Uri multiplicity: 0..* isOrdered: false isUnique: true defaultValue: None isNullable: False |
| externalDataTypeSchema | URI of the schema to parse a type of external management data. The detailed schema definition for the different types of external management data is out of scope of this specification. allowedValues: NA | Type: String multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| externalDataScope | It describes the concrete scope which the external management data is applicable. | type: ExternalDataScope multiplicity: * isOrdered: False isUnique: True defaultValue: None isNullable: False |
| geoAreas | It describes the concrete geographical area(s) | type: GeoArea multiplicity: * isOrdered: False isUnique: True defaultValue: None isNullable: False |
| objectInstancesIncluded | List of managed object instances to which the described data are related. Each object instance is identified by its DN. allowedValues: N/A | type: DN multiplicity: * isOrdered: False isUnique: True defaultValue: None isNullable: False |
| objectInstancesExcluded | List of managed object instances which are not considered in relation to the described data. Each object instance is identified by its DN. allowedValues: N/A | type: DN multiplicity: * isOrdered: False isUnique: True defaultValue: None isNullable: False |
| supportedManagementData | This attribute defines the list of management data that can be supported. The management data is a choice between: - a list of data categories (attribute mgtDataCategory) - a list of management data identified with their name (attribute "mgtDataName"). | Type: ManagementData multiplicity: * isOrdered: False isUnique: True defaultValue: None isNullable: False |
| supportedGranularityPeriods | Granularity periods supported for the production of associated management data. The period is defined in seconds. | Type: Integer multiplicity: * isOrdered: False isUnique: True defaultValue: None isNullable: False |
| supportedReportingPeriods | Reporting periods supported for the associated management data. The period is defined in seconds. | Type: Integer multiplicity: * isOrdered: False isUnique: True defaultValue: None isNullable: False |
| historicalDataPeriod | This attribute describes the maximum period of the requested historical data. The period is defined in seconds. When the value of this attribute is NULL, which means the capability of querying historical data is not supported. | Type: Integer multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: TRUE |
| supportedReportingMethod | List of supported reporting methods for the associated management data. AllowedValues: - FILE - STREAM | type: ENUM multiplicity: 1..* isOrdered: False isUnique: True defaultValue: None isNullable: False |
| supportedDataScope | List of supported sub counter capabilities for the associated management data Allowed Values: - SNSSAI - 5QI - PLMN | type: ENUM multiplicity: 1..* isOrdered: False isUnique: True defaultValue: None isNullable: False |
| supportedDataRequestMnSRef | List of DN of MnSInfo for the MnS instance(s) which can be used to request the associated management data | type: DN multiplicity: 1..* isOrdered: False isUnique: True defaultValue: None isNullable: False |
| supportedDataReportingMnSRef | List of DN of MnSInfo for the MnS instance(s) which can be used to report the associated management data | type: DN multiplicity: 1..* isOrdered: False isUnique: True defaultValue: None isNullable: False |
| MgmtDataInfoRef | List of DN of MgmtDataInfo instance(s) which are associated the MnSInfo which represent a management service instance | type: DN multiplicity: 1..* isOrdered: False isUnique: True defaultValue: None isNullable: False |
| NOTE 1: The value of this attribute is identical to that of the same attribute in clause 9.4.2 of ETSI GS NFV-IFA 008 [16]. NOTE 2: The value of this attribute is identical to that of the attribute isAutoscaleEnabled included in vnfConfigurableProperty in clause 9.4.2 of ETSI GS NFV-IFA 008 [16]. NOTE 3: The presence of the attribute vnfParametersList, whose vnfInstanceId with a string length of zero, in createMO operation can trigger the instantiation of the related VNF/VNFC instances. NOTE 4: The GP defines the measurement data production rate. The supported rates are dependent on the capacity of the producer involved (e.g. the processing power of the producer, the complexity of the measurement type involved etc) and therefore, it cannot be standardized for all producers involved. The supported GPs reflects the agreement between producer and the consumer involved. NOTE 5: The monitoring granularity period defines the measurements monitoring period. The supported monitoring periods are dependent on the capacity of the producer involved (e.g. the processing power of the producer, the complexity of the measurement type involved etc) and therefore, it cannot be standardized for all producers involved. The supported monitoring GPs reflect the agreement between producer and the consumer involved. NOTE 6: The supported threshold levels are dependent on the capacity of the producer involved (e.g. the processing power of the producer, number of measurements being measured by the producer at the time, the complexity of the measurement type involved etc) and therefore, it cannot be standardized for all producers involved. The supported levels can only reflect the negotiated agreement between producer and the consumer involved. NOTE 7: The above values can be further extended by the implementations, as appropriate. NOTE 8: The ueCoreMeasGranularityPeriod defines the measurement data production rate. The supported rates are dependent on the capacity of the producer involved (e.g. the processing power of the producer, the complexity of the measurement type involved etc) and therefore, it cannot be standardized for all producers involved. The supported Granularity periods reflects the agreement between producer and the consumer involved. |  |  |  |

End of changes