3GPP TSG-SA5 Meeting #162	S5-253651

# Stor-Göteborg, Sweden, 25th Aug 2025 - 29th Aug 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 28.532 | CR | 0393 | rev | - | Current version: | 19.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Rel-19 CR 28.532 Correct notification name notifyDataNodeTreeSyncRecommended |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson Hungary Ltd |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S5 |  |  |  |  |  |  |  |  |  |
| Work item code: | SBMA_Ph3 |  |  |  |  |  | Date: |  |  | 2025-08-15 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | The name of the notification notifyDataNodeTreeSyncRecommended is spelled differently in stage 2 and stage 3. In stage 2 it is spelled with Synch in YAML as Sync without the 'H'. The 2 names should be the same. Sync is the preferred spelling because: - it is the more common abbreviation of Synchronisation - it is shorter - there is less chance of disturbing existing implementations if stage-3 is unchanged Correct the value of notificationType for notifyDataNodeTreeSyncRecommended |  |  |  |  |  |  |  |  |
| Summary of change: |  | Correct notification name notifyDataNodeTreeSyncRecommended |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Misunderstandings, interoperability problems, mismatching stage 2 and 3 |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 11.1.1.13, 11.1.1.13.1, 11.1.1.13.2, 12.1.1.2.1, 12.1.1.2.8 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

First change

#### 11.1.1.13	Notification notifyDataNodeTreeSyncRecommended

##### 11.1.1.13.1	Definition

This notification is sent to subscribed MnS consumer(s) if the MnS producer recommends to the MnS consumer to synchronize his copy of the data node tree on the MnS producer with the current data node tree on the MnS producer.

The notification should be sent in numerous scenarios and for various reasons:

- If the data node tree on the MnS producer becomes reliable again after it was unreliable, and a "notifyPotentialFaultyDataNodeTree" notification was sent.

- If the MnS producer realizes that the data node tree was unreliable only when it is reliable again, and no previous "notifyPotentialFaultyDataNodeTree" notification was sent.

- If, for whatever reason, not all data node tree change notifications, that should have been sent based on the changes in the data node tree, are sent, and the fault causing this erroneous behaviour is gone.

- If the connection from the MnS producer to a MnS consumer was down and the MnS producer determines it is up again.

The notification allows to specify the parts of the data node tree, that should be synchronized, using the parameters "objectInstance" and "scope". The usage of these parameters is defined in clause 11.1.1.12.1.

If the "notifyPotentialFaultyDataNodeTree" notification is supported, the "notifyDataNodeTreeSyncRecommended" notification shall be supported as well. Furthermore, if a "notifyPotentialFaultyDataNodeTree" is sent to a subscribed consumer, a subsequent "notifyDataNodeTreeSyncRecommended" notifications shall be sent to the consumer, when the data node tree is reliable again.

##### 11.1.1.13.2	Input parameters

| Parameter Name | S | Documentation and Allowed Values | Properties |
| --- | --- | --- | --- |
| objectClass | M | See clause 11.0.2 | Type: String multiplicity: 1 isOrdered: N/A isUnique: N/A |
| objectInstance | M | If the "scope" parameter is absent, this parameter identifies the root object of the subtree that should be synchronized. If the "scope" parameter is present, this parameter identifies the base object of the scope. | Type: DN multiplicity: 1 isOrdered: N/A isUnique: N/A |
| notificationId | M | See clause 11.0.2 | Type: Integer multiplicity: 1 isOrdered: N/A isUnique: N/A |
| notificationType | M | "notifyDataNodeTreeSyncRecommended " | Type: String multiplicity: 1 isOrdered: N/A isUnique: N/A |
| eventTime | M | See clause 11.0.2 | Type: DateTime multiplicity: 1 isOrdered: N/A isUnique: N/A |
| systemDN | M | See clause 11.0.2 | Type: DN multiplicity: 1 isOrdered: N/A isUnique: N/A |
| sequenceNo | O | See clause 11.0.2 | Type: integer multiplicity: 1 isOrdered: N/A isUnique: N/A |
| subscriptionId | O | See clause 11.0.2 | Type: DN multiplicity: 1 isOrdered: N/A isUnique: N/A |
| scope | O | This parameter specifies the set of data nodes that should be synchronized. The base object of the scope is specified by the "objectInstance" parameter. The parameter has the same structure and semantics as the "Scope" data type in TS 28.622 [11], clause 4.3.23 CHOICE_1.1 scopeType CHOICE_1.2 scopeLevel CHOICE_2.1 dataNodeSelector If the parameter is absent, the scope is the full subtree under "objectInstance". | Type: Scope multiplicity: 0..1 isOrdered: N/A isUnique: N/A |

Next change

##### 12.1.1.2.1	Introduction

The IS notifications are mapped to SS equivalents according to table 12.1.1.2.1-1.

Table 12.1.1.2.1-1: Mapping of IS notifications to SS equivalents

| IS notification | HTTP Method | Resource URI | S |
| --- | --- | --- | --- |
| notifyMOICreation | POST | {notificationTarget} | M |
| notifyMOIDeletion | POST | {notificationTarget} | M |
| notifyMOIAttributeValueChanges | POST | {notificationTarget} | M |
| notifyMOIChanges | POST | {notificationTarget} | M |
| notifyEvent | POST | {notificationTarget} | M |
| notifyPotentialFaultyDataNodeTree | POST | {notificationTarget} | M |
| notifyDataNodeTreeSyncRecommended | POST | {notificationTarget} | M |

First change

##### 12.1.1.2.8	Notification notifyDataNodeTreeSyncRecommended

Principles:

- 	Only information not documented in the OpenAPI files is included in this clause.

- 	The following items are documented in the OpenAPI files: HTTP-Method, parameter name and type.

- 	The name of each parameter is the same in the stage 2 information model (clause 11.1) and in the stage 3 OpenAPI definition. Exceptions, if any, are listed below.

Table 12.1.1.2.8-1: Mapping of IS notification input parameters to SS equivalents (HTTP POST)

| IS parameter name | SS parameter location | SS parameter name | SS parameter type |
| --- | --- | --- | --- |
| objectClass | request body | href | Uri |
| objectInstance |  |  |  |

End of  changes