3GPP TSG-SA5 Meeting #162	S5-253720

# Stor-Göteborg, Sweden, 25th Aug 2025 - 29th Aug 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 28.540 | CR | 0049 | rev |  | Current version: | 19.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Rel-20 CR TS 28.540 Add concept and background for management of Ambient IoT |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Samsung R&D Institute UK |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA5 |  |  |  |  |  |  |  |  |  |
| Work item code: | AdNRM_Ph4-OAM |  |  |  |  |  | Date: |  |  | 2025-08-15 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-20 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | TS 23.369 requires support from OAM to enable AIOT services in 5GC. Clause 5.12 of TS 28.540 states requirements for management of Ambient IoT but the concept and background for same in missing in this TS in relevant section. Hence it is proposed to add some concept and background about management of Ambient IoT by enhancing related configurations of 5GC NRM in OAM. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add concept and background for management of Ambient IoT. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The concept and background for Ambient IoT management is missing. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 2, 4.X (new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

1st Change

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP TS 23.501: "System Architecture for the 5G System".

[3]	3GPP TS 38.300: "NR; Overall description; Stage-2".

[4]	3GPP TS 38.401: "NG-RAN; Architecture description".

[5]	3GPP TS 37.340: "NR; Multi-connectivity; Overall description; Stage 2".

[6]	3GPP TS 28.531: "Management and orchestration of 5G networks; Provisioning".

[7]	3GPP TS 23.502: "Procedures for the 5G System; Stage 2".

[8]	GSMA NG.116 – "Generic Network Slice Template" v1.0 (2019-05-23).

[9]	3GPP TS 28.533: "Management and orchestration; Architecture framework".

[10]	3GPP TS 38.331: "NR; Radio Resource Control (RRC); Protocol specification".

[11]	3GPP TS 23.369: " Ambient power-enabled Internet of Things; Stage 2".

[X]                       3GPP TR 38.769: “Study on solutions for Ambient IoT (Internet of Things) in NR”

Next Change

## 4.X	Management of Ambient IoT

The 5G System (5GS) architecture for AIoT encompasses core network functions, AIoT Radio Access Network (RAN) and Reader architecture, and AIoT devices. However, to enable AIoT services such as inventory service and command service, as defined in the 3GPP SA2 specification 23.369[11], it is required to correctly identify the appropriate AIoT RAN node instance to ensure accurate identification of AIoT devices for correctly triggering various AIoT related operations. This requires an effective mapping between the expected external target area (provided by the Application Function) and the internal target area served by the 5G Core (5GC) and RAN nodes and readers.

The AIoT service operations can work flawlessly with the help of configurations provided by Operations, Administration, and Management (OAM). The NEF can obtain internal area mapping corresponding to the external target area from OAM and use it for further processes like identifying of correct AIOTF with the help of NRF. This ensures that the NEF can accurately identify and interact with the correct internal network segments, leading to targeted service delivery. The NRF can obtain the AIOTF DN corresponding to the internal area for sending service requests to the correct AIOTF. This mapping ensures that data and service requests are routed accurately, reducing latency and improving the overall performance of AIoT applications.

As stated in TS 23.369[11], AIOTF also can obtain AIoT RAN-related information from OAM (e.g. AIoT Service Area, Reader ID list) to select the correct AIoT RAN nodes or readers for accurate identification of AIoT devices for trigerring required services. This capability maintains the integrity and reliability of the ambient IoT networks, as it allows the AIOTF to dynamically adapt to changing RAN network conditions or configurations and ensure that ambient IoT devices are always connected to the suitable RAN nodes and hence AIoT network.

End of Change