3GPP TSG-SA5 Meeting #162	S5-253983

Goteborg, Sweden, 25 - 29 August 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 28.540 | CR | 0047 | rev | 1 | Current version: | 19.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Rel-19 CR TS 28.540 add missing feature concepts and requirements |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, CATT |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI19 |  |  |  |  |  | Date: |  |  | 2025-08-15 |
| Category: | C |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | In TS 28.541, the NRM fragment to support Prose and SON function management has been already defined. However, there is no corresponding requirements description in the current TS 28.540 specification. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add missing Prose, SON function management requirements. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The use case and requirements for Prose and SON function management will not clear. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 2, 5.1, 5.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

1st Change

2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP TS 23.501: "System Architecture for the 5G System".

[3]	3GPP TS 38.300: "NR; Overall description; Stage-2".

[4]	3GPP TS 38.401: "NG-RAN; Architecture description".

[5]	3GPP TS 37.340: "NR; Multi-connectivity; Overall description; Stage 2".

[6]	3GPP TS 28.531: "Management and orchestration of 5G networks; Provisioning".

[7]	3GPP TS 23.502: "Procedures for the 5G System; Stage 2".

[8]	GSMA NG.116 – "Generic Network Slice Template" v1.0 (2019-05-23).

[9]	3GPP TS 28.533: "Management and orchestration; Architecture framework".

[10]	3GPP TS 38.331: "NR; Radio Resource Control (RRC); Protocol specification".

[11]	3GPP TS 23.369: " Ambient power-enabled Internet of Things; Stage 2".

[x1]	3GPP TS 23.304: "Proximity based Services (ProSe) in the 5G System (5GS)".

Next Change

5.1	Requirements for management of NG-RAN

The following specific requirements apply to NG-RAN:

REQ-NGRAN_NRM-CON-001: The NRM definitions shall support management of NG-RAN, containing gNB or/and ng-eNB.

REQ-NGRAN_NRM-CON-002: The NRM definitions shall support management of either gNB without split function or gNB with split functions defined in 3GPP TS 38.401 [4].

REQ-NGRAN_NRM-CON-003: The NRM definitions shall support management of virtualized network functions that are part of gNB, e.g. virtualized gNB-CU.

REQ-NGRAN_NRM-CON-004: The NRM definitions shall support management of intra-NG-RAN handover between any combinations of gNB and ng-eNB.

REQ-NGRAN_NRM-CON-005: The NRM definitions shall support management of inter-system handover between 5GS and EPS.

REQ-NGRAN_NRM-CON-006: The NRM definitions shall support management of network slicing feature in NG-RAN.

REQ-NGRAN_NRM-CON-007: The NRM definitions shall have a read-only representation of NR beam properties in NG-RAN.

REQ-NGRAN_NRM-CON-008: The NRM definitions shall support configuration of sector carrier coverage properties in NG-RAN.

REQ-NGRAN_NRM-CON-009: The NRM definitions shall support management of radio access network sharing feature in NG-RAN.

REQ-NGRAN_NRM-CON-00x: The NRM definitions shall support management of self optimization network feature in NG-RAN.

Next Change

5.3	Requirements for management of 5GC NFs

The following specific requirements apply to management of 5GC NFs:

REQ-5GC_NRM-CON-001: The NRM definitions shall support management of 5GC containing variety of network functions defined in 3GPP TS 23.501 [2].

REQ-5GC_NRM-CON-002: The NRM definitions should support management of all the relationships between network functions and corresponding data storage functions where the network functions store/retrieve their data.

REQ-5GC_NRM-CON-003: The NRM definitions shall support management of interworking between 5GC and EPC.

REQ-5GC_NRM-CON-004: The NRM definitions shall support management of network slicing feature in 5GC.

REQ-5GC_NRM-CON-005: The NRM definitions shall support management of 5GC network function services independently to align with requirements of 3GPP TS 23.501 [2].

REQ-5GC_NRM-CON-006: The NRM definitions shall support NF service instance registration, deregistration, update, etc., with NRF during NF service instance lifecycle to align with requirements of 3GPP TS 23.501 [2] and 3GPP TS 23.502 [7].

REQ-5GC_NRM-CON-007: The NRM definitions shall support configuration of NF profile for NF Service registration and discovery to align with requirements of 3GPP TS 23.501 [2] and 3GPP TS 23.502 [7].

REQ-5GC_NRM-CON-00x: The NRM definitions should support 5G ProSe Direct Discovery and Communication to align with requirements of 3GPP TS 23.304 [x1].

End of changes