3GPP TSG-SA5 Meeting #163	S5-254577

Wuhan, China, 13 - 17 October 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 28.622 | CR | 0607 | rev | - | Current version: | 19.5.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | x | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Rel-19 CR TS 28.622 Corrections for Condition Monitor |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA5 |  |  |  |  |  |  |  |  |  |
| Work item code: | MADCOL_Ph2 |  |  |  |  |  | Date: |  |  | 2025-09-29 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | ConditionMonitor definition does not provide information on how conditions on performance metrics shall be constructed. This CR provides clarity on conditions on performance metrics as part of ConditionMonitor. Removal of editor’s notes related to condition on performance metrics. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Provides clarity on how conditions on performance metrics can be constructedby using Condition Monitor. - Remove editor’s notes related to condition on performance metrics. - Replace attribute condition by conditionMonitorRef. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Ambiguity on how to construct conditions on performance metrics. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.3.47.1, 4.3.47.2, 4.3.65.1, 4.3.65.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

Start of change

### 4.3.47	ManagementDataCollection

#### 4.3.47.1	Definition

This IOC represents a management data collection request job. The requested data is of kind Trace, MDT (Minimization of Drive Test), RLF (Radio Link Failure) report, RCEF (RRC Connection Establishment Failure) report, RRC report, PM (performance measurements), KPI (end-to-end key performance indicators) or a combination of these.

The attribute managementData defines the management data which shall be reported. This may either include a list of data categories or a list of management data identified with their name. For further details see clause 4.3.50.

The targetNodeFilter attribute can be used to target object instance(s) producing the required management data. It is assumed that the consumer may not have detailed knowledge of the network and hence may not identify the exact object instance producing the required management data. In this case consumer can request management data, specified by 3GPP, produced by certain object instance (s) based on a particular location, the domain (CN or RAN) of the object instances, and the handled traffic (CP or UP) of the object instances.

To activate the production of the requested data, a MnS consumer has to create a ManagementDataCollection object instance on the MnS producer.

The attribute jobId is used to identify a management data collection request.

The production and reporting of the management data can be constrained by conditions such that only when the conditions are satisfied shall management data collection be enabled. For example, a MnS consumer can request to create two ManagementDataCollection instances. One can be configured with high data producing and reporting period on a set of conditions (e.g, to reduce transmission cost when network performance metric is in normal range). Another can be configured with low data producing and reporting period on another set of conditions (e.g. to enable network optimization when network performance metric is in abnormal range).

The MnS producer may derive multiple jobs (PerfMetricJob, TraceJob) from a single ManagementDataCollection job for collecting the required management data. The jobId of the request is used to identify the output.  The value is also used to correlate the derived jobs which will have the same jobId value as the ManagementDataCollection.

If the MnS producer receives the collected data from multiple sources, it will consolidate the data into a set of management data for reporting based on the value of the attribute consolidateOutput.

For consolidation of file-based management data the attribute consolidateOutput controls:

- 	True: the MnS Producer will combine the file output from jobs used to collect the required management data into a single output file as follows:

- 	File is in compressed format, i.e. zip.

-	File shall contain individual output files from each configured job

- 	Each file retains its original filename

-	Each file retains it original content

-	Consolidated filename uses naming convention defined in [27], clause 11.3.2.1.4 with the following:

-	<Type> is a combination of the management data types included in the consolidated output

-	<UniqueIdList> is omitted

-	False: the MnS Producer will not combine the output from jobs used to collect the required management data.  The MnS Consumer will receive separate output from the derived jobs.

Subject to the reporting method, the MnS Consumer may receive file related notifications.  When consolidated output is selected the MnS Producer will create file notifications for the consolidated files.  When consolidated output is not selected, the MnS Producer will create the notification subscriptions on behalf of the MnS Consumer and the MnS Consumer will receive notifications directly from the derived jobs.

The attribute collectionTimeWindow specifies the time window for which the management data should be reported. The attributes startTime and the endTime can be in the past, present or in the future.

The attribute reportingCtrl specifies the method and associated control parameters for reporting the produced management data to MnS consumers. Three methods are available: file-based reporting with selection of the file location by the MnS producer, file-based reporting with selection of the file location by the MnS consumer and stream-based reporting.

The attribute dataScope configures, whether the management data should be reported per S-NSSAI or per 5QI or per PLMN, if applicable.

The attribute processMonitor allows the MnS consumer to monitor the status of the management data collection represented by the object ManagementDataCollection.

The MnS producer indicates in the attribute progressStateInfo the state of the management data collection:

-	NOT_STARTED

- 	RUNNING

- 	CANCELING

and indicates in the attribute resultStateInfo:

-	FINISHED

-	FAILED

-	PARTIALLY_FAILED

-	CANCELLED

#### 4.3.47.2	Attributes

The ManagementDataCollection IOC includes the attributes inherited from Top IOC (defined in clause 4.3.29) and the following attributes:

| Attribute Name | S | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| managementData | M | T | T | T | N/A |
| targetNodeFilter | M | T | T | T | N/A |
| collectionTimeWindow | M | T | T | T | N/A |
| reportingCtrl | M | T | T | T | N/A |
| dataScope | O | T | T | T | N/A |
| processMonitor | O | T | F | F | T |
| consolidateOutput | M | T | T | T | T |
| jobId | M | T | T | T | T |
| Attributes related to role |  |  |  |  |  |
| conditionMonitorRef | O | T | T | F | T |

#### 4.3.47.3	Attribute constraints

None.

#### 4.3.47.4	Notifications

The common notifications defined in clause 4.5 are valid for this IOC. In addition, the following set of notifications is also valid.

| Name | S | Notes |
| --- | --- | --- |
| notifyFileReady | M | -- |
| notifyFilePreparationError | M | -- |

Next change

### 4.3.65	ConditionMonitor

#### 4.3.65.1	Definition

The "ConditionMonitor" monitors conditions in a data node tree. Conditions are expressed on the value of nodes in the data node tree or on the presence or absence of nodes. The evaluation of a condition expression yields either "False" or "True". The condition expression is specified in the "conditionExpression" attribute. The current evaluation result is made available with the "conditionStatus" attribute.

The "evaluationPeriod" attribute specifies the interval of time between two consecutive condition expression evaluations. The unit is seconds.

The notation for expressing a condition is solution set specific.

The set of data nodes that can be accessed by a condition is called accessible tree. The accessible tree is given by the subtree, whose root object is the object name-containing the condition monitor. The format of a data node tree is defined by (stage 2) NRM definitions and (stage 3) schema definitions.

Information that has no representation in the data node tree cannot be used in condition expressions except for performance metrics, that are not modeled in the NRMs but whose values can be nevertheless used in condition expressions.

Performance metrics are always related to a managed object instance (the measured object). A specific performance metric is represented by its name and its value. A performance metric can be thought of as an additional attribute of a managed object, whose name is equal to the performance metric name and whose value is equal to the last measured value. With this logical construct, performance metrics can be accessed and used in condition expressions like any other attribute of a managed object.

A "ConditionMonitor" creation request shall be rejected, if the performance metrics requested to be used in a condition expression are not supported by the specified measured object. If the performance metrics are supported but not collected when the request is received, the system may decide to start collecting the required data and accept the request. Otherwise, the request shall be rejected.

Note that the evaluation period may be configured to be much smaller than the granularity or reporting period of the monitored performance metric.

#### 4.3.65.2	Attributes

The ConditionMonitor IOC includes the attributes inherited from Top IOC (defined in clause 4.3.29) and the following attributes:

| Attribute name | S |  | isReadable |  | isWritable |  | isInvariant |  | isNotifyable |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| conditionExpression | M |  | T |  | T |  | F |  | T |  |
| evaluationPeriod | M |  | T |  | T |  | ? |  | ? |  |
| conditionStatus | M |  | T |  | F |  | F |  | T |  |

#### 4.3.65.3	Attribute constraints

None.

#### 4.3.65.4	Notifications

The configuration notifications defined in clause 4.5.2 are valid for this IOC.

End of change