3GPP TSG-SA5 (Telecom Management)	S5-110161

SA5#75, 24 - 28 Jan 2011; Sorrento, Italy	revision of S5-11abcd

| CR-Form-v9.7 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 32251 | CR | CRNum | rev | - | Current version: | a.3.0 |  |
| For HELP on using this form look at the pop-up text over the symbols. Comprehensive instructions on how to use this form can be found at http://www.3gpp.org/specs/CR.htm. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | LIPA SIPTO charging description |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | China Mobile, Huawei |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S5 |  |  |  |  |  |  |  |  |  |
| Work item code: | LIPA-SIPTO |  |  |  |  |  |  | Date: |  | 28/01/2011 |
| Category: | B |  |  |  |  |  |  | Release: |  | Rel-10 |
|  | Use one of the following categories: F (correction) A (corresponds to a correction in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: R99 (Release 1999) Rel-4 (Release 4) Rel-5 (Release 5) Rel-6 (Release 6) Rel-7 (Release 7) Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) |  |
| Reason for change: |  | New Context id for Rc usage of Credit Control needs to be introduced |  |  |  |  |  |  |  |  |
| Summary of change: |  | The New Context Id was introduced for ABMF invocation of Rc |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | There will be no Conext Information for Rc |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7.1.12 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | × | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | × | Test specifications |  |  |  |  |  |  |
|  |  |  | × | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

First change

### 5.1.3	Identifiers and correlation

The EPC Charging Identifier assigned per IP-CAN bearer, is used for correlation purpose within PS domain, as specified in 3GPP TS 32.240 [1].

Within a PDN connection, the EPS default bearer remains established throughout the lifetime of this PDN connection and is assigned with its "EPS default bearer Charging Identifier". Other additional IP-CAN bearers (i.e dedicated bearers) which may be activated and deactivated during this PDN connection, are each assigned with their own "IP-CAN bearer Charging Identifier". For correlation of charging information for the whole PDN connection, a "PDN connection identifier" is shared by all these IP-CAN bearers charging sessions activated during this PDN connection.

During handover of a PDN connection between a GTP based connectivity-access, and a PMIP based connectivity- access for the P-GW (and reversely),  the "PDN connection identifier" allocated on the source access is transferred by the P-GW to the target access, in order to ensure correlation of charging information for the whole PDN connection over the different accesses.

When multiple simultaneous PDN connections are established for a given APN, each PDN connection is associated with its own "PDN connection identifier" and processed independantly from the other PDN connections.

When a "MAPCON capable UE", as defined in 3GPP TS 23.402 [209], has simultaneous PDN connections through different access networks, each PDN connection is associated with its own "PDN connection identifier" over the selected access for the PDN connection.When selective transfer of PDN connections between the different accesses is performed, each PDN connection is transferred, as for a single PDN connection.

When an "IFOM capable UE", as defined in 3GPP TS 23.402 [209], is simultaneously connected to 3GPP access and WLAN access for different IP flows within the same PDN connection, each service data flow is uniquely identified by a PCC Rule within the PDN connection "PDN connection identifier".

When SIPTO funtion applies, as defined in 3GPP TS 23.060[201], during the GW selection of the most appropriate for that UE’s current location, if the PDN Context is de-actived, the charging procedure will be same as normal PDN context activation.  If the PDN context is activated, the during the GW selection cased by user relocation, SGSN will deactivate the impacted PDN, close the ongoing CDR, and open a new CDR upon “reactivation requested" as specified in 3GPP TS 23.060[201]. If the Gateways selection, the CDR generated at GW from which PDN connection is deactivated, may be correlated with the CDR at which the PDN connection is activated by “Charging ID(any other information which can be shared between SGSN, or GGSN for this correlation purpose, for furhter study)” .

Editor’s Notes: The online charging corelation between P-GW during GW selection caused by SIPTO when there is activated PDN connection is for FFS

End of Change