3GPP TSG-SA WG6 Meeting #36-BIS-e	S6-200531

E-meeting, 31st March – 8th April 2020	(revision of S6-xxxxxx)

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.280 | CR | 0243 | rev | - | Current version: | 17.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Additional details to the Location information report |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | BDBOS |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S6 |  |  |  |  |  |  |  |  |  |
| Work item code: | enh3MCPTT |  |  |  |  |  | Date: |  |  | 2020-03-27 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | The current information flow does not provide any information, which kind of information may reported as location information. 3GPP TS 22.280 defines “Location” in clause 3.1 as “The currentposition and if available information about the instantaneous velocity and direction of the MCX UE.”, but other information as “coordinates” are not considered so far. 3GPP TS 22.280 is asking with the requirement: “[R-5.9a-015] The MCX Service system shall allow an MCX Service Administrator to make use of information (e.g. operational schedules, locations, velocity or direction) from external sources to create or delete a functional alias.” for more information, than just the coordinates. 3GPP TS 22.280 is asking with the requirement: “[R-5.11-002a] The MCX Service shall be able to provide a mechanism for obtaining high accuracy Location information by integrating position information from multiple external sources (e.g. magnetometers, orientation sensors, GNSS)” for more information than just the coordinates. 3GPP TS 22.280 is asking with the requirement: “[R-5.11-006] The conveyed Location information shall be the most recently obtained information about the position of the MCX UE at the time of the Location information conveyance.” for the most current location, but only coordinates does not allow any evaluation. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Adding a more detailed list of possible information elements, as part of the location information. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Clarity about the location information elements is missing, while detailed requirements are exist. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 2, 10.9.1.1 (new), 10.9.2.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * First Change * * * *

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP TS 22.179: "Mission Critical Push to Talk (MCPTT); Stage 1".

[3]	3GPP TS 22.280: "Mission Critical Services Common Requirements (MCCoRe); Stage 1".

[4]	3GPP TS 22.281: "Mission Critical Video services".

[5]	3GPP TS 22.282: "Mission Critical Data services".

[6]	3GPP TS 23.002: "Network Architecture".

[7]	3GPP TS 23.179: "Functional architecture and information flows to support mission critical communication services; Stage 2"

[8]	3GPP TS 23.203: "Policy and charging control architecture".

[9]	3GPP TS 23.228: "IP Multimedia Subsystem (IMS); Stage 2".

[10]	3GPP TS 23.237: "IP Multimedia Subsystem (IMS) Service Continuity; Stage 2".

[11]	3GPP TS 23.246: "Multimedia Broadcast/Multicast Service (MBMS); Architecture and functional description".

[12]	3GPP TS 23.281: "Functional architecture and information flows to support Mission Critical Video (MCVideo); Stage 2".

[13]	3GPP TS 23.282: "Functional architecture and information flows to support Mission Critical Data (MCData); Stage 2".

[14]	3GPP TS 23.303: "Proximity-based services (ProSe); Stage 2".

[15]	3GPP TS 23.335: "User Data Convergence (UDC); Technical realization and information flows".

[16]	3GPP TS 23.379: "Functional architecture and information flows to support Mission Critical Push To Talk (MCPTT); Stage 2".

[17]	3GPP TS 23.401: "General Packet Radio Service (GPRS) enhancements for Evolved Universal Terrestrial Radio Access Network (E-UTRAN) access".

[18]	3GPP TS 23.468: "Group Communication System Enablers for LTE (GCSE_LTE); Stage 2".

[19]	3GPP TS 29.283: "Diameter Data Management Applications".

[20]	Void

[21]	3GPP TS 36.300: "Evolved Universal Terrestrial Radio Access (E-UTRA) and Evolved Universal Terrestrial Radio Access Network (E-UTRAN); Overall description; Stage 2".

[22]	IETF RFC 5245 (April 2010): "Interactive Connectivity Establishment (ICE): A Protocol for Network Address Translator (NAT) Traversal for Offer/Answer Protocols".

[23]	GSMA PRD IR.92 v10.0: "IMS Profile for Voice and SMS".

[24]	GSMA PRD IR.88 v15.0: "LTE and EPC Roaming Guidelines".

[25]	3GPP TS 33.180: "Security of the mission critical service".

[26]	IETF RFC 6733 (October 2012): "Diameter Base Protocol".

[27]	3GPP TS 29.214: "Policy and Charging Control over Rx reference point".

[28]	3GPP TS 22.011: "Service accessibility".

[29]	3GPP TS 23.271: "Functional stage 2 description of Location Services (LCS)".

[X]	3GPP TS 25.305: " Stage 2 functional specification of User Equipment (UE) positioning in UTRAN".

[Y]	3GPP TS 23.032: "Universal Geographical Area Description (GAD)".

* * * Next Change * * * *

### 10.9.1	General

Location information of MC service user shall be provided by the location management client to the location management server. The location information reporting triggers are based on the location reporting configuration. Different type of location information can be provided.

#### 10.9.1.1	Terms and Definitions

For the purposes of the following clauses, the terms and definitions given in 3GPP TS 23.271 [29], 3GPP TS 25.305 [X], 3GPP TS 23.032 [Y] and the following apply.

Timestamp: Time expressed with a certain precision to reflect the moment of the location measurement. The timestamp shall allow the comparison of associated location information in respect to timestamp and is therefore based on a common clock.

Accuracy: Reflects the uncertainty of the location in the moment of location measurement with a defined unit and scale.

Speed: Movement in the moment of location measurement with a defined unit and scale. Speed is the magnitude or absolute value of velocity.

Bearing: Direction in the moment of location measurement with a defined unit and scale. The direction shall also have a reference, like "true north" or "magnetic north".

Altitude: Third dimension for the geographical coordinates in the moment of location measurement with a defined ground level, unit and scale. The altitude shall also have a reference, like "sea level (MSL)" or "local ground level (above ground level, or AGL)".

Measurement code: This part of the location information element shall give an indication for what reason the location could not be determined from the positioning system on the UE and gives an indication of how to correctly interpret the provided values, e.g. Measurement code = "no satellite using default value set 1" and in this case the Latitude / Longitude may be set to "Null" or to "last known position".

* * * Next Change * * * *

#### 10.9.2.2	Location information report

Table 10.9.2.2-1 describes the information flow from the location management client to the location management server as well as from the location management server to the MC service server or location management client for the location information reporting.

Table 10.9.2.2-1: Location information report

| Information element | Status | Description |
| --- | --- | --- |
| Set of MC service IDs | M | Set of identities of the reporting MC service user on the MC service UE (e.g. MCPTT ID, MCVideo ID, MCData ID) |
| Functional alias(es) (see NOTE 1) | O | Functional alias that corresponds to the MC service ID. |
| Triggering event | M | Identity of the event that triggered the sending of the report |
| Location Information (see NOTE 2) | M | Location information of the individual MC service user |
| NOTE 1: Each functional alias corresponds to an individual MC service ID. NOTE 2: This may include timestamp, ECGI, MBMS SAIs, longitude, latitude, accuracy, speed, bearing, altitude, measurement code and additional other location information. |  |  |

* * * Next Change * * * *

* * * End of Change * * * *