3GPP TSG-SA WG6 Meeting #39-bis-e	S6-201843

e-meeting, 12th – 20th October 2020	(revision of S6-xxxxxx)

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.379 | CR | 0284 | rev | - | Current version: | 17.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Location management mechanism backward compatibility |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, Hisilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S6 |  |  |  |  |  |  |  |  |  |
| Work item code: | enh3MCPTT |  |  |  |  |  | Date: |  |  | 2020-10-05 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | In R13, the user location is reported to and handled at MCPTT server. In R14, the location mangement server and location management client are introduced to management the user location for all the service including MCPTT, MCVideo and MCData, and the MCPTT server acquires the user location from the location management server. For MCVideo and MCData servicem introduced in R14, there is no compatibility issue as R14 location mechanism is the only one option. For MCPTT, there are two location mechanisms since R14. How to handle the two location mechanism at MCPTT server and MCPTT UE with the backward compatibility is not clear. Since R14, the MCPTT server may get the UE location either from the MCPTT UE or from the location management server, and the location report configuration set by the LMS to LMC is triggered by the MCPTT server. So it is the MCPTT server’s responsibility to determine which options to use. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add descriptions about the selections of location management options. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Location management mechanism backward compatibility is not clear. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7.4.2.3.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * First Change * * * *

## 10.13	Location information (on-network)

The MCPTT system makes use of all of the procedures for location management as specified in 3GPP TS 23.280 [16], utilising the CSC-14 reference point between the location management client and location management server and the CSC-15 reference point between the MCPTT server and location management server.

-	The MC service client is the MCPTT client;

-	The MC service server is the MCPTT server;

-	The MC service group is the MCPTT group;

-	The MC service ID is the MCPTT ID; and

-	The MC service group ID is the MCPTT group ID.

For the backward compatibility with Release 13 location management mechanism, the MCPTT server determines to interact with the location management server to acquire the UE location as described in clause 10.9 of 3GPP TS 23.280 [16] or utilize the MCPTT-1 reference point and the MCPTT-4 reference point to acquire the UE location based on the MCPTT client capability, local policy.

* * * End of Change * * * *