3GPP TSG-SA WG6 Meeting #39 BIS-e	S6-201726

E-meeting, 12 – 20 October 2020

Source:	ETRI, Samsung

Title:	EDGEAPP: Proposal to modify the API service operation names

Spec:	3GPP TS 23.558 v1.0.0

Agenda item:	7.6

Document for:	Approval

Contact:	Seungik Lee <seungiklee@etri.re.kr>; Jong-Hwa Yi <jhyiee@etri.re.kr>

# 1	Introduction

The APIs specified in TS 23.558 have some ambiguity since they are named of reference points not of service producers. For example, EDGE3_EAS_Discovery is a service supported by EES, which is hardly figured out from its name.

This paper proposes to modify the existing APIs specified in TS 23.558 in the form of service-based interfaces by exploiting the same convention used in TS 23.502.

In Annex A.2 of TS 23.502, the service and service operation naming is defined as follows:

-	Nnfname_ServiceName, where Nnfname is the service-based interface where the NF service is invoked. See TS 23.501 [2] clause 4.2.5 for the list of service-based interfaces in the 5GS Architecture.

Figure A.2.2-1: Service Operation Naming and its Methods

An example implementation of NF services in TS 23.502 is as follows:

Table 5.2.8.1-1: NF services provided by the SMF

| Service Name | Service Operations | Operation Semantics | Example Consumer(s) |
| --- | --- | --- | --- |
| Nsmf_PDUSession | Create | Request/Response | V-SMF/I-SMF |
|  | Update | Request/Response | V-SMF/I-SMF, H-SMF |
|  | Release | Request/Response | V-SMF/I-SMF |
|  | CreateSMContext | Request/Response | AMF |
|  | UpdateSMContext | Request/Response | AMF |
|  | ReleaseSMContext | Request/Response | AMF |
|  | SMContextStatusNotify | Subscribe/Notify | AMF |
|  | StatusNotify | Subscribe/Notify | V-SMF/I-SMF |
|  | ContextRequest | Request/Response | AMF, I-SMF, SMF |
|  | ContextPush | Request/Response | SMF |
|  | SendMOData | Request/Response | AMF |
| Nsmf_EventExposure | Subscribe | Subscribe/Notify | NEF, AMF, NWDAF |
|  | Unsubscribe |  | NEF, AMF, NWDAF |
|  | Notify |  | NEF, AMF, NWDAF |
|  | AppRelocationInfo |  | AF |
| Nsmf_NIDD | Delivery | Request/Response | NEF |

# 2	Discussion

The service operations specified in TS 23.558 can be summarized as follows:

| Service operations | Consumer | Producer | Semantics | References | Given API |
| --- | --- | --- | --- | --- | --- |
| Service provisioning | EEC | ECS | R/R | 8.3.2.3 | - |
| Service provisioning subscription | EEC | ECS | S/N | 8.3.2.4.2 | - |
| Service provisioning notification | EEC | ECS | S/N | 8.3.2.4.3 | - |
| Service provisioning subscription update | EEC | ECS | S/N | 8.3.2.4.4 | - |
| Service provisioning unsubscribe | EEC | ECS | S/N | 8.3.2.4.5 | - |
| Edge Enabler Client registration | EEC | EES | R/R | 8.4.2.2.2 | - |
| Edge Enabler Client registration update | EEC | EES | R/R | 8.4.2.2.3 | - |
| Edge Enabler Client de-registration | EEC | EES | R/R | 8.4.2.2.4 | - |
| Edge Application Server registration | EAS | EES | R/R | 8.4.3.2.2 | - |
| Edge Application Server registration update | EAS | EES | R/R | 8.4.3.2.3 | - |
| Edge Application Server de-registration | EAS | EES | R/R | 8.4.3.2.4 | - |
| Edge Enabler Server registration | EES | ECS | R/R | 8.4.4.2.2 | - |
| Edge Enabler Server registration update | EES | ECS | R/R | 8.4.4.2.3 | - |
| Edge Enabler Server de-registration | EES | ECS | R/R | 8.4.4.2.4 | - |
| Edge Application Server discovery | EEC EAS | EES EES | R/R R/R | 8.5.2 8.8.3.2 | - EDGE3_EAS_Discovery Discover_EAS |
| UE location | EAS | EES | R/R | 8.6.2.2.2 | EDGE3_UE_location API Request |
| UE location subscribe | EAS | EES | S/N | 8.6.2.2.3.2 | EDGE3_UE_location API Subscribe |
| UE location API notification | EAS | EES | S/N | 8.6.2.2.3.3 | EDGE3_UE_location API Notify |
| UE location subscription update | EAS | EES | S/N | 8.6.2.2.3.4 | EDGE3_UE_location API Subscription_Update |
| UE location unsubscribe | EAS | EES | S/N | 8.6.2.2.3.5 | EDGE3_UE_location API Unsubscribe |
| User plane path management event subscribe | EAS | EES | S/N | 8.6.3.2.2 | EDGE3_User_plane_management_event API Subscribe |
| User plane path management event notification | EAS | EES | S/N | 8.6.3.2.3 | EDGE3_User_plane_management_event API Notify |
| User plane path management event subscription update | EAS | EES | S/N | 8.6.3.2.4 | EDGE3_User_plane_management_event API Update |
| User plane path management event unsubscribe | EAS | EES | S/N | 8.6.3.2.5 | EDGE3_User_plane_management_event API Unsubscribe |
| Application Client information subscription | EAS | EES | S/N | 8.6.4.2.2 | - |
| Application Client information notification | EAS | EES | S/N | 8.6.4.2.3 | - |
| UE identifier API | EAS | EES | R/R | 8.6.5.2 | EDGE3_UE_Identifier API Request |
| Session with QoS create | EAS | EES | R/R | 8.6.6.2.2 | EDGE3_Session_with_QoS API Create |
| Session with QoS update | EAS | EES | R/R | 8.6.6.2.3 | EDGE3_Session_with_QoS API Update |
| Session with QoS revoke | EAS | EES | R/R | 8.6.6.2.4 | EDGE3_Session_with_QoS API Revoke |
| Session with QoS event notification | EAS | EES | S/N | 8.6.6.2.5 | EDGE3_Session_with_QoS API Notify |
| Retrieve Edge Enabler Server | EES | ECS | R/R | 8.8.3.3 | - |

* R/R: Request/Response, S/N: Subscribe/Notify

Using a similar convention as given in TS 23.502, the service operations of ECS and EES can be re-named as follows:

-	Efename_APIName, where Efename is the name of the functional entity exposing the API prefixed with 'E' (denoting 'Edge').

1. ECS services

| API Name | API Operations | Operation Semantics | Example Consumer(s) | References |
| --- | --- | --- | --- | --- |
| Eecs_ServiceProvisioning | Request | Request/Response | EEC | 8.3.2.3 |
|  | Subscribe | Subscribe/Notify | EEC | 8.3.2.4.2 |
|  | UpdateSubscription |  | EEC | 8.3.2.4.4 |
|  | Unsubscribe |  | EEC | 8.3.2.4.5 |
|  | Notify |  | EEC | 8.3.2.4.3 |
| Eecs_EESRegistration | Request | Request/Response | EES | 8.4.4.2.2 |
|  | Update | Request/Response | EES | 8.4.4.2.3 |
|  | Deregister | Request/Response | EES | 8.4.4.2.4 |
|  | Retrieve | Request/Response | EES | 8.8.3.3 |

2. EES services

| API Name | API Operations | Operation Semantics | Example Consumer(s) | References |
| --- | --- | --- | --- | --- |
| Eees_EECRegistration | Request | Request/Response | EEC | 8.4.2.2.2 |
|  | Update | Request/Response | EEC | 8.4.2.2.3 |
|  | Deregister | Request/Response | EEC | 8.4.2.2.4 |
| Eees_EASRegistration | Request | Request/Response | EAS | 8.4.3.2.2 |
|  | Update | Request/Response | EAS | 8.4.3.2.3 |
|  | Deregister | Request/Response | EAS | 8.4.3.2.4 |
| Eees_EASDiscovery | Request | Request/Response | EEC, EAS | 8.5.2 8.8.3.2 |
| Eees_UELocation | Get | Request/Response | EAS | 8.6.2.2.2 |
|  | Subscribe | Subscribe/Notify | EAS | 8.6.2.2.3.2 |
|  | UpdateSubscription |  | EAS | 8.6.2.2.3.4 |
|  | Unsubscribe |  | EAS | 8.6.2.2.3.5 |
|  | Notify |  | EAS | 8.6.2.2.3.3 |
| Eees_UPPathManagementEvent | Subscribe | Subscribe/Notify | EAS | 8.6.3.2.2 |
|  | UpdateSubscription |  | EAS | 8.6.3.2.4 |
|  | Unsubscribe |  | EAS | 8.6.3.2.5 |
|  | Notify |  | EAS | 8.6.3.2.3 |
| Eees_AppClientInformation | Subscribe | Subscribe/Notify | EAS | 8.6.4.2.2 |
|  | Notify |  | EAS | 8.6.4.2.3 |
| Eees_UEIdentifier | Get | Request/Response | EAS | 8.6.5.2 |
| Eees_SessionWithQoS | Create | Request/Response | EAS | 8.6.6.2.2 |
|  | Update | Request/Response | EAS | 8.6.6.2.3 |
|  | Revoke | Request/Response | EAS | 8.6.6.2.4 |
|  | Notify | Subscribe/Notify | EAS | 8.6.6.2.5 |
| Eees_TargetEASDiscovery | Request | Request/Response | EAS | 8.8.3.2 |

# 3		Proposal

It is proposed to modify the text of TS 23.558 v1.0.0 as follows.

1st CHANGE

2nd CHANGE

3rd CHANGE

4th CHANGE

5th CHANGE

6th CHANGE

#### 8.6.2.4	APIs

##### 8.6.2.4.1	General

Editor's Note: Merging request and subscribe API operation in a single API operation is FFS.

Table 8.6.2.4.1-1 illustrates the API for UE location.

Table 8.6.2.4.1-1: Eees_UELocation API

| API Name | API Operations | Operation Semantics | Consumer(s) |
| --- | --- | --- | --- |
| Eees_UELocation | Get | Request/Response | EAS |
|  | Subscribe | Subscribe/Notify | EAS |
|  | Notify |  |  |
|  | UpdateSubscription |  |  |
|  | Unsubscribe |  |  |

##### 8.6.2.4.2	Eees_UELocation_Get operation

API operation name: Eees_UELocation_Get

Description: The consumer requests one time report of a UE's location.

Inputs: See clause 8.6.2.3.1.

Outputs: See clause 8.6.2.3.2.

See clause 8.6.2.2.2 for details of usage of this operation.

##### 8.6.2.4.3	Eees_UELocation_Subscribe operation

API operation name: Eees_UELocation_Subscribe

Description: The consumer subscribes for continuous reporting of a UE's location.

Inputs: See clause 8.6.2.3.3.

Outputs: See clause 8.6.2.3.4.

See clause 8.6.2.2.3.2 for details of usage of this operation.

##### 8.6.2.4.4	Eees_UELocation_Notify operation

API operation name: Eees_UELocation_Notify

Description: The consumer is notified with report of the UE's location.

Inputs: See clause 8.6.2.3.5.

Outputs: None.

See clause 8.6.2.2.3.3 for details of usage of this operation.

##### 8.6.2.4.5	Eees_UELocation_UpdateSubscription operation

API operation name: Eees_UELocation_UpdateSubscription

Description: The consumer updates an existing subscription for continuous reporting of a UE's location.

Inputs: See clause 8.6.2.3.6.

Outputs: See clause 8.6.2.3.7.

See clause 8.6.2.2.3.4 for details of usage of this operation.

##### 8.6.2.4.6	Eees_UELocation_Unsubscribe operation

API operation name: Eees_UELocation_Unsubscribe

Description: The consumer cancels an existing subscription for continuous reporting of a UE's location.

Inputs: See clause 8.6.2.3.8.

Outputs: See clause 8.6.2.3.9.

See clause 8.6.2.2.3.5 for details of usage of this operation.

7th CHANGE

#### 8.6.3.4	APIs

##### 8.6.3.4.1	General

Table 8.6.3.4.1-1 illustrates the API for user plane path management event.

Table 8.6.3.4.1-1: Eees_UPPathManagementEvent API

| API Name | API Operations | Operation Semantics | Consumer(s) |
| --- | --- | --- | --- |
| Eees_UPPathManagementEvent | Subscribe | Subscribe/Notify | EAS |
|  | Notify |  |  |
|  | UpdateSubscription |  |  |
|  | Unsubscribe |  |  |

##### 8.6.3.4.2	Eees_UPPathManagementEvent_Subscribe operation

API operation name: Eees_UPPathManagementEvent_Subscribe

Description: The consumer subscribes to receive a user plane management event.

Inputs: See clause 8.6.3.3.1.

Outputs: See clause 8.6.3.3.2.

See clause 8.6.3.2.2 for details of usage of this operation.

##### 8.6.3.4.3	Eees_UPPathManagementEvent_Notify operation

API operation name: Eees_UPPathManagementEvent_Notify

Description: The consumer is notified of an event by the Edge Enabler Server.

Inputs: See clause 8.6.3.3.3.

Outputs: None.

See clause 8.6.3.2.3 for details of usage of this operation.

##### 8.6.3.4.4	Eees_UPPathManagementEvent_UpdateSubscription operation

API operation name: Eees_UPPathManagementEvent_UpdateSubscription

Description: The consumer updates an existing subscription for a user plane management event.

Inputs: See clause 8.6.2.3.4.

Outputs: See clause 8.6.2.3.5.

See clause 8.6.3.2.4 for details of usage of this operation.

##### 8.6.3.4.5	Eees_UPPathManagementEvent_Unsubscribe operation

API operation name: Eees_UPPathManagementEvent_Unsubscribe

Description: The consumer unsubscribes for the previously subscribed events.

Inputs: See clause 8.6.3.3.6.

Outputs: See clause 8.6.3.3.7.

See clause 8.6.3.2.5 for details of usage of this operation.

8th CHANGE

#### 8.6.4.4	APIs

##### 8.6.4.4.1	General

Table 8.6.4.4.1-1 illustrates the API for application client information exposure.

Table 8.6.4.4.1-1: Eees_AppClientInformation API

| API Name | API Operations | Operation Semantics | Consumer(s) |
| --- | --- | --- | --- |
| Eees_AppClientInformation | Subscribe | Subscribe/Notify | EAS |
|  | Notify |  |  |

9th CHANGE

#### 8.6.5.4	APIs

##### 8.6.5.4.1	General

Table 8.6.5.4.1-1 illustrates the APIs for UE Identifier.

Table 8.6.5.4.1-1: Eees_UE_Identifier API

| API Name | API Operations | Operation Semantics | Consumer(s) |
| --- | --- | --- | --- |
| Eees_UEIdentifier | Get | Request/Response | EAS |

##### 8.6.5.4.2	Eees_UEIdentifier_Get operation

API operation name: Eees_UEIdentifier_Get

Description: The consumer requests identifier of a UE.

Inputs: See clause 8.6.5.3.1.

Outputs: See clause 8.6.5.3.2.

See clause 8.6.5.2 for details of usage of this operation.

10th CHANGE

#### 8.6.6.4	APIs

##### 8.6.6.4.1	General

Table 8.6.6.4.1-1 illustrates the API for Session with QoS.

Table 8.6.6.4.1-1: Eees_SessionWithQoS API

| API Name | API Operations | Operation Semantics | Consumer(s) |
| --- | --- | --- | --- |
| Eees_SessionWithQoS | Create | Request/Response | EAS |
|  | Update |  |  |
|  | Revoke |  |  |
|  | Notify | Subscribe/Notify (NOTE) | EAS |
| NOTE: The Subscribe operation is embedded in the Request operation of the Create and Update API operation. |  |  |  |

##### 8.6.6.4.2	Eees_SessionWithQoS_Create operation

API operation name: Eees_SessionWithQoS_Create

Description: The consumer requests a data session with the UE with a specific QoS and may also subscribe to certain event notifications related to the user plane traffic.

Inputs: See clause 8.6.6.3.1.

Outputs: See clause 8.6.6.3.2.

See clause 8.6.6.2.2 for details of usage of this operation.

##### 8.6.6.4.3	Eees_SessionWithQoS_Update operation

API operation name: Eees_SessionWithQoS_Update

Description: The consumer requests a modification of the QoS of the data session with the UE and may also update event notifications related to the user plane traffic.

Inputs: See clause 8.6.6.3.3.

Outputs: See clause 8.6.6.3.4.

See clause 8.6.6.2.3 for details of usage of this operation.

##### 8.6.6.4.4	Eees_SessionWithQoS_Revoke operation

API operation name: Eees_SessionWithQoS_Revoke

Description: The consumer requests a revoke of the data session with QoS with the UE and unsubscribes to all related event notifications.

Inputs: See clause 8.6.6.3.5.

Outputs: See clause 8.6.6.3.6.

See clause 8.6.6.2.4 for details of usage of this operation.

##### 8.6.6.4.5	Eees_SessionWithQoS_Notify operation

API operation name: Eees_SessionWithQoS_Notify

Description: The consumer is notified by the Edge Enabler Server of an event related to the user plane traffic.

Inputs: See clause 8.6.6.3.7.

Outputs: None.

See clause 8.6.6.2.5 for details of usage of this operation.

11th CHANGE

### 8.8.5	APIs

#### 8.8.5.1	General

Table 8.8.5.1-1 illustrates the API for fetch target Edge Application Server.

Table 8.8.5.1-1: Eees_TargetEASDiscovery API

| API Name | API Operations | Operation Semantics | Consumer(s) |
| --- | --- | --- | --- |
| Eees_TargetEASDiscovery | Request | Request/Response | EAS |

#### 8.8.5.2	Eees_TargetEASDiscovery_Request operation

API operation name: Eees_TargetEASDiscovery_Request

Description: The consumer requests for the target Edge Application Server information from the Edge Enabler Server.

Inputs: See clause 8.8.4.1.

Outputs: See clause 8.8.4.2.

See clause 8.8.3.2 for details of usage of this operation.

END OF CHANGES