3GPP TSG-SA WG6 Meeting #42-BIS-e	S6-210994

e-meeting, 12th – 20th April 2021	(revision of S6-210749)

Source:	Tencent

Title:	Procedure for UAV and UAV-C Pairing and C2 QoS Provisioning using Group ID

Spec:	3GPP TS 23.255 v0.3.0

Agenda item:	7.10

Document for:	Approval

Contact:	Shuai Zhao, <shuaiizhao@tencent.com>

1. Introduction

This contribution adds a new feature in UASAPP TS 23.255 for UAV and UAV-C grouping.

2. Reason for Change

In TR 23.755, solution#5 provides a grouping approach for C2 QoS provisioning. Based on the study evaluations, the proposed group method enables UAE-S for QoS management using a group-based approach.

3. Proposal

It is proposed for adoption to 3GPP TS 23.255 v0.3.0.

* * * Change * * * *

## 6.x 	UAV and UAV-C Pairing and C2 QoS Provisioning using Group ID

### 6.x.1	General

This clause describes the procedure for UAV and UAV-C grouping C2 QoS provisioning using subsequent group id.

### 6.x.2	Procedures

#### 6.x.2.1	Procedure for group creation for one pair of UAV and UAV-C

Pre-conditions:

-	Both UAV-C and UAV have successfully connected to the UAE server.

-	A CAA-level UAV ID may be assigned during the 3GPP network registration process.

Figure 6.x.2.1-1 illustrates a high-level procedure for group creation.

Figure 6.x.2.1-1: Procedure for group creation for one pair of UAV and UAV-C

- The UAE server recognizes a unique pair of UAV and UAV-C either by 3GPP UE ID or CAA-level UAV ID.

Editor’s Note: How UAE-S recognizes a pair of UAV-C and UAV is FFS

- The UAE server sends a group creation request to the SEAL GM server, if there is no pre-assigned group ID, by using the GM-S reference link as specified in 3GPP TS 23.434 [11] using the procedure defined in clause 10.3. The SEAL GM server creates one group ID for one pair of UAV and UAV-C as specified in 3GPP TS 23.434 [11].

3.	The UAE server uses the returned group ID for UAS for QoS management.

#### 6.x.2.2	Procedure for group-based approach for C2 QoS provisioning

Figure 6.x.2.2-1 illustrates a high-level workflow of group-based C2 QoS provisioning.

Editor's note:	QoS provisioning when C2 is in dynamically changing mode is FFS.

Pre-conditions:

-	Both UAV and UAV-C have registered to 3GPP 5G network respectively. C2 communication is established.

-	The SEAL group manager has been configured and a group ID for a pair of UAV and UAV-C has been assigned using the procedure specified in 6.x.2.1.

Figure 6.x.2.2-1: Procedure of group-based approach for C2 QoS provisioning.

- The UAE server monitors the QoS for a UAV and UAV-C.

Editor’s Note: The details of UAE-S QoS monitoring is FFS

- In cases where the network condition for C2 communication does not satisfy the pre-defined QoS requirement, the UAE server may choose to send QoS adaptation request to the SEAL NRM server using the NRM-S reference point as specified in 3GPP TS 23.434 [11]. The QoS adaptation request needs to be sent per group ID for a pair of UAV and UAV-C created in the procedure specified in 6.x.2.1. The subsequent network resource adaptation procedure is triggered by the UAE-S as specified in clause 14.3.3.3.1 of TS 23.434 [11].

3. 	The UAE-C and UAE-S established communication based on new QoS requirements as specified in clause 14.3.3.2.1.2-1 of 3GPP TS 23.434 [11].

4. 	UAS application layer adapts the updated QoS assignment.

Editor’s Note: How the UAS application layer is adapting newly assigned QoS out of 3GPP’s scope.

### 6.x.3	Information flows

Editor's Note:	This clause will describe the information flow tables