3GPP TSG-SA WG6 Meeting #44	S6-211582_rev3

e-meeting, 12th – 20th July 2021	(revision of S6-21xxxx)

Source:	one2many

Title:	Pseudo-CR on Changes to clause 8.3

Spec:	3GPP TS 23.554 v1.0.0

Agenda item:	8.2

Document for:	Approval

Contact:	peter.sanders@everbridge.com

1. Introduction

In SA6-43e-bis pCR S6-211337 introduced changes to clause 8.3 but the part about the store-and forward (in 8.x) were not agreeable, so the changes in clause 8.3 were not included in the new version of the TS.

2. Reason for Change

The present pCR proposes the same changes in clause 8.3 as were proposed in S6-211337, however, against the V0.4.0 baseline.

The text about verifying the message (topic) in step 1 above table 8.3.2-1 is removed because step 1 because checking if a topic exists is not part of the scope of step 2. Text in step 3 is modified to explain authentication and authorization.

Text for steps that have been removed from the figure are deleted.

In table 8.3.3-1 the "Message delivery based on Messaging Topic indication"IE is changed to Message Topic ID and Group ID is also included to allow the recipient to respond to the all topic subscribers or group members.

3. Conclusions

<Conclusion part (optional)>

4. Proposal

It is proposed to agree the following changes to 3GPP TS 23.554 v1.0.0.

* * * First Change * * * *

## 8.3	Message delivery procedures into and from MSGin5G Server

### 8.3.1	General

All MSGin5G message traffic is routed via the MSGin5G Server. The present clause specifies all MSGin5G message traffic towards and from the MSGin5G Server.

Editor's Note:	Alignment of this solution with the architecture and the conclusions from the study is FFS.

### 8.3.2	MSGin5G inbound messages into the MSGin5G Server

Figure 8.3.2-1 shows the procedure for a MSGin5G UE that initiates a MSGin5G message request.

Figure 8.3.2-1: New MSGin5G message request from UE

Figure 8.3.2-2 shows the procedure for an Application Server that initiates an API request for sending a MSGin5G message to UE.

Editor's Note:	The APIs provided by MSGin5G Server to App Server are to be specified in another section of the TS.

Figure 8.3.2-2: Application Server initiates a request for sending a MSGin5G message

Figure 8.3.2-3 shows the procedure for a Legacy 3GPP Message Gateway or a non-3GPP Message Gateway that sends a new MSGin5G message request to the MSGin5G Server on behalf of a Legacy 3GPP UE or Non-3GPP UE.

Figure 8.3.2-3: New MSGin5G message request sending from Message Gateway

The following procedure applies to the above figures 8.3.2-1, 8.3.2-2 and 8.3.2-3 with the exception that step 1 only applies to figure 8.3.2-1.

1.	The Application Client-1 in the UE sends a request to the MSGin5G Client for invoking the MSGin5G Client to send a new MSGin5G message to a recipient or to multiple recipients.

Editor's Note:	Whether the APIs provided by the MSGin5G Client to the Application Client is to be specified in another section of the TS is FFS.

2.	As shown in figure 8.3.2-1 or 8.3.2-3, the MSGin5G Client or Message Gateway sends the MSGin5G message request to the MSGin5G Server and includes the IEs as listed in table 8.3.2-1 in the request; or as shown in figure 8.3.2-2, the Application Server sends an API request to the MSGin5G Server for sending a MSGin5G message, the API request includes the IEs as listed in table 8.3.2-1.

Table 8.3.2-1: Request to MSGin5G Server for sending MSGin5G message

| Information element | Status | Description |
| --- | --- | --- |
| Originating MSGin5G Service ID | M | The service identity of the sending MSGin5G Client, Legacy 3GPP UE, Non-3GPP UE or the sending Application Server. |
| Recipient MSGin5G Service ID (see NOTE) | O | The service identity of the receiving MSGin5G Client, Legacy 3GPP UE, Non-3GPP UE or the receiving Application Server. This IE is mandatory for Point-to-Point messaging, Application-to-Point messaging, AOMT messaging and MOAT messaging and is not present in other message scenarios. |
| Recipient Group Service ID (see NOTE) | O | The service identifier of the target MSGin5G Group. This IE is mandatory for a Group Message and is not present in other message scenarios. |
| Recipient Broadcast Area ID (see NOTE) | O | The service identifier of the Broadcast Service Area where the message needs to be broadcast. This IE is mandatory in the Broadcast Message and is not present in other message scenarios. |
| Messaging Topic (see NOTE) | O | Indicates which Messaging Topic this message is related to. This IE is mandatory for a message distribution based on topic and is not present in other message scenarios. |
| Application ID | O | Identifies the application(s) for which the payload is intended. This list of Application IDs IE is required when the message is sent to one or multiple Application Clients served by same MSGin5G Client. This list of Application IDs IE may be included when the message is sent to an Application Server or to an Application Client. MSGin5G Server is unaware of the content. |
| Message ID | M | Unique identifier of this message. If the request is a delivery report (as shown in step 6) the Message ID IE is copied from the MSGin5G message that is being acknowledged. |
| Security credentials | O | Security information required by the MSGin5G Server. This is a placeholder for SA3 security information. |
| Delivery status required | O | Indicates if delivery acknowledgement from the recipient is requested. This IE is not present if the MSGin5G message request is a delivery report. |
| Failure cause | O | If the request is a delivery report (as shown in step 6) the Failure Cause indicates the failure reason, if applicable. |
| Payload | O | Payload of the message. MSGin5G Server/Client is unaware of the content. If the request is sent from MSGin5G Client or Message Gateway to the MSGin5G server, the maximum size of this IE is a configurable value that shall not exceed 2048 octets. |
| Priority type | O | Application priority level requested for this message. |
| Message is segmented | O | Indicates this message is part of a segmented message. |
| Segmentation set identifier | O | All segmented messages associated within the same set of segmented messages (i.e. associated with the same MSGin5G message) are assigned the same unique identifier. Mandatory IE to be present in every segmented message. |
| Total number of message segments | O | Indicates the total number of segments for the message. The Total Segments needs to be included only in the first segment of the message. |
| Message segment number | O | An incrementing message segment number that indicates segmented message number of each segmented message within a set of segmented messages |
| Last segment flag | O | An indicator of whether this segmented message is the last segment in the set of segmented messages or not. The Last Segment Flag needs to be included only in the last segment of the message. Message segment number of the segment with "Last Segment Flag" set can be considered as total segments. |
| NOTE: Only one of these IEs shall be included to represent the type of message request. |  |  |

3.	The MSGin5G Server authenticates the message and verifies that the sender is authorized to send the message and determines if the recipient(s) are available to receive the message or if the message needs to be stored for deferred delivery.

4.	The MSGin5G Server may send a Message response to the originating entity if the message is rejected or stored and includes the IEs as listed in table 8.3.2-2 in the response.

Table 8.3.2-2: Message Response

| Information element | Status | Description |
| --- | --- | --- |
| Recipient MSGin5G Service ID | M | The identity of the MSGin5G Client, Legacy 3GPP UE, Non-3GPP UE or the identity of the Application Server that initiated the previous Request. |
| Message ID | M | Identifier of the initiating Request. |
| Delivery status | O | Indicates if delivery is a failure, or if the message is stored for deferred delivery. |
| Failure Cause | O | The reason for failure |

### 8.3.3	MSGin5G outbound messages from the MSGin5G Server

Figure 8.3.3-1 shows the procedure for the MSGin5G Server that forwards a MSGin5G message.

Figure 8.3.3-1: MSGin5G message towards UE

Figure 8.3.3-2 shows the same procedure (step 1 only), however for the MSGin5G Server that sends the message to an Application Server by application request.

Figure 8.3.3-2: message towards an Application Server

Figure 8.3.3-3 shows the procedure for the MSGin5G Server that sends a MSGin5G message to a Legacy 3GPP Message Gateway or a Non-3GPP Message Gateway.

Figure 8.3.3-3: MSGin5G message towards a Message Gateway

The following procedure applies to the above figures 8.3.3-1, 8.3.3-2 and 8.3.3-3 with the exception that step 2 only applies to figure 8.3.3-1.

1.	The MSGin5G Server sends the MSGin5G message request and includes the IEs as listed in table 8.3.3-1.

Table 8.3.3-1: MSGin5G message request from MSGin5G Server

| Information element | Status | Description |
| --- | --- | --- |
| Originating MSGin5G Service ID | M | The service identity of the originating MSGin5G Client, Legacy 3GPP UE, Non-3GPP UE or the originating Application Server. This IE is copied from the associated inbound message. |
| Recipient MSGin5G Service ID (see NOTE) | O | The service identity of the receiving entity. This IE is mandatory for Point-to-Point messaging, Application-to-Point messaging, AOMT messaging and MOAT messaging and is not present in other message scenarios. This IE is copied from the associated inbound message. |
| Recipient Broadcast Area ID (see NOTE) | O | The identifier of the Service Area where the message needs to be broadcast. This IE is mandatory in the Broadcast Message and is not present in other message scenarios. This IE is copied from the associated inbound message. |
| Application ID | O | Identifies the application for which the payload is intended. This list of Application ID(s) IE is required when the message is sent to one or multiple Application Clients served by same MSGin5G Client. This list of Application ID(s) IE may be included when the message is sent to an Application Server or to an Application Client. This list of IEs is copied from the associated inbound message. |
| Message ID | M | Unique identifier of this message. This IE is copied from the associated inbound message. |
| Security credentials | O | Security information. |
| Delivery status required | O | Indicates if delivery acknowledgement from the recipient is requested. This IE is copied from the associated inbound message. |
| Payload | O | Payload of the message. This IE is copied from the associated inbound message. |
| Message is segmented | O | Indicates this message is part of a segmented message. |
| Group Service ID | O | The service identifier of a Group. This IE is included if message delivery is based on Group Messaging. This IE is copied from the Recipient Group Service ID IE in the associated inbound message. |
| Messaging Topic | O | Indicates which Messaging Topic this message is related to. This IE is included if message delivery is based on a Messaging Topic subscription. This IE is copied from the Messaging Topic IE in the associated inbound message. |
| Segmentation set identifier | O | All segmented messages associated within the same set of segmented messages (i.e. associated with the same MSGin5G message) are assigned the same unique identifier. Mandatory IE to be present in every segmented message. |
| Total number of message segments | O | Indicates the total number of segments for the message. The Total Segments needs to be included only in the first segment of the message. |
| Message segment number | O | An incrementing message segment number that indicates segmented message number of each segmented message within a set of segmented messages. |
| Last segment flag | O | An indicator of whether this segmented message is the last segment in the set of segmented messages or not. The Last Segment Flag needs to be included only in the last segment of the message. Message segment number of the segment with "Last Segment Flag" set can be considered as total segments. |
| NOTE: Only one of these IEs shall be included to represent the type of message request. |  |  |

If the received MSGin5G message request is for Group Message, the MSGin5G Server shall replace the Recipient Group Service ID IE with the Recipient MSGin5G Service ID of each individual group member, excluding the message originator.

If the received MSGin5G message request is for a Topic Message, the MSGin5G Server shall replace the Messaging Topic IE with the Recipient MSGin5G Service ID of each individual Topic subscriber, excluding the message originator

* * * End of Changes * * * *