3GPP TSG-WG SA6 Meeting #45 bis	S6-212245

# Online, October 11 – 19, 2021	(revision of S6-210xxxx)

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.289 | CR | XXXX | rev | - | Current version: | 17.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | MBS architectural and functionalities |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | CBN, Huawei, Hisilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA6 |  |  |  |  |  |  |  |  |  |
| Work item code: | MCOver5MBS |  |  |  |  |  | Date: |  |  | 2021-10-11 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | TS 23.247 on 5G MBS stage 2 in SA2 was approved in the last SA plenary, and according to cover page of SA2 MBS (S2-2106682), there is no SA2 internal outstanding issues. Therefore, the architecture now is stable and it is sufficient for SA6 to conclude the architectural for 5G MBS for transport only mode. In TR 23.783, the description of Key Issue #13 includes the architectural related descriptions, which can be used as the basis for architectural conclusion. Moreover, Sol#8 (Application architecture over 5G MBS for transport only mode) and Sol#9 (General use of 5G MBS services for MC service group communications) further include the functionality descriptions. In fact Sol#8 and Sol#9 are complementary to each other, and both of the solutions can be adopted as the selected ones. For Service Mode, more coordination with SA4 is needed: The working in SA4 (i.e., 5MBUSA) is still on-going and according to the clause 7.1.1.1 of TS 23.247, "services offered by the MBSF and related interactions based on that service between MBSF and AF or NEF (if MBSF and NEF are split as shown in configuration 2) are specified in TS 26.502, and detailed interactions between the MBSF or NEF and the MBSTF are specified in TS 26.502". Therefore, it is proposed to conclude Application architecture without MBSF/MBSTF for transport only mode. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Introduce Architectural and functional description of TS 23.783 in TS 23.289. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | There is no description for Architectural and functionalities in current technical specification. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7.X (new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# * * * * First change * * * *

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP TS 23.228: "IP Multimedia Subsystem (IMS); Stage 2".

[3]	3GPP TS 23.280: "Common functional architecture to support mission critical services; Stage 2".

[4]	3GPP TS 23.281: "Functional architecture and information flows to support Mission Critical Video (MCVideo); Stage 2".

[5]	3GPP TS 23.282: "Functional architecture and information flows to support Mission Critical Data (MCData); Stage 2".

[6]	3GPP TS 23.379: "Functional architecture and information flows to support Mission Critical Push To Talk (MCPTT); Stage 2".

[7]	3GPP TS 23.501: "System architecture for the 5G System (5GS)".

[8]	3GPP TS 23.002: "Network Architecture".

[9]	3GPP TS 23.503: "Policy and Charging Control Framework for the 5G System (5GS); Stage 2".

[10]	3GPP TS 23.502: "Procedures for the 5G System (5GS)".

[11]	3GPP TS 22.179: "Mission Critical Push to Talk (MCPTT); Stage 1".

[12]	3GPP TS 22.280: "Mission Critical Services Common Requirements (MCCoRe); Stage 1".

[13]	3GPP TS 22.281: "Mission Critical (MC) Video".

[14]	3GPP TS 22.282: "Mission Critical (MC) Data".

[xx]	3GPP TS 23.247: "Architectural enhancements for 5G multicast-broadcast services; Stage 2".

# * * * * First change * * * *

## 7.X	MC service over 5G MBS

### 7.X.Y	Architecture, reference point and functional entities

Editor's Note:	For the case when Full Service Mode is used, coordination with SA4 is needed.

#### 7.X.Y.A1	General architecture for transport only mode

5G MBS is designed to support multiple services (e.g., MC service, V2X service, media streaming service), and not all the MBS features are required for MC services.

Figure 7.X.Y.A1-1 depicts the 5G MBS reference architecture for transport only mode.

UE

MC Service client

MC Service server

MCX (PTT, Video, Data) - 1

N3mb

NG-RAN

UPF

MB-UPF

MB-SMF

SMF

AMF

UDM

N1

Uu

N3

N2

N4

N4mb

N19mb

N6mb

Nmbsmf

Nsmf

Namf

Naf

NRF

UDM

NEF

Nudm

Nnrf

Nudm

Nnef

Figure 7.X.Y.A1-1: 5G System architecture for Multicast and Broadcast Service for Transport Only Mode.

Figure 7.X.Y.A1-2 depicts the 5G system architecture for MBS using the reference point representation.

UE

MC Service client

MC Service server

MCX (PTT, Video, Data) - 1

N3mb

NG-RAN

UPF

MB-UPF

MB-SMF

SMF

AMF

UDM

PCF

NEF

N1

Uu

N3

N2

N11

N16mb

N4

N4mb

N11mb

N19mb

N6mb

N7mb

N10

Nmb13

N33

N30

N29mb

Figure 7.X.Y.A1-2: 5G System architecture for Multicast and Broadcast Service in reference point representation.

Editor's Note:	For the case when dynamic PCC is deployed and NEF is not used, whether N5 interface (between MC service server and PCF) applies to MBS will be aligned with SA2.

#### 7.X.Y.A2	General reference point and functional entities

##### 7.X.Y.A2.Z1	Functional Entities

###### 7.X.Y.A2.Z1.1	MC service server

The MC service server performs the following functions to support MBS:

-	determining whether to use 5G Multicast, Broadcast, or/and unicast of each group call;

-	interfacing the PCF/NEF to configure the MBS service information/QoS requirements if dynamic PCC is applicable;

-	interfacing with the MB-SMF/NEF to allocate the TMGI, configure the 5G MBS session, and manage the 5G MBS session to request the multicast/broadcast resource for media distribution;

-	announcing the MBS service information to MC service UEs involved in the group communication;

-	announcing the assignment of MBS session for a specific group call to involved MC service UEs;

-	determining for each MC service UE involved in a given call whether to use unicast or MBS delivery;

-	signalling with MC service UE for receiving the broadcast MBS session reception report and the multicast MBS session status report;

-	interfacing with the MB-SMF/NEF for local MBS session configuration and inform the MC service UE about the local MBS session information; and

-	informing the media distribution function of the media streams requiring support for a given call.

# * * * * Second change * * * *

### 7.X.Y	Architectural related for specific MC services

#### 7.X.Y.A1	General

5G MBS is designed to support multiple services (e.g., MC service, V2X service, media streaming service), and not all the MBS features are required for MC services.

Figure 7.X.Y.A1-1 demonstrates an MC service application architecture over 5G MBS for transport only mode.

Figure 7.X.Y.A1-1 MC service application architecture over 5G MBS for transport only mode in reference point representation

#### 7.X.Y.A3	Architecture and reference point specific for MCPTT

##### 7.X.Y.A3.Z1	Architecture

Figure 7.X.Y.A3.Z1-1 depicts the MCPTT functional model for application plane supporting MBS.

Figure 7.X.Y.A3.Z1-1 MCPTT functional model for application plane supporting MBS

##### 7.X.Y.A3.Z2	Reference point

###### 7.X.Y.A3.Z2.1	MCPTT-5A

The MCPTT-5A reference point, which exists between the MCPTT server and the 5GS, is used, subject to the conditions below, by the MCPTT server to obtain unicast network resource with appropriate QoS from the 5GS. It utilises the N5/N33 interface of the 5GS according to 3GPP TS 23.503 [bb].

MCPTT-5A is not used when the MCPTT service provider and the PLMN operator do not have an operational agreement for QoS control to be provided directly from the MCPTT service provider domain.

MCPTT-5A may be used when the MCPTT service provider and the PLMN operator have an operational agreement where QoS control is provided directly from the MCPTT service provider domain.

MCPTT-5A is also used by the MCPTT server to configure the MBS service information/QoS requirements if dynamic PCC is applicable.

NOTE:	Any coordination between the P-CSCF use of N5 and the MCPTT server use of N5 (via MCPTT-5A) from the MCPTT service provider domain is not specified in this release of this specification.

###### 7.X.Y.A3.Z2.2	MCPTT-6A

The MCPTT-6A reference point, which exists between the MCPTT server and the 5GS, is used to request the allocation and activation of MBS transport resources for MCPTT application usage. The MCPTT-6A reference point uses the Nmb13/N33 interface as defined in 3GPP TS 23.247 [xx].

###### 7.X.Y.A3.Z2.3	MCPTT-8

The MCPTT-8 reference point definition in 3GPP TS 23.379 [3] is applied. And when MC service is over 5GS, the MCPTT-8 reference point uses the N6mb interface defined in 3GPP TS 23.247[xx].

###### 7.X.Y.A3.Z2.4	MCPTT-9

The MCPTT-9 reference point definition in 3GPP TS 23.379 [3] is applied. And when MC service is over 5GS, the MCPTT-9 reference point uses the N6mb interface defined in 3GPP TS 23.247 [xx].

#### 7.X.Y.A4	Architecture and Reference point specific for MCVideo

##### 7.X.Y.A4.Z1	Architecture

Figure 7.X.Y.A4.Z1-1 depicts the MCVideo functional model for application plane supporting MBS.

Figure 7.X.Y.A4.Z1-1 MCVideo functional model for application plane supporting MBS

##### 7.X.Y.A4.Z2	Reference point

###### 7.X.Y.A4.Z2.1	MCVideo-5A

The MCVideo-5A reference point, which exists between the MCVideo server and the 5GS, is used, subject to the conditions below, by the MCVideo server to obtain unicast network resource with appropriate QoS from the 5GS. It utilises the N5/N33 interface of the 5GS according to 3GPP TS 23.503 [12].

MCVideo-5A is not used when the MCVideo service provider and the PLMN operator do not have an operational agreement for QoS control to be provided directly from the MCVideo service provider domain.

MCVideo-5A may be used when the MCVideo service provider and the PLMN operator have an operational agreement where QoS control is provided directly from the MCVideo service provider domain.

MCVideo-5A is also used by the MVideo server to configure the MBS service information/QoS requirements if dynamic PCC is applicable.

NOTE:	Any coordination between the P-CSCF use of N5 and the MCVideo server use of N5 (via MCVideo-5A) from the MCVideo service provider domain is not specified in this release of this specification.

###### 7.X.Y.A4.Z2.2	MCVideo-6A

The MCVideo-6A reference point, which exists between the MCVideo server and the 5GS, is used to request the allocation and activation of MBS transport resources for MCVideo application usage. The MCVideo-6A reference point uses the Nmb13/N33 interface as defined in 3GPP TS 23.247 [xx].

###### 7.X.Y.A4.Z2.3	MCVideo-8A

The MCVideo-8 reference point definition in 3GPP TS 23.281 [4] is applied. And when MC service is over 5GS, the MCVideo-8 reference point uses the N6mb interface defined in 3GPP TS 23.247 [xx].

###### 7.X.Y.A4.Z2.4	MCVideo-9

The MCVideo-9 reference point definition in 3GPP TS 23.281 [4] is applied. And when MC service is over 5GS, the MCVideo-9 reference point uses the N6mb interface defined in 3GPP TS 23.247 [xx].

#### 7.X.Y.A5	Architecture and Reference point specific for MCData

##### 7.X.Y.A5.Z1	Architecture

Figure 7.X.Y.A5.Z1-1 depicts the MCData functional model for application plane supporting MBS.

Figure 7.X.Y.A5.Z1-1 MCVideo functional model for application plane supporting MBS

##### 7.X.Y.A5.Z2	Reference point

###### 7.X.Y.A5.Z2.1	MCData-5A

The MCData-5A reference point, which exists between the MCData server and the 5GS, is used, subject to the conditions below, by the MCData server to obtain unicast network resource with appropriate QoS from the 5GS. It utilises the N5/N33 interface of the 5GS according to 3GPP TS 23.503 [12].

MCData-5A is not used when the MCData service provider and the PLMN operator do not have an operational agreement for QoS control to be provided directly from the MCData service provider domain.

MCData-5A may be used when the MCData service provider and the PLMN operator have an operational agreement where QoS control is provided directly from the MCData service provider domain.

MCVideo-5A is also used by the MVideo server to configure the MBS service information/QoS requirements if dynamic PCC is applicable.

NOTE:	Any coordination between the P-CSCF use of N5 and the MCData server use of N5 (via MCData-5A) from the MCData service provider domain is not specified in this release of this specification.

###### 7.X.Y.A5.Z2.2	MCData-6A

The MCData-6A reference point, which exists between the MCData server and the 5GS, is used to request the allocation and activation of MBS transport resources for MCData application usage. The MCData-6A reference point uses the Nmb13/N33 interface as defined in 3GPP TS 23.247 [xx].

# * * * * End of changes * * * *