3GPP TSG-SA WG6 Meeting #45-bis-e	S6-212351

e-meeting, 11th – 19th October 2021	(revision of S6-21xxxx)

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | TS23.554 | CR |  | rev | 0 | Current version: | 17.0.1 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | 5GMARCH store and forward |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Convida Wireless LLC |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S6 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5GMARCH |  |  |  |  |  | Date: |  |  | 2021-10-06 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | Add 5GMARCH store-and-forward procedure, consistent with the approved Rel-17 5GMARCH exception sheet |  |  |  |  |  |  |  |  |
| Summary of change: |  | Two alternatives are proposed: Alternative A re-introduces the solution proposed at SA6 #45 in clause 8.3.x of S6-211904_rev4 Alternative B addresses all the remaining blocking comments received at SA6#45 for S6-211904_rev4 and in the final review period for S6_212118 (i.e, comments addressing Alternative A) as described with italics: A new pre-condition for this procedure should be the terminating UE is offline or not available. The figure should show the terminating UE and the statue of the UE in different steps (not available then available) As suggested the core of this procedure, and most important part, is how the MSGin5G Server handles the s/f and as I commented in previous email (yellow highlighted/underlined below) to provide clarity and the needed architectural requirements to guide stage 3 development in CT. 1. On the new 8.3.x procedure: a. You put all the processing in step 4 makes it hard to follow. It should follow other procedures to have different steps to make them clear such as i. Step 4 as the UE is not available (should be a pre-condition to begin with) the server checks if s/f should apply to the message ii. Step 5 feedback to the sender the decision on s/f iii. Step 6 when the UE is available, server delivers the message and still valid to be delivered iv. Step 7 feedback on the message disposition to sender Alternative A is preferred because: - It is confusing to make the UE availability a pre-condition to the procedure. That would imply that the MSGin5G server knows what the UE availability is before receiving the message, which is incorrect. As shown in alternative A, we consider that making the availability check part of step 4 (alt A) is a more accurate description of the functionality at the MSGin5G server. - Alternative A can be amended to include feedback to the sender occurring twice, as shown in Alternative B, subject to SA6 decision. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | A 5GMARCH exception sheet item remains open. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.6 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

### *****************Alternative A - Change 1************************

### 8.3.x	MSGin5G Store and Forward

Figure 8.3.x-1 shows the procedure for providing store and forward services for MSGin5G message requests.

This procedure applies when an MSGin5G inbound message into the MSGin5G Server is received and the value of the Store and forward flag IE (see table 8.3.2-1) indicates that store and forwarding services are requested; otherwise, the procedure detailed in clause 8.3.2 applies.

Pre-conditions:

1. The MSGin5G Client or Application Server has registered to the MSGin5G Server.

Figure 8.3.x-1: Store and forward procedure

1.	Same as step 1 in 8.3.2. The API message may be optionally received only at the MSGin5G Client.

2.	Same as step 2 in 8.3.2. The value of the Store and forward flag IE (see Table 8.3.2-1) in the received message indicates that store and forward services are requested by the sender.

3.  Same as step 3 in 8.3.2

4.  The MSGin5G Server checks the registration information of the recipient UE. If the Store and forward option IE (see Table 8.2.1-3)  indicates that the recipient UE opts out of store and forward services, go to step 6.

Since the value of the Store and forward flag IE in the received message indicates that store and forward services are requested by the sender, the MSGin5G server determines if the recipients are available to receive the message. The MSGin5G Server may use e.g, UE reachability status monitoring specified in clause 8.2 or the recipient’s Communication Availability information specified in clause 8.9.2  to determine whether the recipient is available. How the MSGin5G server makes this determination is out of scope of the current specification. If no store and forward processing is required, go to step 6.

5.  If store and forward processing is required, the MSGin5G Server stores the message request with the relevant delivery information. If the recipients are not available and Application specific store and forward information IE is provided, the MSGin5G Server uses this information to determine storage and forwarding. If the recipients are not available and the Store and forward parameters IE has not been provided, the message is discarded.

6.	Same as step 4 in 8.3.2. The message response includes store and forward status information in the Delivery Status IE, e.g., that the delivery had been deferred, or that the message was discarded.

7.  The MSGin5G Server attempts delivery of the request (using the procedure specified in clause 8.3.3) at or before the message expiration time or when the recipient becomes available. If the message is not successfully delivered prior to the expiration time, the message is discarded at expiration time.

### *****************Alternative B - Change 1************************

### 8.3.x	MSGin5G Store and Forward

Figure 8.3.x-1 shows the procedure for providing store and forward services for MSGin5G message requests.

This procedure applies when an MSGin5G inbound message is received at the MSGin5G Server and the recipient UE is not available; otherwise, the procedure detailed in clause 8.3.2 applies.

Pre-conditions:

- The MSGin5G Client or Application Server has registered to the MSGin5G Server.

- The MSGin5G server has determined that the recipient UE is not available.

NOTE: The MSGin5G Server may use e.g,, UE reachability status monitoring specified in clause 8.2 or the recipient’s Communication Availability information specified in clause 8.9.2  to determine whether the recipient is available. How the MSGin5G server makes this determination is out of scope of the current specification.

Figure 8.3.x-1: Store and forward procedure

1.	Same as step 1 in 8.3.2. The API message may be optionally received only at the MSGin5G Client.

2.	Same as step 2 in 8.3.2. The value of the Store and forward flag IE (see Table 8.3.2-1) in the received message indicates that store and forward services are requested by the sender.

3.  Same as step 3 in 8.3.2

4.  The MSGin5G Server checks the registration information of the recipient UE. If the Store and forward option IE (see Table 8.2.1-3) indicates that the recipient UE opts out of store and forward services, the message is discarded and the procedure ends. If the Store and forward flag IE (see Table 8.3.2-1) in the received message indicates that store and forward services are not requested by the sender, the message is discarded and the procedure ends.

If store and forward processing is required, the MSGin5G Server uses the Application specific store and forward information IE (see Table 8.3.2-2) to determine storage and forwarding.

5.	Same as step 4 in 8.3.2. The message response includes store and forward status information in the Delivery Status IE, e.g., that the delivery had been deferred, or that the message was discarded.

6.  When the recipient UE becomes available the MSGin5G Server attempts delivery of the request using the procedure specified in clause 8.3.3. The MSGin5G Server may also attempt delivery of the request at or before the message expiration time. If the message is not successfully delivered prior to the expiration time, the message is discarded at expiration time.

7.   Same as step 4 in 8.3.2. This step is executed after attempted delivery or after the message is discarded at message expiration time. The message response includes store and forward status information in the Delivery Status IE, e.g., that the delivery has been attempted, or that the message was discarded.