3GPP TSG-SA WG6 Meeting #47-e	S6-220320

e-meeting, 14th – 22nd February 2022	(revision of S6-220177 Rev1)

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.554 | CR | 0028 | rev | - | Current version: | 17.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Clarification and correction on clause 8.11 Constrained devices |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | China Mobile |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S6 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5GMARCH |  |  |  |  |  | Date: |  |  | 2022-01-30 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | When both MSGin5G UE-1 and UE-2 support MSGin5G Client (i.e. UE-2 is MSGin5G UE-2), MSGin5G Client in MSGin5G UE-1 acts as a relay for the MSGin5G Client in MSGin5G UE-2 to receive MSGin5G Service over MSGin5G-6 reference point. When UE-2 does not support an MSGin5G Client and only MSGin5G UE-1 supports an MSGin5G Client, MSGin5G Client in MSGin5G UE-1 acts as a gateway for the Application Client in UE-2 to receive MSGin5G Service over MSGin5G-5 reference point. The current text in clause 8.11 only covers the procedures over MSGin5G-5 reference and the procedure over MSGin5G-6 is not specified in Rel-17, so some clarification should be made. In addition, some text and figures in clause 8.11 is needed to be corrected accordingly, e.g. MSGin5G UE-2 in the existing text is needed to be converted to UE-2. NOTE: as per the definition, MSGin5G UE is the UE that uses MSGin5G Client in MSGin5G Service. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Clarification and correction on clause 8.11. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The text of 8.11 is not very clair since two architectural choices for constrained device is mentioned in clause 5, but only the procedure in MSGin5G-5 reference point is mentioned in clause 8.11. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.11 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

*****************Change 1************************

8.11	Constrained devices in MSGin5G Service

8.11.1	General

This clause specifies the procedures for the constrained device UE-2 which does not support an MSGin5G Client to perform registration to use MSGin5G UE-1 as gateway UE and to send messages and receive messages using MSGin5G UE-1 which is acting as a gateway UE for the UE-2. The communications between MSGin5G UE-1 and UE-2 is over MSGin5G-5 reference point.

NOTE: The procedure in this clause is also applicable to UE that is out of network coverage.

Editor's note: The API definition for the procedures defined in this clause is FFS.

Editor's note: Whether the procedures in this clause are applicable to Application Client within MSGin5G UE-1 is FFS.

8.11.2	Constrained device registration to use gateway UE

The signalling flow for registration of Application Client-2 on the UE-2 (which is a constrained device) with MSGin5G Client-1 on MSGin5G UE-1 to use it as a gateway UE is illustrated in figure 8.11.2-1.

Pre-conditions:

1.	The MSGin5G UE-1 is configured with information to recognize and authorize UE-2.

2.	The UE-2 is a constrained device and does not have access network connection to connect with the MSGin5G Server.

3.	The UE-2 has discovered MSGin5G UE-1 is having connectivity to the MSGin5G Server and offers gateway UE functionality.

4.	The UE-2 is using NR-PC5 to communicate with MSGin5G UE-1.

Figure 8.11.2-1: Registration of application client on UE-2 with MSGin5G Client-1 to use gateway UE functionality

1)	An Application Client-2 on the UE-2 registers with MSGin5G Client-1 in MSGin5G UE-1 to request MSGin5G Client-1 to act as a gateway UE. The request message includes information elements as specified in Table 8.11.2-1.

Table 8.11.2-1: Information elements for Registration to gateway UE request

| Information element | Status | Description |
| --- | --- | --- |
| Layer-2 ID | M | Layer-2 identity of UE-2 |
| Application ID | M | Application ID of the application client on UE-2 |
| Credential information | M | UE-2 credential information |

Editor's note: The security parameters to include in the message between UE-2 and MSGin5G UE-1 are FFS.

2)	Upon receiving the request from the Application Client-2, the MSGin5G Client-1 authorizes the Application Client-2 on UE-2 to use gateway functionality and MSGin5G Client-1 stores the mapping between Application ID and Layer-2 ID of the UE-2.

NOTE 1:	The MSGin5G Client-1 may reject the request for registration to use gateway UE functionality based on local condition (like available power or connectivity to access network or any other reason outside the scope of 3GPP).

3)	The MSGin5G Client-1 sends response to the Application Client-2. The response message includes information elements as specified in Table 8.11.2-2.

Table 8.11.2-2: Information elements for Registration to gateway UE response

| Information element | Status | Description |
| --- | --- | --- |
| Result | M | Indicates success or failure of the request |
| Registration ID | O | Identifies registration. This IE is included only if Result IE is set to success. |
| Failure reason | O | Indicates failure reason. This IE is included only if Result IE is set to failure. |

NOTE 2:	If MSGin5G Client-1 decided to reject the request for registration to use gateway UE functionality or if authorization fails as in step 2), the MSGin5G Client-1 sends a failure response to the Application Client-2. Otherwise, the MSGin5G Client-1 sends a success response to the Application Client-2.

NOTE 3:	The MSGin5G Client-1 may act as Gateway UE for multiple constrained devices, at the same time.

8.11.3	Constrained device Deregistration from using gateway UE

The signalling flow for deregistration of Application Client-2 on the UE-2 (which is a constrained device) with MSGin5G Client-1 to discontinue use of gateway UE functionality is illustrated in figure 8.11.3-1.

Pre-conditions:

1.	The UE-2 is a constrained device and is successfully registered with MSGin5G UE-1 acting as a gateway UE.

Figure 8.11.3-1: Deregistration of Application Client on UE-2 with MSGin5G Client-1 on MSGin5G UE-1 to discontinue use of gateway UE functionality

1)	An Application Client-2 on the UE-2 deregisters with MSGin5G Client-1 in MSGin5G UE-1 to discontinue usage of gateway UE functionality of MSGin5G UE-1. The request message includes information elements as specified in Table 8.11.3-1.

Table 8.11.3-1: Information elements for De-registration to gateway UE request

| Information element | Status | Description |
| --- | --- | --- |
| Registration ID | M | Identifies the registration |

2)	Upon receiving the request from the Application Client-2, the MSGin5G Client-1 removes the mapping between Application ID and Layer-2 ID of the UE-2. 	The MSGin5G Client-1 sends response to the Application Client-2 on UE-2. The response message includes information elements as specified in Table 8.11.2-2.

8.11.4	Constrained device sending message using Gateway UE

The signalling flow for the Application Client-2 on the UE-2 (which is a constrained device) to send message using gateway UE functionality on MSGin5G UE-1 is illustrated in figure 8.11.4-1.

Pre-conditions:

1.	The MSGin5G UE-1 is connected to an access network that provides connectivity to the MSGin5G Server.

2.	The UE-2 is constrained device and is successfully registered with MSGin5G UE-1 acting as a gateway UE.

Figure 8.11.4-1: Application Client-2 on UE-2 sends message using gateway UE functionality on MSGin5G UE-1

1)	An Application Client-2 on the UE-2 sends a request to send MSGin5G message to the MSGin5G Client-1. The information elements defined in Table 8.11.4-1 are included in the message.

Table 8.11.4-1: Information elements for Request to send MSGin5G message

| Information element | Status | Description |
| --- | --- | --- |
| Recipient address (see NOTE) | O | Indicates details of the recipient. This IE is mandatory to initiate Point-to-Point messaging and Point-to-Application messaging This IE contains the information for MSGin5G Client-1 to generate the Recipient UE Service ID/AS Service ID in the MSGin5G message. |
| Recipient Groupaddress (see NOTE) | O | Indicates target group. This IE is mandatory to initiate Group messaging. |
| Application ID | O | Identifies the application(s) for which the payload is intended. |
| Payload | M | Payload of the message. MSGin5G Server/Client is unaware of the content. |
| Delivery status required | O | Indicates whether delivery status is required or not |
| NOTE: These addresses may be different from the IEs specified in clause 8.3.2, e.g. it may be without the domain informantion within the MSGin5G service. The MSGin5G Client-1 knows the mapping between the addresses in this table and the IEs specified in clause 8.3.2. If the addresses in this table are different from the IEs specified in clause 8.3.2, the MSGin5G Client-1 will generate the IEs specified in clause 8.3.2 based on the addresses in this table to in step 2. |  |  |

Editor's note:	If Priority type needs to be communicated from application clients is FFS.

2)	Upon receiving the request from the Application Client-2 in UE-2, the MSGin5G Client-1 generate the IEs specified in clause 8.3.2 based on the addresses in the request from UE-2 and sends Point-to-Point Message, Point-to-Application Message or Group Message based on these IEs.

a)	if the size of the received message exceeds the maximum allowed packet size, the MSGin5G Client-1 sends the message as specified in clause 8.5; and

b)	If the size of the received message does not exceed the maximum allowed packet size, the MSGin5G Client-1 sends the message as specified in clause 8.7.

NOTE 1:	The MSGin5G Client-1 may also reject the request to send MSGin5G message based on local condition (like available power or connectivity to access network or any other reason outside the scope of 3GPP).

3)	The MSGin5G Client-1 sends response to send MSGin5G message to Application Client-2 on UE-2. The response message includes information elements as specified in Table 8.11.4-2.

Table 8.11.4-2: Information elements for Response to send MSGin5G message

| Information element | Status | Description |
| --- | --- | --- |
| Result | M | Indicates success or failure of the request |
| Failure reason | O | Indicates failure reason. This IE is included only if Result IE is set to failure. |

NOTE 2:	If the MSGin5G Client-1 has decided to reject the request to send the message or the MSGin5G Client-1 received reject response from MSGin5G Server in step 2, the MSGin5G Client-1 sends failure response to the Application Client-2 and stops performing further steps.

4)	If delivery status is requested while sending the message in step 1, the MSGin5G Client-1 may receive MSGin5G message delivery status report from the MSGin5G Server.

5)	Upon receiving the MSGin5G message delivery status report, the MSGin5G Client-1 sends the message delivery status report to the Application Client-2 on UE-2. The message delivery status report includes information elements as specified in Table 8.11.4-3.

Table 8.11.4-3: Information elements for MSGin5G message delivery status

| Information element | Status | Description |
| --- | --- | --- |
| Delivery status | M | Indicates delivery status |

8.11.5	Constrained device receiving message via Gateway UE

The signalling flow for Application Client-2 on the UE-2 (which is constrained device) to receive message (Group message, Point-to-Point message or Application-to-Point message) using gateway UE functionality on MSGin5G UE-1 is illustrated in figure 8.11.5-1.

Pre-conditions:

1.	The MSGin5G UE-1 is connected to an access network that provides connectivity to the MSGin5G Server.

2.	The UE-2 is constrained device and is successfully registered with MSGin5G UE-1 acting as a gateway UE.

Figure 8.11.5-1: UE-2 receives message using gateway UE functionality on MSGin5G UE-1

1)	The MSGin5G Client-1 receives either a Group message or a Point-to-Point message or Application-to-Point message as specified in clause 8.2.3 for the Application Client-2 on UE-2 for which the MSGin5G Client-1 is acting as Gateway UE. The MSGin5G Client-1 performs reassembly if the received message is segmented. The MSGin5G Client-1 may also perform segment recovery procedure as specified in clause 8.4.6 to recover missing segments.

2)	Upon successfully receiving a message for the Application Client-2 on UE-2, the MSGin5G Client-1 sends message received request to Application Client-2 based on Application ID on the UE-2. The request includes information elements as specified in Table 8.11.5-1.

Table 8.11.5-1: Information elements for Message received request

| Information element | Status | Description |
| --- | --- | --- |
| Originator address (see NOTE) | O | Indicates details of the originator. This IE is mandatory for Point-to-Point messaging and Application-to-Point messaging. i.e. Originating UE Service ID/AS Service ID. |
| Group Service address (see NOTE) | O | Indicates group for which the message is received. This IE is mandatory for Group Message. |
| Payload | M | Payload of the message. MSGin5G Server/Client is unaware of the content. |
| Delivery status required | O | Indicates whether delivery status is required or not |
| Priority type | O | Application priority level requested for this message. |
| NOTE: These addresses may be different from the IEs specified in clause 8.3.2 e.g. it may be without the domain informantion within the MSGin5G service. |  |  |

3)	Upon successfully receiving the message, the Application Client-2 on UE-2 sends the message received response to the MSGin5G Client-1.

4)	If delivery status is requested in the received message, the Application client-2 on UE-2 sends message delivery status to the MSGin5G Client-1.

5)	Upon receiving the delivery status, the MSGin5G Client-1 sends message delivery status report to the MSGin5G Server.