3GPP TSG-SA WG6 Meeting #49-e	S6-221031

# E-Meeting, 16th – 25th May 2022

# Title:	LS on the applicability of hold and forward function in DS-TT ports for
5G-native systems

# Response to:	-

# Release:	Release 17

# Work Item:	eSEAL

Source:	SA6

To:	SA2

Cc:	-

Contact Person:

Name: 	Aron Szabo

E-mail Address:	 aron.szabo@ericsson.com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

# Attachments:	None

# 1. Overall Description:

The implementation of the hold and forward function in DS-TTs for 5G-native systems is discussed in SA6. The investigated setup is a specific case of 5G TSC for UE-to-UE time-sensitive and deterministic communication.

The original architecture of a 5G system as a TSN Bridge from TS 23.501 [1] serves as a basis of the proposed setup. The original and the proposed architectures are shown in Figure 1 and Figure 2, respectively. With the addition of a second TSN end system to the device side of the original architecture, two separate TSN end systems communicate with each other while no TSN data transfer exists on the N6 / NW-TT side.

The functional architecture of the proposed implementation is demonstrated in Figure 3 where no external TSN System is present. With the lack of an external TSN system, C-plane and U-plane, the proposed modification is indeed under the umbrella of TSC, however, the proposed schema is a 5G native case where DS-TTs are synched with TSN End Systems. This is achieved by using the same mechanisms as those defined for the integration of a 5GS with an external TSN system. SA6 impact is realized through SEAL NRM and VAL Server as shown in Figure 3.

Please note that the architectures in Figure 2 and Figure 3 are equivalent, highlighting the implementation in Figure 2 and the functionality in Figure 3.

According to subclause 5.27.4 of TS 23.501, DS-TT ports and NW-TT ports support a hold and forward mechanism to schedule traffic only if a 5G system is to participate transparently as a bridge in a TSN network.

TS 23.501 does not specify the support of hold and forward function in DS-TT ports and in NW-TT ports for more general cases of TSN-integrated 5G systems, in particular does not specify for the TSN-integrated case with two communicating TSN end systems on the device side and no TSN data transfer on the N6 side of the TSN Bridge as demonstrated in Figure 2 and Figure 3.

Therefore, SA6 has the following question for SA2 on the applicability of hold and forward function:

Question 1: Does SA2 accept the usage of hold and forward function in DS-TT ports for TSN-integrated 5G systems with the following additions to the originally described "System architecture view with 5GS appearing as TSN bridge” in Figure 4.4.8.2-1 of TS 23.501:

- Two communicating TSN end systems are on the device side instead of a single TSN system and these two TSN end systems communicate with each other.

- No TSN data transfer exists on the N6 / NW-TT side.

The implementation and the functionality of the proposed architecture are demonstrated in Figure 2 and in Figure 3, respectively.

# 2. Actions:

To SA2 group.

ACTION: 	SA6 kindly requests SA2 to consider the above question and kindly inform SA6 on the decision or recommendation.

# 3. Dates of Next SA6 Meetings:

3GPP TSG SA6#49-bis-e		06/2022	E-Meeting

3GPP TSG SA6#50			08/2022	Meeting (Wroclaw, PL)

# Appendix – list of figures:

Figure 1: Original architecture from TS 23.501 [1], Figure 4.4.8.2-1 "System architecture view with 5GS appearing as TSN bridge”

Figure 2: Implementation of the proposed functionality, based on Figure 1

Figure 3: Functional diagram of the proposed implementation equivalent to Figure 2