3GPP TSG-SA WG6 Meeting #49-e	S6-221120

e-meeting, 16th – 25th May 2022	(revision of S6-22xxxx)

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.554 | CR | 0043 | rev | - | Current version: | 17.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network |  | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Messaging Topic handling between different MSGin5G Servers |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | China Mobile |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S6 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5GMARCH_Ph2 |  |  |  |  |  | Date: |  |  | 2022-05-06 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | This CR is proposed to add the procedures for handling Messaging Topic between different MSGin5G Servers. It is included in the first objective of MSGIn5G_Ph2 WID. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Adds the procedures for handling Messaging Topic between different MSGin5G Servers. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The related feature can not be supported. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.8.x (new), |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

*****************Change 1************************

8.8.x	Messaging Topic handling between different MSGin5G Servers

8.8.x.1	General

When Messaging Topic(s) is handled between different MSGin5G Servers, the MSGin5G Server 1 may work in the following models:

a.	Mod.A: the MSGin5G UE/Application Server served by MSGin5G Server 1 subscribes Messaging Topic(s) on MSGin5G Server 2 directly. In this case, the MSGin5G Server 1 forwards Messaging Topic subscription request from the MSGin5G UE/Application Server served by it to MSGin5G Server 2 (Mod.A) or

b.	Mod.B: the MSGin5G Server 1 subscribes the Messaging Topic on behalf of all MSGin5G UE/Application Server served by it.

The MSGin5G Server may choose the work model based on the service policy. The MSGin5G Server 1 and MSGin5G Server 2 can be located in the same PLMN or different PLMNs.

8.8.x.2	Messaging Topic list subscription

Before the subscribing of Messaging Topic(s), the MSGin5G Server 1 should obtain the available Messaging Topic list on the MSGin5G Server 2 to determine whether forwards the Messaging Topic subscription request to MSGin5G 2, or subscribes Messaging Topic on behalf of all MSGin5G UE/Application Server served by it on MSGin5G Server 2.

Figure 8.8.x.2-1 shows the MSGin5G Server 1 subscribing to Messaging Topic list on the MSGin5G Server 2.

NOTE 1:	If the MSGin5G Server 1 and MSGin5G Server 2 are located in the same PLMN, the synchronization of Messaging Topic list between MSGin5G Servers may also implementation specific.

Pre-conditions:

1.	MSGin5G Server 1 and MSGin5G Server 2 have established a secured connection.

Figure 8.8.x.2-1: MSGin5G Server 1 subscribes to Messaging Topic list on the MSGin5G Server 2

1.	The MSGin5G Server 1 sends a Messaging Topic list subscription request to the MSGin5G Server 2. The request includes the information elements listed in Table 8.8.x.2-1.

Table 8.8.x.2-1: Messaging Topic list subscription request

| Information element | Status | Description |
| --- | --- | --- |
| MSGin5G Server address | M | The MSGin5G Server which subscribes the Messaging Topic list. |
| Security credentials | O | Security information required by the MSGin5G Server 2. If the MSGin5G Server 1 and MSGin5G Server 2 are located in the same PLMN, this information element is not mandatory. This is a placeholder for SA3 security information. |
| Expiration | O | The date and time when the subscription expires. If this IE is included, the value of it should be larger than 0. If this IE is not included, the expiration time is subject to operator policy. |
| Messaging Topic list version | O | The version of Messaging Topic list of MSGin5G Server 2 stored on MSGin5G Server 1. If the MSGin5G Server 1 subscribes the Messaging Topic list on the MSGin5G Server 2 first time, the Messaging Topic list version included in the Messaging Topic list subscription request should be set to 0. If the MSGin5G Server 1 has subscribed the Messaging Topic list on the MSGin5G Server 2, the Messaging Topic list version included in this request should be set to the local stored MSGin5G Server 2’s Messaging Topic list version on MSGin5G Server 1, |

2.	Upon receiving the Messaging Topic list subscription request, the MSGin5G Server 2 validates this request and verifies the security credentials.

NOTE 2:	If the MSGin5G Server 1 and MSGin5G Server 2 are located in the same PLMN, the step 2 can be skipped.

3.	The MSGin5G Server 2 checks the local stored Messaging Topic list subscription (s).

a)	If the MSGin5G Server 1’s subscription has already been created, the MSGin5G Server 2 updates the validity time of this subscription.

b)	If the MSGin5G Server 1’s subscription has not been created, the MSGin5G Server 2 creates the subscription.

4.	The MSGin5G Server 2 sends a Messaging Topic list Subscription response to MSGin5G Server 1. The response includes the information listed in Table 8.8.x.2-2.

Table 8.8.x.2-2: Messaging Topic list Subscription response

| Information element | Status | Description |
| --- | --- | --- |
| Subscription status | M | Indicates whether the subscription was successfully added or deleted on the MSGin5G Server 2. |
| Expiration | O | The validity date and time of this Messaging Topic list subscription set by the MSGin5G Server 2. |

5.	The MSGin5G Server 2 checks whether Messaging Topic list notification is needed, e.g. whether the Messaging Topic list version included in the newest Messaging Topic list subscription request is smaller than the Messaging Topic list version on MSGin5G Server 2, or the local Messaging Topic(s) on the MSGin5G Server 2 are updated, e.g. new Messaging Topic(s) has been created or existing Messaging Topic(s) has been deleted.

NOTE 3:	When the Messaging Topic(s) on the MSGin5G Server 2 are updated, the MSGin5G Server 2 should increase the local Messaging Topic list version.

6.	If Messaging Topic list notification is needed, the MSGin5G Server 2 sends a Messaging Topic list notification request to MSGin5G Server 1 and stores the Messaging Topic list version which has been notified to MSGin5G Server 1. The request includes the information listed in Table 8.8.x.2-3.

Table 8.8.x.2-3: Messaging Topic list notification

| Information element | Status | Description |
| --- | --- | --- |
| Expiration | O | The new validity date and time of this subscription set by the MSGin5G Server 2. |
| Messaging Topic list version | M | The version of Messaging Topic list on MSGin5G Server 2. |
| >Messaging Topic list | M | A list of Messaging Topic(s) that is(are) existing on the MSGin5G Server 2. This list includes the deviation of Messaging Topic(s) since the last notification. Each element in this list contains information as specified in Table 8.8.x.2-4. Based on the service, the MSGin5G Server 2 may only choose a part of Messaging Topic(s) which are allowed to be subscribe by MSGin5G Server 1 in the notification |

Table 8.8.x.2-4: Individual Messaging Topic

| Information element | Status | Description |
| --- | --- | --- |
| Messaging Topic | M | Unique identifier of this Messaging Topic. |
| Update status | M | Identifies the Messaging Topic is newly created on the MSGin5G Server 2, or .newly deleted on the MSGin5G Server 2 |

7.	Upon receiving the Messaging Topic list notification, the MSGin5G Server 1 updates the local stored Messaging Topic list:

a)	if the Update status of a Messaging Topic is created, the MSGin5G Server 1 adds the Messaging Topic to thelocal stored Messaging Topic list;

b)	if the Update status of a Messaging Topic is deleted, the MSGin5G Server 1 removes the Messaging Topic from thelocal stored Messaging Topic list; and

c)	the MSGin5G Server 1 updates the local stored Messaging Topic list version to the Messaging Topic list version included in the Messaging Topic list notification.

NOTE 4	The MSGin5G Server should not send Messaging Topic list notification to other MSGin5G Servers if its local stored Messaging Topic list is updated by receiving a Messaging Topic list notification.

8.8.x.3	Messaging Topic Subscription between different MSGin5G Servers

If the MSGin5G Server 1 works in Mod.A, upon receiving a Messaging Topic subscription request from MSGin5G Client or Application Server, if the Messaging Topic is included in the Messaging Topic list of MSGin5G Server 2, the MSGin5G Server 1 forwards the Messaging Topic subscription request to MSGin5G Server 2, Otherwise the MSGin5G Server 1 handles the Messaging Topic subscription request as specified in clause 8.8.1.

If the MSGin5G Server 1 works in Mod.B, it may subscribe one or more Messaging Topic(s) from the Messaging Topic list by using the procedure specified in clause 8.8.1 with the clarification listed below:

1.	The MSGin5G Server 1 includes the information elements listed in Table 8.8.x.3-1 instead of the information elements listed in Table 8.8.1-1.

Table 8.8.x.3-1: Messaging Topic subscription request

| Information element | Status | Description |
| --- | --- | --- |
| Originating UE Service ID/AS Service ID (see NOTE 1) | O | The service identity of the sending MSGin5G Client or the sending Application Server. This IE should be included if MSGin5G Server 1 forwards Messaging Topic subscription request from the MSGin5G UE/Application Server served by it to MSGin5G Server 2. |
| MSGin5G Server address (see NOTE 1) | O | The MSGin5G Server which subscribes the Messaging Topic(s). This IE should be included if MSGin5G Server 1 subscribe the Messaging Topic on behalf of all MSGin5G UE/Application Server served by it. |
| Security credentials | O | Security information required by the MSGin5G Server 2. If the MSGin5G Server 1 and MSGin5G Server 2 are located in the same PLMN, this information element is not mandatory. This is a placeholder for SA3 security information. |
| Messaging Topic (see NOTE 2) | M | A list of Messaging Topic(s) that is to be subscribed. The number of Messaging Topic(s) included in this IE can be one or more. |
| Expiration | O | The date and time when the subscription expires. This date and time apply to all Messaging Topic(s) subscribed in this request. If this IE is included, the value of it should be larger than 0. If this IE is not included, the expiration time is subject to operator policy. |
| NOTE 1: Only one of these IEs shall be included. NOTE 2: The content of the Messaging Topic is out of scope of the present document. |  |  |

2.	Upon receiving the Messaging Topic subscription request, the MSGin5G Server 2 validates this request and verifies the security credentials.

NOTE:	If the MSGin5G Server 1 and MSGin5G Server 2 are located in the same PLMN, the step 2 can be skipped.

3.	The MSGin5G Server 2 handles the Originating UE Service ID/AS Service ID or MSGin5G Server address included in the Messaging Topic subscription request as the UE Service ID/AS Service ID included in Table 8.8.1-1.

8.8.x.4	Messaging Topic Unsubscription between different MSGin5G Servers

If the MSGin5G Server 1 works in Mod.A, upon receiving a Messaging Topic unsubscription request from MSGin5G Client or Application Server, if the Messaging Topic is included in the Messaging Topic list of MSGin5G Server 2, the MSGin5G Server 1 forwards the Messaging Topic unsubscription request to MSGin5G Server 2, Otherwise the MSGin5G Server 1 handles the Messaging Topic subscription request as specified in clause 8.8.1.

If the MSGin5G Server 1 works in Mod.B, it may also unsubscribe one or more Messaging Topic(s) from the Messaging Topic list by using the procedure specified in clause 8.8.3 with the clarification listed below:

1.	The MSGin5G Server 1 includes the information elements listed in Table 8.8.x.4-1 instead of the information elements listed in Table 8.8.3-1.

Table 8.8.x.4-1: Messaging Topic subscription request

| Information element | Status | Description |
| --- | --- | --- |
| Originating UE Service ID/AS Service ID (see NOTE 1) | O | The service identity of the sending MSGin5G Client or the sending Application Server. This IE should be included if MSGin5G Server 1 forwards Messaging Topic unsubscription request from the MSGin5G UE/Application Server served by it to MSGin5G Server 2. |
| MSGin5G Server address (see NOTE 1) | O | The MSGin5G Server which unsubscribes the Messaging Topic(s). This IE should be included if MSGin5G Server 1 subscribe the Messaging Topic on behalf of all MSGin5G UE/Application Server served by it. |
| Security credentials | O | Security information required by the MSGin5G Server 2. If the MSGin5G Server 1 and MSGin5G Server 2 are located in the same PLMN, this information element is not mandatory. This is a placeholder for SA3 security information. |
| Messaging Topic | M | A list of Messaging Topic(s) that is to be unsubscribed. The number of Messaging Topic(s) included in this IE can be one or more. |
| NOTE 1: Only one of these IEs shall be included. |  |  |

2.	Upon receiving the Messaging Topic unsubscription request, the MSGin5G Server 2 validates this request and verifies the security credentials.

NOTE:	If the MSGin5G Server 1 and MSGin5G Server 2 are located in the same PLMN, the step 2 can be skipped.

3.	The MSGin5G Server 2 handles the Originating UE Service ID/AS Service ID or MSGin5G Server address included in the Messaging Topic unsubscription request as the UE Service ID/AS Service ID included in Table 8.8.3-1.